CAN1 Internal Loopback Project

STM32F407 bxCAN initialization, internal loopback, TX mailbox, accept-all filter, and FIFO0 receive test

Document intent: Code-grounded embedded reference for STM32F407 internal CAN loopback. It focuses on CAN1 initialization mode, bit timing, loopback mode, filter bank 0, TX mailboxes, FIFO0 receive, and UART validation.

| Field | Value |
| --- | --- |
| Project scope | Internal CAN1 initialization and loopback test |
| CAN controller | STM32F407 internal bxCAN CAN1 |
| Loopback node label | Node3 in main.c |
| CAN pins configured | PB8 = CAN1_RX, PB9 = CAN1_TX, AF9 |
| Loopback test frame | Standard ID 0x301, DLC 2, data 0x31 0x32 |
| CAN timing | PCLK1 32 MHz, prescaler 8, 8 TQ total, 500 kbps, sample point 75 percent |
| Source authority | Finalized files under CAN1_Loopback |
| Evidence boundary | No separate screenshot file was attached in this turn; shared chat records loopback output |

# 1. Document Control / Metadata

| Item | Details |
| --- | --- |
| Design preset | compact_reference_guide |
| Primary learning source | https://chatgpt.com/share/6a390d75-f15c-83e8-927f-6d2623cce324 |
| Device references | STM32F407 datasheet and RM0090 reference manual |
| Firmware reference | D:\Embedded Communication Protocols\CAN1_Loopback |
| Code fidelity rule | Register values, functions, constants, and flow are taken from finalized source files |
| Comment encoding note | Code comments with encoding artifacts are normalized for document readability; register statements are unchanged |
| Footer rule | Embedded Learning Document |

# 2. Learning Objectives

Understand the difference between MCP2515 external CAN and STM32F407 internal bxCAN.

Trace the initialization-mode handshake using CAN_MCR.INRQ and CAN_MSR.INAK.

Explain the CAN_BTR bit timing values used for 500 kbps at 32 MHz PCLK1.

Understand what CAN_BTR.LBKM does in internal loopback mode.

Configure an accept-all 32-bit filter in filter bank 0 for FIFO0.

Pack a standard CAN ID and payload into a bxCAN TX mailbox.

Read the looped-back frame from FIFO0 and release the FIFO output mailbox.

Identify current project limitations before moving to normal external CAN mode.

# 3. Project Objective

The project validates STM32F407 internal CAN1 loopback without requiring an external CAN transceiver or another CAN node. The application initializes CAN1, enables loopback mode, sends a standard frame from a CAN1 transmit mailbox, receives the same frame through FIFO0, and prints the result over USART2.

# 4. Problem Statement

Core engineering problem: Internal loopback proves the bxCAN register path: initialization, timing, mailbox transmit, filter acceptance, FIFO receive, and FIFO release. It does not prove CANH/CANL physical wiring, ACK from another node, or transceiver behavior.

# 5. System Overview

USART2 debug output
                              |
                              v
                       +--------------+
                       | STM32F407 MCU |
                       +--------------+
                              |
                              v
                       +--------------+
                       | CAN1 / bxCAN  |
                       +--------------+
                         |          |
                 TX mailbox       RX FIFO0
                         |          ^
                         v          |
                  internal loopback path
                         |
                  filter bank 0 accepts all

External pins configured for later normal-mode work:
PB8  = CAN1_RX, AF9
PB9  = CAN1_TX, AF9

Other project modules still initialized:
MCP2515 Node1/Node2 over SPI and MAX7219 display over SPI1.

The current test is a single-MCU internal CAN exercise. PB8/PB9 are configured correctly for CAN1, but internal loopback does not require a transceiver, CANH/CANL, termination, or another receiver.

# 6. Requirements

| Requirement | Project value | Verification point |
| --- | --- | --- |
| CAN peripheral | CAN1 on APB1 | RCC->APB1ENR bit 25 enabled |
| GPIO | PB8 RX, PB9 TX, AF9 | CAN1_GPIO_Init configures GPIOB AFR[1] = 9 |
| Initialization mode | INRQ set, INAK waited | CAN1_InitializationMode |
| Bit timing | 500 kbps at PCLK1=32 MHz | BRP=7, TS1=4, TS2=1, SJW=0 |
| Loopback | LBKM set, SILM clear | CAN1->BTR ¦= CAN_BTR_LBKM |
| Filter | Bank 0 accept-all to FIFO0 | FR1=0, FR2=0, mask mode, 32-bit scale |
| Transmit | Any empty TX mailbox | TME0/TME1/TME2 checked |
| Receive | FIFO0 polling | FMP0 checked, RFOM0 releases output mailbox |

# 7. Hardware Components

| Component | Role | Important caution |
| --- | --- | --- |
| STM32F407 MCU | Runs CAN1 internal loopback and UART logging | Internal loopback is inside bxCAN |
| PB8 / PB9 | Configured as CAN1_RX / CAN1_TX AF9 | Pins are prepared but external bus is not validated in loopback |
| USART2 | Prints init, TX, RX, and result messages | Blocking output can slow repeated loop tests |
| MCP2515 modules | Legacy external CAN nodes still initialized | They are not part of the internal CAN loopback result |
| MAX7219 | Visual side module on SPI1 | Blinking adds delays to the test loop |
| CAN transceiver | Not used for internal loopback | Required later for real CANH/CANL normal mode |

# 8. Software and Firmware Tools

STM32CubeIDE or equivalent for build and flash.

UART terminal connected to USART2 for loopback result logs.

Optional debugger watch expressions for CAN1->MSR, BTR, TSR, RF0R, and FIFO registers.

Logic analyzer is not required for internal loopback, but useful later for normal mode.

RM0090 reference manual for bxCAN register semantics.

# 9. System Block Diagrams

## 9.1 bxCAN Internal Loopback Data Path

Application
  |
  +-- CAN1_SendStdMessage(0x301, 31 32, 2)
          |
          v
      Empty TX mailbox selected by CAN_TSR.TME0/TME1/TME2
          |
          v
      TIR  = standard ID shifted into STID[10:0]
      TDTR = DLC
      TDLR/TDHR = payload bytes
          |
          v
      TXRQ set
          |
          v
      bxCAN loopback engine
          |
          v
      Filter bank 0 accepts frame
          |
          v
      FIFO0 pending count FMP0 increments
          |
          v
      CAN1_ReadMessage_FIFO0()
          |
          v
      Read RIR/RDTR/RDLR/RDHR and set RFOM0 to release FIFO0

## 9.2 Initialization State Flow

CAN1_Init()
  |
  +-- Enable CAN1 APB1 clock
  +-- CAN1_InitializationMode()
          |
          +-- leave sleep: clear SLEEP
          +-- request init: set INRQ
          +-- wait for INAK
  |
  +-- Configure MCR behavior bits
  +-- Configure BTR timing and loopback
  +-- Configure filter bank 0
  +-- leave init: clear INRQ
  +-- wait until INAK clears

# 10. Firmware Architecture

| Module | Responsibility | Important APIs / data |
| --- | --- | --- |
| can.c | CAN1 GPIO, init mode, filters, loopback transmit/receive | CAN1_Init, CAN1_SendStdMessage, CAN1_ReadMessage_FIFO0 |
| can.h | CAN1 API and reset mask | CAN1_RESET_MASK, function prototypes |
| main.c | Project startup and Node3 loopback test loop | node3_msg, CAN1_Init, CAN1_SendStdMessage |
| common.h | Return status macros | E_OK, E_NOT_OK |
| usart.c | Blocking USART2 debug output | USART2_SendString |
| spi/mcp2515/max7219 modules | Legacy project modules still initialized | Not authoritative for internal CAN loopback logic |

# 11. Communication Protocol Used

The protocol under test is Classic CAN using the STM32F407 internal bxCAN controller. The frame is a standard data frame: 11-bit ID 0x301, DLC 2, payload bytes 0x31 and 0x32. In loopback mode the controller internally returns transmitted messages to the receive path, so the protocol registers are exercised without physical arbitration or ACK from another node.

# 12. Core Concept Deep Dive

## 12.1 MCP2515 vs Internal bxCAN

| Topic | MCP2515 external CAN | STM32 internal bxCAN |
| --- | --- | --- |
| Register access | Over SPI commands | Direct MCU peripheral registers |
| CAN controller location | External chip | Inside STM32F407 |
| Loopback path | MCP2515 internal TX-to-RX | CAN1 internal TX mailbox to FIFO path |
| Physical pins | MCP2515 TXCAN/RXCAN to transceiver | PB8/PB9 configured for CAN1 but not needed in internal loopback |
| Receive storage | RXB0/RXB1 | FIFO0/FIFO1 receive mailboxes |
| Transmit storage | TXB0/TXB1/TXB2 | Three transmit mailboxes |

## 12.2 Initialization Mode

bxCAN timing, mode, and filter setup should be done while CAN is in initialization mode. The source clears sleep, sets INRQ, waits for INAK, writes MCR/BTR/filter configuration, then clears INRQ and waits for INAK to clear.

## 12.3 Bit Timing Used

| Parameter | Source value | Meaning |
| --- | --- | --- |
| PCLK1 | 32 MHz | SystemClock_Config sets SYSCLK 64 MHz and APB1 divider /2 |
| Prescaler | 8 | BRP register field is 7 because hardware encoding is prescaler - 1 |
| BS1 | 5 TQ | TS1 field is 4 |
| BS2 | 2 TQ | TS2 field is 1 |
| SJW | 1 TQ | SJW field is 0 |
| Total bit time | 1 + 5 + 2 = 8 TQ | Sync segment + BS1 + BS2 |
| Bitrate | 32 MHz / (8 x 8) = 500 kbps | Matches code comment and BTR setup |
| Sample point | (1 + 5) / 8 = 75 percent | Good practical starting point for a lab loopback test |

## 12.4 Loopback Mode Meaning

The source sets CAN_BTR_LBKM and clears CAN_BTR_SILM. This means the controller transmits internally back into its receive path while not using silent listen-only behavior. For this project, success proves register-level frame movement and FIFO handling, not physical CAN bus behavior.

## 12.5 Filter Bank 0 Accept-All

CAN reception still passes through the filter block in loopback mode. Filter bank 0 is configured in mask mode, 32-bit scale, assigned to FIFO0, with ID and mask both zero. This intentionally accepts every incoming standard/extended candidate during bring-up.

# 13. Module-Wise Implementation

| Phase | Final-source behavior | Engineering purpose |
| --- | --- | --- |
| GPIO setup | PB8/PB9 AF9, PB8 pull-up | Prepare CAN1 pins for loopback and later normal mode |
| Clock setup | CAN1 APB1 clock enabled | Allow CAN1 register access |
| Init entry | Clear sleep, set INRQ, wait INAK | Enter protected configuration state |
| MCR setup | TTCM/ABOM/AWUM/NART/RFLM/TXFP cleared | Simple deterministic bring-up behavior |
| BTR setup | BRP/TS1/TS2/SJW plus LBKM | 500 kbps loopback mode |
| Filter setup | Bank 0 accept-all FIFO0 | Ensure looped-back frame is not rejected |
| Transmit | Find empty mailbox and set TXRQ | Request standard CAN frame transmission |
| Receive | Read FIFO0 if FMP0 nonzero and release RFOM0 | Consume looped-back frame cleanly |

# 14. Main Algorithm and Flowchart

Power-on
  |
  +-- HAL_Init and SystemClock_Config
  +-- USART2_Init
  +-- SPI/MCP2515/MAX7219 legacy initialization
  +-- CAN1_GPIO_Init
  +-- CAN1_Init
        |
        +-- enter initialization mode
        +-- configure MCR, BTR, loopback, filter bank 0
        +-- leave initialization mode
  |
  +-- Forever:
        CAN1_SendStdMessage(0x301, {0x31, 0x32}, 2)
        delay 10 ms
        CAN1_ReadMessage_FIFO0()
        print ID/DLC/DATA over USART2
        MAX7219_CharBlinking('A')

# 15. Code Explanation

Code authority: The snippets below are taken from CAN1_Loopback finalized source files. Corrections and recommendations are documented separately and do not silently replace the project code.

## 15.1 CAN1 GPIO Setup

void CAN1_GPIO_Init(void)
{
    /* Enable GPIOB clock */
    RCC->AHB1ENR |= RCC_AHB1ENR_GPIOBEN;

    /*
     * PB8  = CAN1_RX
     * PB9  = CAN1_TX
     * AF9  = CAN1
     */

    /* Clear mode bits */
    GPIOB->MODER &= ~((3UL << (8 * 2)) |
                      (3UL << (9 * 2)));

    /* Alternate function mode */
    GPIOB->MODER |=  ((2UL << (8 * 2)) |
                      (2UL << (9 * 2)));

    /* Push-pull */
    GPIOB->OTYPER &= ~((1UL << 8) |
                       (1UL << 9));

    /* High speed */
    GPIOB->OSPEEDR &= ~((3UL << (8 * 2)) |
                        (3UL << (9 * 2)));

    GPIOB->OSPEEDR |=  ((2UL << (8 * 2)) |
                        (2UL << (9 * 2)));

    /*
     * CAN RX recessive is high.
     * Pull-up on RX is useful.
     */
    GPIOB->PUPDR &= ~((3UL << (8 * 2)) |
                      (3UL << (9 * 2)));

    GPIOB->PUPDR |= (1UL << (8 * 2));   /* PB8 pull-up */

    /* AF9 for PB8 and PB9 */
    GPIOB->AFR[1] &= ~((0xFUL << ((8 - 8) * 4)) |
                       (0xFUL << ((9 - 8) * 4)));

    GPIOB->AFR[1] |=  ((9UL << ((8 - 8) * 4)) |
                       (9UL << ((9 - 8) * 4)));
}

## 15.2 CAN1 Initialization and Loopback Setup

uint8_t CAN1_Init(void)
{
	uint32_t timeout = 0;

	//Enable clock for the CAN peripheral
	RCC->APB1ENR |= (1 << 25);

	if(CAN1_InitializationMode())
	{
		USART2_SendString("Internal CAN1 Initialization Mode OK!!!\r\n");
	}

	/*
	     * MCR configuration
	     *
	     * TTCM = 0: time-triggered mode disabled
	     * ABOM = 0: automatic bus-off management disabled
	     * AWUM = 0: automatic wakeup disabled
	     * NART = 0: automatic retransmission enabled
	     * RFLM = 0: FIFO not locked
	     * TXFP = 0: transmit priority by identifier
	     */
	    CAN1->MCR &= ~(CAN_MCR_TTCM |
	                   CAN_MCR_ABOM |
	                   CAN_MCR_AWUM |
	                   CAN_MCR_NART |
	                   CAN_MCR_RFLM |
	                   CAN_MCR_TXFP);

	    /*
	     * Bit timing for PCLK1 = 32 MHz, 500 kbps
	     *
	     * Baudrate = PCLK1 / [Prescaler × (1 + BS1 + BS2)]
	     *
	     * Prescaler = 8
	     * BS1       = 5
	     * BS2       = 2
	     * SJW       = 1
	     *
	     * Total TQ = 1 + 5 + 2 = 8
	     * Baudrate = 32 MHz / (8 × 8) = 500 kbps
	     *
	     * Register encoding:
	     * BRP = Prescaler - 1 = 7
	     * TS1 = BS1 - 1 = 4
	     * TS2 = BS2 - 1 = 1
	     * SJW = SJW - 1 = 0
	     */
	    CAN1->BTR = 0;

	    CAN1->BTR |= (7UL << CAN_BTR_BRP_Pos);   /* Prescaler - 1 */
	    CAN1->BTR |= (4UL << CAN_BTR_TS1_Pos);   /* BS1 - 1 */
	    CAN1->BTR |= (1UL << CAN_BTR_TS2_Pos);   /* BS2 - 1 */
	    CAN1->BTR |= (0UL << CAN_BTR_SJW_Pos);   /* SJW - 1 */

	    /*
	     * Loopback mode enabled.
	     * Silent mode disabled.
	     */
	    CAN1->BTR |= CAN_BTR_LBKM;
	    CAN1->BTR &= ~CAN_BTR_SILM;

	    /*
		 * Configure filters before leaving init mode
		 */
		CAN1_Filter_AcceptAll_FIFO0();

		/*
		 * Leave initialization mode
		 */
		CAN1->MCR &= ~CAN_MCR_INRQ;

		timeout = 1000000;
		while((CAN1->MSR & CAN_MSR_INAK) && timeout)
		{
			timeout--;
		}

		if(timeout == 0)
		{
			return 0;
		}

		return 1;

}

## 15.3 Initialization-Mode Handshake

uint8_t CAN1_InitializationMode(void)
{
	uint32_t timeout;

	//leave sleep mode
	CAN1->MCR &= ~(1 << 1);

	//initialization Request
	CAN1->MCR |= (1 << 0);

	timeout = 1000000;
	while(((CAN1->MSR & CAN_MSR_INAK) == 0) && timeout)
	{
		timeout--;
	}

	if(timeout == 0)
	{
		return 0;
	}

	return 1;

}

## 15.4 Filter Bank 0 Accept-All

void CAN1_Filter_AcceptAll_FIFO0(void)
{
    /*
     * Enter filter initialization mode.
     * Reception is disabled while FINIT = 1.
     */
    CAN1->FMR |= CAN_FMR_FINIT;

    /* Deactivate filter bank 0 */
    CAN1->FA1R &= ~(1UL << 0);

    /*
     * Identifier mask mode:
     * 0 = mask mode
     */
    CAN1->FM1R &= ~(1UL << 0);

    /*
     * 32-bit scale:
     * 1 = one 32-bit filter
     */
    CAN1->FS1R |= (1UL << 0);

    /*
     * Assign filter bank 0 to FIFO0:
     * 0 = FIFO0
     */
    CAN1->FFA1R &= ~(1UL << 0);

    /*
     * Accept all:
     * ID = 0
     * Mask = 0
     */
    CAN1->sFilterRegister[0].FR1 = 0x00000000;
    CAN1->sFilterRegister[0].FR2 = 0x00000000;

    /* Activate filter bank 0 */
    CAN1->FA1R |= (1UL << 0);

    /* Leave filter initialization mode */
    CAN1->FMR &= ~CAN_FMR_FINIT;
}

## 15.5 Standard Frame Transmit

uint8_t CAN1_SendStdMessage(uint16_t id, uint8_t *data, uint8_t len)
{
    uint8_t mailbox;
    uint32_t tdlr = 0;
    uint32_t tdhr = 0;

    if(id > 0x7FF)
    {
        return E_NOT_OK;
    }

    if(len > 8)
    {
        return E_NOT_OK;
    }

    if((data == 0) && (len > 0))
    {
        return E_NOT_OK;
    }

    /*
     * Find empty transmit mailbox.
     * TME0 = bit 26
     * TME1 = bit 27
     * TME2 = bit 28
     */
    if(CAN1->TSR & CAN_TSR_TME0)
    {
        mailbox = 0;
    }
    else if(CAN1->TSR & CAN_TSR_TME1)
    {
        mailbox = 1;
    }
    else if(CAN1->TSR & CAN_TSR_TME2)
    {
        mailbox = 2;
    }
    else
    {
        return E_NOT_OK;
    }

    /*
     * Prepare identifier register.
     *
     * Standard ID goes to STID[10:0] = bits [31:21]
     * IDE = 0 for standard ID
     * RTR = 0 for data frame
     * TXRQ = 0 initially
     */
    CAN1->sTxMailBox[mailbox].TIR = ((uint32_t)id << 21);

    /*
     * DLC in bits [3:0]
     */
    CAN1->sTxMailBox[mailbox].TDTR = (len & 0x0F);

    /*
     * Pack data bytes.
     */
    if(len > 0) tdlr |= ((uint32_t)data[0] << 0);
    if(len > 1) tdlr |= ((uint32_t)data[1] << 8);
    if(len > 2) tdlr |= ((uint32_t)data[2] << 16);
    if(len > 3) tdlr |= ((uint32_t)data[3] << 24);

    if(len > 4) tdhr |= ((uint32_t)data[4] << 0);
    if(len > 5) tdhr |= ((uint32_t)data[5] << 8);
    if(len > 6) tdhr |= ((uint32_t)data[6] << 16);
    if(len > 7) tdhr |= ((uint32_t)data[7] << 24);

    CAN1->sTxMailBox[mailbox].TDLR = tdlr;
    CAN1->sTxMailBox[mailbox].TDHR = tdhr;

    /*
     * Request transmission.
     */
    CAN1->sTxMailBox[mailbox].TIR |= CAN_TI0R_TXRQ;

    return E_OK;
}

## 15.6 FIFO0 Receive and Release

uint8_t CAN1_ReadMessage_FIFO0(uint16_t *id, uint8_t *data, uint8_t *len)
{
    uint32_t rir;
    uint32_t rdtr;
    uint32_t rdlr;
    uint32_t rdhr;
    uint8_t dlc;

    /*
     * FMP0[1:0] = number of pending messages in FIFO0.
     */
    if((CAN1->RF0R & CAN_RF0R_FMP0) == 0)
    {
        return E_NOT_OK;
    }

    rir  = CAN1->sFIFOMailBox[0].RIR;
    rdtr = CAN1->sFIFOMailBox[0].RDTR;
    rdlr = CAN1->sFIFOMailBox[0].RDLR;
    rdhr = CAN1->sFIFOMailBox[0].RDHR;

    /*
     * Standard ID:
     * STID[10:0] is in RIR[31:21]
     */
    *id = (uint16_t)((rir >> 21) & 0x7FF);

    /*
     * DLC:
     * bits [3:0]
     */
    dlc = (uint8_t)(rdtr & 0x0F);

    if(dlc > 8)
    {
        dlc = 8;
    }

    *len = dlc;

    /*
     * Unpack data bytes.
     */
    if(dlc > 0) data[0] = (uint8_t)((rdlr >> 0)  & 0xFF);
    if(dlc > 1) data[1] = (uint8_t)((rdlr >> 8)  & 0xFF);
    if(dlc > 2) data[2] = (uint8_t)((rdlr >> 16) & 0xFF);
    if(dlc > 3) data[3] = (uint8_t)((rdlr >> 24) & 0xFF);

    if(dlc > 4) data[4] = (uint8_t)((rdhr >> 0)  & 0xFF);
    if(dlc > 5) data[5] = (uint8_t)((rdhr >> 8)  & 0xFF);
    if(dlc > 6) data[6] = (uint8_t)((rdhr >> 16) & 0xFF);
    if(dlc > 7) data[7] = (uint8_t)((rdhr >> 24) & 0xFF);

    /*
     * Release FIFO0 output mailbox.
     * This is like clearing RX flag in MCP2515.
     */
    CAN1->RF0R |= CAN_RF0R_RFOM0;

    return E_OK;
}

## 15.7 Main Loopback Application Code

//Node 3
	uint8_t node3_msg[] = {0x31,0x32};
	uint16_t node3_id;
	uint8_t node3_dlc, node3_data[8];

CAN1_GPIO_Init();
  if(CAN1_Init())
  {
      USART2_SendString("CAN1 loopback init OK\r\n");
  }
  else
  {
      USART2_SendString("CAN1 loopback init failed\r\n");
  }

//CAN Internal Node3 transmit message
	  if(CAN1_SendStdMessage(0x301,node3_msg,2) == E_OK)
	  {
	      USART2_SendString("CAN1 TX requested\r\n");
	  }
	  else
	  {
	      USART2_SendString("CAN1 TX failed\r\n");
	  }

	  HAL_Delay(10);

	  //Node3 receive message
	  if(CAN1_ReadMessage_FIFO0(&node3_id, node3_data, &node3_dlc) == E_OK)
	  {
	      char node3_msg[100];

	      sprintf(node3_msg,
	              "Node3 : ID=0x%03X DLC=%d DATA0=0x%02X DATA1=0x%02X\r\n",
				  node3_id, node3_dlc, node3_data[0], node3_data[1]);

	      USART2_SendString(node3_msg);
	  }
	  else
	  {
	      USART2_SendString("CAN1 RX no message\r\n");
	  }

## 15.8 Support Definitions

/*
 * can.h
 *
 *  Created on: 20-Jun-2026
 *      Author: HP
 */

#ifndef INC_CAN_H_
#define INC_CAN_H_

#include "stm32f407xx.h"
#include "common.h"

#define CAN1_RESET_MASK		(1UL << 15UL)

void CAN1_GPIO_Init(void);
uint8_t CAN1_Init(void);
void CAN1_Filter_AcceptAll_FIFO0(void);
uint8_t CAN1_InitializationMode(void);
void CAN1_Reset(void);
uint8_t CAN1_SendStdMessage(uint16_t id,
							uint8_t *data,
							uint8_t len);
uint8_t CAN1_ReadMessage_FIFO0(uint16_t *id,
							uint8_t *data,
							uint8_t *len);

#endif /* INC_CAN_H_ */

/*
 * common.h
 *
 *  Created on: 21-Jun-2026
 *      Author: HP
 */

#ifndef INC_COMMON_H_
#define INC_COMMON_H_


#define E_NOT_OK 	0u
#define E_OK		1



#endif /* INC_COMMON_H_ */

## 15.9 USART Debug Output

void USART2_SendString(char *s)
{
	while(*s != '\0')
	{
		while(!(USART2->SR & USART_SR_TXE));
		USART2->DR = *s++;
	}

	//checking for the last character
	while(!(USART2->SR & USART_SR_TC));
}

# 16. Testing and Validation

| Test | Expected observation | Meaning |
| --- | --- | --- |
| CAN1 init mode | Internal CAN1 Initialization Mode OK!!! | INRQ/INAK handshake succeeded |
| CAN1 loopback init | CAN1 loopback init OK | BTR/filter configured and init mode exited |
| TX request | CAN1 TX requested | Empty mailbox found and TXRQ set |
| FIFO0 pending | CAN1_ReadMessage_FIFO0 returns E_OK | Looped-back frame reached FIFO0 |
| Received ID | 0x301 | STID packing/unpacking is correct |
| Received DLC | 2 | DLC transfer path is correct |
| Received data | 0x31 0x32 | TDLR/RDLR byte packing is correct |
| FIFO release | Subsequent reads do not repeat same frame unless retransmitted | RFOM0 release works |

# 17. Results

Observed result from the shared chat: The shared learning notes report a correct loopback result: ID 0x301, DLC 2, data 0x31 and 0x32. No separate screenshot was attached, so the expected terminal output is recorded without embedding a new image.

Expected terminal pattern:

Internal CAN1 Initialization Mode OK!!!
CAN1 loopback init OK
CAN1 TX requested
Node3 : ID=0x301 DLC=2 DATA0=0x31 DATA1=0x32

# 18. Problems Faced and Debugging Notes

| Symptom | Likely cause | Debug action |
| --- | --- | --- |
| CAN1 init failed | INAK did not set or clear | Check CAN clock enable, sleep bit, INRQ sequence, and timeout |
| TX failed | No empty mailbox | Inspect CAN_TSR TME0/TME1/TME2 and pending TXRQ bits |
| No RX message | Filter not active or FIFO0 not assigned | Check FMR.FINIT, FA1R, FFA1R, FR1/FR2, RF0R.FMP0 |
| Wrong ID | Incorrect STID shift | Standard ID must be placed/read at bits [31:21] |
| Wrong data bytes | Incorrect TDLR/TDHR or RDLR/RDHR byte packing | Check little-endian byte positions in lower and higher data registers |
| Same frame repeats | FIFO output mailbox not released | Set CAN_RF0R_RFOM0 after reading |
| Physical analyzer shows nothing | Internal loopback does not need external CAN pins | Use UART/debug registers; use analyzer later in normal mode |

# 19. Embedded Engineering Review

| Finding | Impact | Recommended action |
| --- | --- | --- |
| CAN1_InitializationMode result is not handled as a failure | CAN1_Init continues if init request fails | Return failure immediately if initialization mode entry fails |
| No TX completion/error status check | TX request success does not prove TXOK | Inspect RQCPx/TXOKx/ALSTx/TERRx after TX |
| No CAN ESR diagnostics | Bus-off, warning, passive, and last-error-code states are invisible | Log CAN1->ESR on failures |
| Accept-all filter is good for bring-up only | Later normal-mode tests need intentional filtering | Add ID-specific filter once loopback is stable |
| MCP2515 and MAX7219 still initialize before CAN1 test | Unrelated hardware can delay or block a pure CAN1 lab | Separate demos or guard legacy modules with compile-time switches |
| MAX7219_CharBlinking adds 1 second per loop | CAN test cadence is slowed | Disable display blink during register-debug sessions |
| CAN1_Reset is not used in main path | Reset helper is unverified in this test | Use only if a deliberate peripheral reset sequence is needed |
| spi.h has no include guard | Can cause duplicate declarations in larger builds | Add a conventional include guard |

# 20. Limitations

This is internal loopback, not external normal-mode CAN.

No physical CANH/CANL voltage, transceiver, termination, arbitration, or ACK from another node is verified.

Transmit completion and error status are not yet checked.

Receive is polling-based, not interrupt-driven.

Only standard data frames are implemented in the current helper.

Filter configuration accepts all frames and is not production-selective.

# 21. Future Improvements

Add TX status verification using RQCPx, TXOKx, ALSTx, and TERRx.

Add CAN1->ESR diagnostics for LEC, BOFF, EPVF, and EWGF.

Create a pure CAN1 loopback build path without MCP2515/MAX7219 initialization.

Add receive FIFO overrun/full diagnostics using RF0R flags.

Move from accept-all filter to a standard-ID filter for 0x301.

Add CAN RX FIFO interrupt handling after polling loopback is stable.

Move to normal mode with a transceiver, CANH/CANL, termination, and another node.

# 22. Conclusion

Conclusion: The finalized project demonstrates the core bxCAN loopback path: init mode, 500 kbps timing, accept-all filtering, TX mailbox loading, FIFO0 receive, and FIFO release. Next, add TX/error diagnostics before external normal-mode CAN.

# 23. Appendix: Source Reference

| File / source | Used for |
| --- | --- |
| D:\Embedded Communication Protocols\CAN1_Loopback\Core\Src\can.c | Finalized firmware source/header used for code grounding |
| D:\Embedded Communication Protocols\CAN1_Loopback\Core\Src\main.c | Finalized firmware source/header used for code grounding |
| D:\Embedded Communication Protocols\CAN1_Loopback\Core\Src\max7219.c | Finalized firmware source/header used for code grounding |
| D:\Embedded Communication Protocols\CAN1_Loopback\Core\Src\mcp2515.c | Finalized firmware source/header used for code grounding |
| D:\Embedded Communication Protocols\CAN1_Loopback\Core\Src\spi.c | Finalized firmware source/header used for code grounding |
| D:\Embedded Communication Protocols\CAN1_Loopback\Core\Src\usart.c | Finalized firmware source/header used for code grounding |
| D:\Embedded Communication Protocols\CAN1_Loopback\Core\Inc\can.h | Finalized firmware source/header used for code grounding |
| D:\Embedded Communication Protocols\CAN1_Loopback\Core\Inc\common.h | Finalized firmware source/header used for code grounding |
| D:\Embedded Communication Protocols\CAN1_Loopback\Core\Inc\main.h | Finalized firmware source/header used for code grounding |
| D:\Embedded Communication Protocols\CAN1_Loopback\Core\Inc\max7219.h | Finalized firmware source/header used for code grounding |
| D:\Embedded Communication Protocols\CAN1_Loopback\Core\Inc\mcp2515.h | Finalized firmware source/header used for code grounding |
| D:\Embedded Communication Protocols\CAN1_Loopback\Core\Inc\spi.h | Finalized firmware source/header used for code grounding |
| D:\Embedded Communication Protocols\CAN1_Loopback\Core\Inc\usart.h | Finalized firmware source/header used for code grounding |
| https://chatgpt.com/share/6a390d75-f15c-83e8-927f-6d2623cce324 | Shared learning thread for CAN1 internal initialization and loopback theory |
| https://www.st.com/resource/en/datasheet/stm32f407vg.pdf | Official STM32F407 datasheet for CAN alternate-function/pin reference |
| https://www.st.com/resource/en/reference_manual/dm00031020.pdf | Official STM32F4 reference manual for bxCAN behavior and registers |

# 24. Appendix: Revision Checklist

| Check | Status |
| --- | --- |
| Shared ChatGPT link downloaded | Done |
| CAN1 loopback chunks extracted | Done |
| All user-named source/header files read | Done |
| Source-first frame values used: ID 0x301, data 0x31 0x32 | Done |
| System-level bxCAN loopback diagrams included | Done |
| BTR timing math explained | Done |
| Filter bank 0 accept-all explained | Done |
| TX mailbox and FIFO0 receive explained | Done |
| Engineering review findings included | Done |
| No screenshot invented | Done |
| Footer name set to Embedded Learning Document | Done |