CAN1 Normal Mode RX Interrupt Project

STM32F407 bxCAN normal-mode bus operation, FIFO0 receive interrupt, and two MCP2515 nodes on the same CAN network

Document intent: Long-term embedded learning reference for the finalized CAN1_RX_Interrupt firmware. It explains the STM32F407 internal bxCAN normal-mode setup, 500 kbps timing, filter/FIFO0 reception, CAN1_RX0 interrupt strategy, and the surrounding two-MCP2515-node test system.

| Field | Value |
| --- | --- |
| Project scope | CAN internal normal mode and reception interrupt |
| Primary controller | STM32F407 internal bxCAN CAN1, used as Node3 |
| External nodes | MCP2515 Node1 on SPI2 and MCP2515 Node2 on SPI1 |
| CAN pins configured by source | PD0 = CAN1_RX, PD1 = CAN1_TX, AF9 |
| CAN bitrate | 500 kbps from PCLK1 32 MHz, prescaler 8, total 8 TQ |
| Receive path | Filter bank 0 accept-all -> FIFO0 -> CAN1_RX0_IRQHandler -> main-loop service |
| Footer rule | Embedded Learning Document |
| Shared-chat boundary | Shared ChatGPT URL was not present in this message; content is grounded in finalized source files plus CAN/bxCAN fundamentals. |

# 1. Document Control / Metadata

| Item | Details |
| --- | --- |
| Design preset | compact_reference_guide |
| Firmware reference | D:\Embedded Communication Protocols\CAN1_RX_Interrupt |
| Code fidelity rule | Only user-attached finalized source files are treated as code authority |
| Theory source | Shared ChatGPT URL was not present in this message; content is grounded in finalized source files plus CAN/bxCAN fundamentals. |
| Reference baseline | STM32F407 bxCAN behavior from standard STM32F4 documentation concepts |
| Device references | https://www.st.com/resource/en/datasheet/stm32f407vg.pdf https://www.st.com/resource/en/reference_manual/dm00031020.pdf |
| Filename rule | No 'advanced' term is used in the output filename |

# 2. Learning Objectives

Understand how STM32F407 internal bxCAN moves from loopback learning to real normal-mode CAN bus communication.

Explain the complete receive interrupt chain: physical bus -> transceiver -> CAN1 RX -> filter -> FIFO0 -> CAN1_RX0_IRQn -> main loop.

Verify the 500 kbps bit-timing values used by CAN1 and MCP2515 nodes.

Understand why normal mode requires a real CAN transceiver, termination, and ACK-capable nodes.

Explain how FIFO0 message pending interrupt is enabled, temporarily masked in the ISR, and re-enabled after software drains FIFO0.

Differentiate STM32 bxCAN RX interrupt handling from MCP2515 active-low INT handling through EXTI0/EXTI1.

Review source-level corrections and risks without silently changing the finalized code.

# 3. Project Objective

The project validates a real multi-node CAN network where STM32F407 CAN1 operates in normal mode as an internal CAN node while two MCP2515 controllers also transmit and receive frames. The STM32 node uses FIFO0 receive-message-pending interrupt support: the ISR records that FIFO0 has data, masks the interrupt, and the main loop reads all pending frames before re-enabling the interrupt.

# 4. Problem Statement

Core engineering problem: Normal-mode CAN reception is more than enabling an IRQ. The peripheral must be timed correctly, connected to a valid physical CAN bus, configured with acceptance filters, protected against FIFO re-entry during service, and drained/released correctly so new frames can arrive.

# 5. System Overview

UART2 debug terminal
                                  |
                                  v
                         +------------------+
                         | STM32F407 MCU    |
                         | CAN1 / bxCAN     |
                         | Node3            |
                         +---------+--------+
                                   |
                         PD1 TX / PD0 RX, AF9
                                   |
                         +---------v--------+
                         | CAN transceiver  |
                         +---------+--------+
                                   |
              +--------------------+--------------------+
              |          CANH / CANL bus, 500 kbps      |
              +--------------------+--------------------+
                                   |
            +----------------------+----------------------+
            |                                             |
   +--------v---------+                         +---------v--------+
   | MCP2515 Node1    |                         | MCP2515 Node2    |
   | SPI2, CS PB12    |                         | SPI1, CS PA4     |
   | INT -> PA1/EXTI1 |                         | INT -> PA0/EXTI0 |
   +------------------+                         +------------------+

Support modules:
SPI1 also drives MAX7219 through PA15 CS.
SPI2 also drives W25Q flash through PB9 CS.

Unlike the previous internal loopback project, this source clears CAN_BTR.LBKM and CAN_BTR.SILM, so CAN1 depends on the external CAN bus. A transceiver is required between PD0/PD1 logic-level CAN pins and the differential CANH/CANL bus.

# 6. Requirements

| Requirement | Project value | Verification point |
| --- | --- | --- |
| CAN mode | Normal mode | CAN1_Init_Normal_500kbps clears LBKM and SILM |
| CAN pins | PD0 RX, PD1 TX, AF9 | CAN1_GPIO_Init uses GPIOD->MODER and GPIOD->AFR[0] |
| CAN bitrate | 500 kbps | PCLK1 32 MHz, BRP 7, TS1 4, TS2 1, SJW 0 |
| CAN filter | Bank 0 accept-all, FIFO0 | FR1=0, FR2=0, mask mode, 32-bit scale |
| CAN RX interrupt | FIFO0 message pending interrupt | CAN1->IER ¦= CAN_IER_FMPIE0 |
| NVIC | CAN1_RX0_IRQn priority 1 | NVIC_SetPriority and NVIC_EnableIRQ |
| ISR policy | Set pending flag, mask FMPIE0 | CAN1_RX0_IRQHandler |
| Service policy | Drain FIFO0 in main loop, then re-enable FMPIE0 | while(CAN1_ReadMessage_FIFO0(...)) |
| MCP2515 nodes | Both normal mode | MCP2515_SetMode(MCP2515_MODE_NORMAL) verified by CANSTAT |

# 7. Hardware Components

| Component | Role | Important caution |
| --- | --- | --- |
| STM32F407 | Runs CAN1 internal bxCAN Node3 and application supervisor | CAN1 pins are logic-level and need a CAN transceiver for normal mode |
| CAN transceiver | Converts CAN1 TX/RX logic to differential CANH/CANL | Mandatory for real normal-mode communication |
| CANH/CANL bus | Shared multi-node CAN network | Use correct termination at the bus ends, commonly 120 ohm each end |
| MCP2515 Node1 | External CAN controller through SPI2 | INT line is PA1/EXTI1 according to source |
| MCP2515 Node2 | External CAN controller through SPI1 | INT line is PA0/EXTI0 according to source |
| MAX7219 | Visual heartbeat/demo on SPI1 | Blinking introduces delay and is not part of CAN correctness |
| W25Q64FV | SPI flash exercise on SPI2 | Supporting module; not part of CAN reception logic |
| USART2 | Serial debug output | Blocking UART prints can change timing during dense CAN traffic |

# 8. Software and Firmware Tools

STM32CubeIDE or equivalent for build and flash.

UART terminal connected to USART2 for init, TX, RX, and error logs.

Logic analyzer or CAN analyzer for normal-mode bus validation.

Debugger watch windows for CAN1->MCR, MSR, BTR, IER, RF0R, TSR, ESR, and FIFO registers.

RM0090 reference manual for bxCAN register descriptions and interrupt behavior.

# 9. System-Level Diagrams

## 9.1 Normal-Mode CAN Data Path

MCP2515 Node1 sends ID 0x101
MCP2515 Node2 sends ID 0x201
STM32 bxCAN Node3 sends ID 0x301

Each transmitted frame:
  TX controller
      |
      v
  CAN transceiver
      |
      v
  CANH/CANL bus arbitration + ACK
      |
      v
  Other nodes' transceivers
      |
      v
  Controller receive state machine
      |
      v
  Acceptance filters
      |
      v
  RX buffer/FIFO
      |
      v
  Interrupt or polling service code

## 9.2 STM32 CAN1 RX Interrupt Flow

CAN frame reaches CAN1 RX
  |
  v
Filter bank 0 accepts the frame
  |
  v
FIFO0 pending count FMP0 becomes nonzero
  |
  v
CAN_IER_FMPIE0 allows CAN1_RX0_IRQn
  |
  v
CAN1_RX0_IRQHandler()
  |
  +-- if FMP0 != 0: can1_rx0_pending = 1
  +-- mask FMPIE0 to avoid repeated ISR entry while main loop drains FIFO0
  |
  v
main while(1)
  |
  +-- if can1_rx0_pending:
        clear software flag
        while(CAN1_ReadMessage_FIFO0(...)):
            decode ID, DLC, data
            release FIFO0 using RFOM0
        re-enable FMPIE0

## 9.3 MCP2515 RX Interrupt Flow

MCP2515 receives a CAN frame
  |
  v
CANINTF.RX0IF or RX1IF is set
  |
  v
MCP2515 INT pin pulls low
  |
  v
STM32 EXTI falling edge:
  PA1/EXTI1 -> mcp2515_node1_int_pending = 1
  PA0/EXTI0 -> mcp2515_node2_int_pending = 1
  |
  v
main loop selects matching SPI bus/node
  |
  v
MCP2515_ReadMessage drains RXB0/RXB1 and clears CANINTF flag

# 10. Firmware Architecture

| Module | Responsibility | Important APIs / data |
| --- | --- | --- |
| can.c | CAN1 GPIO, normal-mode init, filter, RX interrupt enable, TX/RX frame helpers | CAN1_Init_Normal_500kbps, CAN1_RX0_Interrupt_Enable, CAN1_ReadMessage_FIFO0 |
| stm32f4xx_it.c | ISR flags for MCP2515 INT lines and CAN1 FIFO0 | EXTI0_IRQHandler, EXTI1_IRQHandler, CAN1_RX0_IRQHandler |
| main.c | System startup and test loop for Node1, Node2, and Node3 | mcp2515_node*_int_pending, can1_rx0_pending |
| mcp2515.c | External CAN controllers over SPI | MCP2515_Init, MCP2515_SendMessage, MCP2515_ReadMessage |
| spi.c | SPI1/SPI2 configuration and blocking transfers | SPI1_TransferByte, SPI2_TransferByte |
| usart.c | Debug print path | USART2_SendString |
| max7219.c | Display heartbeat/demo | MAX7219_CharBlinking |
| w25q64fv.c | SPI flash bring-up test | W25Q_FlashReadJEDEC, W25Q_FlashPageProgram |

# 11. Communication Protocol Used

The primary protocol is Classic CAN using 11-bit standard identifiers and up to 8 data bytes. The implemented test frames are ID 0x101 from MCP2515 Node1 with data 0x11 0x12, ID 0x201 from MCP2515 Node2 with data 0x21 0x22, and ID 0x301 from STM32 bxCAN Node3 with data 0x31 0x32. SPI is used as a support protocol to configure and access the MCP2515 controllers, MAX7219 display, and W25Q flash.

# 12. Core Concept Deep Dive

## 12.1 Normal Mode vs Loopback Mode

| Topic | Loopback mode | Normal mode in this project |
| --- | --- | --- |
| CAN_BTR.LBKM | Set | Cleared |
| CAN_BTR.SILM | Normally cleared unless listen-only is needed | Cleared |
| External bus required | No | Yes |
| ACK required | No real external ACK | Yes, another CAN node must acknowledge |
| Validation scope | Internal controller path | Controller + transceiver + bus + other nodes |
| Analyzer visibility | No physical CAN frame required | Frames should be visible on CANH/CANL |

## 12.2 CAN1 Bit Timing

| Parameter | Source value | Meaning |
| --- | --- | --- |
| PCLK1 | 32 MHz | SystemClock_Config sets SYSCLK 64 MHz and APB1 divider /2 |
| Prescaler | 8 | BRP field is 7 because hardware encoding is prescaler - 1 |
| BS1 | 5 TQ | TS1 field is 4 |
| BS2 | 2 TQ | TS2 field is 1 |
| SJW | 1 TQ | SJW field is 0 |
| Total TQ | 1 + 5 + 2 = 8 | Sync segment + BS1 + BS2 |
| Bitrate | 32 MHz / (8 x 8) = 500 kbps | Matches the code comments and BTR setup |
| Sample point | 75 percent | (1 + BS1) / total TQ = 6 / 8 |

## 12.3 Filter Bank 0 Accept-All

The finalized CAN1 receive filter uses filter bank 0 in mask mode, 32-bit scale, assigned to FIFO0, with FR1 and FR2 both zero. This is intentional for bring-up because every frame is accepted into FIFO0. It is excellent for learning and system validation, but production firmware should later narrow filters to the expected IDs.

## 12.4 FIFO0 Message Pending Interrupt

CAN_IER_FMPIE0 enables an interrupt when FIFO0 has at least one pending message. In the ISR, the source checks CAN_RF0R_FMP0, sets can1_rx0_pending, then disables FMPIE0. That design keeps the ISR short and prevents repeated interrupt entry while the main loop drains all pending messages. After the receive loop finishes, the main loop re-enables FMPIE0.

## 12.5 Why RFOM0 Matters

Reading RIR, RDTR, RDLR, and RDHR copies the received frame into software variables, but it does not remove the frame from FIFO0. Setting CAN_RF0R_RFOM0 releases the FIFO0 output mailbox so the next pending frame can move forward. Missing RFOM0 is a classic reason for seeing the same received frame repeatedly or stalling FIFO processing.

## 12.6 MCP2515 Interrupts vs bxCAN Interrupts

| Topic | MCP2515 path | STM32 bxCAN path |
| --- | --- | --- |
| Interrupt source | External INT pin from MCP2515 | Internal CAN1_RX0 interrupt line |
| STM32 IRQ | EXTI0/EXTI1 | CAN1_RX0_IRQn |
| Pending flag | mcp2515_node1_int_pending / mcp2515_node2_int_pending | can1_rx0_pending |
| Data drain | SPI reads RXB0/RXB1 and clears CANINTF | Direct reads FIFO0 registers and sets RFOM0 |
| Configuration register | MCP2515 CANINTE.RX0IE/RX1IE | CAN1->IER.FMPIE0 |

# 13. Module-Wise Implementation

| Phase | Final-source behavior | Engineering purpose |
| --- | --- | --- |
| GPIO setup | GPIOD clock enabled, PD0/PD1 set to AF9 | Route CAN1 RX/TX to physical pins |
| Init entry | CAN1 clock enabled, sleep cleared, INRQ set, INAK waited | Enter safe configuration mode |
| MCR setup | TTCM, ABOM, AWUM, NART, RFLM, TXFP cleared | Simple bring-up behavior with automatic retransmission enabled |
| BTR setup | BRP=7, TS1=4, TS2=1, SJW=0; LBKM/SILM cleared | 500 kbps normal mode |
| Filter setup | Accept-all filter bank 0 to FIFO0 | Receive all test frames from all three nodes |
| Interrupt enable | FMPIE0 enabled, CAN1_RX0_IRQn priority 1 | Trigger when FIFO0 becomes nonempty |
| ISR | Set can1_rx0_pending and mask FMPIE0 | Short ISR, deferred frame parsing |
| Main service | Drain FIFO0 through CAN1_ReadMessage_FIFO0 and re-enable FMPIE0 | Consume all pending frames safely |
| MCP2515 service | EXTI flags are drained through SPI reads | Handle Node1/Node2 received frames |

# 14. Main Algorithm and Flowchart

Power-on
  |
  +-- HAL_Init and SystemClock_Config
  +-- USART2_Init
  +-- SPI2 + SPI1 initialization
  +-- MCP2515 INT lines: PA0/EXTI0 and PA1/EXTI1
  +-- MCP2515 Node1 normal mode
  +-- MCP2515 Node2 normal mode
  +-- MAX7219 display test
  +-- CAN1_GPIO_Init
  +-- CAN1_Init_Normal_500kbps
  +-- CAN1_RX0_Interrupt_Enable
  +-- W25Q flash test
  |
  +-- Forever:
        STM32 CAN1 sends ID 0x301
        MCP2515 Node1 sends ID 0x101
        MCP2515 Node2 sends ID 0x201
        if Node1 INT pending: drain MCP2515 Node1 RX buffers
        if Node2 INT pending: drain MCP2515 Node2 RX buffers
        if CAN1 RX0 pending: drain CAN1 FIFO0 and re-enable FMPIE0
        blink 'A' on MAX7219

# 15. Code Explanation

Code authority: The snippets below are copied from the finalized CAN1_RX_Interrupt source files. Comments with visible encoding artifacts are normalized only for readability in the document; the register-level behavior is unchanged.

## 15.1 CAN1 GPIO Setup

void CAN1_GPIO_Init(void)
{
    /* Enable GPIOB clock */
    RCC->AHB1ENR |= RCC_AHB1ENR_GPIODEN;

    /*
     * PD0  = CAN1_RX
     * PD1  = CAN1_TX
     * AF9  = CAN1
     */

    /* Clear mode bits */
    GPIOD->MODER &= ~((3UL << (0 * 2)) |
                      (3UL << (1 * 2)));

    /* Alternate function mode */
    GPIOD->MODER |=  ((2UL << (0 * 2)) |
                      (2UL << (1 * 2)));

    /* Push-pull */
    GPIOD->OTYPER &= ~((1UL << 0) |
                       (1UL << 1));

    /* High speed */
    GPIOD->OSPEEDR &= ~((3UL << (0 * 2)) |
                        (3UL << (1 * 2)));

    GPIOD->OSPEEDR |=  ((2UL << (0 * 2)) |
                        (2UL << (1 * 2)));

    /*
     * CAN RX recessive is high.
     * Pull-up on RX is useful.
     */
    GPIOD->PUPDR &= ~((3UL << (0 * 2)) |
                      (3UL << (1 * 2)));

    GPIOD->PUPDR |= (1UL << (0 * 2));   /* PB8 pull-up */

    /* AF9 for PB8 and PB9 */
    GPIOD->AFR[0] &= ~((GPIO_AFRL_AFSEL0) |
                       (GPIO_AFRL_AFSEL1));

    GPIOD->AFR[0] |=  (9UL << (GPIO_AFRL_AFSEL0_Pos)) |
    				  (9UL << (GPIO_AFRL_AFSEL1_Pos));
}

Verified correction: The executable code configures GPIOD pins PD0 and PD1 for CAN1 AF9. Some comments still mention PB8/PB9 from an older variant. This document treats PD0/PD1 as the correct source-grounded pins.

## 15.2 CAN1 Normal-Mode 500 kbps Initialization

uint8_t CAN1_Init_Normal_500kbps(void)
{
    uint32_t timeout;

    RCC->APB1ENR |= RCC_APB1ENR_CAN1EN;

    /* Leave sleep mode */
    CAN1->MCR &= ~CAN_MCR_SLEEP;

    /* Request initialization mode */
    CAN1->MCR |= CAN_MCR_INRQ;

    timeout = 1000000;
    while(((CAN1->MSR & CAN_MSR_INAK) == 0) && timeout)
    {
        timeout--;
    }

    if(timeout == 0)
    {
        return 0;
    }

    CAN1->MCR &= ~(CAN_MCR_TTCM |
                   CAN_MCR_ABOM |
                   CAN_MCR_AWUM |
                   CAN_MCR_NART |
                   CAN_MCR_RFLM |
                   CAN_MCR_TXFP);

    /*
     * Same 500 kbps timing as loopback:
     * PCLK1 = 32 MHz
     * Prescaler = 8
     * BS1 = 5
     * BS2 = 2
     * Total TQ = 8
     */
    CAN1->BTR = 0;
    CAN1->BTR |= (7UL << CAN_BTR_BRP_Pos);
    CAN1->BTR |= (4UL << CAN_BTR_TS1_Pos);
    CAN1->BTR |= (1UL << CAN_BTR_TS2_Pos);
    CAN1->BTR |= (0UL << CAN_BTR_SJW_Pos);

    /*
     * Normal mode:
     * loopback disabled
     * silent disabled
     */
    CAN1->BTR &= ~CAN_BTR_LBKM;
    CAN1->BTR &= ~CAN_BTR_SILM;

    /* In case of Normal mode , no need of RX pull up .
     * Recessive bits will be received from the bus
     */
    GPIOD->OTYPER &= ~(3UL << (0 * 2));

    /* Initialize filter config */
    CAN1_Filter_AcceptAll_FIFO0();

    /* Leave initialization mode */
    CAN1->MCR &= ~CAN_MCR_INRQ;

    timeout = 1000000;
    while((CAN1->MSR & CAN_MSR_INAK) && timeout)
    {
        timeout--;
    }

    if(timeout == 0)
    {
        return 0;
    }


    return 1;
}

## 15.3 CAN1 RX0 Interrupt Enable

void CAN1_RX0_Interrupt_Enable(void)
{
    CAN1->IER |= CAN_IER_FMPIE0;

    NVIC_SetPriority(CAN1_RX0_IRQn, 1);
    NVIC_EnableIRQ(CAN1_RX0_IRQn);
}

## 15.4 CAN1 FIFO0 Interrupt Handler

void CAN1_RX0_IRQHandler(void)
{
    /*
     * FMP0[1:0] = number of pending messages in FIFO0.
     */
    if((CAN1->RF0R & CAN_RF0R_FMP0) != 0)
    {
        can1_rx0_pending = 1;
    }

    CAN1->IER &= ~CAN_IER_FMPIE0;
}

## 15.5 Filter Bank 0 Accept-All

void CAN1_Filter_AcceptAll_FIFO0(void)
{
    /*
     * Enter filter initialization mode.
     * Reception is disabled while FINIT = 1.
     */
    CAN1->FMR |= CAN_FMR_FINIT;

    /* Deactivate filter bank 0 */
    CAN1->FA1R &= ~(1UL << 0);

    /*
     * Identifier mask mode:
     * 0 = mask mode
     */
    CAN1->FM1R &= ~(1UL << 0);

    /*
     * 32-bit scale:
     * 1 = one 32-bit filter
     */
    CAN1->FS1R |= (1UL << 0);

    /*
     * Assign filter bank 0 to FIFO0:
     * 0 = FIFO0
     */
    CAN1->FFA1R &= ~(1UL << 0);

    /*
     * Accept all:
     * ID = 0
     * Mask = 0
     */
    CAN1->sFilterRegister[0].FR1 = 0x00000000;
    CAN1->sFilterRegister[0].FR2 = 0x00000000;

    /* Activate filter bank 0 */
    CAN1->FA1R |= (1UL << 0);

    /* Leave filter initialization mode */
    CAN1->FMR &= ~CAN_FMR_FINIT;
}

## 15.6 Standard CAN Transmit

uint8_t CAN1_SendStdMessage(uint16_t id, uint8_t *data, uint8_t len)
{
    uint8_t mailbox;
    uint32_t tdlr = 0;
    uint32_t tdhr = 0;

    if(id > 0x7FF)
    {
        return E_NOT_OK;
    }

    if(len > 8)
    {
        return E_NOT_OK;
    }

    if((data == 0) && (len > 0))
    {
        return E_NOT_OK;
    }

    /*
     * Find empty transmit mailbox.
     * TME0 = bit 26
     * TME1 = bit 27
     * TME2 = bit 28
     */
    if(CAN1->TSR & CAN_TSR_TME0)
    {
        mailbox = 0;
    }
    else if(CAN1->TSR & CAN_TSR_TME1)
    {
        mailbox = 1;
    }
    else if(CAN1->TSR & CAN_TSR_TME2)
    {
        mailbox = 2;
    }
    else
    {
        return E_NOT_OK;
    }

    /*
     * Prepare identifier register.
     *
     * Standard ID goes to STID[10:0] = bits [31:21]
     * IDE = 0 for standard ID
     * RTR = 0 for data frame
     * TXRQ = 0 initially
     */
    CAN1->sTxMailBox[mailbox].TIR = ((uint32_t)id << 21);

    /*
     * DLC in bits [3:0]
     */
    CAN1->sTxMailBox[mailbox].TDTR = (len & 0x0F);

    /*
     * Pack data bytes.
     */
    if(len > 0) tdlr |= ((uint32_t)data[0] << 0);
    if(len > 1) tdlr |= ((uint32_t)data[1] << 8);
    if(len > 2) tdlr |= ((uint32_t)data[2] << 16);
    if(len > 3) tdlr |= ((uint32_t)data[3] << 24);

    if(len > 4) tdhr |= ((uint32_t)data[4] << 0);
    if(len > 5) tdhr |= ((uint32_t)data[5] << 8);
    if(len > 6) tdhr |= ((uint32_t)data[6] << 16);
    if(len > 7) tdhr |= ((uint32_t)data[7] << 24);

    CAN1->sTxMailBox[mailbox].TDLR = tdlr;
    CAN1->sTxMailBox[mailbox].TDHR = tdhr;

    /*
     * Request transmission.
     */
    CAN1->sTxMailBox[mailbox].TIR |= CAN_TI0R_TXRQ;

    return E_OK;
}

## 15.7 FIFO0 Receive and Release

uint8_t CAN1_ReadMessage_FIFO0(uint16_t *id, uint8_t *data, uint8_t *len)
{
    uint32_t rir;
    uint32_t rdtr;
    uint32_t rdlr;
    uint32_t rdhr;
    uint8_t dlc;

    /*
     * FMP0[1:0] = number of pending messages in FIFO0.
     */
    if((CAN1->RF0R & CAN_RF0R_FMP0) == 0)
    {
        return E_NOT_OK;
    }

    rir  = CAN1->sFIFOMailBox[0].RIR;
    rdtr = CAN1->sFIFOMailBox[0].RDTR;
    rdlr = CAN1->sFIFOMailBox[0].RDLR;
    rdhr = CAN1->sFIFOMailBox[0].RDHR;

    /*
     * Standard ID:
     * STID[10:0] is in RIR[31:21]
     */
    *id = (uint16_t)((rir >> 21) & 0x7FF);

    /*
     * DLC:
     * bits [3:0]
     */
    dlc = (uint8_t)(rdtr & 0x0F);

    if(dlc > 8)
    {
        dlc = 8;
    }

    *len = dlc;

    /*
     * Unpack data bytes.
     */
    if(dlc > 0) data[0] = (uint8_t)((rdlr >> 0)  & 0xFF);
    if(dlc > 1) data[1] = (uint8_t)((rdlr >> 8)  & 0xFF);
    if(dlc > 2) data[2] = (uint8_t)((rdlr >> 16) & 0xFF);
    if(dlc > 3) data[3] = (uint8_t)((rdlr >> 24) & 0xFF);

    if(dlc > 4) data[4] = (uint8_t)((rdhr >> 0)  & 0xFF);
    if(dlc > 5) data[5] = (uint8_t)((rdhr >> 8)  & 0xFF);
    if(dlc > 6) data[6] = (uint8_t)((rdhr >> 16) & 0xFF);
    if(dlc > 7) data[7] = (uint8_t)((rdhr >> 24) & 0xFF);

    /*
     * Release FIFO0 output mailbox.
     * This is like clearing RX flag in MCP2515.
     */
    CAN1->RF0R |= CAN_RF0R_RFOM0;

    return E_OK;
}

## 15.8 Main Initialization Path

USART2_Init();

  USART2_SendString("UART Initialized....!!\r\n");

  SPI2_GPIO_Init();
  SPI2_Init();

  SPI1_GPIO_Init();
  SPI1_Init();
  USART2_SendString("SPI Initialized.....!!\r\n");

  //Enable Exti interrupt in PA0 for node2
  MCP2515_INT_PA0_Init();

  //Enable Exti interrupt in PA1 for node1
  MCP2515_INT_PA1_Init();

  MCP2515_SelectNode(MCP2515_NODE_1);
  MCP2515_SelectSpiBus(MCP2515_SPI_BUS_2);
  SPI2_Test();

  MCP2515_SelectNode(MCP2515_NODE_2);
  MCP2515_SelectSpiBus(MCP2515_SPI_BUS_1);
  SPI1_Test();

  // Initialize MCP node1 and move to normal mode
  MCP2515_SelectNode(MCP2515_NODE_1);
  MCP2515_SelectSpiBus(MCP2515_SPI_BUS_2);
  MCP2515_Init();

  //Enter normal mode
  MCP2515_SetMode(MCP2515_MODE_NORMAL);

  //check and wait till normal mode
  while((MCP2515_ReadRegister(MCP2515_CANSTAT) & MCP2515_MODE_MASK) != MCP2515_MODE_NORMAL);

  uint8_t canstat = MCP2515_ReadRegister(MCP2515_CANSTAT);

  if((canstat & MCP2515_MODE_MASK) == MCP2515_MODE_NORMAL)
  {
      USART2_SendString("MCP2515 Node1 Normal mode OK\r\n");
  }
  else
  {
      USART2_SendString("MCP2515 Node1 Normal mode failed\r\n");
  }

  // Initialize MCP node2 and move to normal mode
  MCP2515_SelectNode(MCP2515_NODE_2);
  MCP2515_SelectSpiBus(MCP2515_SPI_BUS_1);
  MCP2515_Init();

  //Enter Normal mode
  MCP2515_SetMode(MCP2515_MODE_NORMAL);

  //check and wait till loopback mode
  while((MCP2515_ReadRegister(MCP2515_CANSTAT) & MCP2515_MODE_MASK) != MCP2515_MODE_NORMAL);

  uint8_t canstat2 = MCP2515_ReadRegister(MCP2515_CANSTAT);

  if((canstat2 & MCP2515_MODE_MASK) == MCP2515_MODE_NORMAL)
  {
      USART2_SendString("MCP2515 Node2 Normal mode OK\r\n");
  }
  else
  {
      USART2_SendString("MCP2515 Node2 Normal mode failed\r\n");
  }

  // Simple hardware test
  MAX7219_Write(0x0F, 0x01);   // All LEDs ON
  HAL_Delay(2000);
  MAX7219_Write(0x0F, 0x00);   // Test OFF

  MAX7219_Init();
  MAX7219_Clear();

  CAN1_GPIO_Init();
  if(CAN1_Init_Normal_500kbps())
  {
      USART2_SendString("CAN1 Normal Mode init OK\r\n");
  }
  else
  {
      USART2_SendString("CAN1 Normal Mode init failed\r\n");
  }

  CAN1_RX0_Interrupt_Enable();

  W25Q_FlashReset();
  W25Q_FlashReadJEDEC(jedec_id);
  W25Q_FlashRead_StatusRegister(w25q_status);

  //Programming sequence - Erase sector,program page, read data
  W25Q_FlashSectorErase(W25Q_TEST_ADDR);
  W25Q_FlashPageProgram(W25Q_TEST_ADDR,tx_data,5);
  W25Q_FlashRead_Data(W25Q_TEST_ADDR, rx, 5);

## 15.9 Main Loop CAN Test Path

/*********TEST : CAN message transmission and reception**********/\r\n");
	  	  USART2_SendString("\r\n");

	  	  //CAN Internal Node3 transmit message
	  	  if(CAN1_SendStdMessage(0x301,node3_msg,2) == E_OK)
	  	  {
	  	      USART2_SendString("CAN1 TX requested\r\n");
			  sprintf(uart_msg_tx,"bxCAN Message Transmitted : ID = 0x%03X, DLC = %u, DATA1 = 0x%02X, DATA2 = 0x%02X\r\n",0x301,2,node3_msg[0],node3_msg[1]);
			  USART2_SendString(uart_msg_tx);
	  	  }
	  	  else
	  	  {
	  	      USART2_SendString("CAN1 TX failed\r\n");
	  	  }

	  	//Node1 transmit message
		  MCP2515_SelectNode(MCP2515_NODE_1);
		  MCP2515_SelectSpiBus(MCP2515_SPI_BUS_2);

		  if(MCP2515_SendMessage(0x101,msg,dlc_len))
		  {
			  HAL_Delay(10);

			  if(MCP2515_PrintTxStatus(current_txctrl_addr))
			  {
				  USART2_SendString("Node1 Message Transmitted Successfully\r\n");
			  }
			  else
			  {
				  USART2_SendString("Node1 Tx Error\r\n");
			  }

			  sprintf(uart_msg_tx,"Node1 Message Transmitted : ID = 0x%03X, DLC = 0x%02X, DATA1 = 0x%02X, DATA2 = 0x%02X\r\n",0x101,dlc_len,msg[0],msg[1]);
			  USART2_SendString(uart_msg_tx);
		  }
		  else
		  {
			  USART2_SendString("Node1 : Message Tx Failed\r\n");
		  }

		  //Node2 transmit message
		  MCP2515_SelectNode(MCP2515_NODE_2);
		  MCP2515_SelectSpiBus(MCP2515_SPI_BUS_1);

		  if(MCP2515_SendMessage(0x201,msg2,dlc_len))
		  {
			  HAL_Delay(10);

			  if(MCP2515_PrintTxStatus(current_txctrl_addr))
			  {
				  USART2_SendString("Node2 Message Transmitted Successfully\r\n");
			  }
			  else
			  {
				  USART2_SendString("Node2 Tx Error\r\n");
			  }
			  sprintf(uart_msg_tx,"Node2 Message Transmitted : ID = 0x%03X, DLC = 0x%02X, DATA1 = 0x%02X, DATA2 = 0x%02X\r\n",0x201,dlc_len,msg2[0],msg2[1]);
			  USART2_SendString(uart_msg_tx);
		  }
		  else
		  {
			  USART2_SendString("Node2 : Message Tx Failed\r\n");
		  }

	  	  HAL_Delay(10);

	  	  if(mcp2515_node1_int_pending)
	  	  {
	  		  mcp2515_node1_int_pending = 0;

	  		  MCP2515_SelectNode(MCP2515_NODE_1);
	  		  MCP2515_SelectSpiBus(MCP2515_SPI_BUS_2);

	  		  //Receiving frame
	  		  while(MCP2515_ReadMessage(&rx_id,rx_msg,&rx_dlc_len))
	  		  {
	  			  sprintf(uart_msg_rx,"Node1 Message Received    : ID = 0x%03X, DLC = 0x%02X, DATA1 = 0x%02X, DATA2 = 0x%02X\r\n",rx_id,rx_dlc_len,rx_msg[0],rx_msg[1]);
	  			  USART2_SendString(uart_msg_rx);
	  		  }
	  	  }

	  	  if(mcp2515_node2_int_pending)
	  	  {
	  		  mcp2515_node2_int_pending = 0;

	  		 //Node 2 Receives
	  		  MCP2515_SelectNode(MCP2515_NODE_2);
	  		  MCP2515_SelectSpiBus(MCP2515_SPI_BUS_1);

	  		  //Receiving frame
	  		  while(MCP2515_ReadMessage(&rx_id2,rx_msg2,&rx_dlc_len2))
	  		  {
	  			  sprintf(uart_msg_rx2,"Node2 Message Received    : ID = 0x%03X, DLC = 0x%02X, DATA1 = 0x%02X, DATA2 = 0x%02X\r\n",rx_id2,rx_dlc_len2,rx_msg2[0],rx_msg2[1]);
	  			  USART2_SendString(uart_msg_rx2);
	  		  }
	  	  }

	  	  if(can1_rx0_pending)
	  	  {
	  		can1_rx0_pending = 0;

			  while(CAN1_ReadMessage_FIFO0(&rx_id3, rx_msg3, &rx_dlc_len3))
			  {
				  sprintf(uart_msg_rx3,"bxCAN Message Received    : ID = 0x%03X, DLC = 0x%02X, DATA1 = 0x%02X, DATA2 = 0x%02X\r\n",rx_id3,rx_dlc_len3,rx_msg3[0],rx_msg3[1]);
				  USART2_SendString(uart_msg_rx3);
			  }

			  //Enable interrupt for next msg reception
			  CAN1->IER |= (CAN_IER_FMPIE0);

	  	  }

## 15.10 MCP2515 Normal-Mode and RX Interrupt Support

void MCP2515_Init(void)
{
	// Reset to reach config mode
	MCP2515_Reset();

	//check and wait till config mode
	while((MCP2515_ReadRegister(MCP2515_CANSTAT) & MCP2515_MODE_MASK) != MCP2515_MODE_CONFIG);

	/*Baudrate config , 500kbps ,8 N_TQ, SyncSeg = 1 TQ,
	PropSeg = 2 TQ ,PS1     = 3 TQ ,PS2     = 2 TQ
	Total   = 8 TQ*/
	MCP2515_WriteRegister(MCP2515_CNF1, 0x00);
	MCP2515_WriteRegister(MCP2515_CNF2, 0x91);
	MCP2515_WriteRegister(MCP2515_CNF3, 0x01);

	//Filter,mask settings, receive any message ID
	MCP2515_BitModify(MCP2515_RXB0CTRL, 0x60 , 0x60);
	MCP2515_BitModify(MCP2515_RXB1CTRL, 0x60 , 0x60);

	//enable RXB0 rollover to RXB1 */
	MCP2515_BitModify(MCP2515_RXB0CTRL, 0x04, 0x04);

	/* Clear interrupt flags */
	MCP2515_WriteRegister(MCP2515_CANINTF, 0x00);

	/* Enable Receive interrupt for buffer 0 and buffer 1 */
	MCP2515_BitModify(MCP2515_CANINTE, 0x03, 0x03);
}

void MCP2515_INT_PA0_Init(void)
{
    /*
     * MCP2515 INT pin:
     * Active low
     * Normal state = high
     * Interrupt event = falling edge
     */

    /* Enable GPIOA clock */
    RCC->AHB1ENR |= RCC_AHB1ENR_GPIOAEN;

    /* Enable SYSCFG clock */
    RCC->APB2ENR |= RCC_APB2ENR_SYSCFGEN;

    /*
     * PA0 as input
     * MODER0 = 00
     */
    GPIOA->MODER &= ~GPIO_MODER_MODER0;

    /*
     * Pull-up on PA0
     * PUPDR0 = 01
     */
    GPIOA->PUPDR &= ~GPIO_PUPDR_PUPD0;
    GPIOA->PUPDR |=  GPIO_PUPDR_PUPD0_0;

    /*
     * Map EXTI0 to Port A.
     * EXTICR[0], EXTI0 field = bits [3:0]
     * Port A = 0000
     */
    SYSCFG->EXTICR[0] &= ~SYSCFG_EXTICR1_EXTI0;

    /*
     * Unmask EXTI0 interrupt
     */
    EXTI->IMR |= EXTI_IMR_MR0;

    /*
     * Falling edge trigger enable.
     * MCP2515 INT is active-low.
     */
    EXTI->FTSR |= EXTI_FTSR_TR0;

    /*
     * Disable rising edge trigger
     */
    EXTI->RTSR &= ~EXTI_RTSR_TR0;

    /*
     * Clear pending flag before enabling NVIC.
     * Write 1 to clear.
     */
    EXTI->PR = EXTI_PR_PR0;

    /*
     * Optional but recommended: set priority
     */
    NVIC_SetPriority(EXTI0_IRQn, 2);

    /*
     * Enable EXTI line 0 interrupt in NVIC.
     */
    NVIC_EnableIRQ(EXTI0_IRQn);
}

void MCP2515_INT_PA1_Init(void)
{
    /*
     * MCP2515 INT pin:
     * Active low
     * Normal state = high
     * Interrupt event = falling edge
     */

    /* Enable GPIOA clock */
    RCC->AHB1ENR |= RCC_AHB1ENR_GPIOAEN;

    /* Enable SYSCFG clock */
    RCC->APB2ENR |= RCC_APB2ENR_SYSCFGEN;

    /*
     * PA1 as input
     * MODER0 = 00
     */
    GPIOA->MODER &= ~GPIO_MODER_MODER1;

    /*
     * Pull-up on PA1
     * PUPDR0 = 01
     */
    GPIOA->PUPDR &= ~GPIO_PUPDR_PUPD1;
    GPIOA->PUPDR |=  GPIO_PUPDR_PUPD1_0;

    /*
     * Map EXTI0 to Port A.
     * EXTICR[0], EXTI0 field = bits [3:0]
     * Port A = 0000
     */
    SYSCFG->EXTICR[0] &= ~SYSCFG_EXTICR1_EXTI1;

    /*
     * Unmask EXTI0 interrupt
     */
    EXTI->IMR |= EXTI_IMR_MR1;

    /*
     * Falling edge trigger enable.
     * MCP2515 INT is active-low.
     */
    EXTI->FTSR |= EXTI_FTSR_TR1;

    /*
     * Disable rising edge trigger
     */
    EXTI->RTSR &= ~EXTI_RTSR_TR1;

    /*
     * Clear pending flag before enabling NVIC.
     * Write 1 to clear.
     */
    EXTI->PR = EXTI_PR_PR1;

    /*
     * Optional but recommended: set priority
     */
    NVIC_SetPriority(EXTI1_IRQn, 2);

    /*
     * Enable EXTI line 0 interrupt in NVIC.
     */
    NVIC_EnableIRQ(EXTI1_IRQn);
}

void EXTI0_IRQHandler(void)
{
    if(EXTI->PR & EXTI_PR_PR0)
    {
        EXTI->PR = EXTI_PR_PR0;
        mcp2515_node2_int_pending = 1;
    }
}

void EXTI1_IRQHandler(void)
{
    if(EXTI->PR & EXTI_PR_PR1)
    {
        EXTI->PR = EXTI_PR_PR1;
        mcp2515_node1_int_pending = 1;
    }
}

## 15.11 Key Headers

/*
 * can.h
 *
 *  Created on: 20-Jun-2026
 *      Author: HP
 */

#ifndef INC_CAN_H_
#define INC_CAN_H_

#include "stm32f407xx.h"
#include "common.h"

#define CAN1_RESET_MASK		(1UL << 15UL)

void CAN1_GPIO_Init(void);
uint8_t CAN1_Init(void);
void CAN1_Filter_AcceptAll_FIFO0(void);
uint8_t CAN1_InitializationMode(void);
void CAN1_Reset(void);
uint8_t CAN1_SendStdMessage(uint16_t id,
							uint8_t *data,
							uint8_t len);
uint8_t CAN1_ReadMessage_FIFO0(uint16_t *id,
							uint8_t *data,
							uint8_t *len);
uint8_t CAN1_Init_Loopback_500kbps(void);
uint8_t CAN1_Init_Normal_500kbps(void);
void CAN1_RX0_Interrupt_Enable(void);

#endif /* INC_CAN_H_ */

/*
 * common.h
 *
 *  Created on: 21-Jun-2026
 *      Author: HP
 */

#ifndef INC_COMMON_H_
#define INC_COMMON_H_


#define E_NOT_OK 	0u
#define E_OK		1



#endif /* INC_COMMON_H_ */

/*
 * spi.h
 *
 *  Created on: 09-Jun-2026
 *      Author: HP
 */

#include "stm32f407xx.h"
#include "common.h"

typedef enum
{
	MCP2515_NODE_1 = 0,
	MCP2515_NODE_2
}MCP2515_Node_t;

typedef enum
{
    MCP2515_SPI_BUS_1 = 0,
    MCP2515_SPI_BUS_2
} MCP2515_SpiBus_t;

void SPI1_Init(void);
void SPI1_GPIO_Init(void);
uint8_t SPI1_TransferByte(uint8_t data);

void SPI2_GPIO_Init(void);
void SPI2_Init(void);
uint8_t SPI2_TransferByte(uint8_t byte);
void SPI2_TransmitBuffer(const uint8_t *tx_buf, uint16_t len);
void SPI2_Test(void);
void SPI2_ReceiveBuffer(uint8_t *rx_buf, uint16_t len);
void SPI2_TransferBuffer(const uint8_t *tx_buf,
                         uint8_t *rx_buf,
                         uint16_t len);
void SPI2_WaitNotBusy(void);


void MCP2515_SelectNode(MCP2515_Node_t node);
void MCP2515_SelectSpiBus(MCP2515_SpiBus_t bus);


#define MCP2515_CS_LOW()   			(GPIOB->BSRR = GPIO_BSRR_BR12)
#define MCP2515_CS_HIGH()  			(GPIOB->BSRR = GPIO_BSRR_BS12)

#define MAX7219_CS_LOW()   			(GPIOA->BSRR = GPIO_BSRR_BR15)
#define MAX7219_CS_HIGH()  			(GPIOA->BSRR = GPIO_BSRR_BS15)

#define MCP2515_NODE2_CS_LOW()   	(GPIOA->BSRR = GPIO_BSRR_BR4)
#define MCP2515_NODE2_CS_HIGH()  	(GPIOA->BSRR = GPIO_BSRR_BS4)

#define W25Q_FLASH_CS_LOW()   		(GPIOB->BSRR = GPIO_BSRR_BR9)
#define W25Q_FLASH_CS_HIGH()  		(GPIOB->BSRR = GPIO_BSRR_BS9)

# 16. Testing and Validation

| Test | Expected observation | Meaning |
| --- | --- | --- |
| CAN1 normal init | CAN1 Normal Mode init OK | CAN1 entered and exited init mode with normal BTR setup |
| MCP2515 Node1 mode | MCP2515 Node1 Normal mode OK | Node1 accepted mode change through CANSTAT |
| MCP2515 Node2 mode | MCP2515 Node2 Normal mode OK | Node2 accepted mode change through CANSTAT |
| CAN1 TX request | bxCAN Message Transmitted : ID = 0x301 | CAN1 found a free transmit mailbox and requested TX |
| Node1 TX request | Node1 Message Transmitted : ID = 0x101 | MCP2515 Node1 TX buffer loaded and requested |
| Node2 TX request | Node2 Message Transmitted : ID = 0x201 | MCP2515 Node2 TX buffer loaded and requested |
| CAN1 RX interrupt | can1_rx0_pending becomes 1 and bxCAN Message Received is printed | FIFO0 message pending interrupt path works |
| FIFO release | while(CAN1_ReadMessage_FIFO0(...)) exits cleanly | RFOM0 releases each FIFO0 output mailbox |
| MCP2515 EXTI | Node1/Node2 Message Received lines appear | External active-low INT and SPI drain path works |

# 17. Results / Expected Logs

Expected UART log pattern:

UART Initialized....!!
SPI Initialized.....!!
MCP2515 Node1 Normal mode OK
MCP2515 Node2 Normal mode OK
CAN1 Normal Mode init OK

/*********TEST : CAN message transmission and reception**********/
CAN1 TX requested
bxCAN Message Transmitted : ID = 0x301, DLC = 2, DATA1 = 0x31, DATA2 = 0x32
Node1 Message Transmitted : ID = 0x101, DLC = 0x02, DATA1 = 0x11, DATA2 = 0x12
Node2 Message Transmitted : ID = 0x201, DLC = 0x02, DATA1 = 0x21, DATA2 = 0x22
Node1 Message Received    : ID = ...
Node2 Message Received    : ID = ...
bxCAN Message Received    : ID = ..., DLC = ..., DATA1 = ..., DATA2 = ...

Result interpretation: In normal mode, the exact receive order can vary because CAN arbitration, TX request timing, interrupt timing, and main-loop service order all contribute. The important validation is that all expected nodes can transmit, acknowledge, interrupt, and drain received frames without FIFO or TX errors.

# 18. Problems Faced and Debugging Notes

| Symptom | Likely cause | Debug action |
| --- | --- | --- |
| CAN1 init failed | INAK did not set or clear | Check CAN1 clock, sleep bit, INRQ sequence, and timeout |
| No frame on CAN analyzer | Missing transceiver, wiring, termination, or normal-mode node ACK | Check PD0/PD1 to transceiver and CANH/CANL bus |
| CAN1 TX requested but no receive | Bus error, no ACK, filter/FIFO issue, or interrupt not enabled | Inspect CAN1->ESR, TSR, IER, RF0R, and analyzer decode |
| CAN1_RX0_IRQHandler repeats too often | FIFO not drained or interrupt not masked during service | Confirm FMPIE0 masking and RFOM0 release |
| Received frame repeats | FIFO0 output mailbox not released | Confirm CAN1->RF0R ¦= CAN_RF0R_RFOM0 executes |
| Node1/Node2 INT stuck low | MCP2515 CANINTF flag not cleared | Drain RXB0/RXB1 and clear RX0IF/RX1IF through bit modify |
| Wrong CAN ID | Incorrect standard ID packing/unpacking | Verify STID is shifted by 21 in bxCAN and SIDH/SIDL in MCP2515 |
| Only one node receives | Filter or physical wiring asymmetry | Check MCP2515 accept-all config, bus wiring, and common ground |

# 19. Embedded Engineering Review

| Finding | Impact | Recommended action |
| --- | --- | --- |
| Source comments mention PB8/PB9 but code configures PD0/PD1 | Can cause incorrect wiring if comments are followed | Use PD0/PD1 as source-grounded truth or update comments in code |
| Normal init says no RX pull-up but CAN1_GPIO_Init leaves PD0 pull-up enabled | Usually harmless with transceiver, but documentation should be precise | Clear PD0 PUPDR in normal mode if external transceiver biasing is preferred |
| ISR masks FMPIE0 and main loop re-enables it | Good deferred-processing pattern, but lost re-enable would stop RX interrupts | Keep re-enable close to FIFO drain completion and audit all early exits |
| No CAN ESR logging | Bus-off, error-passive, ACK error, stuff/form errors are invisible | Print CAN1->ESR on TX/RX failure |
| TX request success does not check TXOK/TERR/ALST | TXRQ set is not the same as successful bus transmission | Check TSR RQCPx/TXOKx/TERRx/ALSTx after transmit |
| Accept-all filter is broad | Good for learning; noisy for production | Replace with standard-ID filters after multi-node bring-up |
| MCP2515 SPI1_Test selects Node1 while SPI1 bus is used for Node2 | Potentially confusing during analyzer-based SPI validation | Use active node selection consistent with the bus under test |
| W25Q_FlashSectorErase drives CS low before WriteEnable | SPI flash helper is outside CAN scope but sequencing appears inconsistent | Review flash CS sequencing separately if flash behavior matters |
| spi.h has no include guard | Can cause duplicate declarations in larger builds | Add a conventional include guard later |

# 20. Limitations

The shared ChatGPT learning link was not available in the current message, so the document could not extract chat-specific wording.

No final screenshots were attached in this turn; the Results section documents expected UART/analyzer behavior instead of embedding images.

The code uses an accept-all filter, which is appropriate for bring-up but not selective.

The CAN1 ISR defers all parsing to the main loop; high bus load could overflow FIFO0 if main-loop delays are too long.

MAX7219 blinking and UART blocking prints add timing delays that are acceptable for learning but not for high-rate bus validation.

The document does not modify source files; it documents review findings separately.

# 21. Future Improvements

Add CAN1 error-state logging from ESR and transmit result logging from TSR.

Add FIFO0 full/overrun handling and counters for received, dropped, and overflowed frames.

Replace accept-all filtering with exact filters for IDs 0x101, 0x201, and 0x301.

Move UART logging out of time-critical sections or use a buffered/DMA UART path.

Split support modules behind compile-time switches so CAN-only tests can run without display/flash side effects.

Add a wiring checklist for transceiver VCC, ground, STB/EN pins, CANH/CANL polarity, and termination.

Add a logic-analyzer/CAN-analyzer evidence section when screenshots are attached.

# 22. Conclusion

Conclusion: The finalized project demonstrates STM32F407 CAN1 in normal mode with receive interrupt deferral while two MCP2515 controllers act as bus nodes. The key learning outcome is the receive path: filter bank 0, FIFO0 pending interrupt, ISR flagging, main-loop FIFO drain, and RFOM0 release.

# 23. Appendix: Source Reference

| File / source | Used for |
| --- | --- |
| D:\Embedded Communication Protocols\CAN1_RX_Interrupt\Core\Src\can.c | Finalized firmware source/header used for code grounding |
| D:\Embedded Communication Protocols\CAN1_RX_Interrupt\Core\Src\main.c | Finalized firmware source/header used for code grounding |
| D:\Embedded Communication Protocols\CAN1_RX_Interrupt\Core\Src\max7219.c | Finalized firmware source/header used for code grounding |
| D:\Embedded Communication Protocols\CAN1_RX_Interrupt\Core\Src\mcp2515.c | Finalized firmware source/header used for code grounding |
| D:\Embedded Communication Protocols\CAN1_RX_Interrupt\Core\Src\spi.c | Finalized firmware source/header used for code grounding |
| D:\Embedded Communication Protocols\CAN1_RX_Interrupt\Core\Src\stm32f4xx_it.c | Finalized firmware source/header used for code grounding |
| D:\Embedded Communication Protocols\CAN1_RX_Interrupt\Core\Src\usart.c | Finalized firmware source/header used for code grounding |
| D:\Embedded Communication Protocols\CAN1_RX_Interrupt\Core\Src\w25q64fv.c | Finalized firmware source/header used for code grounding |
| D:\Embedded Communication Protocols\CAN1_RX_Interrupt\Core\Inc\can.h | Finalized firmware source/header used for code grounding |
| D:\Embedded Communication Protocols\CAN1_RX_Interrupt\Core\Inc\common.h | Finalized firmware source/header used for code grounding |
| D:\Embedded Communication Protocols\CAN1_RX_Interrupt\Core\Inc\main.h | Finalized firmware source/header used for code grounding |
| D:\Embedded Communication Protocols\CAN1_RX_Interrupt\Core\Inc\max7219.h | Finalized firmware source/header used for code grounding |
| D:\Embedded Communication Protocols\CAN1_RX_Interrupt\Core\Inc\mcp2515.h | Finalized firmware source/header used for code grounding |
| D:\Embedded Communication Protocols\CAN1_RX_Interrupt\Core\Inc\spi.h | Finalized firmware source/header used for code grounding |
| D:\Embedded Communication Protocols\CAN1_RX_Interrupt\Core\Inc\stm32f4xx_it.h | Finalized firmware source/header used for code grounding |
| D:\Embedded Communication Protocols\CAN1_RX_Interrupt\Core\Inc\usart.h | Finalized firmware source/header used for code grounding |
| D:\Embedded Communication Protocols\CAN1_RX_Interrupt\Core\Inc\w25q64fv.h | Finalized firmware source/header used for code grounding |
| Shared ChatGPT URL was not present in this message; content is grounded in finalized source files plus CAN/bxCAN fundamentals. | Theory-source availability note |
| https://www.st.com/resource/en/datasheet/stm32f407vg.pdf | Official STM32F407 datasheet reference |
| https://www.st.com/resource/en/reference_manual/dm00031020.pdf | Official STM32F4 reference manual reference for bxCAN |

# 24. Appendix: Revision Checklist

| Check | Status |
| --- | --- |
| All user-named source/header files read | Done |
| Shared ChatGPT URL checked in current message | Unavailable - no URL visible |
| CAN1 normal mode explained | Done |
| RX interrupt flow explained | Done |
| System-level diagrams included | Done |
| MCP2515 Node1/Node2 interaction included | Done |
| PD0/PD1 correction documented | Done |
| BTR timing math verified | Done |
| Filter bank 0 / FIFO0 / RFOM0 explained | Done |
| Engineering review findings included | Done |
| No invented screenshots | Done |
| Footer name set to Embedded Learning Document | Done |