MCP2515 Two-Node CAN Project

Loopback Foundation, Normal-Mode Bus Communication, ACK Behavior, and STM32 SPI Multi-Node Driver

Document intent: Embedded learning reference built from the finalized two-node MCP2515 firmware, shared learning chat, and Microchip data sheet. It focuses on node routing, SPI mapping, Normal mode, ACK, buffers, and bus validation.

| Field | Value |
| --- | --- |
| Project scope | MCP2515 two-node loopback foundation and two-node Normal mode over CAN bus |
| Controller | One STM32F407-style MCU controlling two MCP2515 devices |
| Node 1 | MCP2515 on SPI2, CS PB12 |
| Node 2 | MCP2515 on SPI1, CS PA4 |
| Display/debug side module | MAX7219 on SPI1, CS PA15 |
| CAN test frames | Node1 TX ID 0x101; Node2 TX ID 0x201; payload 0x11 0x22 |
| Authoritative files | main.c, mcp2515.c, spi.c, usart.c, max7219.c and their headers |

# 1. Document Control / Metadata

| Item | Details |
| --- | --- |
| Version | v1.0 |
| Shared learning source | https://chatgpt.com/share/6a343a5d-ad38-83ee-bfe1-0123d30a3356 |
| Primary device source | Microchip MCP2515 Family Data Sheet DS20001801K |
| Code fidelity rule | Code snippets are taken only from the ten finalized files named by the user |
| Design preset | compact_reference_guide |
| Evidence boundary | No separate screenshot file was attached in this turn; the shared chat records terminal/analyzer observations |

# 2. Learning Objectives

Understand why single-node loopback is a controlled bring-up step and why two-node Normal mode is the real CAN-bus validation step.

Map the actual STM32 system: SPI2 controls Node 1, SPI1 controls Node 2 and MAX7219, and USART2 reports status.

Explain how MCP2515 normal mode differs from loopback mode, especially CANH/CANL, ACK, transceivers, and termination.

Trace both directions: Node1 TX ID 0x101 to Node2 RX, then Node2 TX ID 0x201 to Node1 RX.

Decode TXB0SIDH/TXB0SIDL, TXB0DLC, TXREQ, TXERR, MLOA, ABTF, RX0IF, RXB0SIDH/RXB0SIDL, and RXB0D0.

Understand the analyzer's decoded CAN frame versus actual differential CANH/CANL voltages.

Identify system-level risks and cleanup items before moving toward filters, interrupts, and production-style driver structure.

# 3. Project Objective

The project validates MCP2515 two-node CAN communication using polling SPI. Both MCP2515 controllers use 8 MHz / 500 kbps timing, enter Normal mode, and exchange standard frames in both directions.

# 4. Problem Statement

Core engineering problem: Loopback proves the internal CAN engine. Normal mode adds physical bus requirements: matching bit timing, transceivers, CANH/CANL polarity, common ground, termination, and receiver ACK.

# 5. System Overview

UART2 debug
                              |
                              v
                       +--------------+
                       | STM32F407 MCU |
                       +--------------+
                         |          |
             SPI2 PB13/14/15      SPI1 PA5/6/7
                 CS PB12              |------------------+
                 |                    |                  |
                 v                    v                  v
           +------------+       +------------+       +----------+
           | MCP2515 #1 |       | MCP2515 #2 |       | MAX7219  |
           |   Node 1   |       |   Node 2   |       |  Matrix  |
           +------------+       +------------+       +----------+
                 | TXCAN/RXCAN        | TXCAN/RXCAN
                 v                    v
           +------------+       +------------+
           | CAN XCVR 1 |-------| CAN XCVR 2 |
           +------------+       +------------+
                 CANH/CANL bus with proper termination

The two MCP2515 devices are independent CAN controllers. The STM32 selects the active logical node and active SPI peripheral before every MCP2515 register transaction. This is why node selection and SPI-bus selection are first-class concepts in the driver.

# 6. Requirements

| Requirement | Project value | Verification point |
| --- | --- | --- |
| MCP2515 oscillator | 8 MHz assumption from the learning path | CNF values are valid only for this oscillator |
| CAN bit rate | 500 kbps | CNF1=0x00, CNF2=0x91, CNF3=0x01 |
| Both modes covered | Loopback theory plus Normal mode final code | Loopback is internal; Normal mode uses bus and ACK |
| Node 1 route | SPI2 + PB12 CS | MCP2515_NODE_1 and MCP2515_SPI_BUS_2 |
| Node 2 route | SPI1 + PA4 CS | MCP2515_NODE_2 and MCP2515_SPI_BUS_1 |
| CAN physical layer | Two transceivers, CANH/CANL, common GND, termination | Required for Normal mode only |
| Receive policy | RXB0/RXB1 receive all valid messages | RXB0CTRL/RXB1CTRL RXM[1:0] set to 11 |

# 7. Hardware Components

| Component | Role | Important caution |
| --- | --- | --- |
| STM32F407 board | Controls both MCP2515 chips and UART output | One MCU drives both logical CAN nodes in this lab setup |
| MCP2515 Node 1 | CAN controller connected through SPI2 | CS is PB12 |
| MCP2515 Node 2 | CAN controller connected through SPI1 | CS is PA4 |
| CAN transceivers | Convert TXCAN/RXCAN to differential CANH/CANL | Normal mode will not work without active transceivers |
| CAN bus termination | 120 ohm at each physical end | Short two-node tests often need both module terminators enabled |
| MAX7219 | Visual side module on SPI1 | Shares SPI1 with Node 2; all CS lines must remain mutually exclusive |
| Logic analyzer / scope | Protocol and physical-layer validation | Analyzer decodes frames; oscilloscope is better for CANH/CANL voltages |

# 8. Software and Firmware Tools

STM32CubeIDE or equivalent build/debug environment.

UART terminal connected to USART2 for initialization, TX, RX, and error messages.

CAN-capable logic analyzer for ID, DLC, data, CRC, ACK, and bit-rate decoding.

Oscilloscope or differential probe when validating CANH/CANL physical levels.

Microchip MCP2515 data sheet for mode, buffer, SPI instruction, and error flag verification.

# 9. System Block Diagrams

## 9.1 Hardware Wiring Diagram

STM32 side                                CAN side

Node 1:
PB12 ---- CS -----------------------> MCP2515 #1 CS
PB13 ---- SPI2 SCK -----------------> MCP2515 #1 SCK
PB15 ---- SPI2 MOSI ----------------> MCP2515 #1 SI
PB14 <--- SPI2 MISO ---------------- MCP2515 #1 SO
                                      MCP2515 #1 TXCAN/RXCAN <-> CAN transceiver #1

Node 2:
PA4  ---- CS -----------------------> MCP2515 #2 CS
PA5  ---- SPI1 SCK -----------------> MCP2515 #2 SCK
PA7  ---- SPI1 MOSI ----------------> MCP2515 #2 SI
PA6  <--- SPI1 MISO ---------------- MCP2515 #2 SO
                                      MCP2515 #2 TXCAN/RXCAN <-> CAN transceiver #2

CAN bus:
Transceiver #1 CANH ---------------- Transceiver #2 CANH
Transceiver #1 CANL ---------------- Transceiver #2 CANL
Common GND ------------------------- Common GND
120 ohm termination at both bus ends for a two-node bench setup

## 9.2 Firmware Routing Diagram

Application code
    |
    +-- MCP2515_SelectNode(NODE_1 or NODE_2)
    +-- MCP2515_SelectSpiBus(SPI_BUS_2 or SPI_BUS_1)
            |
            +-- MCP2515_SPI_TransferByte()
                    |
                    +-- SPI1_TransferByte() when active bus is SPI1
                    +-- SPI2_TransferByte() when active bus is SPI2
            |
            +-- MCP2515_CS_Low()
                    |
                    +-- PB12 low for Node 1
                    +-- PA4 low for Node 2

## 9.3 Normal-Mode CAN Frame Flow

Node1 -> Node2:
STM32 selects Node1/SPI2 -> load TXB0 with ID 0x101 and data 11 22
    -> set TXREQ
    -> MCP2515 #1 waits for idle bus and transmits through transceiver #1
    -> CANH/CANL carry dominant/recessive bit stream
    -> transceiver #2 drives RXCAN into MCP2515 #2
    -> MCP2515 #2 validates CRC and drives ACK dominant
    -> MCP2515 #2 stores frame in RXB0 and sets RX0IF
    -> STM32 selects Node2/SPI1 and reads RXB0

Node2 -> Node1:
Same sequence in reverse, using ID 0x201.

# 10. Firmware Architecture

| Module | Responsibility | Important APIs / constants |
| --- | --- | --- |
| main.c | Initializes peripherals and exercises bidirectional CAN traffic | MCP2515_SelectNode, MCP2515_SelectSpiBus, SendMessage, ReadMessage |
| mcp2515.c | Active node/bus selection, SPI command layer, modes, TXB0, RXB0, TX status | active_mcp_node, active_mcp_spi_bus, MCP2515_PrintTxB0Status |
| spi.c | SPI1/SPI2 setup and polling byte transfers | SPI1_Init, SPI2_Init, SPI1_TransferByte, SPI2_TransferByte |
| usart.c | USART2 polling debug output | USART2_Init, USART2_SendString |
| max7219.c | LED matrix side display on SPI1 | MAX7219_Write, MAX7219_CharBlinking |
| mcp2515.h / spi.h | Modes, commands, registers, masks, node and bus enums, CS macros | MCP2515_MODE_NORMAL, NODE_1, NODE_2, SPI_BUS_1, SPI_BUS_2 |

# 11. Communication Protocol Used

The STM32 communicates with each MCP2515 over SPI using command-byte transactions. The MCP2515 then communicates on the CAN bus through its TXCAN/RXCAN pins and an external CAN transceiver. This project therefore has two communication layers: local SPI register access and external CAN frame transfer.

| Layer | Signals | Purpose | Validated by |
| --- | --- | --- | --- |
| SPI1 | PA5 SCK, PA6 MISO, PA7 MOSI, PA4 CS | Access Node 2 MCP2515 registers | SPI1_Test and Node2 normal-mode traffic |
| SPI2 | PB13 SCK, PB14 MISO, PB15 MOSI, PB12 CS | Access Node 1 MCP2515 registers | SPI2_Test and Node1 normal-mode traffic |
| CAN logic | TXCAN/RXCAN | MCP2515 to transceiver logic interface | Successful TX/RX and ACK |
| CAN physical | CANH/CANL | Differential two-wire bus | Analyzer/scope and two-node communication |
| UART | USART2 PA2/PA3 | Human-readable debug output | Terminal log |

# 12. Core Concept Deep Dive

## 12.1 Loopback vs Normal Mode

| Topic | Loopback mode | Normal mode |
| --- | --- | --- |
| Data path | TX buffer internally returns to RX buffer | TX buffer drives TXCAN/transceiver/CANH-CANL and another node receives |
| Bus dependency | No external CAN bus required | Requires transceiver, wiring, termination, matching bit timing, and active receiver |
| ACK | ACK bit is ignored; no external ACK needed | Receiver drives dominant ACK slot after valid CRC |
| TXCAN | Recessive/silent | Actively transmits frames |
| Best use | Bring-up and internal frame-path proof | Real two-node communication proof |
| Failure meaning | Likely SPI, register, or internal driver problem | Could be SPI, bit timing, transceiver, wiring, termination, ACK, or filters |

Datasheet verification: The MCP2515 data sheet describes Loopback as an internal TX-to-RX test mode with TXCAN recessive, and Normal mode as the only mode where the MCP2515 transmits messages over the CAN bus.

## 12.2 ACK Slot Behavior

The ACK shown by the CAN analyzer is part of the transmitted frame, but it is contributed by a receiver. The transmitting node sends recessive during the ACK slot. Any node that correctly receives the frame and validates the CRC drives the bus dominant during that slot. The transmitter only knows that at least one node acknowledged; it does not know exactly which node did it.

CAN frame field order:
SOF -> Arbitration ID -> Control/DLC -> Data -> CRC -> ACK slot -> ACK delimiter -> EOF

ACK slot:
Transmitter: releases/sends recessive
Receiver: drives dominant if frame is valid
Bus result: dominant wins, transmitter sees ACK

## 12.3 Analyzer Decode vs CANH/CANL Voltage

| View | What it shows | What it cannot fully show |
| --- | --- | --- |
| CAN protocol decoder | ID, DLC, payload, CRC, ACK, bit timing, frame boundaries | Exact analog CANH/CANL voltage margins |
| Digital RXD/CAN decode line | Dominant/recessive logic after transceiver thresholding | Differential voltage and common-mode behavior |
| Oscilloscope on CANH/CANL | CANH rises, CANL falls, differential voltage during dominant bits | Decoded register state inside MCP2515 |

Typical CAN transceivers hold both CANH and CANL near 2.5 V in recessive state. During dominant bits, CANH rises and CANL falls, producing a positive differential voltage. A protocol analyzer may display dominant as logic 0 and recessive as logic 1; that is not the same thing as directly viewing both analog bus wires.

## 12.4 Bit Timing Used by Both Nodes

| Parameter | Value | Reasoning |
| --- | --- | --- |
| FOSC | 8 MHz | Module oscillator assumed/verified in the learning path |
| Target bit rate | 500 kbps | 1 bit = 2 us |
| BRP | 0 | TQ = 2 x (BRP+1) / FOSC = 250 ns |
| Total TQ | 8 | 8 x 250 ns = 2 us |
| Segments | Sync=1, Prop=2, PS1=3, PS2=2 | Total 8 TQ, sample point 75 percent |
| Registers | CNF1=0x00, CNF2=0x91, CNF3=0x01 | Written in MCP2515_Init() while in Configuration mode |

# 13. Module-Wise Implementation

| Phase | Final-source behavior | Engineering purpose |
| --- | --- | --- |
| GPIO/SPI init | SPI2 and SPI1 are configured as Mode 0, 8-bit, MSB-first | Local register access to Node 1 and Node 2 |
| Node selection | active_mcp_node chooses PB12 or PA4 CS | One shared driver can address both MCP2515 chips |
| SPI selection | active_mcp_spi_bus chooses SPI1 or SPI2 transfer | One command layer can run on two SPI peripherals |
| MCP init | Reset, wait Config, write CNF, accept all, enable rollover, clear CANINTF | Common setup for both nodes |
| Normal mode | Both nodes request MCP2515_MODE_NORMAL and poll CANSTAT | Enables real CAN bus TX/RX |
| Node1 TX / Node2 RX | ID 0x101, DLC 2, data 0x11 0x22 | Proves Node1-to-Node2 path and ACK |
| Node2 TX / Node1 RX | ID 0x201, same payload | Proves reverse bus path |
| Status/debug | TXB0CTRL checked; RX0IF checked; USART messages printed | Visible lab feedback |

# 14. Main Algorithm and Flowchart

Power-on
  |
  +-- HAL_Init and SystemClock_Config
  +-- USART2_Init
  +-- SPI2_GPIO_Init / SPI2_Init
  +-- SPI1_GPIO_Init / SPI1_Init
  +-- SPI2_Test for Node1 path
  +-- SPI1_Test for Node2/SPI1 path review
  +-- Select Node1 + SPI2 -> MCP2515_Init -> Normal mode -> verify CANSTAT
  +-- Select Node2 + SPI1 -> MCP2515_Init -> Normal mode -> verify CANSTAT
  +-- MAX7219 test/init/clear
  |
  +-- Forever:
        Node1 + SPI2 -> send ID 0x101 -> check TXB0 status
        Node2 + SPI1 -> read RXB0 -> print ID/DLC/data
        Blink MAX7219 A
        Node2 + SPI1 -> send ID 0x201 -> check TXB0 status
        Node1 + SPI2 -> read RXB0 -> print ID/DLC/data
        Blink MAX7219 A

# 15. Code Explanation

Code authority: All code blocks in this section are copied from the finalized files attached in this request. Corrections and recommendations are documented separately and do not silently replace the source.

## 15.1 Application Test Variables

uint16_t id = 0x123;
	uint8_t dlc_len = 0x2;
	uint8_t msg[] = {0x11, 0x22};

	uint16_t rx_id = 0;
	uint8_t rx_dlc_len = 0;
	uint8_t rx_msg[8] = {0,0,0,0,0,0,0,0};

	char uart_msg_tx[100];
	char uart_msg_rx[100];

## 15.2 Node and SPI Bus Selection

static uint8_t MCP2515_SPI_TransferByte(uint8_t data)
{
    if(active_mcp_spi_bus == MCP2515_SPI_BUS_1)
    {
        return SPI1_TransferByte(data);
    }
    else
    {
        return SPI2_TransferByte(data);
    }
}

static void MCP2515_CS_Low(void)
{
	if(active_mcp_node == MCP2515_NODE_1)
	{
		MCP2515_CS_LOW();
	}
	else
	{
		MCP2515_NODE2_CS_LOW();
	}
}

static void MCP2515_CS_High(void)
{
	if(active_mcp_node == MCP2515_NODE_1)
	{
		MCP2515_CS_HIGH();
	}
	else
	{
		MCP2515_NODE2_CS_HIGH();
	}
}

void MCP2515_SelectSpiBus(MCP2515_SpiBus_t bus)
{
    active_mcp_spi_bus = bus;
}

void MCP2515_SelectNode(MCP2515_Node_t node)
{
	active_mcp_node = node;
}

This is the key architectural change from the earlier one-node projects. The same MCP2515 register functions operate on different hardware depending on the active node and active SPI bus.

## 15.3 MCP2515 Initialization and Mode Control

void MCP2515_Init(void)
{
	// Reset to reach config mode
	MCP2515_Reset();

	//check and wait till config mode
	while((MCP2515_ReadRegister(MCP2515_CANSTAT) & MCP2515_MODE_MASK) != MCP2515_MODE_CONFIG);

	/*Baudrate config , 500kbps ,8 N_TQ, SyncSeg = 1 TQ,
	PropSeg = 2 TQ ,PS1     = 3 TQ ,PS2     = 2 TQ
	Total   = 8 TQ*/
	MCP2515_WriteRegister(MCP2515_CNF1, 0x00);
	MCP2515_WriteRegister(MCP2515_CNF2, 0x91);
	MCP2515_WriteRegister(MCP2515_CNF3, 0x01);

	//Filter,mask settings, receive any message ID
	MCP2515_BitModify(MCP2515_RXB0CTRL, 0x60 , 0x60);
	MCP2515_BitModify(MCP2515_RXB1CTRL, 0x60 , 0x60);

	//enable RXB0 rollover to RXB1 */
	MCP2515_BitModify(MCP2515_RXB0CTRL, 0x04, 0x04);

	/* Clear interrupt flags */
	MCP2515_WriteRegister(MCP2515_CANINTF, 0x00);
}

void MCP2515_SetMode(uint8_t mode)
{
    MCP2515_BitModify(MCP2515_CANCTRL,
                      MCP2515_MODE_MASK,
                      mode);
}

uint8_t MCP2515_SetModeVerify(uint8_t mode)
{
    uint32_t timeout = 100000;

    MCP2515_SetMode(mode);

    while(timeout--)
    {
        if(MCP2515_GetMode() == mode)
        {
            return 1;   // success
        }
    }

    return 0;           // failed
}

## 15.4 SPI Command Layer

void MCP2515_Reset(void)
{
	MCP2515_CS_Low();
	MCP2515_SPI_TransferByte(MCP2515_CMD_RESET);
	MCP2515_CS_High();

	// Needed for oscillator stabilization
	HAL_Delay(100);
}

uint8_t MCP2515_ReadRegister(uint8_t reg)
{
	uint8_t reg_data;

	MCP2515_CS_Low();
	MCP2515_SPI_TransferByte(MCP2515_CMD_READ);
	MCP2515_SPI_TransferByte(reg);
	reg_data = MCP2515_SPI_TransferByte(0xFF);
	MCP2515_CS_High();

	return reg_data;
}

void MCP2515_WriteRegister(uint8_t address, uint8_t data)
{
	MCP2515_CS_Low();
	MCP2515_SPI_TransferByte(MCP2515_CMD_WRITE);
	MCP2515_SPI_TransferByte(address);
	MCP2515_SPI_TransferByte(data);
	MCP2515_CS_High();
}

void MCP2515_BitModify(uint8_t address,uint8_t mask,uint8_t data)
{
	MCP2515_CS_Low();
	MCP2515_SPI_TransferByte(MCP2515_CMD_BIT_MODIFY);
	MCP2515_SPI_TransferByte(address);
	MCP2515_SPI_TransferByte(mask);
	MCP2515_SPI_TransferByte(data);
	MCP2515_CS_High();
}

## 15.5 Frame Transmission

uint8_t MCP2515_SendMessage(uint16_t id, uint8_t *data, uint8_t len)
{
    uint8_t sidh;
    uint8_t sidl;

    /* Standard CAN ID is 11-bit */
    if(id > 0x7FF)
    {
        return 0;
    }

    /* CAN data length max is 8 bytes */
    if(len > 8)
    {
        return 0;
    }

    if((data == 0) && (len > 0))
    {
        return 0;
    }

    /* Check TXB0 is free: TXREQ must be 0 */
    if(MCP2515_ReadRegister(MCP2515_TXB0CTRL) & 0x08)
    {
        return 0;
    }

    /*
     * Standard ID mapping:
     * TXB0SIDH = SID10..SID3
     * TXB0SIDL[7:5] = SID2..SID0
     */
    sidh = (uint8_t)(id >> 3);
    sidl = (uint8_t)((id & 0x07) << 5);

    /* Load standard ID */
    MCP2515_WriteRegister(MCP2515_TXB0SIDH, sidh);
    MCP2515_WriteRegister(MCP2515_TXB0SIDL, sidl);

    /* Standard data frame, DLC = len */
    MCP2515_WriteRegister(MCP2515_TXB0DLC, len & 0x0F);

    /* Load data bytes */
    for(uint8_t i = 0; i < len; i++)
    {
        MCP2515_WriteRegister(MCP2515_TXB0D0 + i, data[i]);
    }

    /* Request transmission: set TXB0CTRL.TXREQ */
    MCP2515_BitModify(MCP2515_TXB0CTRL, 0x08, 0x08);

    return 1;
}

| Code step | Register behavior | Why it matters |
| --- | --- | --- |
| Check id <= 0x7FF | Standard 11-bit frame only | Prevents invalid standard ID packing |
| Check len <= 8 | Classic CAN payload limit | Prevents writing past TXB0D0..D7 |
| Read TXB0CTRL & 0x08 | TXREQ must be clear | Avoids overwriting pending TXB0 |
| SIDH=id>>3; SIDL=(id&7)<<5 | Packs SID10..SID0 | Matches MCP2515 standard ID layout |
| Write TXB0DLC and data bytes | Loads DLC and payload | Defines what appears on the CAN frame |
| Set TXREQ | Requests transmission | Transmission starts when bus is available |

## 15.6 Receive Path

uint8_t MCP2515_ReadMessage(uint16_t *id, uint8_t *data, uint8_t *len)
{
    uint8_t sidh;
    uint8_t sidl;
    uint8_t dlc;
    uint8_t canintf;

    /* Check if RXB0 has received message */
    canintf = MCP2515_ReadRegister(MCP2515_CANINTF);

    if((canintf & MCP2515_CANINTF_RX0IF_MASK) == 0)
    {
    	USART2_SendString("No message available\r\n");
        return 0;   /* No message available */
    }

    USART2_SendString("Message available\r\n");

    /* Read standard identifier registers */
    sidh = MCP2515_ReadRegister(MCP2515_RXB0SIDH);
    sidl = MCP2515_ReadRegister(MCP2515_RXB0SIDL);

    /*
     * Reconstruct 11-bit standard ID:
     * SIDH contains SID10..SID3
     * SIDL[7:5] contains SID2..SID0
     */
    *id = ((uint16_t)sidh << 3) | (sidl >> 5);

    /* Read DLC */
    dlc = MCP2515_ReadRegister(MCP2515_RXB0DLC);
    dlc = dlc & 0x0F;

    if(dlc > 8)
    {
        dlc = 8;
    }

    *len = dlc;

    /* Read data bytes */
    for(uint8_t i = 0; i < dlc; i++)
    {
        data[i] = MCP2515_ReadRegister(MCP2515_RXB0D0 + i);
    }

    /*
     * Clear RX0IF after reading message.
     * This tells MCP2515 RXB0 is free again.
     */
    MCP2515_BitModify(MCP2515_CANINTF,
                      MCP2515_CANINTF_RX0IF_MASK,
                      0x00);

    return 1;
}

The receive function currently checks only CANINTF.RX0IF and reads RXB0. Since MCP2515_Init() enables RXB0 rollover to RXB1, a later production driver should also check RX1IF and read RXB1 when rollover occurs.

## 15.7 TXB0 Status Check

uint8_t MCP2515_PrintTxB0Status(void)
{
	uint8_t TxErrorFlag = 1;
    uint8_t txctrl = MCP2515_ReadRegister(MCP2515_TXB0CTRL);


    if(txctrl & MCP2515_TXB0CTRL_TXREQ)
    {
        USART2_SendString("TXB0 still pending\r\n");
        TxErrorFlag = 0;
    }

    if(txctrl & MCP2515_TXB0CTRL_TXERR)
    {
        USART2_SendString("TXB0 transmit error\r\n");
        TxErrorFlag = 0;
    }

    if(txctrl & MCP2515_TXB0CTRL_MLOA)
    {
        USART2_SendString("TXB0 lost arbitration\r\n");
        TxErrorFlag = 0;
    }

    if(txctrl & MCP2515_TXB0CTRL_ABTF)
    {
        USART2_SendString("TXB0 aborted\r\n");
        TxErrorFlag = 0;
    }

    return TxErrorFlag;
}

## 15.8 Main Initialization Code

USART2_Init();

  USART2_SendString("UART Initialized....!!\r\n");

  SPI2_GPIO_Init();
  SPI2_Init();

  SPI1_GPIO_Init();
  SPI1_Init();
  USART2_SendString("SPI Initialized.....!!\r\n");

  MCP2515_SelectNode(MCP2515_NODE_1);
  MCP2515_SelectSpiBus(MCP2515_SPI_BUS_2);
  SPI2_Test();

  MCP2515_SelectNode(MCP2515_NODE_2);
  MCP2515_SelectSpiBus(MCP2515_SPI_BUS_1);
  SPI1_Test();

  // Initialize MCP node1 and move to normal mode
  MCP2515_SelectNode(MCP2515_NODE_1);
  MCP2515_SelectSpiBus(MCP2515_SPI_BUS_2);
  MCP2515_Init();

  //Enter normal mode
  MCP2515_SetMode(MCP2515_MODE_NORMAL);

  //check and wait till loopback mode
  while((MCP2515_ReadRegister(MCP2515_CANSTAT) & MCP2515_MODE_MASK) != MCP2515_MODE_NORMAL);

  uint8_t canstat = MCP2515_ReadRegister(MCP2515_CANSTAT);

  if((canstat & MCP2515_MODE_MASK) == MCP2515_MODE_NORMAL)
  {
      USART2_SendString("MCP2515 Node1 Normal mode OK\r\n");
  }
  else
  {
      USART2_SendString("MCP2515 Node1 Normal mode failed\r\n");
  }

  // Initialize MCP node2 and move to loopback mode
  MCP2515_SelectNode(MCP2515_NODE_2);
  MCP2515_SelectSpiBus(MCP2515_SPI_BUS_1);
  MCP2515_Init();

  //Enter Normal mode
  MCP2515_SetMode(MCP2515_MODE_NORMAL);

  //check and wait till loopback mode
  while((MCP2515_ReadRegister(MCP2515_CANSTAT) & MCP2515_MODE_MASK) != MCP2515_MODE_NORMAL);

  uint8_t canstat2 = MCP2515_ReadRegister(MCP2515_CANSTAT);

  if((canstat2 & MCP2515_MODE_MASK) == MCP2515_MODE_NORMAL)
  {
      USART2_SendString("MCP2515 Node2 Normal mode OK\r\n");
  }
  else
  {
      USART2_SendString("MCP2515 Node2 Normal mode failed\r\n");
  }

  // Simple hardware test
  MAX7219_Write(0x0F, 0x01);   // All LEDs ON
  HAL_Delay(2000);
  MAX7219_Write(0x0F, 0x00);   // Test OFF

  MAX7219_Init();
  MAX7219_Clear();

## 15.9 Main Bidirectional Loop

//Transmitting standard CAN frame
	  MCP2515_SelectNode(MCP2515_NODE_1);
	  MCP2515_SelectSpiBus(MCP2515_SPI_BUS_2);
	  if(MCP2515_SendMessage(0x101,msg,dlc_len))
	  {
		  HAL_Delay(10);

		  if(MCP2515_PrintTxB0Status())
		  {
			  USART2_SendString("Node1 Message Transmitted Successfully\r\n");
		  }
		  else
		  {
			  USART2_SendString("Tx Error\r\n");
		  }

		  sprintf(uart_msg_tx,"Node1 Message Transmitted : ID = 0x%03X, DLC = 0x%02X, DATA1 = 0x%02X, DATA2 = 0x%02X\r\n",0x101,dlc_len,msg[0],msg[1]);
		  USART2_SendString(uart_msg_tx);
	  }
	  else
	  {
		  USART2_SendString("Node1 : Message Tx Failed\r\n");
	  }

	 //Node 2 Receives
	  MCP2515_SelectNode(MCP2515_NODE_2);
	  MCP2515_SelectSpiBus(MCP2515_SPI_BUS_1);

	  //Receiving frame
	  if(MCP2515_ReadMessage(&rx_id,rx_msg,&rx_dlc_len))
	  {
		  sprintf(uart_msg_rx,"Node2 Message Received    : ID = 0x%03X, DLC = 0x%02X, DATA1 = 0x%02X, DATA2 = 0x%02X\r\n",rx_id,rx_dlc_len,rx_msg[0],rx_msg[1]);
		  USART2_SendString(uart_msg_rx);
	  }
	  else
	  {
		  USART2_SendString("Node2 : No Message Received\r\n");
	  }

	  MAX7219_CharBlinking('A');

	  //This time Node 2 Transmits and Node 1 Receives
	  MCP2515_SelectNode(MCP2515_NODE_2);
	  MCP2515_SelectSpiBus(MCP2515_SPI_BUS_1);

	  if(MCP2515_SendMessage(0x201,msg,dlc_len))
	  {
		  HAL_Delay(10);

		  if(MCP2515_PrintTxB0Status())
		  {
			  USART2_SendString("Node2 Message Transmitted Successfully\r\n");
		  }
		  else
		  {
			  USART2_SendString("Tx Error\r\n");
		  }
		  sprintf(uart_msg_tx,"Node2 Message Transmitted : ID = 0x%03X, DLC = 0x%02X, DATA1 = 0x%02X, DATA2 = 0x%02X\r\n",0x201,dlc_len,msg[0],msg[1]);
		  USART2_SendString(uart_msg_tx);
	  }
	  else
	  {
		  USART2_SendString("Node1 : Message Tx Failed\r\n");
	  }

	  MCP2515_SelectNode(MCP2515_NODE_1);
	  MCP2515_SelectSpiBus(MCP2515_SPI_BUS_2);

	  //Receiving frame
	  if(MCP2515_ReadMessage(&rx_id,rx_msg,&rx_dlc_len))
	  {
		  sprintf(uart_msg_rx,"Node2 Message Received    : ID = 0x%03X, DLC = 0x%02X, DATA1 = 0x%02X, DATA2 = 0x%02X\r\n",rx_id,rx_dlc_len,rx_msg[0],rx_msg[1]);
		  USART2_SendString(uart_msg_rx);
	  }
	  else
	  {
		  USART2_SendString("Node2 : No Message Received\r\n");
	  }

	  MAX7219_CharBlinking('A');

	  //MAX7219_StringBlinking("Hello Via Matrix");

	  //MAX7219_ScrollText_RtoL("HELLO", 100);

## 15.10 SPI1 and SPI2 Configuration

void SPI1_GPIO_Init(void)
{
    /* Enable GPIOB clock */
    RCC->AHB1ENR |= RCC_AHB1ENR_GPIOAEN;

    /*
     * PA15 = CS_MAX7219
     * PA4 = CS_MCP2515_NODE1
     * PA5 = SPI1_SCK AF
     * PA6 = SPI1_MISO AF
     * PA7 = SPI1_MOSI AF
     */

    /* Clear mode bits */
    GPIOA->MODER &= ~((3UL << (15 * 2))|
    				  (3UL << (4 * 2)) |
                      (3UL << (5 * 2)) |
                      (3UL << (6 * 2)) |
                      (3UL << (7 * 2)));

    /* Set modes */
    GPIOA->MODER |=  ((1UL << (15 * 2)) |
    				  (1UL << (4 * 2))  |   /* PA4 output */
                      (2UL << (5 * 2)) |   /* PA5 AF */
                      (2UL << (6 * 2)) |   /* PA6 AF */
                      (2UL << (7 * 2)));   /* PA7 AF */

    /* Push-pull */
    GPIOA->OTYPER &= ~((1UL << 15)  |
    				   (1UL << 4)   |
                       (1UL << 5)   |
                       (1UL << 6)   |
                       (1UL << 7));

    /* Speed: medium/high for SPI */
    GPIOA->OSPEEDR &= ~((3UL << (15 * 2)) |
    					(3UL << (4 * 2)) |
                        (3UL << (5 * 2)) |
                        (3UL << (6 * 2)) |
                        (3UL << (7 * 2)));

    GPIOA->OSPEEDR |=  ((2UL << (15 * 2)) |
    					(2UL << (4 * 2)) |
                        (2UL << (5 * 2)) |
                        (2UL << (6 * 2)) |
                        (2UL << (7 * 2)));

    /* Pull config: CS pull-up, others no pull */
    GPIOA->PUPDR &= ~((3UL << (15 * 2)) |
    				  (3UL << (4 * 2)) |
                      (3UL << (5 * 2)) |
                      (3UL << (6 * 2)) |
                      (3UL << (7 * 2)));

    GPIOA->PUPDR |=  (1UL << (4 * 2));     /* PA4 pull-up */
    GPIOA->PUPDR |=  (1UL << (15 * 2));    /* PA15 pull-up */

    /* AF5 for PA5, PA6, PA7 */
    GPIOA->AFR[0] &= ~(GPIO_AFRL_AFSEL5 |
                       GPIO_AFRL_AFSEL6 |
                       GPIO_AFRL_AFSEL7);

    GPIOA->AFR[0] |=  (5UL << GPIO_AFRL_AFSEL5_Pos) |
                       (5UL << GPIO_AFRL_AFSEL6_Pos) |
                       (5UL << GPIO_AFRL_AFSEL7_Pos);

    /* Keep CS high initially */
    MCP2515_NODE2_CS_HIGH();
    MAX7219_CS_HIGH();

}

void SPI1_Init(void)
{
    /* Enable SPI2 clock - APB2 peripheral clock*/
    RCC->APB2ENR |= RCC_APB2ENR_SPI1EN;

    /* Disable SPI before configuration */
    SPI1->CR1 &= ~SPI_CR1_SPE;

    /* Clear CR1 configuration bits we care about */
    SPI1->CR1 = 0;

    /*
     * Master mode
     * Software slave management
     * Internal slave select high
     */
    SPI1->CR1 |= SPI_CR1_MSTR;
    SPI1->CR1 |= SPI_CR1_SSM;
    SPI1->CR1 |= SPI_CR1_SSI;

    /*
     * Mode 0: CPOL = 0, CPHA = 0
     */
    SPI1->CR1 &= ~SPI_CR1_CPOL;
    SPI1->CR1 &= ~SPI_CR1_CPHA;

    /*
     * 8-bit frame, MSB first
     */
    SPI1->CR1 &= ~SPI_CR1_DFF;
    SPI1->CR1 &= ~SPI_CR1_LSBFIRST;

    /*
     * Baud rate = PCLK / 32
     * If PCLK1 = 32 MHz, SCK = 1 MHz
     */
    SPI1->CR1 &= ~SPI_CR1_BR;
    SPI1->CR1 |= SPI_CR1_BR_2;

    /*
     * Full duplex 2-line
     */
    SPI1->CR1 &= ~SPI_CR1_BIDIMODE;
    SPI1->CR1 &= ~SPI_CR1_RXONLY;

    /* Enable SPI */
    SPI1->CR1 |= SPI_CR1_SPE;
}

void SPI2_GPIO_Init(void)
{
    /* Enable GPIOB clock */
    RCC->AHB1ENR |= RCC_AHB1ENR_GPIOBEN;

    /*
     * PB12 = CS_MCP2515_NODE1
     * PB13 = SPI2_SCK AF
     * PB14 = SPI2_MISO AF
     * PB15 = SPI2_MOSI AF
     */

    /* Clear mode bits */
    GPIOB->MODER &= ~((3UL << (12 * 2)) |
                      (3UL << (13 * 2)) |
                      (3UL << (14 * 2)) |
                      (3UL << (15 * 2)));

    /* Set modes */
    GPIOB->MODER |=  ((1UL << (12 * 2)) |   /* PB12 output */
                      (2UL << (13 * 2)) |   /* PB13 AF */
                      (2UL << (14 * 2)) |   /* PB14 AF */
                      (2UL << (15 * 2)));   /* PB15 AF */

    /* Push-pull */
    GPIOB->OTYPER &= ~((1UL << 12) |
                       (1UL << 13) |
                       (1UL << 14) |
                       (1UL << 15));

    /* Speed: medium/high for SPI */
    GPIOB->OSPEEDR &= ~((3UL << (12 * 2)) |
                        (3UL << (13 * 2)) |
                        (3UL << (14 * 2)) |
                        (3UL << (15 * 2)));

    GPIOB->OSPEEDR |=  ((2UL << (12 * 2)) |
                        (2UL << (13 * 2)) |
                        (2UL << (14 * 2)) |
                        (2UL << (15 * 2)));

    /* Pull config: CS pull-up, others no pull */
    GPIOB->PUPDR &= ~((3UL << (12 * 2)) |
                      (3UL << (13 * 2)) |
                      (3UL << (14 * 2)) |
                      (3UL << (15 * 2)));

    GPIOB->PUPDR |=  (1UL << (12 * 2));     /* PB12 pull-up */

    /* AF5 for PB13, PB14, PB15 */
    GPIOB->AFR[1] &= ~((0xFUL << ((13 - 8) * 4)) |
                       (0xFUL << ((14 - 8) * 4)) |
                       (0xFUL << ((15 - 8) * 4)));

    GPIOB->AFR[1] |=  ((5UL << ((13 - 8) * 4)) |
                       (5UL << ((14 - 8) * 4)) |
                       (5UL << ((15 - 8) * 4)));

    /* Keep CS high initially */
    MCP2515_CS_HIGH();

}

void SPI2_Init(void)
{
    /* Enable SPI2 clock */
    RCC->APB1ENR |= RCC_APB1ENR_SPI2EN;

    /* Disable SPI before configuration */
    SPI2->CR1 &= ~SPI_CR1_SPE;

    /* Clear CR1 configuration bits we care about */
    SPI2->CR1 = 0;

    /*
     * Master mode
     * Software slave management
     * Internal slave select high
     */
    SPI2->CR1 |= SPI_CR1_MSTR;
    SPI2->CR1 |= SPI_CR1_SSM;
    SPI2->CR1 |= SPI_CR1_SSI;

    /*
     * Mode 0: CPOL = 0, CPHA = 0
     */
    SPI2->CR1 &= ~SPI_CR1_CPOL;
    SPI2->CR1 &= ~SPI_CR1_CPHA;

    /*
     * 8-bit frame, MSB first
     */
    SPI2->CR1 &= ~SPI_CR1_DFF;
    SPI2->CR1 &= ~SPI_CR1_LSBFIRST;

    /*
     * Baud rate = PCLK / 32
     * If PCLK1 = 32 MHz, SCK = 1 MHz
     */
    SPI2->CR1 &= ~SPI_CR1_BR;
    SPI2->CR1 |= SPI_CR1_BR_2;

    /*
     * Full duplex 2-line
     */
    SPI2->CR1 &= ~SPI_CR1_BIDIMODE;
    SPI2->CR1 &= ~SPI_CR1_RXONLY;

    /* Enable SPI */
    SPI2->CR1 |= SPI_CR1_SPE;
}

uint8_t SPI1_TransferByte(uint8_t data)
{
    uint8_t received;

    while(!(SPI1->SR & SPI_SR_TXE));

    *((volatile uint8_t *)&SPI1->DR) = data;

    while(!(SPI1->SR & SPI_SR_RXNE));

    received = *((volatile uint8_t *)&SPI1->DR);

    while(SPI1->SR & SPI_SR_BSY);

    return received;
}

uint8_t SPI2_TransferByte(uint8_t byte)
{
    uint8_t received;

    //waiting for TX buffer empty
    while(!(SPI2->SR & SPI_SR_TXE));
    *((volatile uint8_t *)&SPI2->DR) = byte;

    //waiting for RX buffer Not empty
    while(!(SPI2->SR & SPI_SR_RXNE));
    received = *((volatile uint8_t *)&SPI2->DR);

    //Wait till SPI completes
    while(SPI2->SR & SPI_SR_BSY);

    return received;
}

## 15.11 USART and MAX7219 Roles

void USART2_Init(void)
{
    /* Enable clocks */
    RCC->AHB1ENR |= RCC_AHB1ENR_GPIOAEN;
    RCC->APB1ENR |= RCC_APB1ENR_USART2EN;

    /* Configure PA2(TX) and PA3(RX) as Alternate Function */
    GPIOA->MODER &= ~(0x3UL << GPIO_MODER_MODER2_Pos);
    GPIOA->MODER &= ~(0x3UL << GPIO_MODER_MODER3_Pos);

    GPIOA->MODER |=  (0x2UL << GPIO_MODER_MODER2_Pos);
    GPIOA->MODER |=  (0x2UL << GPIO_MODER_MODER3_Pos);

    /* Select AF7 for USART2 */
    GPIOA->AFR[0] &= ~(0xFUL << GPIO_AFRL_AFSEL2_Pos);
    GPIOA->AFR[0] &= ~(0xFUL << GPIO_AFRL_AFSEL3_Pos);

    GPIOA->AFR[0] |=  (0x7UL << GPIO_AFRL_AFSEL2_Pos);
    GPIOA->AFR[0] |=  (0x7UL << GPIO_AFRL_AFSEL3_Pos);

    /* Configure baud rate */
    USART2->BRR = 278;

    /* Configure frame format: 8N1 */
    USART2->CR1 &= ~USART_CR1_M;
    USART2->CR1 &= ~USART_CR1_PCE;

    USART2->CR2 &= ~USART_CR2_STOP;

    /* Enable transmitter and receiver */
    USART2->CR1 |= USART_CR1_TE;
    USART2->CR1 |= USART_CR1_RE;

    /* Enable USART */
    USART2->CR1 |= USART_CR1_UE;

    /* Enable RX interrupt, and USART2 interrupt in NVIC*/
    //NVIC_EnableIRQ(USART2_IRQn);
}

void USART2_SendString(char *s)
{
	while(*s != '\0')
	{
		while(!(USART2->SR & USART_SR_TXE));
		USART2->DR = *s++;
	}

	//checking for the last character
	while(!(USART2->SR & USART_SR_TC));
}

void MAX7219_Write(uint8_t address, uint8_t data)
{
	MAX7219_CS_LOW();

	SPI1_TransferByte(address);
	SPI1_TransferByte(data);

	MAX7219_CS_HIGH();

}

void MAX7219_CharBlinking(char ch)
{
	MAX7219_DisplayChar(ch);
	HAL_Delay(500);
	MAX7219_Clear();
	HAL_Delay(500);

}

# 16. Testing and Validation

| Test | Expected observation | Meaning |
| --- | --- | --- |
| Node1 register access | SPI2 read/write works with PB12 CS | Node1 command layer valid |
| Node2 register access | SPI1 read/write works with PA4 CS | Node2 command layer valid |
| Node1 Normal mode | CANSTAT & 0xE0 == 0x00 | Node1 can transmit on CAN bus |
| Node2 Normal mode | CANSTAT & 0xE0 == 0x00 | Node2 can transmit on CAN bus |
| Node1 TX status | TXREQ clears, TXERR/MLOA/ABTF clear | Frame was sent and acknowledged |
| Node2 RX | RX0IF set, received ID 0x101, data 11 22 | Node1-to-Node2 path works |
| Node2 TX status | TXREQ clears, TXERR/MLOA/ABTF clear | Reverse transmit works |
| Node1 RX | RX0IF set, received ID 0x201, data 11 22 | Node2-to-Node1 path works |
| Analyzer ACK | ACK slot dominant in decoded frame | At least one receiver acknowledged |

# 17. Results

Observed result from the learning chat: Shared learning data reports successful Normal-mode communication both ways: Node1 sent ID 0x101 to Node2, and Node2 sent ID 0x201 to Node1. Analyzer ACK confirms receiver acknowledgment.

Expected terminal pattern:

MCP2515 Node1 Normal mode OK
MCP2515 Node2 Normal mode OK
Node1 Message Transmitted Successfully
Node1 Message Transmitted : ID = 0x101, DLC = 0x02, DATA1 = 0x11, DATA2 = 0x22
Node2 Message Received    : ID = 0x101, DLC = 0x02, DATA1 = 0x11, DATA2 = 0x22
Node2 Message Transmitted Successfully
Node2 Message Transmitted : ID = 0x201, DLC = 0x02, DATA1 = 0x11, DATA2 = 0x22
Node1 Message Received    : ID = 0x201, DLC = 0x02, DATA1 = 0x11, DATA2 = 0x22

# 18. Problems Faced and Debugging Notes

| Symptom | Likely cause | Debug action |
| --- | --- | --- |
| Normal mode OK but no RX | Physical bus issue or no ACK | Check CANH/CANL polarity, common ground, termination, transceiver enable |
| TXREQ remains set | No bus availability, no ACK, wiring fault, bitrate mismatch | Read TXB0CTRL and EFLG; inspect analyzer ACK |
| TXERR set | Transmit error on CAN bus | Check bit timing and physical layer |
| MLOA set | Lost arbitration | Expected if another node transmits a lower ID simultaneously |
| ABTF set | Abort occurred | Check ABAT and TXREQ handling |
| RX0IF never set | Receiver did not accept/store frame | Check RXB0CTRL/RXB1CTRL, CANINTF, filters, and normal mode |
| Only occasional receive | RXB0 full or rollover to RXB1 | Add RX1IF/RXB1 read path and overflow checks |
| Analyzer shows ACK but UART says no RX | Frame may be acknowledged by another node or stored in RXB1 | Read CANINTF, EFLG, RXB1 registers |
| SPI1 test confusing | SPI1_Test() selects NODE_1 internally | Correct test helper to keep Node2 selected when testing SPI1 Node2 path |

# 19. Embedded Engineering Review

| Finding | Impact | Recommended action |
| --- | --- | --- |
| SPI1_Test() internally selects MCP2515_NODE_1 | It does not truly test the Node2/PA4 CS path even when caller selected Node2 | Change SPI1_Test() to select NODE_2 or avoid changing active node inside test helpers |
| RXB0 rollover enabled but RXB1 not read | Messages can be missed if RXB0 is full and a frame rolls to RXB1 | Check RX1IF and implement RXB1 readback |
| Mode waits are unbounded in MCP2515_Init() and main.c | Disconnected or failed node can hang firmware | Use MCP2515_SetModeVerify-style timeouts for config and normal mode |
| Print labels have Node1/Node2 copy-paste mistakes | Debug output can mislead during reverse-direction tests | Update Node2 TX failure and Node1 RX messages |
| No EFLG/error-counter reporting | Bus-off, warning, passive/active error states are invisible | Read EFLG, TEC, REC after TX/RX failures |
| MAX7219 blinking in CAN loop | Adds 1 second of delay per blink and uses SPI1 shared with Node2 | Keep for visual demo or move CAN test timing away from display delays |
| No interrupt-driven receive yet | Polling may miss timing in busier systems | Use MCP2515 INT pin and CANINTE.RX0IE/RX1IE later |
| No filter/mask learning yet | All IDs are accepted | Add ID 0x101/0x201 filters as the next learning step |
| spi.h header has no include guard | Can create duplicate declarations/macro issues in larger builds | Add conventional include guard |

# 20. Limitations

Only standard data frames are supported in the current send/read APIs.

Only TXB0 and RXB0 are actively used for frame transfer.

RXB1 rollover is configured but not consumed by the receive API.

All receive filters are effectively bypassed for bring-up.

CAN physical-layer validation is mostly external to the firmware and must be checked with wiring, termination, and instruments.

The current application is a polling lab/demo, not yet a non-blocking production driver.

# 21. Future Improvements

Fix SPI1_Test() so the SPI1 path validates Node2 with PA4 CS.

Replace all endless mode waits with timeout-return APIs.

Implement RXB1 read support and overflow/error diagnostics.

Add EFLG, TEC, and REC reporting after transmit or receive failures.

Add standard ID filters so Node2 accepts only ID 0x101 and Node1 accepts only ID 0x201.

Move receive handling from polling to the MCP2515 INT pin.

Separate MAX7219 demo timing from CAN transport testing.

Add extended ID and remote-frame support after standard data frames are stable.

# 22. Conclusion

Conclusion: The project demonstrates two MCP2515 controllers, controlled by one STM32 over two SPI peripherals, communicating over a real CAN bus. Loopback is the foundation; Normal mode proves the bus path, ACK, TX status, and RX readback.

# 23. Appendix: Source Reference

| File / source | Used for |
| --- | --- |
| D:\Embedded Communication Protocols\SPI_MCP2515_NormalMode_2Nodes\Core\Src\main.c | Application sequence, node mode setup, bidirectional CAN test loop, UART prints |
| D:\Embedded Communication Protocols\SPI_MCP2515_NormalMode_2Nodes\Core\Src\mcp2515.c | Node/SPI selection, MCP2515 register commands, init, TX/RX, TX status |
| D:\Embedded Communication Protocols\SPI_MCP2515_NormalMode_2Nodes\Core\Src\spi.c | SPI1/SPI2 GPIO and peripheral configuration, polling transfers |
| D:\Embedded Communication Protocols\SPI_MCP2515_NormalMode_2Nodes\Core\Src\usart.c | USART2 debug output |
| D:\Embedded Communication Protocols\SPI_MCP2515_NormalMode_2Nodes\Core\Src\max7219.c | MAX7219 shared-SPI visual side task |
| D:\Embedded Communication Protocols\SPI_MCP2515_NormalMode_2Nodes\Core\Inc\mcp2515.h | Register, mode, mask, node, status, and API declarations |
| D:\Embedded Communication Protocols\SPI_MCP2515_NormalMode_2Nodes\Core\Inc\spi.h | SPI declarations and chip-select macros |
| https://chatgpt.com/share/6a343a5d-ad38-83ee-bfe1-0123d30a3356 | Shared learning thread: 2-node normal-mode path, ACK interpretation, analyzer understanding |
| https://ww1.microchip.com/downloads/aemDocuments/documents/APID/ProductDocuments/DataSheets/MCP2515-Family-Data-Sheet-DS20001801K.pdf | Official MCP2515 modes, TX/RX behavior, SPI interface, flags, and constraints |

# 24. Appendix: Revision Checklist

| Check | Status |
| --- | --- |
| All ten finalized source/header files read | Done |
| Shared ChatGPT conversation downloaded and extracted | Done |
| Microchip MCP2515 data sheet checked | Done |
| System-level hardware and firmware diagrams included | Done |
| Loopback vs Normal mode explained | Done |
| ACK slot and analyzer interpretation explained | Done |
| Node1 ID 0x101 and Node2 ID 0x201 flows covered | Done |
| Exact source snippets included for key functions | Done |
| Review corrections separated from finalized code | Done |
| No separate screenshot image embedded because none was attached in this turn | Done |