MCP2515 Two-Node Receive Interrupt Project

Interrupt-driven CAN receive path using MCP2515 INT pins, STM32 EXTI0/EXTI1, and SPI register service

Document intent: Code-grounded embedded reference for MCP2515 two-node receive interrupts. It focuses on CANINTE/CANINTF, active-low INT, STM32 EXTI, deferred SPI service, RXB0/RXB1 draining, and two-node system behavior.

| Field | Value |
| --- | --- |
| Project scope | MCP2515 two-node Normal mode with receive interrupt handling |
| Node 1 | MCP2515 on SPI2, CS PB12, INT to PA1 / EXTI1 |
| Node 2 | MCP2515 on SPI1, CS PA4, INT to PA0 / EXTI0 |
| Display/debug side module | MAX7219 on SPI1, CS PA15; USART2 debug output |
| CAN test frames | Node1 TX ID 0x101; Node2 TX ID 0x201; payload 0x11 0x22 |
| Source authority | Finalized project directory SPI_MCP2515_Receive_Interrupt |
| Evidence boundary | No new screenshot file was attached in this turn; shared chat records output observations |

# 1. Document Control / Metadata

| Item | Details |
| --- | --- |
| Design preset | compact_reference_guide |
| Primary learning source | https://chatgpt.com/share/6a353c0a-c490-83ee-a511-e640babeb744 |
| Device reference | Microchip MCP2515 Family Data Sheet DS20001801K |
| Firmware reference | D:\Embedded Communication Protocols\SPI_MCP2515_Receive_Interrupt |
| Code fidelity rule | Code snippets are copied only from the finalized source files listed in the appendix |
| Footer rule | Embedded Learning Document |

# 2. Learning Objectives

Understand why MCP2515 receive interrupts are generated from CANINTE-enabled CANINTF flags.

Separate MCP2515 INT behavior from STM32 EXTI pending behavior.

Trace Node1-to-Node2 and Node2-to-Node1 receive paths without continuous RX polling.

Explain why the ISR should set a pending flag and avoid SPI transactions.

Understand how RXB0/RXB1 draining prevents missed frames after one interrupt edge.

Review the multi-TX-buffer send improvement and the remaining code risks.

# 3. Project Objective

The project upgrades the previous polling receive path into an interrupt-notified receive flow. MCP2515 receive events pull the INT pin low, STM32 EXTI handlers set lightweight pending flags, and the main loop services the MCP2515 over SPI by reading RXB0/RXB1 and clearing the matching CANINTF flags.

# 4. Problem Statement

Core engineering problem: Polling wastes CPU time and can miss timing intent. Receive interrupts move detection to hardware, but correctness now depends on two interrupt domains: STM32 EXTI pending and MCP2515 CANINTF pending.

# 5. System Overview

USART2 debug
                              |
                              v
                       +--------------+
                       | STM32F407 MCU |
                       +--------------+
          EXTI1 PA1 <----- INT                INT -----> PA0 EXTI0
                         |                         |
             SPI2 PB13/14/15              SPI1 PA5/6/7
                 CS PB12                      CS PA4
                 |                            |
                 v                            v
           +------------+                +------------+
           | MCP2515 #1 |                | MCP2515 #2 |
           |   Node 1   |                |   Node 2   |
           +------------+                +------------+
                 | TXCAN/RXCAN                 | TXCAN/RXCAN
                 v                             v
           +------------+ CANH/CANL bus +------------+
           | CAN XCVR 1 |-------------- | CAN XCVR 2 |
           +------------+               +------------+

           MAX7219 shares SPI1 with Node 2 using PA15 chip select.

| Signal / block | Final source mapping | Learning meaning |
| --- | --- | --- |
| Node1 MCP2515 | SPI2 + PB12 CS + PA1 INT | Node1 receive event reaches EXTI1 |
| Node2 MCP2515 | SPI1 + PA4 CS + PA0 INT | Node2 receive event reaches EXTI0 |
| MCP2515 INT | Active-low output | Low until enabled MCP2515 interrupt flags are cleared |
| STM32 EXTI | Falling-edge input interrupt | Detects the INT low transition and sets software pending flag |
| Main loop | Checks node pending flags | Services MCP2515 over SPI outside ISR |
| RX buffers | RXB0 and RXB1 | Read both because rollover and two-buffer receive are enabled |

# 6. Requirements

| Requirement | Project value | Verification point |
| --- | --- | --- |
| CAN mode | Both nodes Normal mode | CANSTAT & 0xE0 equals 0x00 |
| CAN bitrate | 500 kbps with 8 MHz MCP2515 oscillator | CNF1=0x00, CNF2=0x91, CNF3=0x01 |
| Receive interrupts | RX0IE and RX1IE enabled | CANINTE bit modify mask/data 0x03 |
| Receive flags | RX0IF and RX1IF serviced | MCP2515_ReadMessage chooses RXB0 or RXB1 and clears matching flag |
| MCU IRQ wiring | PA0/EXTI0 for Node2, PA1/EXTI1 for Node1 | MCP2515_INT_PA0_Init and MCP2515_INT_PA1_Init |
| ISR design | No SPI in EXTI ISR | ISR only clears EXTI pending and sets software flag |
| TX buffers | TXB0/TXB1/TXB2 free-buffer selection | MCP2515_GetFreeTxBuffer |

# 7. Hardware Components

| Component | Role | Important caution |
| --- | --- | --- |
| STM32F407 board | Runs SPI, EXTI, UART, and application loop | EXTI lines must match INT pins physically |
| MCP2515 Node 1 | CAN controller for Node1 | INT must be wired to PA1, not only SPI pins |
| MCP2515 Node 2 | CAN controller for Node2 | INT must be wired to PA0 |
| CAN transceivers | Convert TXCAN/RXCAN to CANH/CANL | Normal mode needs real bus and ACK |
| CAN termination | 120 ohm at both physical ends | Required for stable two-node bench operation |
| MAX7219 | Visual demo on SPI1 | Shares SPI1; chip selects must be mutually exclusive |
| Logic analyzer / UART | Observe CAN frames and debug messages | No new screenshot was attached for this run |

# 8. Software and Firmware Tools

STM32CubeIDE or equivalent for building and flashing.

UART terminal connected to USART2 for runtime logs.

Logic analyzer or CAN analyzer for frame, ACK, and interrupt correlation.

Oscilloscope if validating MCP2515 INT low pulse/level and CANH/CANL voltage levels.

Microchip MCP2515 data sheet for register and interrupt semantics.

# 9. System Block Diagrams

## 9.1 Interrupt Event Flow

CAN frame arrives at receiver
    |
    v
MCP2515 validates frame and stores it in RXB0 or RXB1
    |
    +-- sets CANINTF.RX0IF or CANINTF.RX1IF
    |
    +-- if CANINTE.RX0IE/RX1IE is enabled, INT pin goes low
            |
            v
      STM32 EXTI falling edge
            |
            v
      EXTI ISR clears STM32 EXTI pending bit
      and sets mcp2515_nodeX_int_pending = 1
            |
            v
      Main loop selects correct node + SPI bus
            |
            v
      while(MCP2515_ReadMessage(...)) drains RXB0/RXB1
            |
            v
      MCP2515_ReadMessage clears RX0IF/RX1IF in CANINTF
            |
            v
      MCP2515 INT releases high when all enabled flags are clear

## 9.2 Two-Node Directional Flow

Node1 -> Node2:
Node1/SPI2 loads a free TX buffer with ID 0x101 and data 11 22
    -> Node1 transmits on CAN bus
    -> Node2 stores the frame in RXB0/RXB1
    -> Node2 MCP2515 INT pulls PA0 low
    -> EXTI0 sets mcp2515_node2_int_pending
    -> main loop selects Node2/SPI1 and drains receive buffers

Node2 -> Node1:
Node2/SPI1 transmits ID 0x201
    -> Node1 MCP2515 INT pulls PA1 low
    -> EXTI1 sets mcp2515_node1_int_pending
    -> main loop selects Node1/SPI2 and drains receive buffers

# 10. Firmware Architecture

| Module | Responsibility | Important APIs / data |
| --- | --- | --- |
| main.c | Initialization and bidirectional CAN test loop | mcp2515_node1_int_pending, mcp2515_node2_int_pending |
| mcp2515.c | MCP2515 SPI command layer, mode, TX, RX, INT GPIO/EXTI init | MCP2515_Init, ReadMessage, GetFreeTxBuffer, INT_PA0/PA1_Init |
| stm32f4xx_it.c | EXTI0/EXTI1 interrupt handlers | Sets node pending flags; no SPI inside ISR |
| spi.c | SPI1/SPI2 GPIO and byte transfers | SPI1_TransferByte, SPI2_TransferByte |
| usart.c | Blocking UART debug output | USART2_SendString |
| max7219.c | LED matrix side module on SPI1 | MAX7219_Write, MAX7219_CharBlinking |

# 11. Communication Protocol Used

The MCU still accesses MCP2515 registers over polling SPI. The change is not SPI interrupt mode; it is MCP2515 receive-event notification through the external INT pin. The actual frame data is still fetched over SPI after the EXTI ISR notifies the main loop.

# 12. Core Concept Deep Dive

## 12.1 MCP2515 Receive Interrupt Registers

| Register / bit | Final source use | Meaning |
| --- | --- | --- |
| CANINTE 0x2B | MCP2515_BitModify(CANINTE, 0x03, 0x03) | Enable receive interrupts for RXB0 and RXB1 |
| CANINTF 0x2C | Read in MCP2515_ReadMessage | Shows pending interrupt source flags |
| RX0IE | CANINTE bit 0 | Allow RXB0 full event to drive INT |
| RX1IE | CANINTE bit 1 | Allow RXB1 full event to drive INT |
| RX0IF | CANINTF bit 0 | RXB0 has a received frame pending |
| RX1IF | CANINTF bit 1 | RXB1 has a received frame pending |

Datasheet verification: Microchip documents that INT is driven low when an enabled interrupt occurs and remains low until the MCU clears the relevant CANINTF flag. Receive interrupts are generated when RXnIE is enabled and a message is loaded into RXBn.

## 12.2 EXTI Pending vs MCP2515 Pending

| Pending source | Where it lives | How it is cleared | What it means |
| --- | --- | --- | --- |
| STM32 EXTI pending | EXTI->PR bit 0 or bit 1 | Write 1 to EXTI->PR bit | The MCU saw the falling edge on PA0/PA1 |
| MCP2515 receive pending | CANINTF.RX0IF/RX1IF | Bit Modify CANINTF with matching mask to 0 | A frame is still waiting in RXB0/RXB1 |
| Software pending flag | mcp2515_nodeX_int_pending | Main loop clears before servicing | A deferred service request for the main loop |

The shared learning correction is important: clearing STM32 EXTI->PR does not clear the MCP2515 interrupt. If CANINTF remains set, the MCP2515 INT pin remains low and another falling edge may not occur.

## 12.3 Why No SPI Inside EXTI ISR

SPI register reads are blocking in this project and can take unbounded time if the SPI state is wrong.

The ISR should stay short so it does not delay SysTick, UART/DMA, or other interrupt paths.

The main loop already has the node/SPI selection context needed to read RXB0/RXB1 safely.

A flag-based ISR is easier to debug and avoids nested chip-select/SPI ownership problems.

## 12.4 RXB0/RXB1 Draining Strategy

Because RXB0 rollover is enabled and both RX0IE/RX1IE are enabled, the service routine must not stop after one read. The main loop uses while(MCP2515_ReadMessage(...)) so both receive buffers are drained before waiting for a new INT edge.

# 13. Module-Wise Implementation

| Phase | Final-source behavior | Engineering purpose |
| --- | --- | --- |
| EXTI setup | PA0 and PA1 configured as input pull-up, falling edge, NVIC enabled | Detect active-low INT transitions |
| MCP2515 init | Clear CANINTF, enable RX0IE/RX1IE | Allow receive-buffer events to assert INT |
| TX path | Select free TXB0/TXB1/TXB2 buffer | Avoid failing just because TXB0 is busy |
| RX path | Read CANINTF, choose RXB0 or RXB1, clear matching flag | Correctly releases MCP2515 INT after data is read |
| Main loop | Check node pending flags before receive service | Avoid continuous polling receive path |
| Status path | Read active TX control address | Report pending, TXERR, MLOA, ABTF |

# 14. Main Algorithm and Flowchart

Power-on
  |
  +-- HAL_Init and clock setup
  +-- USART2_Init
  +-- SPI2 + SPI1 GPIO/peripheral init
  +-- MCP2515_INT_PA0_Init for Node2 INT
  +-- MCP2515_INT_PA1_Init for Node1 INT
  +-- Initialize Node1 over SPI2 and set Normal mode
  +-- Initialize Node2 over SPI1 and set Normal mode
  +-- Initialize MAX7219
  |
  +-- Forever:
        Node1 sends ID 0x101
        if Node2 INT pending:
            clear software flag
            select Node2/SPI1
            drain RXB0/RXB1 by MCP2515_ReadMessage()
        blink MAX7219
        Node2 sends ID 0x201
        if Node1 INT pending:
            clear software flag
            select Node1/SPI2
            drain RXB0/RXB1 by MCP2515_ReadMessage()
        blink MAX7219

# 15. Code Explanation

Code authority: The snippets below are copied from the project files under SPI_MCP2515_Receive_Interrupt. Review corrections are documented separately and are not silently applied to the source.

## 15.1 Application State and Pending Flags

extern const uint8_t font_8x8_digits[10][8];
extern const uint8_t font_8x8_alpha[26][8];
extern const uint8_t font_space[8];


volatile uint8_t mcp2515_node2_int_pending;
volatile uint8_t mcp2515_node1_int_pending;
uint8_t current_txctrl_addr;

## 15.2 Main Initialization

USART2_Init();

  USART2_SendString("UART Initialized....!!\r\n");

  SPI2_GPIO_Init();
  SPI2_Init();

  SPI1_GPIO_Init();
  SPI1_Init();
  USART2_SendString("SPI Initialized.....!!\r\n");

  //Enable Exti interrupt in PA0 for node2
  MCP2515_INT_PA0_Init();

  //Enable Exti interrupt in PA1 for node1
  MCP2515_INT_PA1_Init();

  MCP2515_SelectNode(MCP2515_NODE_1);
  MCP2515_SelectSpiBus(MCP2515_SPI_BUS_2);
  SPI2_Test();

  MCP2515_SelectNode(MCP2515_NODE_2);
  MCP2515_SelectSpiBus(MCP2515_SPI_BUS_1);
  SPI1_Test();

  // Initialize MCP node1 and move to normal mode
  MCP2515_SelectNode(MCP2515_NODE_1);
  MCP2515_SelectSpiBus(MCP2515_SPI_BUS_2);
  MCP2515_Init();

  //Enter normal mode
  MCP2515_SetMode(MCP2515_MODE_NORMAL);

  //check and wait till normal mode
  while((MCP2515_ReadRegister(MCP2515_CANSTAT) & MCP2515_MODE_MASK) != MCP2515_MODE_NORMAL);

  uint8_t canstat = MCP2515_ReadRegister(MCP2515_CANSTAT);

  if((canstat & MCP2515_MODE_MASK) == MCP2515_MODE_NORMAL)
  {
      USART2_SendString("MCP2515 Node1 Normal mode OK\r\n");
  }
  else
  {
      USART2_SendString("MCP2515 Node1 Normal mode failed\r\n");
  }

  // Initialize MCP node2 and move to normal mode
  MCP2515_SelectNode(MCP2515_NODE_2);
  MCP2515_SelectSpiBus(MCP2515_SPI_BUS_1);
  MCP2515_Init();

  //Enter Normal mode
  MCP2515_SetMode(MCP2515_MODE_NORMAL);

  //check and wait till loopback mode
  while((MCP2515_ReadRegister(MCP2515_CANSTAT) & MCP2515_MODE_MASK) != MCP2515_MODE_NORMAL);

  uint8_t canstat2 = MCP2515_ReadRegister(MCP2515_CANSTAT);

  if((canstat2 & MCP2515_MODE_MASK) == MCP2515_MODE_NORMAL)
  {
      USART2_SendString("MCP2515 Node2 Normal mode OK\r\n");
  }
  else
  {
      USART2_SendString("MCP2515 Node2 Normal mode failed\r\n");
  }

  // Simple hardware test
  MAX7219_Write(0x0F, 0x01);   // All LEDs ON
  HAL_Delay(2000);
  MAX7219_Write(0x0F, 0x00);   // Test OFF

  MAX7219_Init();
  MAX7219_Clear();

## 15.3 Main Interrupt-Service Loop

//Transmitting standard CAN frame
	  MCP2515_SelectNode(MCP2515_NODE_1);
	  MCP2515_SelectSpiBus(MCP2515_SPI_BUS_2);
	  if(MCP2515_SendMessage(0x101,msg,dlc_len))
	  {
		  HAL_Delay(10);

		  if(MCP2515_PrintTxStatus(current_txctrl_addr))
		  {
			  USART2_SendString("Node1 Message Transmitted Successfully\r\n");
		  }
		  else
		  {
			  USART2_SendString("Node1 Tx Error\r\n");
		  }

		  sprintf(uart_msg_tx,"Node1 Message Transmitted : ID = 0x%03X, DLC = 0x%02X, DATA1 = 0x%02X, DATA2 = 0x%02X\r\n",0x101,dlc_len,msg[0],msg[1]);
		  USART2_SendString(uart_msg_tx);
	  }
	  else
	  {
		  USART2_SendString("Node1 : Message Tx Failed\r\n");
	  }

	  if(mcp2515_node2_int_pending)
	  {
		  mcp2515_node2_int_pending = 0;

		 //Node 2 Receives
		  MCP2515_SelectNode(MCP2515_NODE_2);
		  MCP2515_SelectSpiBus(MCP2515_SPI_BUS_1);

		  //Receiving frame
		  while(MCP2515_ReadMessage(&rx_id,rx_msg,&rx_dlc_len))
		  {
			  sprintf(uart_msg_rx,"Node2 Message Received    : ID = 0x%03X, DLC = 0x%02X, DATA1 = 0x%02X, DATA2 = 0x%02X\r\n",rx_id,rx_dlc_len,rx_msg[0],rx_msg[1]);
			  USART2_SendString(uart_msg_rx);
		  }
	  }

	  MAX7219_CharBlinking('A');

	  //This time Node 2 Transmits and Node 1 Receives
	  MCP2515_SelectNode(MCP2515_NODE_2);
	  MCP2515_SelectSpiBus(MCP2515_SPI_BUS_1);

	  if(MCP2515_SendMessage(0x201,msg,dlc_len))
	  {
		  HAL_Delay(10);

		  if(MCP2515_PrintTxStatus(current_txctrl_addr))
		  {
			  USART2_SendString("Node2 Message Transmitted Successfully\r\n");
		  }
		  else
		  {
			  USART2_SendString("Node2 Tx Error\r\n");
		  }
		  sprintf(uart_msg_tx,"Node2 Message Transmitted : ID = 0x%03X, DLC = 0x%02X, DATA1 = 0x%02X, DATA2 = 0x%02X\r\n",0x201,dlc_len,msg[0],msg[1]);
		  USART2_SendString(uart_msg_tx);
	  }
	  else
	  {
		  USART2_SendString("Node2 : Message Tx Failed\r\n");
	  }

	  if(mcp2515_node1_int_pending)
	  {
		  mcp2515_node1_int_pending = 0;

		  MCP2515_SelectNode(MCP2515_NODE_1);
		  MCP2515_SelectSpiBus(MCP2515_SPI_BUS_2);

		  //Receiving frame
		  while(MCP2515_ReadMessage(&rx_id,rx_msg,&rx_dlc_len))
		  {
			  sprintf(uart_msg_rx,"Node2 Message Received    : ID = 0x%03X, DLC = 0x%02X, DATA1 = 0x%02X, DATA2 = 0x%02X\r\n",rx_id,rx_dlc_len,rx_msg[0],rx_msg[1]);
			  USART2_SendString(uart_msg_rx);
		  }
	  }

	  MAX7219_CharBlinking('A');

	  //MAX7219_StringBlinking("Hello Via Matrix");

	  //MAX7219_ScrollText_RtoL("HELLO", 100);

## 15.4 Active SPI and Chip-Select Wrappers

static uint8_t MCP2515_SPI_TransferByte(uint8_t data)
{
    if(active_mcp_spi_bus == MCP2515_SPI_BUS_1)
    {
        return SPI1_TransferByte(data);
    }
    else
    {
        return SPI2_TransferByte(data);
    }
}

static void MCP2515_CS_Low(void)
{
	if(active_mcp_node == MCP2515_NODE_1)
	{
		MCP2515_CS_LOW();
	}
	else
	{
		MCP2515_NODE2_CS_LOW();
	}
}

static void MCP2515_CS_High(void)
{
	if(active_mcp_node == MCP2515_NODE_1)
	{
		MCP2515_CS_HIGH();
	}
	else
	{
		MCP2515_NODE2_CS_HIGH();
	}
}

## 15.5 MCP2515 Initialization

void MCP2515_Init(void)
{
	// Reset to reach config mode
	MCP2515_Reset();

	//check and wait till config mode
	while((MCP2515_ReadRegister(MCP2515_CANSTAT) & MCP2515_MODE_MASK) != MCP2515_MODE_CONFIG);

	/*Baudrate config , 500kbps ,8 N_TQ, SyncSeg = 1 TQ,
	PropSeg = 2 TQ ,PS1     = 3 TQ ,PS2     = 2 TQ
	Total   = 8 TQ*/
	MCP2515_WriteRegister(MCP2515_CNF1, 0x00);
	MCP2515_WriteRegister(MCP2515_CNF2, 0x91);
	MCP2515_WriteRegister(MCP2515_CNF3, 0x01);

	//Filter,mask settings, receive any message ID
	MCP2515_BitModify(MCP2515_RXB0CTRL, 0x60 , 0x60);
	MCP2515_BitModify(MCP2515_RXB1CTRL, 0x60 , 0x60);

	//enable RXB0 rollover to RXB1 */
	MCP2515_BitModify(MCP2515_RXB0CTRL, 0x04, 0x04);

	/* Clear interrupt flags */
	MCP2515_WriteRegister(MCP2515_CANINTF, 0x00);

	/* Enable Receive interrupt for buffer 0 and buffer 1 */
	MCP2515_BitModify(MCP2515_CANINTE, 0x03, 0x03);
}

## 15.6 Free TX Buffer Selection

static uint8_t MCP2515_GetFreeTxBuffer(uint8_t *ctrl_addr,uint8_t *sidh_addr,
								uint8_t *sidl_addr,uint8_t *dlc_addr,uint8_t *d0_addr)
{
	uint8_t buffer_available = E_NOT_OK;

	if((MCP2515_ReadRegister(MCP2515_TXB0CTRL)& 0x08) == 0)
	{
		//if buffer available then store register address
		buffer_available = E_OK;
		*ctrl_addr = MCP2515_TXB0CTRL;
		*sidh_addr = MCP2515_TXB0SIDH;
		*sidl_addr = MCP2515_TXB0SIDL;
		*dlc_addr  = MCP2515_TXB0DLC;
		*d0_addr   = MCP2515_TXB0D0;
	}
	else if((MCP2515_ReadRegister(MCP2515_TXB1CTRL)& 0x08) == 0)
	{
		//if buffer available then store register address
		buffer_available = E_OK;
		*ctrl_addr = MCP2515_TXB1CTRL;
		*sidh_addr = MCP2515_TXB1SIDH;
		*sidl_addr = MCP2515_TXB1SIDL;
		*dlc_addr  = MCP2515_TXB1DLC;
		*d0_addr   = MCP2515_TXB1D0;
	}
	else if((MCP2515_ReadRegister(MCP2515_TXB2CTRL)& 0x08) == 0)
	{
		//if buffer available then store register address
		buffer_available = E_OK;
		*ctrl_addr = MCP2515_TXB2CTRL;
		*sidh_addr = MCP2515_TXB2SIDH;
		*sidl_addr = MCP2515_TXB2SIDL;
		*dlc_addr  = MCP2515_TXB2DLC;
		*d0_addr   = MCP2515_TXB2D0;
	}
	else
	{
		buffer_available = E_NOT_OK;
	}

	return buffer_available;
}

uint8_t MCP2515_SendMessage(uint16_t id, uint8_t *data, uint8_t len)
{
    uint8_t ctrl_addr;
    uint8_t sidh_addr;
    uint8_t sidl_addr;
    uint8_t dlc_addr;
    uint8_t d0_addr;

    uint8_t sidh;
    uint8_t sidl;

    /* Standard CAN ID is 11-bit */
    if(id > 0x7FF)
    {
        return 0;
    }

    /* CAN data length max is 8 bytes */
    if(len > 8)
    {
        return 0;
    }

    if((data == 0) && (len > 0))
    {
        return 0;
    }

    //find free buffer
    if(!MCP2515_GetFreeTxBuffer(&ctrl_addr,&sidh_addr,&sidl_addr,&dlc_addr,&d0_addr))
    {
    	return 0;
    }

    //global var for current tx addr
    current_txctrl_addr = ctrl_addr;
    /*
     * Standard ID mapping:
     * TXB0SIDH = SID10..SID3
     * TXB0SIDL[7:5] = SID2..SID0
     */
    sidh = (uint8_t)(id >> 3);
    sidl = (uint8_t)((id & 0x07) << 5);

    /* Load standard ID */
    MCP2515_WriteRegister(sidh_addr, sidh);
    MCP2515_WriteRegister(sidl_addr, sidl);

    /* Standard data frame, DLC = len */
    MCP2515_WriteRegister(dlc_addr, len & 0x0F);

    /* Load data bytes */
    for(uint8_t i = 0; i < len; i++)
    {
        MCP2515_WriteRegister(d0_addr + i, data[i]);
    }

    /* Request transmission: set TXB0CTRL.TXREQ */
    MCP2515_BitModify(ctrl_addr, 0x08, 0x08);

    return 1;
}

## 15.7 RXB0/RXB1 Read and Flag Clear

uint8_t MCP2515_ReadMessage(uint16_t *id, uint8_t *data, uint8_t *len)
{
    uint8_t sidh_addr;
    uint8_t sidl_addr;
    uint8_t dlc_addr;
    uint8_t d0_addr;
    uint8_t clear_mask;
    uint8_t canintf;

    uint8_t sidh;
    uint8_t sidl;
    uint8_t dlc;

    canintf = MCP2515_ReadRegister(MCP2515_CANINTF);

    if(canintf & MCP2515_CANINTF_RX0IF_MASK)
    {
        sidh_addr  = MCP2515_RXB0SIDH;
        sidl_addr  = MCP2515_RXB0SIDL;
        dlc_addr   = MCP2515_RXB0DLC;
        d0_addr    = MCP2515_RXB0D0;
        clear_mask = MCP2515_CANINTF_RX0IF_MASK;
    }
    else if(canintf & MCP2515_CANINTF_RX1IF_MASK)
    {
        sidh_addr  = MCP2515_RXB1SIDH;
        sidl_addr  = MCP2515_RXB1SIDL;
        dlc_addr   = MCP2515_RXB1DLC;
        d0_addr    = MCP2515_RXB1D0;
        clear_mask = MCP2515_CANINTF_RX1IF_MASK;
    }
    else
    {
        return 0;
    }

    sidh = MCP2515_ReadRegister(sidh_addr);
    sidl = MCP2515_ReadRegister(sidl_addr);

    *id = ((uint16_t)sidh << 3) | (sidl >> 5);

    dlc = MCP2515_ReadRegister(dlc_addr) & 0x0F;

    if(dlc > 8)
    {
        dlc = 8;
    }

    *len = dlc;

    for(uint8_t i = 0; i < dlc; i++)
    {
        data[i] = MCP2515_ReadRegister(d0_addr + i);
    }

    MCP2515_BitModify(MCP2515_CANINTF, clear_mask, 0x00);

    return 1;
}

## 15.8 MCP2515 INT GPIO/EXTI Setup

void MCP2515_INT_PA0_Init(void)
{
    /*
     * MCP2515 INT pin:
     * Active low
     * Normal state = high
     * Interrupt event = falling edge
     */

    /* Enable GPIOA clock */
    RCC->AHB1ENR |= RCC_AHB1ENR_GPIOAEN;

    /* Enable SYSCFG clock */
    RCC->APB2ENR |= RCC_APB2ENR_SYSCFGEN;

    /*
     * PA0 as input
     * MODER0 = 00
     */
    GPIOA->MODER &= ~GPIO_MODER_MODER0;

    /*
     * Pull-up on PA0
     * PUPDR0 = 01
     */
    GPIOA->PUPDR &= ~GPIO_PUPDR_PUPD0;
    GPIOA->PUPDR |=  GPIO_PUPDR_PUPD0_0;

    /*
     * Map EXTI0 to Port A.
     * EXTICR[0], EXTI0 field = bits [3:0]
     * Port A = 0000
     */
    SYSCFG->EXTICR[0] &= ~SYSCFG_EXTICR1_EXTI0;

    /*
     * Unmask EXTI0 interrupt
     */
    EXTI->IMR |= EXTI_IMR_MR0;

    /*
     * Falling edge trigger enable.
     * MCP2515 INT is active-low.
     */
    EXTI->FTSR |= EXTI_FTSR_TR0;

    /*
     * Disable rising edge trigger
     */
    EXTI->RTSR &= ~EXTI_RTSR_TR0;

    /*
     * Clear pending flag before enabling NVIC.
     * Write 1 to clear.
     */
    EXTI->PR = EXTI_PR_PR0;

    /*
     * Optional but recommended: set priority
     */
    NVIC_SetPriority(EXTI0_IRQn, 2);

    /*
     * Enable EXTI line 0 interrupt in NVIC.
     */
    NVIC_EnableIRQ(EXTI0_IRQn);
}

void MCP2515_INT_PA1_Init(void)
{
    /*
     * MCP2515 INT pin:
     * Active low
     * Normal state = high
     * Interrupt event = falling edge
     */

    /* Enable GPIOA clock */
    RCC->AHB1ENR |= RCC_AHB1ENR_GPIOAEN;

    /* Enable SYSCFG clock */
    RCC->APB2ENR |= RCC_APB2ENR_SYSCFGEN;

    /*
     * PA1 as input
     * MODER0 = 00
     */
    GPIOA->MODER &= ~GPIO_MODER_MODER1;

    /*
     * Pull-up on PA1
     * PUPDR0 = 01
     */
    GPIOA->PUPDR &= ~GPIO_PUPDR_PUPD1;
    GPIOA->PUPDR |=  GPIO_PUPDR_PUPD1_0;

    /*
     * Map EXTI0 to Port A.
     * EXTICR[0], EXTI0 field = bits [3:0]
     * Port A = 0000
     */
    SYSCFG->EXTICR[0] &= ~SYSCFG_EXTICR1_EXTI1;

    /*
     * Unmask EXTI0 interrupt
     */
    EXTI->IMR |= EXTI_IMR_MR1;

    /*
     * Falling edge trigger enable.
     * MCP2515 INT is active-low.
     */
    EXTI->FTSR |= EXTI_FTSR_TR1;

    /*
     * Disable rising edge trigger
     */
    EXTI->RTSR &= ~EXTI_RTSR_TR1;

    /*
     * Clear pending flag before enabling NVIC.
     * Write 1 to clear.
     */
    EXTI->PR = EXTI_PR_PR1;

    /*
     * Optional but recommended: set priority
     */
    NVIC_SetPriority(EXTI1_IRQn, 2);

    /*
     * Enable EXTI line 0 interrupt in NVIC.
     */
    NVIC_EnableIRQ(EXTI1_IRQn);
}

## 15.9 STM32 EXTI Handlers

void EXTI0_IRQHandler(void)
{
	if(!(EXTI->PR & 0U))
	{
		/*
		 * Clear EXTI pending flag first.
		 * Write 1 to clear.
		*/
		EXTI->PR = (1 << 0);

		/*
		* Do not read MCP2515 over SPI here.
		* Just notify main loop.
		*/

		mcp2515_node2_int_pending = 1;
	}

}

void EXTI1_IRQHandler(void)
{
	if(!(EXTI->PR & 1U))
	{
		/*
		 * Clear EXTI pending flag first.
		 * Write 1 to clear.
		*/
		EXTI->PR = (1 << 1);

		/*
		* Do not read MCP2515 over SPI here.
		* Just notify main loop.
		*/

		mcp2515_node1_int_pending = 1;
	}

}

## 15.10 SPI and UART Support

void SPI1_GPIO_Init(void)
{
    /* Enable GPIOB clock */
    RCC->AHB1ENR |= RCC_AHB1ENR_GPIOAEN;

    /*
     * PA15 = CS_MAX7219
     * PA4 = CS_MCP2515_NODE1
     * PA5 = SPI1_SCK AF
     * PA6 = SPI1_MISO AF
     * PA7 = SPI1_MOSI AF
     */

    /* Clear mode bits */
    GPIOA->MODER &= ~((3UL << (15 * 2))|
    				  (3UL << (4 * 2)) |
                      (3UL << (5 * 2)) |
                      (3UL << (6 * 2)) |
                      (3UL << (7 * 2)));

    /* Set modes */
    GPIOA->MODER |=  ((1UL << (15 * 2)) |
    				  (1UL << (4 * 2))  |   /* PA4 output */
                      (2UL << (5 * 2)) |   /* PA5 AF */
                      (2UL << (6 * 2)) |   /* PA6 AF */
                      (2UL << (7 * 2)));   /* PA7 AF */

    /* Push-pull */
    GPIOA->OTYPER &= ~((1UL << 15)  |
    				   (1UL << 4)   |
                       (1UL << 5)   |
                       (1UL << 6)   |
                       (1UL << 7));

    /* Speed: medium/high for SPI */
    GPIOA->OSPEEDR &= ~((3UL << (15 * 2)) |
    					(3UL << (4 * 2)) |
                        (3UL << (5 * 2)) |
                        (3UL << (6 * 2)) |
                        (3UL << (7 * 2)));

    GPIOA->OSPEEDR |=  ((2UL << (15 * 2)) |
    					(2UL << (4 * 2)) |
                        (2UL << (5 * 2)) |
                        (2UL << (6 * 2)) |
                        (2UL << (7 * 2)));

    /* Pull config: CS pull-up, others no pull */
    GPIOA->PUPDR &= ~((3UL << (15 * 2)) |
    				  (3UL << (4 * 2)) |
                      (3UL << (5 * 2)) |
                      (3UL << (6 * 2)) |
                      (3UL << (7 * 2)));

    GPIOA->PUPDR |=  (1UL << (4 * 2));     /* PA4 pull-up */
    GPIOA->PUPDR |=  (1UL << (15 * 2));    /* PA15 pull-up */

    /* AF5 for PA5, PA6, PA7 */
    GPIOA->AFR[0] &= ~(GPIO_AFRL_AFSEL5 |
                       GPIO_AFRL_AFSEL6 |
                       GPIO_AFRL_AFSEL7);

    GPIOA->AFR[0] |=  (5UL << GPIO_AFRL_AFSEL5_Pos) |
                       (5UL << GPIO_AFRL_AFSEL6_Pos) |
                       (5UL << GPIO_AFRL_AFSEL7_Pos);

    /* Keep CS high initially */
    MCP2515_NODE2_CS_HIGH();
    MAX7219_CS_HIGH();

}

void SPI2_GPIO_Init(void)
{
    /* Enable GPIOB clock */
    RCC->AHB1ENR |= RCC_AHB1ENR_GPIOBEN;

    /*
     * PB12 = CS_MCP2515_NODE1
     * PB13 = SPI2_SCK AF
     * PB14 = SPI2_MISO AF
     * PB15 = SPI2_MOSI AF
     */

    /* Clear mode bits */
    GPIOB->MODER &= ~((3UL << (12 * 2)) |
                      (3UL << (13 * 2)) |
                      (3UL << (14 * 2)) |
                      (3UL << (15 * 2)));

    /* Set modes */
    GPIOB->MODER |=  ((1UL << (12 * 2)) |   /* PB12 output */
                      (2UL << (13 * 2)) |   /* PB13 AF */
                      (2UL << (14 * 2)) |   /* PB14 AF */
                      (2UL << (15 * 2)));   /* PB15 AF */

    /* Push-pull */
    GPIOB->OTYPER &= ~((1UL << 12) |
                       (1UL << 13) |
                       (1UL << 14) |
                       (1UL << 15));

    /* Speed: medium/high for SPI */
    GPIOB->OSPEEDR &= ~((3UL << (12 * 2)) |
                        (3UL << (13 * 2)) |
                        (3UL << (14 * 2)) |
                        (3UL << (15 * 2)));

    GPIOB->OSPEEDR |=  ((2UL << (12 * 2)) |
                        (2UL << (13 * 2)) |
                        (2UL << (14 * 2)) |
                        (2UL << (15 * 2)));

    /* Pull config: CS pull-up, others no pull */
    GPIOB->PUPDR &= ~((3UL << (12 * 2)) |
                      (3UL << (13 * 2)) |
                      (3UL << (14 * 2)) |
                      (3UL << (15 * 2)));

    GPIOB->PUPDR |=  (1UL << (12 * 2));     /* PB12 pull-up */

    /* AF5 for PB13, PB14, PB15 */
    GPIOB->AFR[1] &= ~((0xFUL << ((13 - 8) * 4)) |
                       (0xFUL << ((14 - 8) * 4)) |
                       (0xFUL << ((15 - 8) * 4)));

    GPIOB->AFR[1] |=  ((5UL << ((13 - 8) * 4)) |
                       (5UL << ((14 - 8) * 4)) |
                       (5UL << ((15 - 8) * 4)));

    /* Keep CS high initially */
    MCP2515_CS_HIGH();

}

uint8_t SPI1_TransferByte(uint8_t data)
{
    uint8_t received;

    while(!(SPI1->SR & SPI_SR_TXE));

    *((volatile uint8_t *)&SPI1->DR) = data;

    while(!(SPI1->SR & SPI_SR_RXNE));

    received = *((volatile uint8_t *)&SPI1->DR);

    while(SPI1->SR & SPI_SR_BSY);

    return received;
}

uint8_t SPI2_TransferByte(uint8_t byte)
{
    uint8_t received;

    //waiting for TX buffer empty
    while(!(SPI2->SR & SPI_SR_TXE));
    *((volatile uint8_t *)&SPI2->DR) = byte;

    //waiting for RX buffer Not empty
    while(!(SPI2->SR & SPI_SR_RXNE));
    received = *((volatile uint8_t *)&SPI2->DR);

    //Wait till SPI completes
    while(SPI2->SR & SPI_SR_BSY);

    return received;
}

void USART2_SendString(char *s)
{
	while(*s != '\0')
	{
		while(!(USART2->SR & USART_SR_TXE));
		USART2->DR = *s++;
	}

	//checking for the last character
	while(!(USART2->SR & USART_SR_TC));
}

# 16. Testing and Validation

| Test | Expected observation | Meaning |
| --- | --- | --- |
| Node1 Normal mode | MCP2515 Node1 Normal mode OK | Node1 is active on CAN bus |
| Node2 Normal mode | MCP2515 Node2 Normal mode OK | Node2 is active on CAN bus |
| Node1 TX | TXREQ clears; no TXERR/MLOA/ABTF | Frame transmitted and acknowledged |
| Node2 INT | PA0 falling edge, EXTI0 pending, node2 flag set | Node2 receive event detected |
| Node2 RX | Node2 receives ID 0x101 with data 11 22 | Node1-to-Node2 interrupt path works |
| Node2 TX | TXREQ clears; no TX error flags | Reverse frame transmitted |
| Node1 INT | PA1 falling edge, EXTI1 pending, node1 flag set | Node1 receive event detected |
| Node1 RX | Node1 receives ID 0x201 with data 11 22 | Node2-to-Node1 interrupt path works |
| CANINTF drain | ReadMessage eventually returns 0 | RX0IF/RX1IF are cleared and INT can release |

# 17. Results

Observed result from the shared chat: The shared learning notes report that the interrupt-based receive flow works in both directions. No fresh screenshot file was attached in this turn, so the document records the expected UART/analyzer observations without embedding a new image.

Expected terminal pattern:

MCP2515 Node1 Normal mode OK
MCP2515 Node2 Normal mode OK
Node1 Message Transmitted Successfully
Node1 Message Transmitted : ID = 0x101, DLC = 0x02, DATA1 = 0x11, DATA2 = 0x22
Node2 Message Received    : ID = 0x101, DLC = 0x02, DATA1 = 0x11, DATA2 = 0x22
Node2 Message Transmitted Successfully
Node2 Message Transmitted : ID = 0x201, DLC = 0x02, DATA1 = 0x11, DATA2 = 0x22
Node1 Message Received    : ID = 0x201, DLC = 0x02, DATA1 = 0x11, DATA2 = 0x22

# 18. Problems Faced and Debugging Notes

| Symptom | Likely cause | Debug action |
| --- | --- | --- |
| INT pin stays low | CANINTF flag still set | Read CANINTF; clear RX0IF/RX1IF after reading matching buffer |
| EXTI fires once only | MCP2515 INT never returned high | Drain all enabled interrupt sources before waiting for next edge |
| Pending flag set but no message | Wrong node/SPI selected or CANINTF already cleared | Print active node, bus, and CANINTF |
| RXB1 never read | Receive function stops after RXB0 | Use while(ReadMessage) and RX1IF support |
| TX fails under load | All TX buffers busy or no ACK | Check TXB0/TXB1/TXB2 TXREQ and TXERR flags |
| Wrong receiver label in UART | Copy-paste print string | Fix Node1 receive message text |
| SPI1 test confusing | SPI1_Test selects MCP2515_NODE_1 | Change helper to select NODE_2 for SPI1 test |

# 19. Embedded Engineering Review

| Finding | Impact | Recommended action |
| --- | --- | --- |
| EXTI0_IRQHandler checks EXTI->PR & 0U | Condition is always true inside handler and does not really test PR0 | Use if(EXTI->PR & EXTI_PR_PR0) before clearing PR0 |
| EXTI1_IRQHandler checks bit 0 and inverts the condition | Line 1 pending state is not validated correctly | Use if(EXTI->PR & EXTI_PR_PR1) before clearing PR1 |
| Main reverse receive print says Node2 Message Received | Node1 receive logs can mislead debugging | Change reverse receive string to Node1 Message Received |
| SPI1_Test() selects MCP2515_NODE_1 | SPI1 test can drive wrong CS context | Select MCP2515_NODE_2 or remove node mutation from test helper |
| Mode waits are unbounded | A failed MCP2515 can hang firmware | Use MCP2515_SetModeVerify-style timeout everywhere |
| INT service relies on edge detection | If flags are not fully cleared, no new falling edge may occur | Always drain all CANINTF receive sources and optionally re-read CANINTF |
| No EFLG/TEC/REC diagnostics | CAN error states are invisible | Add error-register reporting after TX/RX failures |
| MAX7219 blink delays receive service | Long display delays reduce responsiveness | Move display demo out of timing-sensitive CAN loop |
| spi.h has no include guard | Can cause duplicate declarations in larger builds | Add a conventional include guard |

# 20. Limitations

The implementation is interrupt-notified receive, not fully interrupt-driven CAN parsing.

SPI transactions remain blocking and occur in the main loop.

Receive filters are bypassed for learning convenience.

The app still uses HAL_Delay and MAX7219 blinking inside the CAN test loop.

Only standard data frames are handled by the current send/read path.

No new screenshot was attached, so screenshot analysis is not included.

# 21. Future Improvements

Fix EXTI0/EXTI1 pending-bit conditions.

Add CANINTF/EFLG/TEC/REC debug prints after interrupt service.

Move display blinking out of the CAN timing path.

Add filters so Node2 accepts ID 0x101 and Node1 accepts ID 0x201.

Use a small receive queue so main-loop work can be decoupled from UART printing.

Add timeout protection to SPI transfer waits and mode transitions.

Add a dedicated low-level board wiring checklist for INT, CS, CANH, CANL, and GND.

# 22. Conclusion

Conclusion: The project is a solid step from polling receive: MCP2515 hardware notifies STM32 on RXB0/RXB1 events, and the main loop services both nodes over SPI. Fix EXTI pending checks, UART labels, and error diagnostics before treating it as robust.

# 23. Appendix: Source Reference

| File / source | Used for |
| --- | --- |
| D:\Embedded Communication Protocols\SPI_MCP2515_Receive_Interrupt\Core\Src\main.c | Finalized firmware source/header used for exact code grounding |
| D:\Embedded Communication Protocols\SPI_MCP2515_Receive_Interrupt\Core\Src\mcp2515.c | Finalized firmware source/header used for exact code grounding |
| D:\Embedded Communication Protocols\SPI_MCP2515_Receive_Interrupt\Core\Src\spi.c | Finalized firmware source/header used for exact code grounding |
| D:\Embedded Communication Protocols\SPI_MCP2515_Receive_Interrupt\Core\Src\usart.c | Finalized firmware source/header used for exact code grounding |
| D:\Embedded Communication Protocols\SPI_MCP2515_Receive_Interrupt\Core\Src\max7219.c | Finalized firmware source/header used for exact code grounding |
| D:\Embedded Communication Protocols\SPI_MCP2515_Receive_Interrupt\Core\Src\stm32f4xx_it.c | Finalized firmware source/header used for exact code grounding |
| D:\Embedded Communication Protocols\SPI_MCP2515_Receive_Interrupt\Core\Inc\main.h | Finalized firmware source/header used for exact code grounding |
| D:\Embedded Communication Protocols\SPI_MCP2515_Receive_Interrupt\Core\Inc\mcp2515.h | Finalized firmware source/header used for exact code grounding |
| D:\Embedded Communication Protocols\SPI_MCP2515_Receive_Interrupt\Core\Inc\spi.h | Finalized firmware source/header used for exact code grounding |
| D:\Embedded Communication Protocols\SPI_MCP2515_Receive_Interrupt\Core\Inc\usart.h | Finalized firmware source/header used for exact code grounding |
| D:\Embedded Communication Protocols\SPI_MCP2515_Receive_Interrupt\Core\Inc\max7219.h | Finalized firmware source/header used for exact code grounding |
| D:\Embedded Communication Protocols\SPI_MCP2515_Receive_Interrupt\Core\Inc\stm32f4xx_it.h | Finalized firmware source/header used for exact code grounding |
| https://chatgpt.com/share/6a353c0a-c490-83ee-a511-e640babeb744 | Shared learning thread for receive-interrupt theory and corrections |
| https://ww1.microchip.com/downloads/aemDocuments/documents/APID/ProductDocuments/DataSheets/MCP2515-Family-Data-Sheet-DS20001801K.pdf | Official MCP2515 interrupt, buffer, SPI, and register behavior |

# 24. Appendix: Revision Checklist

| Check | Status |
| --- | --- |
| Shared ChatGPT link downloaded | Done |
| Receive-interrupt chunks extracted | Done |
| Final source directory inspected | Done |
| MCP2515 data sheet checked | Done |
| System-level INT/EXTI diagrams included | Done |
| RX0IE/RX1IE and RX0IF/RX1IF explained | Done |
| EXTI vs CANINTF clearing explained | Done |
| Engineering review findings included | Done |
| No screenshot invented | Done |
| Footer name set to Embedded Learning Document | Done |