EMBEDDED DRIVER DEVELOPMENT GUIDE

I2C EEPROM Driver Development

AT24C256 Driver Architecture, Page Handling, Sequential Read, Fill Test, and STM32F407 I2C Polling Implementation

Document intent: A long-term professional reference for building and reviewing an AT24C256 EEPROM driver on top of a generic STM32F407 I2C1 polling layer. The attached finalized source files are used as the implementation authority.

| Coverage area | What this document captures |
| --- | --- |
| Driver layering | main.c -> at24c256.c -> i2c.c -> STM32F407 I2C1 registers |
| EEPROM model | 16-bit word address, 32 KiB range, 64-byte page writes, internal write cycle |
| Implemented APIs | WriteByte, ReadByte, WritePage, ReadBuffer, WriteBuffer, Fill, TestAllApis |
| I2C backend | MasterWrite and MasterWriteRead with single-byte, two-byte, and multi-byte receive paths |
| Verification | UART self-test sequence and source-fidelity checks against the attached code |
| Engineering notes | ACK polling not yet implemented, I2C_TIMEOUT unused, malformed unused PAGE_SPACE macro |

# 1. Document Control and Scope

| Control item | Value |
| --- | --- |
| Document type | Embedded EEPROM driver development learning document |
| Shared learning source | https://chatgpt.com/share/6a4df4d1-84cc-83e8-a115-46c6762e22ec |
| Finalized code authority | D:\Embedded Communication Protocols\I2C_EEPROM_Driver_Development\Core |
| Generic protocol reference | NXP UM10204 I2C-bus specification |
| Device reference | Microchip AT24C256C serial EEPROM datasheet |
| Design preset | compact_reference_guide with editorial cover |
| Generated | 08 July 2026 |
| Output footer | Embedded Learning Document |

# 2. Source Files Read

| File | Role in this document |
| --- | --- |
| Core/Src/at24c256.c | EEPROM API implementation and UART self-test |
| Core/Src/i2c.c | Generic STM32F407 I2C1 GPIO/init/read-write backend |
| Core/Src/main.c | System initialization order and test entry point |
| Core/Src/usart.c | USART2 debug output implementation used by TestAllApis |
| Core/Inc/at24c256.h | AT24C256 macros, public API declarations, page helpers |
| Core/Inc/i2c.h | I2C public API declarations and I2C_TIMEOUT macro |
| Core/Inc/usart.h | USART2 public API declarations |

# 3. Learning Objectives

Understand why EEPROM driver code should be split from the generic I2C register driver.

Explain how AT24C256 byte write, random read, page write and sequential read map to I2C bus frames.

Review the finalized source APIs and understand the data path through each layer.

Understand page-boundary logic and why WriteBuffer must split writes into page-safe chunks.

Understand STM32F407 receive handling for 1, 2 and more than 2 bytes.

Know the current implementation limitations before reusing the driver in a larger project.

# 4. System Overview

Application startup
  |
  +-- USART2_Init()                  -> debug terminal output
  +-- SPI/CAN/Flash existing modules -> already present in system
  +-- I2C1_GPIO_Init()               -> PB6=SCL, PB7=SDA, AF4, open-drain
  +-- I2C1_Init_100kHz()             -> PCLK1=32 MHz, CCR=160, TRISE=33
  +-- AT24C256_TestAllApis()         -> EEPROM API verification

I2C bus
  STM32F407 PB6/SCL ----+---- AT24C256 SCL
                        |
                      pull-up

  STM32F407 PB7/SDA ----+---- AT24C256 SDA
                        |
                      pull-up

AT24C256 addressing
  7-bit address used by code: 0x50
  Write bus byte: 0xA0
  Read bus byte : 0xA1

## 4.1 Implemented Project Objective

The project adds an AT24C256 EEPROM driver to an existing embedded communication system. The first goal is a reliable polling-based EEPROM driver that can write one byte, read one byte, write a page, read a buffer, write across page boundaries by chunking, fill the EEPROM, and verify the APIs using USART2 logs.

## 4.2 Hardware and Software Assumptions

| Item | Source-derived value |
| --- | --- |
| MCU family | STM32F407 project code and register names |
| I2C instance | I2C1 |
| I2C pins | PB6 = I2C1_SCL, PB7 = I2C1_SDA |
| Alternate function | AF4 |
| I2C speed | 100 kHz Standard-mode |
| PCLK1 example | 32 MHz |
| EEPROM | AT24C256, 32 KiB, 64-byte page size |
| Debug output | USART2_SendString() and sprintf() in TestAllApis |

# 5. Driver Architecture

main.c
  calls I2C1_GPIO_Init()
  calls I2C1_Init_100kHz()
  calls AT24C256_TestAllApis()

at24c256.c / at24c256.h
  EEPROM-aware layer:
  - validates memory range
  - creates two-byte memory address
  - enforces 64-byte page boundary rules
  - splits large writes into page-safe chunks
  - adds EEPROM write-cycle delay

i2c.c / i2c.h
  generic STM32F407 I2C1 polling layer:
  - configures PB6/PB7
  - configures I2C timing
  - provides MasterWrite()
  - provides MasterWriteRead()

STM32F407 I2C1 peripheral
  generates START, repeated START, STOP, ACK/NACK and shifts bytes

## 5.1 Why This Split Is Correct

| Layer | Should know | Should not know |
| --- | --- | --- |
| Application | Which feature/test to run | I2C flags such as SB, ADDR, TXE, RXNE |
| AT24C256 driver | EEPROM size, page size, word address, write cycle | GPIO registers and I2C timing bits |
| I2C driver | START/STOP, address phase, ACK/NACK, read lengths | EEPROM page boundaries |
| Hardware | Electrical state of SDA/SCL | Application test meaning |

# 6. AT24C256 Device Model

| Device fact | Value in code/document | Driver impact |
| --- | --- | --- |
| Capacity | 32768 bytes | Valid addresses are 0x0000 to 0x7FFF |
| Last address | AT24C256_LAST_ADDR = 0x7FFFU | All APIs reject start/final address above this |
| Page size | AT24C256_PAGE_SIZE = 64U | Page write must not cross a 64-byte boundary |
| Address width | 16-bit memory address | High byte then low byte are sent before data |
| Device address | AT24C256_I2C_ADDR_7BIT = 0x50U | Generic I2C backend shifts it to create bus byte |
| Write cycle | HAL_Delay(5) | Current code waits a fixed 5 ms after write/page write |

## 6.1 Addressing Correction

Engineering correction: The header comment shows the device address format as 1 0 1 0 0 A1 A0 R/W. For the AT24C256 family, the general format is 1010 A2 A1 A0 R/W. The code uses 0x50, which corresponds to A2/A1/A0 all low. Treat 0xA0 and 0xA1 as transmitted bus bytes, not the 7-bit address.

## 6.2 EEPROM Versus SPI NOR Flash

| Aspect | AT24C256 EEPROM | W25Q-style SPI NOR flash |
| --- | --- | --- |
| Erase before write | Not manually required by this driver | Normally required for 0->1 restoration |
| Write granularity | Byte/page write | Page program after sector/block erase |
| Bus selection | I2C address 0x50 | Chip-select line |
| Write completion | Internal write cycle; ACK polling recommended | Busy bit/WIP polling |
| Best learning value | Addressing and page-boundary discipline | Erase/program/read state machine discipline |

# 7. I2C Backend Implementation

## 7.1 GPIO Initialization

void I2C1_GPIO_Init(void)
{
    RCC->AHB1ENR |= RCC_AHB1ENR_GPIOBEN;

    GPIOB->MODER &= ~((3UL << (6U * 2U)) |
                      (3UL << (7U * 2U)));
    GPIOB->MODER |=  ((2UL << (6U * 2U)) |
                      (2UL << (7U * 2U)));

    GPIOB->OTYPER |= ((1UL << 6U) |
                      (1UL << 7U));

    GPIOB->PUPDR |=  ((1UL << (6U * 2U)) |
                      (1UL << (7U * 2U)));

    GPIOB->AFR[0] |=  ((4UL << (6U * 4U)) |
                       (4UL << (7U * 4U)));
}

GPIO note: The final code enables internal pull-ups for first testing and comments that external pull-ups are preferred. That is a good learning setup, but board-level reliability should be checked with real pull-up values and rise-time measurements.

## 7.2 I2C1 100 kHz Initialization

RCC->APB1ENR |= RCC_APB1ENR_I2C1EN;
I2C1->CR1 &= ~I2C_CR1_PE;
I2C1->CR1 |= I2C_CR1_SWRST;
I2C1->CR1 &= ~I2C_CR1_SWRST;
I2C1->CR1 &= ~I2C_CR1_SMBUS;
I2C1->CR1 &= ~I2C_CR1_NOSTRETCH;
I2C1->CR2 &= ~I2C_CR2_FREQ;
I2C1->CR2 |= 32U;
I2C1->CCR = 160U;
I2C1->TRISE = 33U;
I2C1->CR1 |= I2C_CR1_ACK;
I2C1->CR1 |= I2C_CR1_PE;

| Register/action | Implemented value | Meaning |
| --- | --- | --- |
| CR2.FREQ | 32U | APB1 peripheral clock is 32 MHz; this is not the I2C bus speed |
| CCR | 160U | 32 MHz / (2 x 100 kHz) for Standard-mode timing |
| TRISE | 33U | FREQ + 1 for Standard-mode maximum rise-time setting |
| ACK | Enabled by default | Read paths temporarily disable ACK near the final byte |
| NOSTRETCH | Cleared | Allows clock stretching |

## 7.3 ADDR Flag Clearing

static void I2C1_ClearADDR(void)
{
    volatile uint32_t dummy;

    dummy = I2C1->SR1;
    dummy = I2C1->SR2;

    (void)dummy;
}

# 8. Core I2C Transaction Flows

## 8.1 EEPROM Bus Frame Summary

| Operation | Bus frame | Implementation API |
| --- | --- | --- |
| Byte write | START, 0xA0, word high, word low, data, STOP | AT24C256_WriteByte -> I2C1_MasterWrite |
| Random byte read | START, 0xA0, word high, word low, Repeated START, 0xA1, data, NACK, STOP | AT24C256_ReadByte -> I2C1_MasterWriteRead |
| Page write | START, 0xA0, word high, word low, up to 64 data bytes, STOP | AT24C256_WritePage -> I2C1_MasterWrite |
| Sequential read | START, 0xA0, word high, word low, Repeated START, 0xA1, multiple data bytes, NACK, STOP | AT24C256_ReadBuffer -> I2C1_MasterWriteRead |

## 8.2 MasterWrite Flow

Wait SR2.BUSY clear
Generate START
Wait SR1.SB
Write slave_addr_7bit << 1 to DR
Wait SR1.ADDR or handle SR1.AF
Clear ADDR using SR1 then SR2
For each byte:
  wait TXE
  write DR
  check AF
Wait BTF
Generate STOP

## 8.3 MasterWriteRead Flow

Write phase:
  START -> address+W -> ADDR clear -> tx bytes -> BTF

Read phase:
  repeated START -> address+R -> ADDR wait

rx_len == 1:
  clear ACK before clearing ADDR
  clear ADDR
  STOP
  wait RXNE
  read DR
  restore ACK

rx_len == 2:
  set POS and clear ACK before clearing ADDR
  wait BTF
  STOP
  read DR twice
  restore POS/ACK

rx_len > 2:
  ACK normal bytes
  when 3 bytes remain, prepare NACK and STOP timing
  read final three bytes with STM32F4 pipeline-safe ordering

Why the read code is length-specific: At the I2C protocol level, the rule is simple: ACK every byte except the last, NACK the last, then STOP. On STM32F4, the receive pipeline means software must prepare ACK/NACK and STOP early enough for 1-byte, 2-byte, and N-byte cases.

# 9. EEPROM API Layer

| API | Purpose | Key behavior |
| --- | --- | --- |
| AT24C256_WriteByte | Write one byte at one memory address | Builds [addr high, addr low, data], calls MasterWrite, delays 5 ms |
| AT24C256_ReadByte | Read one byte from one memory address | Builds [addr high, addr low], calls MasterWriteRead with rx_len 1 |
| AT24C256_WritePage | Write one page-safe chunk | Rejects NULL, zero len, len > 64, range overflow, and page crossing |
| AT24C256_ReadBuffer | Sequential read from memory | Validates range, writes address, reads len bytes |
| AT24C256_WriteBuffer | Write arbitrary range | Splits into page-safe chunks and calls WritePage repeatedly |
| AT24C256_Fill | Fill entire EEPROM with a byte | Writes a 64-byte page buffer from 0x0000 through 0x7FC0 |
| AT24C256_TestAllApis | UART self-test | Runs byte, page, boundary-crossing buffer, and fill tests |

## 9.1 WriteByte and ReadByte

tx_buf[0] = (uint8_t)(mem_addr >> 8);
tx_buf[1] = (uint8_t)(mem_addr & 0xFFU);
tx_buf[2] = data;

I2C1_MasterWrite(AT24C256_I2C_ADDR_7BIT, tx_buf, 3U);
HAL_Delay(5);

addr_buf[0] = (uint8_t)(mem_addr >> 8);
addr_buf[1] = (uint8_t)(mem_addr & 0xFFU);

I2C1_MasterWriteRead(AT24C256_I2C_ADDR_7BIT,
                     addr_buf,
                     2U,
                     read_data,
                     1U);

## 9.2 WritePage Boundary Protection

offset = PAGE_OFFSET(mem_addr);
page_space = AT24C256_PAGE_SIZE - offset;

if(len > page_space)
{
    return E_NOT_OK;
}

tx_buf[0] = (uint8_t)(mem_addr >> 8);
tx_buf[1] = (uint8_t)(mem_addr & 0xFFU);

for(i = 0U; i < len; i++)
{
    tx_buf[i + 2U] = data[i];
}

Important correction from the learning path: The final WritePage implementation correctly allows offset 0. Page-aligned addresses such as 0x0040 are valid and are often the best case. The earlier idea that offset 0 should be an error was corrected.

## 9.3 WriteBuffer Page-Splitting Logic

while(bytes_remaining > 0U)
{
    page_offset = PAGE_OFFSET(current_addr);
    page_space = AT24C256_PAGE_SIZE - page_offset;

    if(bytes_remaining > page_space)
    {
        chunk_size = page_space;
    }
    else
    {
        chunk_size = bytes_remaining;
    }

    AT24C256_WritePage(current_addr,
                       &data[data_index],
                       chunk_size);

    current_addr += chunk_size;
    data_index += chunk_size;
    bytes_remaining -= chunk_size;
}

## 9.4 ReadBuffer and Sequential Read

ReadBuffer does not need to split at EEPROM page boundaries because sequential reads can continue across address boundaries until the requested length is complete, as long as the final requested address is inside the device range. The code still validates the start address and final address before calling MasterWriteRead.

## 9.5 Fill Operation

Fill prepares one 64-byte page buffer containing the selected value, then writes that page buffer repeatedly from address 0x0000 to 0x7FC0 in 64-byte steps. This tests every EEPROM page, but it also consumes write endurance, so it should be used intentionally.

## 9.6 API Parameter Validation Pattern

The EEPROM APIs consistently reject NULL data pointers, zero lengths, start addresses outside 0x0000 to 0x7FFF, and final-address overflow. WritePage adds an extra rule: the requested length must fit inside the current 64-byte page.

## 9.7 Why WritePage Is Strict but WriteBuffer Is Flexible

WritePage is intentionally strict because a hardware page write that crosses a page boundary can wrap inside the same EEPROM page and corrupt earlier bytes. WriteBuffer is flexible because it calculates safe chunk sizes and calls WritePage repeatedly.

# 10. Test and Validation Flow

AT24C256_TestAllApis()
  TEST 1: WriteByte + ReadByte
    address 0x0010, data 0x5A

  TEST 2: WritePage + ReadBuffer
    address 0x0040, 16 bytes, page-aligned

  TEST 3: WriteBuffer across page boundary + ReadBuffer
    address 0x003C, length 100
    first chunk: 4 bytes to finish page
    remaining bytes continue from 0x0040

  TEST 4: Fill EEPROM with 0xFF
    read first 32 bytes for verification

| Test | Expected pass condition | What it proves |
| --- | --- | --- |
| WriteByte/ReadByte | Read 0x5A from 0x0010 | Byte write, random read, write cycle delay |
| WritePage/ReadBuffer | 16-byte read matches page_write_data | Page write with 16-bit word address |
| WriteBuffer crossing page | 100-byte read matches source | Chunking prevents page rollover corruption |
| Fill | First 32 bytes read back 0xFF | Repeated full-page writes across the device |

# 11. Engineering Review of Final Code

## 11.1 Strengths

Clean separation between generic I2C backend and EEPROM-specific driver.

Correct two-byte memory addressing before EEPROM data operations.

Correct rejection of page-crossing writes in WritePage.

WriteBuffer correctly splits arbitrary-length writes into page-safe chunks.

MasterWriteRead handles STM32F4 1-byte, 2-byte, and N-byte receive cases separately.

TestAllApis verifies byte, page, boundary-crossing buffer, and fill workflows.

## 11.2 Risks and Corrections to Track

| Item | Current state | Recommendation |
| --- | --- | --- |
| I2C_TIMEOUT | Defined in i2c.h but not used in polling loops | Add timeout handling to every BUSY/SB/ADDR/TXE/BTF/RXNE wait |
| ACK polling | Not implemented; fixed HAL_Delay(5) after writes | Replace delay with ACK polling for faster and more deterministic writes |
| PAGE_SPACE macro | Unused and malformed: PAGE_OFFSET has no argument | Fix or remove to prevent future misuse |
| Address comment | Header comment omits A2 | Update comment to 1010 A2 A1 A0 R/W if package pins are used |
| Error flags | AF handled in several address/write waits; BERR/ARLO not broadly handled | Add recovery paths for BERR, ARLO, OVR and stuck BUSY |
| Fill endurance | Writes the full EEPROM | Use sparingly and document endurance impact |
| System blocking | Other subsystem loops also block in main.c | Make driver timeouts a system-wide rule |

## 11.3 Production Hardening Checklist

Convert all polling waits to timeout-based waits returning E_NOT_OK on expiry.

Add ACK polling after WriteByte, WritePage, and every WriteBuffer chunk.

Add I2C bus recovery for SDA/SCL stuck low before initialization.

Centralize I2C flag error handling for AF, BERR, ARLO, OVR and TIMEOUT.

Add API documentation for blocking behavior and maximum write duration.

Avoid full EEPROM fill in normal application startup.

Add read-back verification only where data integrity matters, not blindly after every write.

# 12. Protocol Comparison for Driver Thinking

| Driver concern | I2C EEPROM | SPI flash | UART | CAN |
| --- | --- | --- | --- | --- |
| Device selection | 7-bit address | Chip select | Point-to-point or bus wiring | Message ID and controller filtering |
| Write completion | Internal write cycle / ACK polling | WIP/busy bit polling | TXE/TC flags | Mailbox/status flags |
| Memory geometry | Page write boundary | Erase sectors and program pages | No memory geometry | Frame payload/DLC |
| Failure style | NACK, stuck BUSY, clock stretch | No MISO, CS/timing, WIP stuck | Framing/overrun | ACK/error/passive/bus-off |
| Best abstraction | Device driver over generic I2C | Device driver over generic SPI | Stream driver | Network/frame driver |

# 13. Source-Verified Implementation Facts

| Fact | Verified from finalized source |
| --- | --- |
| I2C pins | I2C1_GPIO_Init() configures PB6/PB7 as AF4 open-drain |
| I2C timing | I2C1_Init_100kHz() uses CR2=32, CCR=160, TRISE=33 |
| Device address | AT24C256_I2C_ADDR_7BIT is 0x50U |
| Memory range | AT24C256_LAST_ADDR is 0x7FFFU |
| Page size | AT24C256_PAGE_SIZE is 64U |
| Write completion | WriteByte and WritePage use HAL_Delay(5) |
| Test entry | main.c calls AT24C256_TestAllApis() after I2C init |

# 14. Primary References

| Reference | Used for |
| --- | --- |
| https://www.nxp.com/docs/en/user-guide/UM10204.pdf | I2C bus model, START/STOP, ACK/NACK, repeated START, open-drain behavior |
| https://ww1.microchip.com/downloads/aemDocuments/documents/MPD/ProductDocuments/DataSheets/AT24C256C-I%C2%B2C-Compatible-%28Two-Wire%29-Serial-EEPROM-256-Kbit-%2832%2C768x8%29-Data-Sheet-DS20006042.pdf | AT24C256C capacity, page size, addressing, write cycle, page write and sequential read behavior |
| https://chatgpt.com/share/6a4df4d1-84cc-83e8-a115-46c6762e22ec | User-specific learning path, API evolution, corrections and driver-development reasoning |
| D:\Embedded Communication Protocols\I2C_EEPROM_Driver_Development\Core | Final code authority for functions, constants, snippets and initialization order |

# 15. Conclusion

This project has crossed the important line from a working I2C transaction to a reusable EEPROM driver. The strongest learning points are the layer split, 16-bit memory addressing, page-safe writes, STM32F4 receive-state-machine timing, and the difference between a learning driver and a production driver. The current code is a solid learning baseline; the next engineering upgrade is timeout handling and ACK polling.