EMBEDDED COMMUNICATION IMPLEMENTATION GUIDE

I2C Initialization, GPIO Setup, and AT24C256 Byte Access

STM32F407 I2C1 on PB6/PB7, Open-Drain GPIO Reasoning, Write 1 Byte, Read 1 Byte, and Debugging Corrections

Document intent: A long-term learning reference for the first practical I2C implementation stage: configure GPIO/I2C correctly, understand the STM32 flag sequence, write one byte to AT24C256 EEPROM, read one byte back, and debug stuck polling loops without memorizing blindly.

| Learning area | Coverage |
| --- | --- |
| GPIO foundation | PB6/PB7 alternate function, open-drain output type, pull-ups, speed, input sampling |
| I2C init | Peripheral clocking, PE disable/configure/enable order, CR2 FREQ, CCR, TRISE, ACK |
| Write 1 byte | START, address write byte, word address high/low, data, BTF, STOP, write cycle |
| Read 1 byte | Dummy write, repeated START, receiver mode, NACK/STOP timing, RXNE read |
| Corrections | 7-bit address vs 8-bit bus byte, ADDR clearing, debugger SFR side effects, WP behavior |
| Debugging | START stuck, ADDR stuck, AF/BERR/ARLO checks, BUSY stuck, timeout discipline |

# 1. Document Control and Scope

| Control item | Value |
| --- | --- |
| Document type | Embedded I2C implementation learning guide |
| Primary learning source | https://chatgpt.com/share/6a4cb37a-a39c-83e8-98e8-778efba94ded |
| Generic protocol authority | NXP UM10204 I2C-bus specification and user manual |
| Device authority | Microchip AT24C256C serial EEPROM data sheet |
| MCU-specific reference | ST RM0090, STM32F405/407 reference manual, I2C chapter |
| MCU assumption | STM32F407 family; I2C1 mapped to PB6 SCL and PB7 SDA |
| Peripheral clock assumption | PCLK1 = 32 MHz for the worked 100 kHz timing example |
| Design preset | compact_reference_guide with editorial cover |
| Generated | 07 July 2026 |

# 2. Scope Boundary

This document is based on the shared theory chat. No finalized project source files were attached for this I2C stage, so the document does not claim that a specific C file or function body is final code. Register names and sequences are included as implementation guidance and must be reconciled with your actual source when you attach it.

Code authority rule: For later project documents, finalized source files should become the authority. For this document, the authority hierarchy is: shared chat for learning intent, NXP for I2C protocol behavior, Microchip for AT24C256 behavior, and STM32F4 RM0090 for I2C1 register/flag semantics.

# 3. Learning Objectives

Explain why PB6/PB7 must be alternate-function open-drain for I2C1.

Understand what the pull-up resistor does electrically and why internal pull-ups are usually not enough for a robust bus.

Write an I2C1 initialization sequence for 100 kHz Standard-mode using PCLK1 = 32 MHz.

Understand the difference between the AT24C256 7-bit address 0x50 and bus bytes 0xA0/0xA1.

Trace a byte write transaction from START to STOP and internal EEPROM write cycle.

Trace a one-byte random read using dummy write, repeated START, NACK, STOP and RXNE.

Debug stuck polling loops by checking line levels, PE, BUSY, SB, ADDR, TXE, RXNE, BTF, AF, BERR and ARLO.

# 4. System-Level Diagram

3.3 V
  |
  +-- Rp(SCL) --+------------------ PB6 / I2C1_SCL / AF4
  |             |
  |             +------------------ AT24C256 SCL
  |
  +-- Rp(SDA) --+------------------ PB7 / I2C1_SDA / AF4
                |
                +------------------ AT24C256 SDA

STM32F407 GND --------------------- AT24C256 GND
AT24C256 A2/A1/A0 ----------------- address select pins
AT24C256 WP ----------------------- write protect input

Only two bus wires are shared. The EEPROM is selected by address,
not by a chip-select pin.

## 4.1 Practical Assumptions

| Item | Working assumption | Why it matters |
| --- | --- | --- |
| I2C instance | I2C1 | Used by the STM32 register names and PB6/PB7 mapping |
| SCL | PB6, AF4 | Clock line generated by controller during transactions |
| SDA | PB7, AF4 | Bidirectional data line; driven by controller or target at different phases |
| Speed | 100 kHz Standard-mode | Simpler timing margin for first bring-up |
| PCLK1 | 32 MHz | Used for CR2 FREQ, CCR and TRISE example values |
| EEPROM address pins | A2/A1/A0 low unless board says otherwise | Produces 7-bit address 0x50 |

# 5. GPIO Understanding for I2C

## 5.1 Why Alternate Function Mode Is Required

GPIO mode selects which internal path owns the pin. For I2C1 on PB6/PB7, the pin must be routed to the alternate-function peripheral path, not used as ordinary software-controlled GPIO output. The I2C hardware state machine then controls when the pin releases high and when it pulls low.

| GPIO field | I2C setting | Reasoning |
| --- | --- | --- |
| MODER | Alternate function | Connects PB6/PB7 to I2C1 instead of GPIO output logic |
| AFRL | AF4 for PB6/PB7 | Selects I2C1/I2C alternate-function mapping for the pins |
| OTYPER | Open-drain | Allows any device to pull the shared line low safely |
| PUPDR | Pull-up or no-pull with external pull-ups | The bus must idle high; external pull-ups are preferred |
| OSPEEDR | Medium/high enough for edge rate | Controls output slew behavior when pulling low |
| IDR | Readable line state | Useful to confirm SCL and SDA are high before START |

## 5.2 Open-Drain Mental Model

Open-drain output can do only two things:

Drive 0  -> transistor ON  -> line forced LOW
Release -> transistor OFF -> pull-up resistor restores HIGH

No I2C device actively drives a shared bus line high.
This prevents direct high-vs-low output contention.

Important correction: Open-drain is required for both SDA and SCL. SDA is obviously shared and bidirectional; SCL also needs open-drain behavior because targets may hold SCL low for clock stretching and multi-controller arbitration must remain electrically safe.

## 5.3 Why Open-Drain Helps with Multiple Devices

Shared bus truth table:

Device A releases, Device B releases -> pull-up makes line HIGH
Device A pulls low, Device B releases -> line LOW
Device A releases, Device B pulls low -> line LOW
Device A pulls low, Device B pulls low -> line LOW

The bus behaves like a wired-AND:
HIGH appears only when every connected device releases the line.

## 5.4 Pull-Up Resistor Sizing Reminder

| Check | Formula or rule | Meaning |
| --- | --- | --- |
| Maximum pull-up | Rp(max) = tr(max) / (0.8473 x Cb) | Too large makes rising edges too slow |
| Minimum pull-up | Rp(min) = (VDD - VOL(max)) / IOL | Too small demands too much sink current |
| 100 kHz Standard-mode | tr(max) commonly 1000 ns | More tolerant during first bring-up |
| 400 kHz Fast-mode | tr(max) commonly 300 ns | Requires stronger pull-up/lower capacitance |
| Internal pull-up | Often tens of kOhm | Useful for idle testing, weak for reliable speed operation |

# 6. I2C1 Initialization Function Understanding

## 6.1 Initialization Flow

Enable GPIOB clock
Configure PB6/PB7 as AF4, open-drain, pulled high, suitable speed
Enable I2C1 peripheral clock on APB1
Keep or force I2C peripheral disabled while timing is configured
Program CR2.FREQ based on APB1 clock in MHz
Program CCR for target SCL speed
Program TRISE for maximum allowed rise time
Enable ACK if receiver operations will be used
Enable peripheral by setting CR1.PE
Verify bus idle: SCL = HIGH and SDA = HIGH

## 6.2 Worked 100 kHz Timing Example

| Register concept | Value for PCLK1 = 32 MHz | Reasoning |
| --- | --- | --- |
| CR2.FREQ | 32 | The I2C peripheral needs the APB1 clock frequency in MHz |
| CCR, Standard-mode | CCR = 32,000,000 / (2 x 100,000) = 160 | Standard-mode uses symmetric low/high timing in the simple case |
| TRISE | TRISE = (1000 ns / 31.25 ns) + 1 = 33 | APB1 period is 1/32 MHz = 31.25 ns |
| CR1.ACK | Set for normal receiver capability | May be cleared temporarily before final byte of a one-byte read |
| CR1.PE | Set after configuration | The peripheral must be enabled before generating START |

Layer separation: I2C protocol timing is generic, but CR2/CCR/TRISE fields are STM32F4 peripheral-specific. Other microcontrollers implement the same bus behavior with different register models.

## 6.3 Initialization Checklist

Confirm the selected pins really support I2C1 alternate function.

Confirm no other peripheral or board function is using PB6/PB7.

Confirm the external module has pull-ups or add pull-ups yourself.

Confirm APB1 clock frequency before calculating CCR/TRISE.

Do not generate START before CR1.PE is set.

Before START, verify SCL and SDA read high through GPIO IDR or a logic analyzer.

# 7. STM32F407 I2C Internal State Machine

Firmware writes CR1.START
        |
        v
Hardware creates START on bus and sets SR1.SB
        |
Firmware writes address byte to DR
        |
        v
Hardware shifts address, samples ACK, sets ADDR on success
        |
Firmware clears ADDR by reading SR1 then SR2
        |
        v
Data phase:
  Transmitter -> TXE/BTF guide writes to DR
  Receiver    -> ACK/NACK setup and RXNE guide reads from DR
        |
Firmware writes CR1.STOP at the correct phase

| Flag | Meaning in this learning stage | How it is cleared or consumed |
| --- | --- | --- |
| BUSY | Bus is occupied | Clears when STOP is detected and bus becomes idle |
| SB | START condition generated | Read SR1 then write DR with address byte |
| ADDR | Address phase completed and ACK received | Read SR1 then SR2 |
| TXE | Data register empty | Write next byte to DR |
| RXNE | Data register not empty | Read DR |
| BTF | Byte transfer finished | Read/write DR or generate STOP/START depending on mode |
| AF | Acknowledge failure/NACK | Clear flag and generate STOP/recover |
| BERR | Bus error | Clear and recover bus/peripheral |
| ARLO | Arbitration lost | Clear and retry when bus free |

Debugger warning: STM32 I2C flags are read-sequence sensitive. A live SFR/register view can read SR1/SR2 in the background and accidentally clear flags such as ADDR. When debugging I2C, trust breakpoints and watch expressions carefully.

# 8. AT24C256 Addressing Model

## 8.1 7-Bit Address Versus Bus Byte

| Concept | Value when A2/A1/A0 are low | Use |
| --- | --- | --- |
| 7-bit target address | 0x50 | Used by driver APIs and address calculations |
| Write bus byte | 0xA0 | Sent on bus as 0x50 << 1 with R/W = 0 |
| Read bus byte | 0xA1 | Sent on bus as (0x50 << 1) ¦ 1 |
| Address byte bits | 1010 A2 A1 A0 R/W | Device type code plus hardware address pins plus direction |

Important correction: 0xA0 is not the 7-bit EEPROM address. The 7-bit address is 0x50 when A2/A1/A0 are low. 0xA0 and 0xA1 are the 8-bit bus bytes transmitted during write and read address phases.

## 8.2 Memory Word Address

| Memory property | AT24C256C value | Implementation effect |
| --- | --- | --- |
| Capacity | 256 Kbit = 32,768 x 8 = 32 KiB | Valid byte addresses are 0x0000 through 0x7FFF |
| Word address bytes | Two bytes | Send high address byte first, then low address byte |
| Page size | 64 bytes | Page writes must not cross a 64-byte page boundary |
| Write cycle | Internal nonvolatile programming after STOP | Use delay or ACK polling before next write/read |
| WP pin | Write protect input | Does not normally prevent address ACK; it prevents actual memory programming |

# 9. Write 1 Byte to AT24C256

## 9.1 Bus-Level Frame

START
0xA0                 -> EEPROM address + Write bit
ACK from EEPROM
Memory address high  -> A14..A8 in lower bits of first word-address byte
ACK from EEPROM
Memory address low   -> A7..A0
ACK from EEPROM
Data byte
ACK from EEPROM
STOP
EEPROM internal write cycle begins

## 9.2 STM32 Polling Sequence

Wait until SR2.BUSY is clear, or use a timeout and recover if the bus is stuck.

Set CR1.START and wait for SR1.SB.

Write 0xA0 to DR. This clears SB and starts the address phase.

Wait for SR1.ADDR. If SR1.AF sets instead, the target NACKed the address.

Clear ADDR by reading SR1 followed by SR2.

Wait TXE, write memory address high byte.

Wait TXE, write memory address low byte.

Wait TXE, write the data byte.

Wait BTF to confirm the final byte moved through the shifter and ACK phase completed.

Generate STOP. Then wait the EEPROM write cycle or use ACK polling before the next access.

## 9.3 Why BTF Matters on the Final Byte

TXE only means the data register can accept another byte. BTF means the byte transfer finished at the bus level. For the final data byte of a write operation, waiting for BTF before STOP helps ensure the data and ACK phase completed. A robust driver should still include a timeout and check AF, BERR and ARLO.

| If polling stops at | Likely meaning | Debug action |
| --- | --- | --- |
| SB wait | START was not generated | Check PE, SCL/SDA high, GPIO AF/open-drain/pull-ups and BUSY |
| ADDR wait | No address ACK or flag was disturbed | Check 0x50/0xA0, A pins, wiring, AF flag and debugger SFR view |
| TXE wait | Data register not ready | Check peripheral state and previous flag clearing |
| BTF wait | Byte not fully transferred | Check clock stretching, NACK, bus waveform and STOP timing |
| After STOP | EEPROM busy internally | Use ACK polling or delay up to the datasheet write-cycle maximum |

# 10. Read 1 Byte from AT24C256

## 10.1 Why a Dummy Write Is Needed

A random read from EEPROM first needs to load the target memory address. That address phase is performed as a write operation without sending data to be stored. The controller then issues a repeated START and changes direction to receiver mode.

Dummy write phase:
START -> 0xA0 -> ACK -> memory high -> ACK -> memory low -> ACK

Direction change:
Repeated START -> 0xA1 -> ACK

Single-byte receive:
Controller prepares NACK for the last byte
Controller clears ADDR
Controller requests STOP
EEPROM shifts data byte
RXNE sets
Firmware reads DR

## 10.2 STM32 One-Byte Read Sequence

Generate START and send 0xA0 to select EEPROM in write direction.

Clear ADDR, then send the two-byte memory address.

Wait until the address bytes have left the transmitter path.

Generate repeated START, wait SB, then send 0xA1.

Wait ADDR for the read address ACK.

Clear ACK before clearing ADDR, because only one byte will be received.

Clear ADDR by reading SR1 then SR2.

Generate STOP before or around the final byte completion according to the STM32 single-byte receive sequence.

Wait RXNE, read DR, then restore ACK for future multi-byte receives.

Why NACK/STOP appear before DR read: For the final byte, the controller must tell the EEPROM that no more data is wanted. On STM32F4, software prepares this before reading DR because the peripheral must control the ninth clock ACK/NACK and STOP timing while the byte is being received.

## 10.3 Generic I2C Frame Versus EEPROM Frame

| Frame type | Generic I2C structure | AT24C256-specific addition |
| --- | --- | --- |
| Write | START, address+W, data bytes, STOP | The first two data bytes are the memory word address |
| Current address read | START, address+R, data, NACK, STOP | Uses EEPROM internal address pointer |
| Random read | Write address phase, repeated START, address+R, data, NACK, STOP | Dummy write loads target memory address |
| Sequential read | Address+R followed by ACK after each byte | EEPROM auto-increments until controller NACKs |

# 11. Debugging the First I2C Bring-Up

## 11.1 START Stuck

Stuck at:
CR1.START = 1
while (!(SR1 & SB)) waits forever

Check:
1. CR1.PE = 1
2. GPIOB clock and I2C1 clock enabled
3. PB6/PB7 are AF4, open-drain and pulled high
4. SCL and SDA physically read HIGH before START
5. SR2.BUSY is not already stuck
6. No other device is holding SDA or SCL low

## 11.2 ADDR Stuck

| Symptom | Meaning | Correction |
| --- | --- | --- |
| ADDR never sets, AF sets | EEPROM did not ACK address | Check address pins, 0x50/0xA0, wiring, power and pull-ups |
| ADDR set in SFR but loop behaves oddly | Debugger may have read SR1/SR2 | Disable live register polling or step carefully |
| Removed ADDR wait | Unsafe workaround | Keep ADDR wait; clear it only after it becomes set |
| WP high | Write protected EEPROM | Address can still ACK; data may not be programmed |

## 11.3 Error Flags to Teach Yourself Early

AF: target NACKed address or data. Clear it and generate STOP/recover.

BERR: illegal bus condition. Check waveform, noise, pin configuration and recovery.

ARLO: another controller won arbitration. Retry only when bus is free.

OVR: receiver data was not read in time in some receive scenarios.

TIMEOUT: do not leave production polling loops without a maximum wait count.

System-level lesson: Your chat mentioned the broader system sometimes gets stuck in another device path. The same fix applies here: every polling driver must have timeout and recovery paths. A single missing timeout can freeze the complete embedded application.

# 12. Important Understandings to Retain

| Understanding | Correct long-term model |
| --- | --- |
| I2C is half-duplex | SDA direction changes by transaction phase; transmit and receive do not happen simultaneously on SDA |
| SCL is generated by controller | Targets may stretch SCL low, so SCL is not simply a push-pull output |
| START requires idle-high lines | SDA falling while SCL high is only possible if both lines can idle high first |
| ADDR clearing is special | On STM32F4, clear ADDR by reading SR1 then SR2 |
| SB clearing is special | Read SR1 then write DR with the address byte |
| RXNE clearing is simple | Read DR |
| Final read byte uses NACK | The controller NACKs the last byte to end the read |
| 0x50 is not 0xA0 | 0x50 is 7-bit address; 0xA0/0xA1 are transmitted bus bytes |
| EEPROM writes are slow | STOP begins the internal write cycle; use ACK polling or delay |

# 13. Verification Checklist Before Trusting the Driver

| Check | Expected result |
| --- | --- |
| Idle line measurement | SCL = high, SDA = high before START |
| START waveform | SDA falls while SCL is high |
| Address write byte | Analyzer decodes 0xA0 for AT24C256 write phase |
| Address read byte | Analyzer decodes 0xA1 after repeated START |
| Memory address order | High byte first, low byte second |
| Write data ACK | EEPROM ACKs the data byte |
| Random read final byte | Controller NACKs last byte and sends STOP |
| Readback | Read byte matches previously written byte after write cycle completion |
| Error handling | Timeouts exist for every polling wait |

# 14. Corrections Applied from Engineering Review

| Area | Correction documented |
| --- | --- |
| AT24C256 address | Use 1010 A2 A1 A0 R/W. Do not call 0xA0 the 7-bit address. |
| ADDR wait | Do not remove the ADDR polling loop. Wait for ADDR, then clear using SR1 followed by SR2. |
| One-byte read | Prepare NACK/STOP according to STM32 receiver timing before the final DR read. |
| BTF confusion | TXE is not the same as BTF. The final write byte should be allowed to complete before STOP. |
| WP pin | WP high protects memory programming; it should not be treated as the normal cause of address NACK. |
| SFR debug | Live register views can perturb read-to-clear flag sequences. |
| Pull-ups | External pull-ups are the professional design reference; internal pull-ups are weak for reliable bus timing. |

# 15. Primary References

| Reference | Used for |
| --- | --- |
| https://www.nxp.com/docs/en/user-guide/UM10204.pdf | Generic I2C protocol, open-drain bus, ACK/NACK, clock stretching, arbitration and pull-up formulas |
| https://ww1.microchip.com/downloads/aemDocuments/documents/MPD/ProductDocuments/DataSheets/AT24C256C-I%C2%B2C-Compatible-%28Two-Wire%29-Serial-EEPROM-256-Kbit-%2832%2C768x8%29-Data-Sheet-DS20006042.pdf | AT24C256C addressing, capacity, byte/page write, random read, sequential read, ACK polling and write cycle |
| ST RM0090, STM32F405/407 reference manual, I2C chapter | STM32F407 I2C1 register names, CR1/CR2/CCR/TRISE/SR1/SR2/DR and flag-clearing sequences |
| https://chatgpt.com/share/6a4cb37a-a39c-83e8-98e8-778efba94ded | User-specific learning path, questions, misconceptions, debugging observations and requested scope |

# 16. Next Learning Stage

Attach the finalized I2C source files when available so the next document can verify exact function names and register writes.

Extend from byte write/read to page write, current-address read, sequential read and ACK polling APIs.

Add timeout return values and bus recovery before treating the driver as reusable.

Capture a logic-analyzer trace and compare every decoded byte against the frame diagrams in this document.

# 17. Conclusion

The first practical I2C driver is mostly about respecting layers. GPIO makes the electrical bus possible, the I2C peripheral state machine generates protocol events, the EEPROM imposes its own addressing and write-cycle rules, and firmware must sequence flags carefully. Once those layers are separated, write-one-byte and read-one-byte stop feeling like magic and become predictable bus transactions.