EMBEDDED COMMUNICATION FOUNDATION

I2C Protocol Foundation

Internal Functionality, Electrical Design, Timing Calculations, and AT24C256C EEPROM Analysis

Document intent: A theory-only, long-term reference for building a correct mental model of I2C before writing firmware. It combines the shared learning chat with primary-source verification and uses the AT24C256C EEPROM as the practical device example.

| Foundation area | Coverage |
| --- | --- |
| Bus behavior | SDA, SCL, open-drain, pull-ups, START, STOP, ACK and NACK |
| Protocol mechanics | Addressing, repeated START, reads, writes, arbitration and clock stretching |
| Calculations | Rise time, pull-up range, bus timing, transfer clocks and EEPROM boundaries |
| Device study | AT24C256C capacity, pins, addressing, page write, ACK polling and read modes |
| Comparisons | I2C versus UART, SPI and CAN |
| Implementation boundary | No driver code or unverified register sequence is included |

# 1. Document Control and Scope

| Control item | Value |
| --- | --- |
| Document type | Theory-only embedded protocol foundation guide |
| Shared learning source | https://chatgpt.com/share/6a4a7784-ed40-83ee-8254-23398e9cf44e |
| Protocol authority | NXP UM10204, I2C-bus specification and user manual, Rev. 7.0 |
| Device authority | Microchip AT24C256C Data Sheet DS20006042C |
| Design preset | compact_reference_guide with editorial cover |
| Terminology | Controller/target are used; master/slave are shown only as familiar legacy terms |
| Theory boundary | STM32F407 timing is an illustration, not finalized firmware |
| Generated | 05 July 2026 |

# 2. Learning Objectives

Explain why I2C needs only two shared wires and no per-device chip-select signal.

Understand open-drain electrical behavior and calculate a valid pull-up resistor range.

Read an I2C waveform as START, address, direction, ACK/NACK, data and STOP phases.

Distinguish 7-bit target addresses from the 8-bit address byte transmitted on the bus.

Understand repeated START, clock stretching, arbitration and bus ownership.

Calculate bus periods, transfer overhead and STM32F407 standard-mode timing values.

Analyze AT24C256C capacity, addressing, page boundaries, write-cycle behavior and read modes.

Choose I2C appropriately by comparing it with UART, SPI and CAN.

# 3. Scope and Non-Goals

This document builds protocol and device foundations. It does not provide a final GPIO initialization function, peripheral initialization function, polling driver, interrupt driver, DMA design or application source code. Those should be created only after the bus-level model is stable.

Reading rule: Treat the NXP specification as the authority for generic I2C behavior and the Microchip datasheet as the authority for AT24C256C-specific behavior. Microcontroller reference manuals govern register sequencing.

# 4. What I2C Is

I2C, Inter-Integrated Circuit, is a synchronous serial bus intended primarily for communication between integrated circuits over short board-level connections. One or more controllers communicate with addressed targets using a shared serial data line, SDA, and serial clock line, SCL.

| Property | I2C foundation |
| --- | --- |
| Wires | Two shared lines: SDA and SCL, plus common ground |
| Clocking | Synchronous; the active controller supplies SCL |
| Selection | Targets are selected by address, not a chip-select pin |
| Direction | SDA is bidirectional; direction can change within a combined transfer |
| Electrical type | Open-drain/open-collector with pull-up resistors |
| Transfer unit | 8 data bits followed by one ACK/NACK bit |
| Common modes | 100 kbit/s Standard-mode, 400 kbit/s Fast-mode, 1 Mbit/s Fast-mode Plus |
| Typical devices | EEPROMs, sensors, RTCs, PMICs, GPIO expanders and ADCs |

# 5. System-Level Bus Diagram

VDD
                          |
                    +-----+-----+
                    |           |
                  Rp(SCL)     Rp(SDA)
                    |           |
Controller           |           |
STM32            SCL +-----------+---- AT24C256C EEPROM
                 SDA +-----------+---- Sensor
                    |           | +---- RTC
                    |           |
                   GND--------- common ground

Every target sees every clock and address.
Only the addressed target acknowledges and participates in the data phase.

## 5.1 Address-Based Selection

All targets remain physically connected to SDA and SCL. Each listens to the address after START. A matching target pulls SDA low for ACK; nonmatching targets release the bus. Addressing replaces the separate chip-select wires used by SPI.

# 6. Open-Drain Electrical Foundation

## 6.1 Push-Pull Versus Open-Drain

| Behavior | Push-pull | Open-drain |
| --- | --- | --- |
| Logic 0 | Actively drives low | Actively pulls low |
| Logic 1 | Actively drives high | Releases line; pull-up creates high |
| Shared-wire safety | Opposite outputs can fight | Any device may safely pull low |
| Typical use | SPI clock/data, UART TX | I2C SDA and SCL |
| Rising edge | Driven by output transistor | Set by pull-up resistance and bus capacitance |

Open-drain line model

VDD
 |
Rp
 |
 +---------- SDA or SCL ---------- receivers
 |                 |
NMOS A           NMOS B
 |                 |
GND               GND

NMOS ON  -> device asserts logic 0.
NMOS OFF -> device releases the line; Rp creates logic 1.

## 6.2 Wired-AND Behavior

The physical bus is low if any connected device pulls it low. The bus is high only when every device releases it. In positive logic this is called wired-AND behavior. It enables acknowledgment, clock stretching and nondestructive arbitration.

| Device A | Device B | Bus result |
| --- | --- | --- |
| Release | Release | HIGH through pull-up |
| Pull LOW | Release | LOW |
| Release | Pull LOW | LOW |
| Pull LOW | Pull LOW | LOW |

## 6.3 Why Both SDA and SCL Are Open-Drain

SDA changes ownership: controller sends address/write data, target sends ACK/read data, and controller ACKs received bytes.

SCL is normally timed by the controller, but a target may optionally hold it low to stretch the clock.

Multiple controllers can synchronize clocks and arbitrate without output-driver conflict.

A released bus naturally idles high and allows START/STOP detection.

# 7. Pull-Up Resistor and Rise-Time Calculations

Pull-up selection is a range problem. The resistor must be small enough to charge the total bus capacitance within the allowed rise time, but large enough that every low-driving device can sink the resulting current while meeting its VOL limit.

Maximum pull-up from rise time:
Rp(max) = tr(max) / (0.8473 x Cb)

Minimum pull-up from sink current:
Rp(min) = (VDD - VOL(max)) / IOL

Where:
tr  = allowed 30%-to-70% rise time
Cb  = total bus capacitance: pins + traces + connectors + probes
VOL = maximum valid low voltage
IOL = guaranteed sink current of the weakest participating device

## 7.1 Worked 3.3 V, 100 pF Example

| Calculation | Standard-mode | Fast-mode | Fast-mode Plus |
| --- | --- | --- | --- |
| Rise-time limit used | 1000 ns | 300 ns | 120 ns |
| Rp(max) for 100 pF | 11.8 kohm | 3.54 kohm | 1.42 kohm |
| Illustrative Rp(min) | 967 ohm at 3 mA | 967 ohm at 3 mA | 145 ohm at 20 mA |
| Interpretation | 4.7-10 kohm may fit | About 2.2-3.3 kohm may fit | Requires Fm+ capable devices |

Engineering caution: The example is not a universal resistor recommendation. Use the real bus capacitance and every device's VOL/IOL limits. A module marked 103 likely has 10 kohm pull-ups, but that value is not automatically valid at 400 kHz or 1 MHz.

## 7.2 What Changes the Rise Time

Longer traces or cables increase capacitance.

Each device pin, connector and analyzer probe adds capacitance.

Larger resistance reduces low-level current but slows rising edges.

Smaller resistance speeds rising edges but increases sink current and power.

The slowest edge and weakest sink capability set the safe operating point.

# 8. Bus States and Special Conditions

| Condition | SDA behavior while SCL is HIGH | Meaning |
| --- | --- | --- |
| Idle | SDA HIGH, SCL HIGH | No transfer is active |
| START (S) | SDA changes HIGH to LOW | New transfer begins |
| Repeated START (Sr) | SDA changes HIGH to LOW without prior STOP | Combined transfer continues with new address/direction |
| STOP (P) | SDA changes LOW to HIGH | Transfer ends and bus is released |
| Normal data | SDA must remain stable | Receiver samples the bit |

## 8.1 Complete Transfer Structure

S | 7-bit target address | R/W | ACK | DATA[7:0] | ACK | ... | P
      7 clocks             1     1       8 clocks     1

Every address or data byte occupies nine SCL pulses:
8 information bits + 1 acknowledge bit.

Data is transferred MSB first.

# 9. ACK and NACK Ownership

After eight bits, the transmitter releases SDA. The receiver owns the ninth clock: pulling SDA low means ACK; leaving it high means NACK. NACK is not always an error. A controller receiving data deliberately NACKs the final byte to tell the target that the read is complete.

| Phase | Transmitter | Who ACKs/NACKs? | Typical meaning |
| --- | --- | --- | --- |
| Address + Write | Controller | Target | Address matched and target accepts write direction |
| Write data | Controller | Target | Byte accepted |
| Address + Read | Controller | Target | Address matched and target will transmit |
| Read data, more wanted | Target | Controller sends ACK | Continue sequential read |
| Read data, final byte | Target | Controller sends NACK | Terminate read, then STOP or repeated START |

# 10. Repeated START and Combined Transfers

A repeated START begins another address phase without first releasing the bus with STOP. It is essential when a transaction must change direction while preserving the target's internal context. Serial-memory random read is the classic example: first write the memory address, then repeated START and read the data.

Random-read direction change

START -> target+W -> word address high -> word address low
      -> REPEATED START -> target+R -> receive byte(s) -> NACK -> STOP

No STOP occurs after the dummy-write address phase.
Therefore the EEPROM does not begin a write cycle.

# 11. Clock Stretching

Clock stretching is optional. A stretching-capable target pauses a transfer by holding SCL low after the controller releases it. The controller must observe the physical SCL level and continue only after SCL actually rises. AT24C256C write completion is normally handled by address ACK polling rather than relying on clock stretching.

Controller releases SCL for HIGH
              |
Target needs time and keeps SCL LOW
              |
Controller observes SCL still LOW and waits
              |
Target releases SCL -> pull-up raises line -> transfer continues

# 12. Multi-Controller Arbitration

Arbitration is nondestructive because controllers monitor SDA while transmitting. A controller that releases SDA for a 1 but observes the bus low has lost to a controller transmitting 0. It stops driving the transfer; the winning message remains valid. Single-controller designs still benefit from understanding arbitration-related error flags and bus ownership.

Controller A sends 1 -> releases SDA
Controller B sends 0 -> pulls SDA LOW
Observed bus is 0

Controller A expected 1 but observed 0 -> loses arbitration
Controller B continues without corrupting its frame

# 13. Speed Modes and Timing Budget

| Mode | Maximum bit rate | Typical use | Rise-time sensitivity |
| --- | --- | --- | --- |
| Standard-mode | 100 kbit/s | Bring-up, EEPROM, low-speed sensors | Most tolerant; tr max 1000 ns |
| Fast-mode | 400 kbit/s | Common production sensor/control bus | Requires stronger pull-up/lower capacitance; tr max 300 ns |
| Fast-mode Plus | 1 Mbit/s | Fm+ capable devices and careful PCB design | tr max 120 ns and stronger sink capability |
| High-speed mode | 3.4 Mbit/s | Specialized systems | Different protocol/timing requirements |

## 13.1 Basic Period Calculation

SCL period = 1 / fSCL

100 kHz -> 10 us per SCL pulse
400 kHz -> 2.5 us per SCL pulse
1 MHz   -> 1 us per SCL pulse

The usable high interval is reduced by the real rising edge.
Therefore nominal divider math alone does not prove timing compliance.

## 13.2 Transfer-Clock Overhead

A useful first estimate counts nine clocks per address or data byte. START, repeated START and STOP add condition timing but are not separate nine-clock bytes.

| AT24C256C operation | Clocked bytes | SCL clocks | At 400 kHz, wire time |
| --- | --- | --- | --- |
| Byte write | device+W, address high, address low, 1 data | 4 x 9 = 36 | About 90 us, plus internal write cycle |
| Random read, 1 byte | device+W, 2 address, device+R, 1 data | 5 x 9 = 45 | About 112.5 us |
| Random sequential read, 16 bytes | 4 address-phase bytes + 16 data | 20 x 9 = 180 | About 450 us |
| Full 64-byte page write | device+W, 2 address, 64 data | 67 x 9 = 603 | About 1.51 ms, plus internal write cycle |

Throughput insight: EEPROM write latency is usually dominated by the self-timed nonvolatile write cycle, not only by the I2C wire time. ACK polling avoids always waiting the full 5 ms maximum.

# 14. STM32F407 Timing Illustration

The shared chat used STM32F407 I2C1 as a concrete timing example. This illustrates how one older STM32 I2C peripheral converts APB1 clock into Standard-mode SCL timing. Newer MCUs may use a TIMINGR-style register instead, so the concept transfers but the registers do not.

Assumptions:
PCLK1 = 32 MHz
Target fSCL = 100 kHz Standard-mode
TPCLK1 = 1 / 32 MHz = 31.25 ns

Standard-mode, approximately 50% duty:
CCR = PCLK1 / (2 x fSCL)
CCR = 32,000,000 / (2 x 100,000) = 160

Maximum Standard-mode rise-time model:
TRISE = (1000 ns / 31.25 ns) + 1 = 33

CR2.FREQ represents 32 MHz input-clock information.
It does not mean that the I2C bus runs at 32 MHz.

Platform boundary: Actual timing also includes line rise/fall time, analog/digital filters and device limits. Always apply the target MCU reference-manual formula and verify the waveform.

# 15. Generic I2C Controller Internal Functionality

CPU / peripheral bus
        |
        v
Control and status registers
        |
        +--> START / STOP generator
        +--> target-address comparator / address sequencer
        +--> ACK / NACK controller
        +--> arbitration and bus-error logic
        +--> clock generator and stretch detector
        |
        v
TX data register -> shift register -> open-drain SDA output
open-drain SDA input -> shift register -> RX data register
        |
        v
Physical SDA and SCL pins

## 15.1 Internal Transmit Sequence

Software requests START while the bus is free.

Hardware creates the START condition and enters controller mode.

Software or hardware supplies the target address and direction bit.

The shift register serializes bits MSB first while the clock generator controls SCL.

The transmitter releases SDA for the ninth clock and samples ACK/NACK.

Data bytes continue, or hardware generates STOP/repeated START as requested.

## 15.2 Internal Receive Sequence

The addressed target places each data bit on SDA while the controller clocks SCL.

The input path samples SDA and shifts eight bits into the receive register.

The controller sends ACK when more bytes are required.

Before the final byte completes, the controller prepares NACK and STOP or repeated START.

Software reads the receive register and handles completion or error status.

## 15.3 Why Register Sequences Differ Between MCUs

I2C protocol behavior is portable, but peripheral state machines are not. STM32F407 uses CR1/CR2/CCR/TRISE/SR1/SR2/DR and has flag-clearing sequences such as reading status registers in a prescribed order. Newer STM32, NXP, Renesas and Microchip controllers expose different timing and event models. Learn the bus first, then the peripheral.

# 16. AT24C256C Device Overview

| Feature | Verified device information |
| --- | --- |
| Memory density | 256 Kbit = 262,144 bits |
| Byte organization | 32,768 x 8 = 32 KiB |
| Address range | 0x0000 through 0x7FFF |
| Page organization | 512 pages x 64 bytes |
| Supply | 1.7 V to 5.5 V |
| Bus speeds | 100 kHz and 400 kHz across 1.7-5.5 V; 1 MHz at 2.5-5.5 V |
| Write cycle | Self-timed, 5 ms maximum |
| Write protection | WP high protects the full array |
| Endurance | 1,000,000 write cycles |
| Data retention | 100 years |

## 16.1 Pin Functions

| Pin | Purpose | Foundation note |
| --- | --- | --- |
| VCC | Device supply | Decouple locally and respect the speed-versus-voltage table |
| GND | Ground reference | Must share ground with the controller |
| SCL | Serial clock input | Connect to pulled-up open-drain SCL |
| SDA | Bidirectional serial data | Open-drain; carries address, data and ACK/NACK |
| A0, A1, A2 | Hardware target-address selection | Connect to known levels; permit up to eight device addresses |
| WP | Whole-array hardware write protect | High blocks memory writes; reads remain available |

# 17. Device Addressing: The 7-Bit Versus 8-Bit Trap

AT24C256C address byte on the wire:

Bit:   7 6 5 4 3  2  1  0
       1 0 1 0 A2 A1 A0 R/W

7-bit target address = binary 1010 A2 A1 A0
Address range        = 0x50 through 0x57

If A2=A1=A0=0:
7-bit address        = 0x50
Transmitted write byte = (0x50 << 1) | 0 = 0xA0
Transmitted read byte  = (0x50 << 1) | 1 = 0xA1

Correction to preserve: The shared discussion omitted A2 in one simplified address diagram. The verified AT24C256C format includes A2, A1 and A0. Most I2C APIs expect 0x50 as the 7-bit address; 0xA0/0xA1 are bus bytes, not two different devices.

# 18. Memory Addressing and Geometry

Capacity conversion:
256 Kbit / 8 = 32 KByte = 32768 byte locations
32768 = 2^15, so the memory word address is 15 bits
Valid range: 0x0000 to 0x7FFF

Two word-address bytes are transmitted:
First byte : X A14 A13 A12 A11 A10 A9 A8
Second byte: A7 A6 A5 A4 A3 A2 A1 A0

## 18.1 Page Boundary Calculations

Page size = 64 bytes = 0x40

page_number = address / 64
page_offset = address & 0x3F
page_base   = address & 0xFFC0
space_left = 64 - page_offset

Example: start address 0x0070
page_offset = 0x30 = 48
space_left  = 64 - 48 = 16 bytes

A 40-byte write must be split:
16 bytes at 0x0070, then 24 bytes at 0x0080.

# 19. Write Operations

## 19.1 Byte Write

START
Device address + W      -> target ACK
Word address high       -> target ACK
Word address low        -> target ACK
Data byte               -> target ACK
STOP                    -> begins internal write cycle

## 19.2 Page Write

A page write accepts one to 64 bytes after the starting word address. The internal page counter increments only within the selected 64-byte page. If the host crosses the boundary without splitting, the counter rolls to the beginning of the same page and can overwrite bytes already loaded in that page.

Boundary rule: The limit is not merely 64 bytes per call. The transfer must also fit in the remaining space of the current page.

## 19.3 ACK Polling

After STOP starts the self-timed write cycle:

Send START + valid device address with R/W=0
    |
    +-- NACK -> write still active; retry within a bounded timeout
    |
    +-- ACK  -> write cycle complete; next operation may begin

ACK polling is more efficient than always delaying for the 5 ms maximum. A production implementation still needs a timeout so a missing or write-protected device cannot stall the system forever.

## 19.4 Hardware Write Protect

With WP high, the full memory array is protected. The device can still acknowledge the device address, word address and data, but no internal write cycle begins when STOP arrives. Therefore ACK alone does not prove that data changed; read-back verification matters.

# 20. Read Operations

| Read type | How address is selected | Important behavior |
| --- | --- | --- |
| Current address read | Uses internal address counter | Counter is retained while powered and points to the next location after prior access |
| Random read | Dummy write loads two-byte word address, then repeated START + read | No STOP after dummy write |
| Sequential read | Begins as current or random read | Controller ACKs each byte except final byte; address auto-increments |
| End-of-array behavior | Sequential read passes 0x7FFF | Counter rolls to 0x0000 |

## 20.1 Random Read Frame

START
Device address + W      -> ACK
Word address high       -> ACK
Word address low        -> ACK
REPEATED START
Device address + R      -> ACK
Data from EEPROM
Controller NACK         -> marks final byte
STOP

## 20.2 Sequential Read Ownership

EEPROM sends byte 0 -> controller ACKs: continue
EEPROM sends byte 1 -> controller ACKs: continue
...
EEPROM sends final byte -> controller NACKs: stop sending
Controller generates STOP or repeated START

# 21. Worked AT24C256C Transactions

## 21.1 Write 0x55 to Word Address 0x0010

Assume A2=A1=A0=0.

START
0xA0  -> device 0x50, write direction -> ACK
0x00  -> word address A14:A8          -> ACK
0x10  -> word address A7:A0           -> ACK
0x55  -> data                          -> ACK
STOP  -> internal write begins
ACK poll until 0xA0 receives ACK

## 21.2 Read Back Word Address 0x0010

START
0xA0 -> ACK
0x00 -> ACK
0x10 -> ACK
REPEATED START
0xA1 -> ACK
EEPROM returns 0x55
Controller NACKs final byte
STOP

# 22. I2C Compared with Other Embedded Protocols

| Property | I2C | SPI | UART | CAN |
| --- | --- | --- | --- | --- |
| Clock | Shared SCL | Controller SCK | Asynchronous | Recovered from bus timing |
| Data wires | One bidirectional SDA | MOSI and MISO | TX and RX | Differential CANH/CANL |
| Device selection | 7/10-bit address | Chip select | Usually point-to-point | Message identifier |
| Electrical drive | Open-drain + pull-ups | Usually push-pull | Usually push-pull | Differential transceiver |
| Duplex | Half-duplex on SDA | Full-duplex | Full-duplex | Shared broadcast bus |
| Acknowledgment | ACK/NACK each byte | Device-protocol specific | None in base UART | Frame ACK slot |
| Multi-node scaling | Many addressed targets on two wires | Extra CS per target | Additional ports/protocol | Many nodes on one bus |
| Typical strength | Low pin count and register-like devices | High throughput and simple framing | Simple links/debug | Robust distributed control |

## 22.1 When I2C Is the Better Choice

Several low/medium-speed devices must share two board-level signal wires.

Devices already provide an I2C register or serial-memory interface.

Pin count matters more than raw throughput.

The bus length and capacitance can be kept within specification.

## 22.2 When I2C Is Not the Better Choice

High sustained throughput or deterministic full-duplex transfer is needed: consider SPI.

A simple long point-to-point serial link is needed: use a suitable UART physical layer such as RS-232/RS-485.

A robust multi-node differential network with message arbitration is needed: consider CAN.

Cabling is long, noisy or highly capacitive without appropriate extenders/buffers.

# 23. Logic-Analyzer Reading Guide

| Analyzer observation | What it proves | What it does not prove |
| --- | --- | --- |
| START and STOP decode correctly | Basic polarity and timing are recognizable | Target operation succeeded |
| Address 0x50 + W/R appears | Analyzer uses 7-bit notation and address phase is correct | A0-A2 wiring is correct unless ACK is also seen |
| ACK after every write byte | Target accepted the protocol byte | EEPROM nonvolatile write completed |
| Repeated START before read | Random-read direction change is correct | Returned data is the intended address unless word bytes are verified |
| Final read byte followed by NACK | Controller terminates read correctly | STOP timing is correct unless waveform is inspected |
| Slow rounded rising edge | RC pull-up behavior is visible | Rise time meets the selected mode without measurement |

# 24. Failure Modes and Debugging

| Symptom | Likely causes | First engineering checks |
| --- | --- | --- |
| Both lines remain low | Short, stuck target, controller pin misconfiguration | Power off resistance check; identify which device holds line |
| Lines never rise cleanly | Missing/weak pull-ups or excessive capacitance | Measure rise time; calculate Rp(max); inspect module pull-ups |
| Address always NACKs | Wrong 7-bit address, A-pin state, power, WP confusion, wiring | Scan 0x50-0x57; verify VCC/GND and A2-A0 |
| Write ACKs but data unchanged | WP high or page rollover overwrote another location | Read WP level; verify page offset and read back |
| EEPROM NACKs immediately after write | Normal internal write cycle | Use bounded ACK polling |
| Random read returns wrong location | Word-address byte order or missing repeated START | Decode high byte, low byte and combined transfer |
| Bus BUSY forever after reset | SDA held low by interrupted transaction | Perform documented bus recovery and reset affected devices |
| Works at 100 kHz, fails at 400 kHz | Rise time or target timing limit | Measure capacitance/rise time; reduce Rp or speed within sink limits |
| Multi-controller transfer aborts | Arbitration lost | Treat as expected contention; retry after bus becomes free |

## 24.1 Bus Recovery Foundation

A target interrupted mid-byte can hold SDA low. A common recovery concept is to temporarily treat SCL as open-drain GPIO, generate up to nine clock pulses while observing SDA, then create a STOP-like release and reinitialize the peripheral. The exact method must follow the controller and target documentation and must include a timeout.

# 25. Foundation Design Checklist

| Check | Acceptance question |
| --- | --- |
| Voltage | Are controller, target and pull-up voltage levels compatible? |
| Topology | Are SDA, SCL and ground shared correctly with short board-level routing? |
| Pull-ups | Do calculated Rp(min) and Rp(max) overlap for the real Cb and speed? |
| Address | Are A2-A0 known, and does software use the 7-bit address convention? |
| Timing | Does every device support the selected mode at the selected supply? |
| Waveform | Are rise time, START/STOP and ACK setup visibly compliant? |
| Timeouts | Can every wait, clock stretch and ACK-poll loop terminate on failure? |
| EEPROM boundaries | Are writes split by remaining page space, not only by 64-byte length? |
| Verification | Are important writes read back and compared? |
| Recovery | Is there a defined response to SDA/SCL stuck low? |

# 26. Important Understandings to Retain

I2C logic high is normally a released line, not an actively driven high output.

Every transmitted byte has a ninth ACK/NACK clock, so byte throughput is below the raw bit rate.

A NACK can mean no target, target busy, protected/invalid operation, or intentional end of a read.

The 7-bit address and the transmitted address byte are related but not interchangeable.

Repeated START is part of one combined transfer and is not equivalent to STOP followed by START for every device use case.

Pull-ups and capacitance are protocol components, not optional board details.

Clock stretching is optional; ACK polling is the normal AT24C256C write-completion mechanism.

AT24C256C page writes must fit the remaining bytes of one 64-byte page.

ACK of transmitted write bytes does not prove nonvolatile data changed, especially when WP is high.

Bus-level understanding is portable; MCU register sequences are not.

# 27. Limitations and Next Learning Stage

This guide does not implement STM32F407 polling APIs or exact flag-clearing code.

10-bit addressing, general call, SMBus/PMBus and High-speed mode are introduced only at boundary level.

Signal-integrity treatment is limited to first-order RC pull-up design.

A later implementation document should use finalized source files and logic-analyzer screenshots.

The natural next project is an I2C scanner, then AT24C256C byte write/random read, followed by page-safe writes and sequential reads.

# 28. Conclusion

Conclusion: I2C is best understood as a shared open-drain state machine, not merely a two-wire byte shifter. Correct designs combine address framing, ACK ownership, repeated START behavior, electrical pull-up calculations and target-specific rules. AT24C256C is an excellent foundation device because it exercises all of those concepts in a compact project.

# 29. Appendix A: Primary References

| Reference | Use in this document |
| --- | --- |
| https://chatgpt.com/share/6a4a7784-ed40-83ee-8254-23398e9cf44e | User's shared I2C learning conversation and emphasis |
| https://www.nxp.com/docs/en/user-guide/UM10204.pdf | Generic I2C electrical, framing, arbitration, clock-stretching and pull-up rules |
| https://ww1.microchip.com/downloads/aemDocuments/documents/MPD/ProductDocuments/DataSheets/AT24C256C-I%C2%B2C-Compatible-%28Two-Wire%29-Serial-EEPROM-256-Kbit-%2832%2C768x8%29-Data-Sheet-DS20006042.pdf | AT24C256C capacity, addressing, page, write, read and electrical behavior |
| C:\Users\HP\Documents\Codex\2026-06-04\i-m-an-embedded-engineer-currently\work\i2c_foundation_clean_source.txt | Filtered shared-chat theory used during authoring |

# 30. Appendix B: Shared-Chat Corrections

| Shared simplification | Verified correction |
| --- | --- |
| Address diagram omitted A2 | AT24C256C uses 1010 A2 A1 A0 R/W |
| 0xA0/0xA1 described as device addresses | 0x50 is the 7-bit address when A2-A0 are low; 0xA0/0xA1 are transmitted bytes |
| 10 kohm is usually okay at 100 kHz | It can be reasonable only if calculated rise time and sink-current limits are met |
| 1 MHz linked generally to a 5 V version | AT24C256C specifies 1 MHz Fast-mode Plus at 2.5-5.5 V |
| Internal pull-ups suggested for testing | External pull-ups are the design reference; internal pulls are weak and should not be assumed sufficient |

# 31. Appendix C: Revision Checklist

| Verification item | Status |
| --- | --- |
| Shared ChatGPT link downloaded and decoded | Complete |
| Scope limited to I2C foundation theory and AT24C256C | Complete |
| Generic protocol facts checked against NXP UM10204 | Complete |
| AT24C256C facts checked against Microchip DS20006042C | Complete |
| Open-drain, pull-up, timing and throughput calculations included | Complete |
| System, electrical, controller and EEPROM flow diagrams included | Complete |
| Protocol comparisons included | Complete |
| No firmware code presented as finalized implementation | Complete |
| Footer set to Embedded Learning Document | Complete |