MAX7219 SPI2 Polling-Based Project

8x8 LED Dot-Matrix Bring-Up, Bitmap Rendering, and Right-to-Left Text Scrolling

Document intent: An industry-grade learning and implementation reference built from the finalized attached firmware, the shared MAX7219 learning chat, and Analog Devices' official MAX7219 documentation.

| Field | Value |
| --- | --- |
| MCU / peripheral | STM32F407-style target, SPI2 master on APB1 |
| Display device | MAX7219 driving one common-cathode 8x8 LED dot matrix |
| Interface style | Blocking/polling, 8-bit SPI transfers grouped into one 16-bit MAX7219 write |
| SPI configuration | Mode 0, MSB-first, approximately 1 MHz when PCLK1 is 32 MHz |
| Display features | Raw bitmap rows, digits, letters, test patterns, blinking, and scrolling text |
| Authoritative code | main.c, max7219.c, spi.c, max7219.h, spi.h |

# 1. Document Control and Source Boundary

| Item | Details |
| --- | --- |
| Version | v1.0 |
| Shared learning source | https://chatgpt.com/share/6a2ef732-b840-83e8-ba44-20bcd1f7db30 |
| Primary device source | Analog Devices MAX7219/MAX7221 Data Sheet, Rev. 6 |
| Code fidelity rule | Quoted code is taken only from the five attached finalized source/header files |
| Evidence boundary | No MAX7219 logic-analyzer screenshot was attached in this turn; the shared chat records earlier analyzer observations |
| Document status | Engineering-reviewed learning reference; hardware limits must still be validated on the exact module |

# 2. Learning Objectives

Understand how the MAX7219 converts a 16-bit serial write into a control-register or display-RAM update.

Configure STM32 SPI2 and GPIO chip select for reliable polling-based writes.

Explain why an 8x8 matrix requires no-decode mode and eight raw row bytes.

Trace character rendering from a font table to the MAX7219 digit registers.

Understand matrix orientation, row order, and bit order without confusing them with SPI errors.

Derive the right-to-left scrolling algorithm as a moving 8-column window over virtual text.

Identify electrical, blocking, maintainability, and shared-bus risks in the finalized project.

# 3. Project Objective

Drive a single MAX7219-based 8x8 LED dot-matrix module from STM32 SPI2 using blocking polling. Validate the physical display, write raw row patterns, render alphanumeric bitmaps, and continuously scroll the text HELLO from right to left.

# 4. Engineering Problem Statement

Core problem: Successful SPI bytes are only the transport layer. A readable matrix also depends on no-decode mode, the module's row/column wiring, the font's row order, the byte's bit order, and a frame-generation algorithm that preserves those conventions.

# 5. System Overview

STM32F407                         MAX7219 module

PB9  GPIO chip select ----------> LOAD / CS
PB13 SPI2_SCK ------------------> CLK
PB15 SPI2_MOSI -----------------> DIN
PB14 SPI2_MISO -----------------> Not required by MAX7219
GND ----------------------------> GND
Supply -------------------------> Module VCC

MAX7219 segment and digit outputs --> Common-cathode 8x8 LED matrix

The STM32 updates registers; the MAX7219 retains the eight display-RAM bytes and performs continuous multiplex scanning and current-controlled LED driving. The MCU therefore refreshes only when a pattern or animation frame changes.

# 6. Requirements and Constraints

| Requirement | Project value | Engineering interpretation |
| --- | --- | --- |
| Serial clock | About 1 MHz | Well below the device's 10 MHz maximum serial-interface rate |
| Clock behavior | Mode 0, MSB-first | DIN is stable for the rising CLK edge used by MAX7219 |
| Command width | 16 serial bits | Address byte followed by data byte while LOAD remains low |
| Matrix mode | Decode mode = 0x00 | All eight display-RAM bytes directly control LED patterns |
| Active scan | Scan limit = 0x07 | All eight matrix rows/digits participate in multiplexing |
| Animation model | Blocking polling + HAL_Delay | Simple and deterministic, but CPU time is unavailable during animation |

# 7. Electrical Compatibility Check

Important hardware limitation: The bare MAX7219 is specified for a 4.0 V to 5.5 V supply. At that supply range, the specified minimum logic-high input voltage is 3.5 V. A 3.3 V STM32 output may work on a particular module but is not guaranteed by the MAX7219 data sheet. Validate the actual module circuit or use a suitable level shifter/buffer.

Share a solid common ground between the MCU, analyzer, and display module.

Check the module's VCC, onboard components, and matrix type rather than assuming all marketplace modules are identical.

The external ISET resistor establishes peak segment current; the intensity register then adjusts average brightness using internal PWM.

Display-test and high-intensity modes can create substantial LED current. Confirm USB/regulator capacity and module temperature.

MAX7219 is intended for common-cathode displays. A differently wired matrix requires different hardware or a compatible driver.

# 8. Hardware and Tooling

| Item | Role | Verification point |
| --- | --- | --- |
| STM32F407 board | SPI master and application processor | Confirm clock tree and 3.3 V GPIO levels |
| MAX7219 8x8 module | Display RAM, multiplexing, and LED current drive | Confirm VCC, ISET resistor, and orientation |
| Logic analyzer | Observe LOAD, CLK, and DIN | Decode address and data as two MSB-first bytes |
| UART terminal | Bring-up diagnostics | Not part of MAX7219 transport itself |
| Multimeter / oscilloscope | Power and signal-level verification | Check VCC, ground, VIH margin, and edge quality |

# 9. MAX7219 Internal Model

DIN -> 16-bit shift register -> LOAD rising edge -> address decoder
                                                     |
                  +----------------------------------+------------------+
                  |                                                     |
          control registers                                      8x8 display RAM
   decode / intensity / scan / shutdown / test             eight addressable bytes
                  |                                                     |
                  +---------------- multiplex scan and current drive ---+
                                                                        |
                                                                  8x8 LED matrix

The term 'digit register' comes from the device's original 7-segment use. On this dot-matrix module, addresses 0x01 through 0x08 are treated as eight raw matrix lines. Each stored byte controls eight LEDs on one line according to the physical module mapping.

# 10. Serial Transaction and Timing

| Phase | LOAD / CS | Serial activity | Meaning |
| --- | --- | --- | --- |
| Idle | High | CLK idle low | No write is in progress |
| Start | Low | Address begins at DIN | Open one 16-bit transaction |
| Shift | Low | 16 rising CLK edges | Address and data shift MSB-first |
| Latch | Rising edge | After the 16th clock | Selected register accepts the data |
| Next write | High then low | New 16-bit word | A separate register update |

Bit position:  D15 D14 D13 D12 | D11 D10 D9 D8 | D7 ... D0
Meaning:       don't-care bits | register address | data byte

Firmware representation:
    SPI2_TransferByte(address);
    SPI2_TransferByte(data);

LOAD remains low across both byte transfers and rises only after the data byte.

Terminology: The project drives the MAX7219 serial interface with an STM32 SPI peripheral. The timing is SPI-like and Mode-0-compatible for this implementation. The data sheet explicitly calls the MAX7221 SPI-compatible; avoid relying on that label alone for MAX7219 and verify the actual edge/latch timing.

# 11. Register Map Used by the Project

| Address | Register | Project value | Purpose |
| --- | --- | --- | --- |
| 0x00 | No-op | Not used | Useful in cascaded chains |
| 0x01-0x08 | Digit 0-7 / display RAM | 0x00-0xFF | Eight raw matrix lines |
| 0x09 | Decode mode | 0x00 | Bypass Code-B and preserve raw bitmap bits |
| 0x0A | Intensity | 0x08 | Digital brightness setting |
| 0x0B | Scan limit | 0x07 | Scan all eight matrix lines |
| 0x0C | Shutdown | 0x01 | Normal operation |
| 0x0F | Display test | 0x00 normally | 0x01 forces every LED on |

# 12. Why No-Decode Mode Is Mandatory Here

Code-B decode is convenient for seven-segment digits because the device translates a numeric code into segment outputs. An 8x8 dot matrix needs independent pixel control, so every data bit must reach the matrix mapping unchanged. The finalized initialization correctly writes 0x00 to the decode-mode register.

Verified correction from the learning history: Earlier seven-segment-oriented explanations used decode mode. The final dot-matrix driver correctly uses no-decode mode and clears rows with 0x00 rather than the Code-B blank value 0x0F.

# 13. Firmware Architecture

| Layer | File | Responsibility |
| --- | --- | --- |
| Application/demo | main.c | Runs hardware tests, fixed patterns, character display, and scrolling loop |
| Display driver | max7219.c/.h | Register writes, initialization, frames, fonts, and animation algorithms |
| SPI transport | spi.c/.h | SPI2 and GPIO initialization, polling byte transfers, and CS macros |
| Platform | STM32 HAL/CMSIS | Clock tree, delay service, device-register definitions, and startup |

main()
  |
  +--> SPI2_GPIO_Init() / SPI2_Init()
  |
  +--> MAX7219_Write() / MAX7219_Init()
  |       |
  |       +--> SPI2_TransferByte(address)
  |       +--> SPI2_TransferByte(data)
  |
  +--> MAX7219_DisplayFrame()
          |
          +--> eight MAX7219_Write() calls
                  |
                  +--> MAX7219 display RAM and internal scan engine

# 14. SPI2 Configuration Review

| Configuration | Final source | Review |
| --- | --- | --- |
| Controller mode | SPI_CR1_MSTR | Correct |
| Software NSS | SSM + SSI | Correct for GPIO-controlled PB9 chip select |
| CPOL / CPHA | 0 / 0 | Compatible with DIN capture on rising clock edge |
| Frame / bit order | 8-bit / MSB-first | Correct; two bytes form one device word |
| Prescaler | PCLK / 32 | About 1 MHz if PCLK1 is 32 MHz |
| Duplex | 2-line full duplex | Works, though MAX7219 does not return application data |
| Polling flags | TXE, RXNE, BSY | Correct basic sequence; no timeout protection |

uint8_t SPI2_TransferByte(uint8_t byte)
{
    uint8_t received;

    //waiting for TX buffer empty
    while(!(SPI2->SR & SPI_SR_TXE));
    *((volatile uint8_t *)&SPI2->DR) = byte;

    //waiting for RX buffer Not empty
    while(!(SPI2->SR & SPI_SR_RXNE));
    received = *((volatile uint8_t *)&SPI2->DR);

    //Wait till SPI completes
    while(SPI2->SR & SPI_SR_BSY);

    return received;
}

Polling implication: Every byte waits synchronously for TXE, RXNE, and BSY. This is easy to debug and fully adequate for a first display driver, but a peripheral fault can trap the CPU forever because the loops have no timeout.

# 15. Core Driver Implementation

Code authority: The excerpts in this section are copied from the attached finalized firmware. Engineering recommendations are kept in a separate review section.

## 15.1 Atomic MAX7219 Register Write

void MAX7219_Write(uint8_t address, uint8_t data)
{
    MAX7219_CS_LOW();

    SPI2_TransferByte(address);
    SPI2_TransferByte(data);

    MAX7219_CS_HIGH();

}

Drive PB9 low to open the MAX7219 serial transaction.

Transmit the register address as the first eight bits.

Transmit the register data as the second eight bits.

Drive PB9 high; this rising edge latches the 16-bit word into the addressed register.

## 15.2 Initialization Sequence

void MAX7219_Init(void)
{
    MAX7219_Write(MAX7219_REG_DISPLAY_TEST, 0x00);   // Display test off
    MAX7219_Write(MAX7219_REG_SHUTDOWN, 0x01);   // Normal operation, exit shutdown
    MAX7219_Write(MAX7219_REG_DECODE_MODE, 0x00);   // No-decode mode for digits 0-7
    MAX7219_Write(MAX7219_REG_INTENSITY, 0x08);   // Medium brightness
    MAX7219_Write(MAX7219_REG_SCAN_LIMIT, 0x07);   // Scan digits 0-7

    MAX7219_Clear();
}

The sequence establishes a known non-test state, exits shutdown, enables raw bitmap operation, selects brightness, scans all eight lines, and clears stale RAM. Display test is intentionally disabled before normal rendering.

## 15.3 Clear, Fill, and Diagnostic Patterns

void MAX7219_Clear(void)
{
    for (uint8_t i = 1; i <= 8; i++)
    {
        MAX7219_Write(i, 0x00);
    }
}

void MAX7219_Fill(void)
{
    for (uint8_t i = 1; i <= 8; i++)
    {
        MAX7219_Write(i, 0xFF);
    }
}

void MAX7219_CheckPosition(void)
{
    MAX7219_Clear();

    MAX7219_Write(0x01, 0x81);   // Two edge pixels on first row
    MAX7219_Write(0x08, 0x81);   // Two edge pixels on last row
}

| Pattern | What it proves | Likely fault if incorrect |
| --- | --- | --- |
| Display test 0x0F/0x01 | Power, driver, and matrix can illuminate | Power, module, or LED hardware |
| One line = 0xFF | Address mapping for one display-RAM line | Row/digit mapping |
| Fill = all 0xFF | Normal-mode raw control of all pixels | Decode/test/shutdown configuration |
| Diagonal 80..01 | Row order and bit order | Orientation or matrix wiring |
| Corner pattern 0x81 | First/last lines and edge columns | Rotation, mirroring, or off-by-one |

## 15.4 Character Lookup and Rendering

void MAX7219_DisplayChar(char ch)
{
    const uint8_t *bitmap = MAX7219_GetCharBitmap(ch);

    for (uint8_t row = 0; row < 8; row++)
    {
        MAX7219_Write(row + 1, bitmap[row]);
    }
}

const uint8_t* MAX7219_GetCharBitmap(char ch)
{
    if (ch >= 'a' && ch <= 'z')
    {
        ch = ch - 'a' + 'A';
    }

    if (ch >= 'A' && ch <= 'Z')
    {
        return font_8x8_alpha[ch - 'A'];
    }

    if (ch >= '0' && ch <= '9')
    {
        return font_8x8_digits[ch - '0'];
    }

    return font_space;
}

The lookup normalizes lowercase input, supports A-Z and 0-9, and safely maps unsupported characters to a blank glyph. This is why spaces in strings no longer need special-case skipping.

## 15.5 Frame Transfer

void MAX7219_DisplayFrame(uint8_t frame[8])
{
    for (uint8_t row = 0; row < 8; row++)
    {
        MAX7219_Write(row + 1, frame[row]);
    }
}

A frame is eight bytes, one per display-RAM line. The function performs eight complete MAX7219 transactions, so LOAD rises eight times per frame. The MAX7219 continues scanning while these writes occur; it has no frame-buffer swap, so a high-speed camera or slow bus could observe partial-frame tearing.

# 16. Bitmap Orientation Deep Dive

A font entry is eight row bytes. The array index selects the matrix line and each set bit selects a column. The final project font is already transformed for the specific module orientation, so direct row writes display readable characters.

Example glyph A from the finalized font:

{0x00, 0x66, 0x66, 0x7E, 0x66, 0x66, 0x3C, 0x18}

Project convention:
    bitmap[0] .. bitmap[7] -> addresses 0x01 .. 0x08
    bit0 behaves as the left-side source column for scrolling
    bit7 behaves as the right-side source column for scrolling

This convention is module-specific, not a universal MAX7219 law.

| Observed symptom | Usually changed by mistake | Correct diagnostic |
| --- | --- | --- |
| Character upside down | Row-address order | Use corners/diagonal before editing fonts |
| Character mirrored | Bit order within each row byte | Compare 0x01 and 0x80 edge pixels |
| Text order reversed | Scroll offset direction | Track character index as offset increases |
| Readable static text, mirrored scroll | Source/destination mask convention | Use same bit convention in render and scroll |
| Random or corrupted shape | Transport or RAM writes | Capture the exact 16-bit address/data words |

# 17. Right-to-Left Scrolling Algorithm

Mental model: Characters do not move inside the MAX7219. Firmware repeatedly constructs a new 8x8 frame from a moving eight-column viewport over a virtual strip of text.

Virtual strip for "HELLO" with one blank column after each character:

H columns 0..7 | space 8 | E columns 9..16 | space 17 | ...

For every visible display column:
    virtual_col = offset + display_col - 8

At offset 0: visible virtual columns are -8..-1 -> blank
At offset 1: virtual column 0 reaches display column 7 -> H enters at right
As offset increases: the viewport advances through H, E, L, L, O
After the text: eight extra columns allow the final glyph to leave the display.

## 17.1 Final Scrolling Function

void MAX7219_ScrollText_RtoL(const char *text, uint16_t speed_ms)
{
    uint8_t frame[8];

    uint16_t text_len = MAX7219_StrLen(text);

    const uint8_t char_width = 8;
    const uint8_t spacing = 1;
    const uint8_t char_step = char_width + spacing;

    uint16_t total_columns = (text_len * char_step) + 8;

    for (uint16_t offset = 0; offset < total_columns; offset++)
    {
        for (uint8_t row = 0; row < 8; row++)
        {
            frame[row] = 0x00;
        }

        for (uint8_t display_col = 0; display_col < 8; display_col++)
        {
            int16_t virtual_col = (int16_t)offset + display_col - 8;

            if (virtual_col < 0)
            {
                continue;
            }

            uint16_t char_index = virtual_col / char_step;
            uint8_t col_in_char = virtual_col % char_step;

            if (char_index >= text_len)
            {
                continue;
            }

            if (col_in_char >= 8)
            {
                continue;   // spacing column
            }

            const uint8_t *bitmap = MAX7219_GetCharBitmap(text[char_index]);

            /*
             * IMPORTANT:
             * Your corrected font uses bit0 as the left-side column.
             */
            uint8_t source_mask = (uint8_t)(1U << col_in_char);
            uint8_t dest_mask   = (uint8_t)(1U << display_col);

            for (uint8_t row = 0; row < 8; row++)
            {
                if (bitmap[row] & source_mask)
                {
                    frame[row] |= dest_mask;
                }
            }
        }

        MAX7219_DisplayFrame(frame);
        HAL_Delay(speed_ms);
    }
}

## 17.2 Variable-by-Variable Explanation

| Variable | Meaning | Why it exists |
| --- | --- | --- |
| frame[8] | Current eight row bytes | A complete visible matrix image |
| char_width | 8 columns | Each glyph occupies an 8x8 bitmap |
| spacing | 1 column | Separates adjacent glyphs |
| char_step | 9 columns | One glyph plus its spacing |
| offset | Viewport position | Advances the text by one column per frame |
| virtual_col | Column in the complete text strip | Maps one physical display column to text content |
| char_index | virtual_col / 9 | Selects the source character |
| col_in_char | virtual_col % 9 | Selects a glyph column or spacing column |
| source_mask | 1 << col_in_char | Reads one pixel column using the corrected font convention |
| dest_mask | 1 << display_col | Places that column into the visible frame |

## 17.3 Complexity and Timing

Every animation frame examines 8 display columns x 8 rows, so frame construction is small and predictable.

Every frame performs eight register writes, each containing two SPI bytes: 128 serial data clocks plus LOAD transitions.

At a 1 MHz serial clock, raw transfer time is small relative to the configured 100 ms frame delay.

The function is fully blocking. A five-character HELLO pass uses 53 frames and approximately 5.3 seconds at 100 ms per frame, excluding transfer overhead.

The extra eight columns blank the display before entry and allow the final character to leave. The one spacing column after the last glyph is also included by the current formula.

# 18. Application Bring-Up Sequence

HAL and system clock initialization
        |
        +--> UART diagnostic initialization
        +--> SPI2 GPIO and peripheral initialization
        +--> legacy/shared-bus SPI and MCP2515 checks
        +--> MAX7219 display test: all LEDs on, then off
        +--> MAX7219 initialization
        +--> one full line
        +--> full matrix
        +--> diagonal pattern
        +--> display character A, then clear
        +--> forever: scroll "HELLO" right-to-left

## 18.1 Final Main-Loop Use

while (1)
{

    //MAX7219_CharBlinking('A');
    //MAX7219_StringBlinking("Hello Via Matrix");

    MAX7219_ScrollText_RtoL("HELLO", 100);
}

The function completes one full blank-to-blank pass, returns, and is immediately called again. This naturally repeats the message without maintaining animation state outside the driver.

# 19. Embedded Engineering Review

| Finding | Impact | Recommended action |
| --- | --- | --- |
| spi.h has no include guard | Repeated inclusion can cause future declaration/macro maintenance problems | Add a conventional #ifndef/#define/#endif guard |
| Polling loops have no timeout | A peripheral fault can deadlock the entire application | Return status and enforce bounded waits |
| 3.3 V logic-high margin is not guaranteed | Temperature/process variation can cause intermittent writes | Verify module circuitry or add level translation |
| MCP2515 startup remains in main.c | MAX7219 demo depends on unrelated hardware and delays focused bring-up | Separate demos or guard each device with a project option |
| SPI2_Test selects MCP2515 CS | It does not validate the MAX7219 transaction path | Use a MAX7219-specific captured command |
| PB14 MISO configured | Unnecessary for MAX7219 alone | Keep only if the SPI bus is intentionally shared |
| DisplayFrame parameter is non-const | API does not express read-only intent | Prefer const uint8_t frame[8] |
| Eight writes per frame | Possible transient partial-frame updates | Accept for learning; consider faster transfer or display blanking for demanding visuals |
| HAL_Delay animation | CPU cannot perform concurrent work | Move to tick-based non-blocking animation or a timer-driven state machine |
| Orientation is embedded in font data | A different module may mirror/rotate all glyphs | Centralize row/column transform in a configurable mapping layer |

# 20. Shared SPI Bus Considerations

PB9 selects MAX7219 and PB12 selects MCP2515. Both chip-select lines are driven high after GPIO setup.

Before selecting one slave, ensure every other slave's chip select is high.

A shared bus is valid only if all active devices tolerate the configured mode and frequency, or the driver reconfigures SPI between devices.

MAX7219 has no MCU-readable status path in this project. SPI RX bytes during its writes are discarded.

Because the source waits for BSY after each byte, the address and data may contain an inter-byte pause while LOAD remains low. That is functionally acceptable; the latch occurs only when LOAD rises.

# 21. Recommended Production Evolution

Add timeout-aware SPI primitives returning an explicit status code.

Create a board-configuration layer for chip selects, orientation, module count, and logic-level strategy.

Represent the current image as a display buffer and update it through a non-blocking animation state machine.

Add setters for intensity, shutdown, and test mode with input validation.

Separate the MCP2515 and MAX7219 examples while retaining a deliberate shared-bus integration test.

For cascaded modules, shift one 16-bit word per device before the single LOAD rising edge and use NO-OP words for untouched devices.

Add host-side unit tests for glyph lookup and frame generation; compare generated frames against known vectors.

Do not silently change finalized learning code: The recommendations above are review findings, not replacements embedded into the quoted source. This preserves traceability between the document and the attached firmware.

# 22. Verification Strategy

| Test | Expected observation | Pass criterion |
| --- | --- | --- |
| Power check | Stable module VCC and common ground | Voltage and current remain within module limits |
| Display test | Every LED turns on, then returns to normal | No dead lines/pixels beyond known hardware defects |
| Analyzer write | 0x0F 0x01 then 0x0F 0x00 | 16 clocks per write and LOAD rises after data |
| Initialization | 0x09=00, 0x0A=08, 0x0B=07, 0x0C=01 | All configuration words decode correctly |
| Line/fill tests | One complete line, then all pixels | No unexpected decode-mode behavior |
| Diagonal/corners | Known orientation markers | Row and column mapping documented |
| Glyph A | Readable, non-mirrored A | Font and direct-write conventions agree |
| HELLO scroll | H enters from right; H-E-L-L-O order; no mirroring | Blank entry and exit frames are visible |
| Long run | Repeated scroll without lockup | No permanent polling stall or thermal issue |

# 23. Logic Analyzer Capture Guide

Recommended channels
    CH0: PB9  / MAX7219 LOAD-CS
    CH1: PB13 / SPI2 SCK
    CH2: PB15 / SPI2 MOSI-DIN
    GND: common system ground

Decoder
    clock idle low
    sample on rising edge
    MSB first
    enable active low
    8 bits per decoded byte

Expected display-test sequence
    CS low -> 0x0F -> 0x01 -> CS high
    delay
    CS low -> 0x0F -> 0x00 -> CS high

Evidence status: The shared chat records correction of an earlier analyzer setup that decoded 0xFF/0xFF because the observed MOSI channel was high. No new MAX7219 screenshot file was supplied with this request, so the final document does not claim a fresh visual capture.

# 24. Troubleshooting Matrix

| Symptom | Likely cause | Check / correction |
| --- | --- | --- |
| Completely blank | Shutdown, power, CS, or missing clock | Read the capture sequence and verify 0x0C=0x01 |
| All LEDs always on | Display-test register left enabled | Write 0x0F=0x00 |
| Numbers/letters look like segments | Decode mode enabled | Write 0x09=0x00 |
| Mirrored glyph | Bit order disagrees with physical module | Use 0x01/0x80 edge tests before modifying fonts |
| Upside-down glyph | Display-RAM address order reversed | Use first/last row corner pattern |
| HELLO appears OLLEH | Offset traversal reversed | Increment offset and keep char_index derived from virtual_col |
| Static glyph correct, scroll mirrored | Scroll masks use opposite bit convention | Use 1U << col for the finalized font |
| Analyzer shows FF FF | MOSI probe/channel is wrong or floating high | Probe PB15/DIN and share ground |
| Occasional corruption | 3.3 V VIH margin, wiring, or supply noise | Measure levels; shorten wiring; level shift if needed |
| Application freezes | Unbounded TXE/RXNE/BSY wait | Add timeout and fault reporting |

# 25. Review Checkpoints

| Question | Expected reasoning |
| --- | --- |
| Why are two SPI bytes sent per write? | MAX7219 accepts one 16-bit word containing address and data |
| Why must LOAD stay low for both bytes? | The word is shifted first and latched only on LOAD's rising edge |
| Why is decode mode zero? | The matrix needs raw pixel bits, not Code-B seven-segment translation |
| Why is MISO ignored? | MAX7219 writes are unidirectional from the MCU; DOUT serves cascaded devices |
| Why can a correct SPI trace still show a mirrored A? | Orientation is a row/column mapping issue above the transport layer |
| Why is 100 ms not an SPI requirement? | It is the human-visible animation frame delay |
| What makes the function blocking? | Busy-wait flag loops plus HAL_Delay occupy the control flow until completion |
| Why is 3.3 V a concern? | The bare device specifies VIH >= 3.5 V at its 4.0-5.5 V supply range |

# 26. Practical Exercises

Capture one MAX7219_Write() and label every address, data, clock, and LOAD event.

Use MAX7219_CheckPosition() to document the physical meaning of addresses 0x01 and 0x08 and data bits 0 and 7.

Predict the eight row bytes for a simple cross, then display and compare them.

Single-step the HELLO scroll for offsets 0, 1, 8, 9, and 17; write down virtual_col, char_index, and col_in_char.

Add a punctuation glyph through a separate lookup table without changing unsupported-character behavior.

Convert the scroll routine into a non-blocking function that emits one frame whenever a millisecond deadline expires.

Add a configurable orientation transform supporting row reverse, bit reverse, and 90-degree rotation.

Run a long-duration test at several intensity settings while monitoring supply voltage and temperature.

# 27. Key Takeaways

The MAX7219 receives a 16-bit address/data word and independently refreshes the LED matrix.

A dot matrix uses no-decode mode and treats the eight digit registers as raw display RAM.

Transport correctness, matrix orientation, font encoding, and animation direction are separate debugging layers.

The finalized scroll algorithm is a moving 8-column viewport and intentionally uses bit0-to-bit7 ordering for this module's corrected font.

Polling is an excellent learning implementation, but timeout handling and non-blocking animation are the next engineering steps.

Electrical logic-level compatibility must be verified; observed operation is not the same as guaranteed data-sheet margin.

# 28. Source Inventory and References

| Source | Use in this document |
| --- | --- |
| Attached main.c | Application flow, patterns, and continuous HELLO scroll |
| Attached max7219.c / max7219.h | Final driver, fonts, frame generation, register constants, and public API |
| Attached spi.c / spi.h | SPI2 configuration, polling transfer, GPIO mapping, and CS controls |
| Shared ChatGPT link | Learning history, analyzer diagnosis, orientation corrections, and scrolling derivation |
| Analog Devices MAX7219/MAX7221 data sheet | Electrical limits, serial timing, registers, RAM, intensity, scan, shutdown, and display test |
| Analog Devices MAX7219 product page | Device summary and feature cross-check |

Shared chat: https://chatgpt.com/share/6a2ef732-b840-83e8-ba44-20bcd1f7db30

Official data sheet: https://www.analog.com/media/en/technical-documentation/data-sheets/MAX7219-MAX7221.pdf

Official product page: https://www.analog.com/en/products/max7219.html

# 29. Final Engineering Assessment

Assessment: The finalized MAX7219 driver is logically coherent for a single 8x8 module and correctly implements no-decode bitmap rendering and right-to-left scrolling using the project's established orientation. The most important remaining risks are 3.3 V input-high margin, unbounded polling, unrelated MCP2515 startup coupling, and module-specific orientation assumptions.