MCP2515 SPI2 Polling-Based Project

Device Bring-Up, SPI Instruction Layer, Register Access, and Operating-Mode Verification

Document intent: An industry-grade learning reference built from the finalized attached source files, the shared MCP2515 learning chat, and Microchip's official MCP2515 data sheet.

| Field | Value |
| --- | --- |
| MCU / SPI | STM32F407-style target, SPI2 controller on APB1 |
| External controller | Microchip MCP2515 stand-alone CAN 2.0B controller |
| Current project scope | Polling SPI command/register bring-up and mode verification |
| SPI configuration | Mode 0, 8-bit, MSB-first, approximately 1 MHz |
| Authoritative code | main.c, mcp2515.c, spi.c, main.h, mcp2515.h, spi.h |
| Not yet covered | CAN bit timing, filters, frame transmit/receive, interrupts |

# 1. Document Control / Metadata

| Item | Details |
| --- | --- |
| Version | v1.0 |
| Shared learning source | https://chatgpt.com/share/6a2d9ad1-9584-83ee-876c-5869eeafb06a |
| Primary device source | Microchip MCP2515 Family Data Sheet DS20001801K |
| Code fidelity rule | Code snippets are taken only from the six attached finalized files |
| Validation boundary | No new screenshot file was attached in this turn; analyzer observations are taken from the shared chat |

# 2. Learning Objectives

Understand the MCP2515 as a stand-alone CAN controller accessed through SPI.

Separate SPI transport validation from CAN bus configuration and communication.

Understand RESET, READ, WRITE, READ STATUS, and BIT MODIFY instructions.

Read CANSTAT and CANCTRL and distinguish requested mode from actual mode.

Implement mode changes safely with a mask, timeout, and verification.

Recognize the next required work: oscillator-aware CAN bit timing and CAN transceiver/bus validation.

# 3. Project Objective

Bring up an MCP2515 through STM32 SPI2 using blocking polling transactions. Prove command framing, register read/write access, reset-to-Configuration-mode behavior, and transition to Normal mode before attempting CAN frame transmission.

# 4. Problem Statement

Core engineering problem: A correct SPI waveform does not prove that the MCP2515's CAN engine is configured. Device bring-up must progress through command validation, register validation, operating-mode validation, CAN bit timing, and finally CAN bus communication.

# 5. System Overview

The STM32 acts as the SPI controller and accesses all MCP2515 configuration/status registers. The MCP2515 contains three main functional areas: SPI interface logic, control/register logic, and the CAN protocol engine with transmit/receive buffers, masks, and filters.

STM32F407                         MCP2515                         CAN transceiver
SPI2 controller                  Stand-alone CAN controller      Physical-layer driver

PB12 CS  ----------------------> CS
PB13 SCK ----------------------> SCK
PB15 MOSI ---------------------> SI
PB14 MISO <--------------------- SO
                                  TXCAN ------------------------> TXD
                                  RXCAN <------------------------ RXD
                                                                   CANH / CANL

# 6. Requirements

| Requirement | Project value | Verification note |
| --- | --- | --- |
| MCP2515 supply | Chip supports 2.7 V to 5.5 V | Module-level regulator/transceiver circuitry must be checked separately |
| SPI mode | Mode 0 | MCP2515 supports SPI modes 0,0 and 1,1 |
| SPI frequency | About 1 MHz | Below the MCP2515's 10 MHz maximum |
| Chip select | Software CS on PB12 | CS remains low for each complete instruction |
| Reset result | Configuration mode expected | Verify CANSTAT.OPMOD[7:5] equals 0x80 |
| Mode change | Configuration to Normal | Write CANCTRL.REQOP[7:5], poll CANSTAT.OPMOD[7:5] |

# 7. Hardware Components

| Component | Role | Important caution |
| --- | --- | --- |
| STM32F407 board | SPI host/controller and UART debug output | Clock tree determines SPI2 peripheral clock |
| MCP2515 module | CAN controller accessed by SPI | Module voltage compatibility may differ from bare IC capability |
| CAN transceiver | Converts TXCAN/RXCAN logic to CANH/CANL | MCP2515 itself is not the physical-layer transceiver |
| Oscillator / crystal | MCP2515 timing source | CAN bit-rate calculation depends on actual oscillator frequency |
| Logic analyzer | Confirms command/address/data sequences | Decode with the selected SPI mode |

# 8. Software and Firmware Tools

| Tool | Use |
| --- | --- |
| STM32CubeIDE / CMSIS | Build, flash, and inspect STM32 registers |
| UART terminal | Display initialization and mode-test results |
| Logic 2 / SPI analyzer | Verify MCP2515 instruction byte streams |
| Microchip data sheet | Verify commands, registers, modes, timing, and reset behavior |

# 9. System Block Diagram

Application / main.c
        |
        +--> MCP2515 driver: reset, read, write, status, bit modify, mode control
                |
                +--> SPI2 byte transfer and CS control
                        |
                        +--> STM32 SPI2 registers / GPIOB pins
                                |
                                +--> MCP2515 SPI protocol block
                                        |
                                        +--> Control registers / CAN protocol engine

# 10. Firmware Architecture

| Module | Responsibility | Key interfaces |
| --- | --- | --- |
| spi.c / spi.h | SPI2 GPIO, peripheral setup, transfer primitives, CS control | SPI2_TransferByte(), buffer helpers |
| mcp2515.c / mcp2515.h | Device command layer and operating-mode management | Reset, ReadRegister, WriteRegister, BitModify, Get/SetMode |
| main.c | Bring-up sequence and UART result reporting | Read CANSTAT/CANCTRL/status, verify Config/Normal mode |
| main.h | Common STM32 HAL declarations | Error_Handler and HAL types |

# 11. MCP2515 Foundation

| Feature | Device capability | Current-project relevance |
| --- | --- | --- |
| CAN support | CAN 2.0B, standard/extended data and remote frames | Future phase |
| TX/RX resources | Three transmit buffers and two receive buffers | Future message handling |
| Acceptance logic | Two masks and six filters | Future receive filtering |
| SPI interface | Up to 10 MHz, modes 0,0 and 1,1 | Current project uses Mode 0 at about 1 MHz |
| Operating modes | Normal, Sleep, Loopback, Listen-only, Configuration | Config and Normal tested |

# 12. SPI Instruction Set Used

| Instruction | Opcode | SPI transaction | Use |
| --- | --- | --- | --- |
| RESET | 0xC0 | CS low -> C0 -> CS high | Reset registers and enter Configuration mode |
| READ | 0x03 | CS low -> 03 -> address -> dummy -> CS high | Read one register |
| WRITE | 0x02 | CS low -> 02 -> address -> data -> CS high | Write one register |
| BIT MODIFY | 0x05 | CS low -> 05 -> address -> mask -> data -> CS high | Change selected bits atomically |
| READ STATUS | 0xA0 | CS low -> A0 -> dummy -> CS high | Read condensed CAN status |

# 13. CANCTRL and CANSTAT Mode Control

| Register field | Meaning | Driver behavior |
| --- | --- | --- |
| CANCTRL.REQOP[7:5] | Requested operating mode | Set with BIT MODIFY and MCP2515_MODE_MASK |
| CANSTAT.OPMOD[7:5] | Current operating mode | Read back and compare with requested mode |
| MCP2515_MODE_MASK | 0xE0 | Isolates the three mode bits |
| Configuration mode | 0x80 | Expected after RESET |
| Normal mode | 0x00 | Allows normal CAN operation after bit timing is configured |

# 14. Main Algorithm and Flowchart

HAL and clock initialization
        |
        +--> USART2 initialization for diagnostic messages
        +--> SPI2 GPIO and peripheral initialization
        +--> SPI2 waveform test
        +--> MCP2515 RESET
        +--> Read CANSTAT, CANCTRL and READ STATUS
        +--> Verify Configuration mode
        +--> Request Normal mode and poll CANSTAT
        +--> Verify Normal mode
        +--> RESET again and confirm Configuration mode

# 15. Code Explanation

Code authority: All code excerpts below are copied from the attached finalized files. Recommendations are shown separately and do not silently replace the source.

## 15.1 Command Constants and Registers

#define MCP2515_CMD_RESET        0xC0
#define MCP2515_CMD_READ         0x03
#define MCP2515_CMD_WRITE        0x02
#define MCP2515_CMD_BIT_MODIFY   0x05
#define MCP2515_CMD_READ_STATUS  0xA0

#define MCP2515_CANSTAT   0x0E
#define MCP2515_CANCTRL   0x0F
#define MCP2515_MODE_MASK 0xE0

## 15.2 Reset and Register Read

void MCP2515_Reset(void)
{
    SPI2_CS_LOW();
    SPI2_TransferByte(MCP2515_CMD_RESET);
    SPI2_CS_HIGH();

    // Needed for oscillator stabilization
    HAL_Delay(120);
}

uint8_t MCP2515_ReadRegister(uint8_t reg)
{
    uint8_t reg_data;

    SPI2_CS_LOW();
    SPI2_TransferByte(MCP2515_CMD_READ);
    SPI2_TransferByte(reg);
    reg_data = SPI2_TransferByte(0xFF);
    SPI2_CS_HIGH();

    return reg_data;
}

Reset timing note: Microchip specifies 128 OSC1 cycles for oscillator startup after reset. The 120 ms delay is very conservative. A production driver can calculate a smaller safe delay from the actual MCP2515 oscillator and add margin.

## 15.3 Write, Status, and Bit Modify

uint8_t MCP2515_ReadStatus(void)
{
    uint8_t reg_data;

    SPI2_CS_LOW();
    SPI2_TransferByte(MCP2515_CMD_READ_STATUS);
    reg_data = SPI2_TransferByte(0xFF);
    SPI2_CS_HIGH();

    return reg_data;
}

void MCP2515_WriteRegister(uint8_t address, uint8_t data)
{
    SPI2_CS_LOW();
    SPI2_TransferByte(MCP2515_CMD_WRITE);
    SPI2_TransferByte(address);
    SPI2_TransferByte(data);
    SPI2_CS_HIGH();
}

void MCP2515_BitModify(uint8_t address,uint8_t mask,uint8_t data)
{
    SPI2_CS_LOW();
    SPI2_TransferByte(MCP2515_CMD_BIT_MODIFY);
    SPI2_TransferByte(address);
    SPI2_TransferByte(mask);
    SPI2_TransferByte(data);
    SPI2_CS_HIGH();
}

## 15.4 Mode Control and Verification

uint8_t MCP2515_GetMode(void)
{
    uint8_t canstat;

    canstat = MCP2515_ReadRegister(MCP2515_CANSTAT);

    return (canstat & MCP2515_MODE_MASK);
}

void MCP2515_SetMode(uint8_t mode)
{
    MCP2515_BitModify(MCP2515_CANCTRL,
                      MCP2515_MODE_MASK,
                      mode);
}

uint8_t MCP2515_SetModeVerify(uint8_t mode)
{
    uint32_t timeout = 100000;

    MCP2515_SetMode(mode);

    while(timeout--)
    {
        if(MCP2515_GetMode() == mode)
        {
            return 1;
        }
    }

    return 0;
}

Using BIT MODIFY for REQOP is a good design choice because it changes only the mode field rather than overwriting unrelated CANCTRL bits.

## 15.5 Main Bring-Up Sequence

MCP2515_Reset();

uint8_t canstat = MCP2515_ReadRegister(MCP2515_CANSTAT);
uint8_t canctrl = MCP2515_ReadRegister(MCP2515_CANCTRL);
uint8_t status  = MCP2515_ReadStatus();

MCP2515_SetModeVerify(MCP2515_MODE_NORMAL);
uint8_t mode = MCP2515_GetMode();

MCP2515_Reset();
canstat = MCP2515_ReadRegister(MCP2515_CANSTAT);

# 16. Embedded Engineering Review and Corrections

| Finding | Current behavior | Recommended correction |
| --- | --- | --- |
| Configuration-mode test | if(canstat & MCP2515_MODE_MASK) | Use (canstat & mask) == MCP2515_MODE_CONFIG |
| SetModeVerify result | Return value is ignored | Branch on success/failure and report timeout explicitly |
| Stale register variables | canctrl/status are read once before mode changes | Re-read registers after each operation before inspecting values |
| Reset delay | Fixed 120 ms | Use oscillator-derived minimum plus margin; keep longer delay only when module startup requires it |
| Mode argument | Caller is expected to pass a masked value | Optionally apply mode &= MCP2515_MODE_MASK before writing/comparing |
| Polling robustness | SPI and mode loops can block | Use time-based timeouts for production firmware |
| Header hygiene | spi.h has no include guard | Add a conventional include guard |
| Dependency boundary | main.c includes usart.h but it was not attached | USART debug implementation is outside this code audit |

## 16.1 Recommended Mode Checks

if((canstat & MCP2515_MODE_MASK) == MCP2515_MODE_CONFIG)
{
    USART2_SendString("MCP2515 Config mode OK\r\n");
}

if(MCP2515_SetModeVerify(MCP2515_MODE_NORMAL))
{
    USART2_SendString("MCP2515 Normal mode OK\r\n");
}
else
{
    USART2_SendString("MCP2515 Normal mode timeout\r\n");
}

Why equality matters: A nonzero masked result also occurs for Sleep, Loopback, and Listen-only modes. Exact equality proves the intended mode.

# 17. Testing and Validation

| Test | Expected SPI transaction / value | Meaning |
| --- | --- | --- |
| RESET | C0 under one CS-low window | Device reset command accepted |
| Read CANSTAT | 03 0E FF; returned mode field 0x80 after reset | SPI read and Configuration-mode state confirmed |
| Read CANCTRL | 03 0F FF | Requested-mode/control register is readable |
| READ STATUS | A0 FF | Special status instruction works |
| WRITE register | 02 address data | Register write path works |
| BIT MODIFY | 05 address mask data | Masked update path works |
| Normal mode | CANSTAT.OPMOD becomes 0x00 | Mode transition accepted |
| Second RESET | CANSTAT.OPMOD returns to 0x80 | Reset-to-Configuration behavior is repeatable |

Analyzer evidence: The shared chat records successful command-level captures, including READ CANSTAT using MOSI 0x03, 0x0E, 0xFF and a returned CANSTAT value consistent with Configuration mode. No new image file was attached in this turn.

# 18. Results

SPI2 transport and MCP2515 command framing are implemented correctly for first bring-up.

Reset, register read, register write, read status, and bit modify APIs are present.

Mode requests use CANCTRL.REQOP through BIT MODIFY.

Actual mode is verified through CANSTAT.OPMOD with a timeout loop.

The project proves SPI/device register communication but does not yet prove CAN bus communication.

# 19. Problems Faced and Debugging Notes

| Symptom | Likely reason | Debug action |
| --- | --- | --- |
| Reads 0xFF from every register | SO/MISO floating, wrong CS, wiring, power, or mode | Check CS/SCK/SI/SO and voltage compatibility |
| CANSTAT not 0x80 after reset | Reset not accepted, oscillator not stable, or bad SPI read | Capture C0 command and delay; then read 0x0E |
| Requested mode changes but old variable remains | Software variable was not refreshed | Re-read CANCTRL/CANSTAT after the operation |
| Normal mode reported but CAN does not communicate | Bit timing/transceiver/bus not configured | Configure CNF1/CNF2/CNF3 and validate CANH/CANL |
| Mode-change loop times out | Invalid mode, hardware state, oscillator issue, or SPI error | Read CANCTRL, CANSTAT, and error/status registers |
| Module works at 5 V but STM32 logic is 3.3 V | Board-level level shifting/regulator behavior varies | Inspect module schematic; do not infer logic safety from IC supply range alone |

# 20. Limitations

CAN bit timing registers CNF1, CNF2, and CNF3 are not configured.

Acceptance masks and filters are not configured.

Transmit/receive CAN buffers are not exercised.

INT-based reception and error handling are not implemented.

No attached screenshot file is available for embedding in this document.

USART source files used for debug output were not attached and were not audited.

# 21. Future Improvements

| Next step | Learning goal |
| --- | --- |
| Read oscillator marking | Establish the exact MCP2515 clock frequency |
| Configure CNF1/CNF2/CNF3 | Set CAN bit rate and sample point |
| Use Loopback mode | Validate CAN frame load/transmit/receive without a physical bus |
| Configure masks/filters | Control which standard/extended identifiers are accepted |
| Implement TX buffer APIs | Load identifier, DLC, payload, and request transmission |
| Implement RX polling | Check RX status, read receive buffers, clear interrupt flags |
| Add INT handling | Move from continuous status polling to event-driven reception |
| Add robust timeouts | Prevent permanent blocking during SPI and mode transitions |

# 22. Conclusion

This project establishes a correct MCP2515 polling bring-up layer. The firmware can reset the controller, execute the core SPI instructions, access CANSTAT/CANCTRL, and request and verify operating modes. The next engineering milestone is CAN timing and frame handling; entering Normal mode alone does not prove that a usable CAN network has been configured.

# Appendix A: Source Files

| File | Role |
| --- | --- |
| main.c | Initialization, diagnostics, reset/register tests, mode transitions |
| mcp2515.c | MCP2515 SPI instruction and operating-mode implementation |
| spi.c | SPI2 GPIO/peripheral configuration and polling transfers |
| main.h | HAL and application declarations |
| mcp2515.h | Commands, modes, registers, mask, public API |
| spi.h | SPI2 public API and CS macros |

# Appendix B: Verification Sources

| Source | Use | URL |
| --- | --- | --- |
| Microchip MCP2515 Family Data Sheet DS20001801K | Device architecture, SPI commands, modes, reset timing, registers | https://ww1.microchip.com/downloads/aemDocuments/documents/APID/ProductDocuments/DataSheets/MCP2515-Family-Data-Sheet-DS20001801K.pdf |
| Shared learning chat | Learning sequence and analyzer-observation context | https://chatgpt.com/share/6a2d9ad1-9584-83ee-876c-5869eeafb06a |
| Attached finalized code | Authoritative implementation snippets | Local files listed in Appendix A |

# Appendix C: Revision Checklist

| Check | Question |
| --- | --- |
| [ ] | Can I explain why MCP2515 is a CAN controller but not a CAN transceiver? |
| [ ] | Can I draw the READ, WRITE, RESET, READ STATUS, and BIT MODIFY byte sequences? |
| [ ] | Can I distinguish CANCTRL.REQOP from CANSTAT.OPMOD? |
| [ ] | Can I explain why entering Normal mode does not prove CAN communication? |
| [ ] | Can I identify why exact mode equality is safer than a nonzero mask test? |
| [ ] | Can I state the next registers required for CAN bit timing? |