MCP2515 SPI2 Loopback Mode Project

Internal CAN Frame Transmission, RXB0 Readback, and Polling-Based Register Verification

Document intent: A professional long-term learning reference for the finalized MCP2515 loopback project. Code snippets are grounded only in the attached source files; concepts are verified against the shared learning chat and Microchip MCP2515 data sheet.

| Field | Value |
| --- | --- |
| MCU / SPI | STM32F407-style target, SPI2 master on APB1 |
| External controller | Microchip MCP2515 stand-alone CAN 2.0B controller |
| Current project scope | Polling SPI, loopback-mode CAN frame transmit/receive validation |
| SPI configuration | Mode 0, 8-bit, MSB-first, approximately 1 MHz when PCLK1 is 32 MHz |
| CAN test frame | Standard ID 0x123, DLC 2, data bytes 0x11 and 0x22 |
| Authoritative code | main.c, mcp2515.c, spi.c, mcp2515.h, spi.h |

# 1. Document Control / Metadata

| Item | Details |
| --- | --- |
| Version | v1.0 |
| Shared learning source | https://chatgpt.com/share/6a3093d7-1ca4-83ee-b534-92ea538b7356 |
| Primary device source | Microchip MCP2515 Family Data Sheet DS20001801K |
| Code fidelity rule | Quoted code is taken only from the five attached finalized files |
| Design preset | compact_reference_guide |
| Validation boundary | No new screenshot file was attached in this turn; UART output and analyzer guidance are documented as expected checks |

# 2. Learning Objectives

Understand MCP2515 loopback mode and why it is useful before connecting a real CAN bus.

Verify the SPI command layer: RESET, READ, WRITE, READ STATUS, and BIT MODIFY.

Explain mode request and mode verification using CANCTRL.REQOP[7:5] and CANSTAT.OPMOD[7:5].

Decode the 8 MHz oscillator, 500 kbps, 8 TQ bit-timing configuration used by the source.

Follow how a standard 11-bit CAN ID maps into TXB0SIDH/TXB0SIDL and back from RXB0SIDH/RXB0SIDL.

Understand TXREQ, RX0IF, DLC, RX buffer flags, and why RX0IF must be cleared after reading.

Identify remaining production risks: unbounded waits, no TX-complete/RX-available timeout, no error-register handling, and shared-bus coupling.

# 3. Project Objective

Initialize the MCP2515, configure it for a verified 8 MHz / 500 kbps bit timing setup, enter loopback mode, transmit a standard CAN data frame from TXB0, receive it internally into RXB0, and print the transmitted and received frame fields over UART.

# 4. Problem Statement

Core engineering problem: A CAN controller can have a working SPI interface while the CAN frame path is still unproven. Loopback mode validates the MCP2515 message engine, TX buffer loading, identifier packing, DLC/data bytes, receive-buffer movement, and status flags without depending on a transceiver, termination, ACK from another node, or live CAN wiring.

# 5. System Overview

STM32F407 application
    |
    +-- UART2 diagnostics
    |
    +-- SPI2 polling driver
            |
            +-- MCP2515 command/register layer
                    |
                    +-- Configuration mode: CNF1/CNF2/CNF3, RX buffer policy
                    +-- Loopback mode: TXB0 -> internal CAN engine -> RXB0
                    +-- UART printout: transmitted frame and received frame

The MCP2515 contains a SPI protocol block, control logic/registers, and a CAN protocol engine with transmit and receive buffers. In this project, the physical CAN bus is intentionally bypassed by loopback mode, but the controller still exercises its transmit and receive paths internally.

# 6. Requirements

| Requirement | Project value | Verification point |
| --- | --- | --- |
| MCP2515 clock | 8 MHz module crystal, verified in the shared learning flow | Bit timing values depend directly on this frequency |
| CAN bit rate | 500 kbps target | CNF1=0x00, CNF2=0x91, CNF3=0x01 with 8 TQ |
| Mode | Loopback, code value 0x40 | CANSTAT & 0xE0 must equal 0x40 |
| SPI mode | Mode 0, MSB-first, 8-bit | MCP2515 supports modes 0,0 and 1,1 |
| SPI clock | Approximately 1 MHz | Below the MCP2515 10 MHz SPI limit |
| Frame type | Standard data frame | ID must be <= 0x7FF and DLC <= 8 |
| Receive policy | RXM[1:0] = 11 for RXB0 and RXB1 | Accept any message for first loopback testing |

# 7. Hardware Components

| Component | Role | Important caution |
| --- | --- | --- |
| STM32F407 board | SPI master and UART diagnostics | Clock tree controls APB1 and SPI2 SCK |
| MCP2515 module | CAN controller under test | Loopback proves controller logic, not external CAN wiring |
| 8 MHz oscillator | MCP2515 time base | A 16 MHz module would require different CNF values |
| CAN transceiver | Not required for loopback | Needed later for Normal mode on CANH/CANL |
| MAX7219 module | Still present in main.c as a visual side demo | Unrelated to MCP2515 loopback verification |
| Logic analyzer | Optional SPI transaction validation | Probe PB12, PB13, PB14, PB15 with common ground |

# 8. Software and Firmware Tools

STM32CubeIDE or equivalent build/debug environment.

UART terminal for the transmitted and received frame printout.

Logic 2 or another SPI analyzer for command/register waveform validation.

Microchip MCP2515 data sheet for register and timing verification.

The finalized source files attached in this request for all code-level details.

# 9. System Block Diagram

STM32 SPI2 pins                       MCP2515

PB12 GPIO CS  -----------------------> CS
PB13 SPI2_SCK -----------------------> SCK
PB15 SPI2_MOSI ----------------------> SI
PB14 SPI2_MISO <---------------------- SO
GND ---------------------------------> VSS
VCC ---------------------------------> VDD

Loopback path inside MCP2515:
TXB0 registers -> CAN protocol engine -> RXB0 registers
TXCAN pin remains recessive; no external CAN traffic is transmitted.

# 10. Firmware Architecture

| Module | Responsibility | Key APIs / data |
| --- | --- | --- |
| main.c | Test application, UART reporting, loopback send/read loop | id=0x123, msg={0x11,0x22}, MCP2515_SendMessage(), MCP2515_ReadMessage() |
| mcp2515.c | Device command layer, mode control, bit timing, frame TX/RX | Reset, ReadRegister, WriteRegister, BitModify, SetMode, SendMessage, ReadMessage |
| mcp2515.h | Commands, modes, register addresses, masks, public prototypes | MCP2515_MODE_LOOPBACK=0x40, MODE_MASK=0xE0, RX0IF mask=0x01 |
| spi.c | SPI2 peripheral and GPIO setup, byte-level polling transfer | SPI2_Init(), SPI2_GPIO_Init(), SPI2_TransferByte() |
| spi.h | SPI declarations and chip-select macros | MCP2515_CS_LOW/HIGH on PB12, MAX7219_CS_LOW/HIGH on PB9 |

# 11. Communication Protocol Used

MCP2515 register access is performed using SPI command transactions. CS must stay low for the entire command, and the first byte after CS goes low is interpreted as the instruction byte. The project uses single-register READ/WRITE operations plus BIT MODIFY for mode and flag manipulation.

| Instruction | Opcode | Source API | Transaction shape |
| --- | --- | --- | --- |
| RESET | 0xC0 | MCP2515_Reset() | CS low -> C0 -> CS high |
| READ | 0x03 | MCP2515_ReadRegister() | CS low -> 03 -> address -> dummy/read -> CS high |
| WRITE | 0x02 | MCP2515_WriteRegister() | CS low -> 02 -> address -> data -> CS high |
| READ STATUS | 0xA0 | MCP2515_ReadStatus() | CS low -> A0 -> dummy/read -> CS high |
| BIT MODIFY | 0x05 | MCP2515_BitModify() | CS low -> 05 -> address -> mask -> data -> CS high |

# 12. Loopback Mode Deep Dive

Loopback purpose: Loopback mode internally routes transmitted messages from the MCP2515 transmit buffers to receive buffers. The device does not drive CAN frames onto TXCAN, ignores ACK behavior for the external bus, and keeps TXCAN recessive. That makes it ideal for validating firmware before adding the transceiver and a second CAN node.

| Mode value | Macro | Meaning |
| --- | --- | --- |
| 0x00 | MCP2515_MODE_NORMAL | Normal external CAN bus operation |
| 0x20 | MCP2515_MODE_SLEEP | Low-power sleep |
| 0x40 | MCP2515_MODE_LOOPBACK | Internal TX-to-RX validation mode |
| 0x60 | MCP2515_MODE_LISTEN_ONLY | Silent monitor mode |
| 0x80 | MCP2515_MODE_CONFIG | Configuration mode for CNF, masks, filters, and setup |

Mode requests are written into CANCTRL.REQOP[7:5]. Actual mode must be confirmed from CANSTAT.OPMOD[7:5]. The source correctly uses MCP2515_MODE_MASK = 0xE0 to isolate those three bits.

# 13. Bit Timing Configuration

The shared learning flow states the module crystal is 8 MHz. The finalized code configures 500 kbps with 8 time quanta per bit. This is a compact, valid first-loopback setup.

MCP2515_WriteRegister(MCP2515_CNF1, 0x00);
MCP2515_WriteRegister(MCP2515_CNF2, 0x91);
MCP2515_WriteRegister(MCP2515_CNF3, 0x01);

| Register | Value | Decoded meaning | Contribution |
| --- | --- | --- | --- |
| CNF1 | 0x00 | BRP=0, SJW=1 TQ | TQ = 2 x (BRP+1) / FOSC = 250 ns at 8 MHz |
| CNF2 | 0x91 | BTLMODE=1, PropSeg=2 TQ, PS1=3 TQ | Sample point after SyncSeg+PropSeg+PS1 = 6 TQ |
| CNF3 | 0x01 | PS2=2 TQ | Completes total bit time to 8 TQ |
| Total | - | 1 + 2 + 3 + 2 = 8 TQ | 8 x 250 ns = 2 us, so bit rate = 500 kbps |
| Sample point | - | 6 / 8 TQ | 75 percent of bit time |

Oscillator dependency: These CNF values are not universal MCP2515 values. They are tied to the verified 8 MHz oscillator. A 16 MHz module would produce a different bit rate with the same CNF bytes.

# 14. Module-Wise Implementation

| Phase | Source behavior | Why it matters |
| --- | --- | --- |
| SPI init | PB12/PB9 CS outputs, PB13/PB14/PB15 AF5, SPI2 Mode 0 | Establishes command transport |
| MCP reset | Send 0xC0 and delay 100 ms | Returns device to default configuration mode |
| Config wait | Poll CANSTAT until 0x80 mode bits | CNF registers are modifiable only in Configuration mode |
| Bit timing | Write CNF1/CNF2/CNF3 | Defines 500 kbps timing for the internal CAN engine |
| Receive policy | Set RXM[1:0]=11 on RXB0/RXB1 and BUKT on RXB0 | Accepts any message for loopback bring-up |
| Mode switch | Bit-modify CANCTRL mode bits to 0x40 | Requests Loopback mode while preserving lower CANCTRL bits |
| Frame TX | Load TXB0 SIDH/SIDL/DLC/data and set TXREQ | Queues standard frame for internal transmission |
| Frame RX | Check RX0IF, read RXB0, clear RX0IF | Confirms internal receive path and frees RXB0 |

# 15. Main Algorithm and Flowchart

HAL_Init and SystemClock_Config
        |
        +--> USART2_Init for diagnostic output
        +--> SPI2_GPIO_Init and SPI2_Init
        +--> SPI2_Test waveform transaction on MCP2515 CS
        +--> MCP2515_Init
              |
              +--> Reset and wait for Configuration mode
              +--> Write CNF1/CNF2/CNF3
              +--> Accept all receive messages
              +--> Clear CANINTF
        +--> MCP2515_SetMode(LOOPBACK)
        +--> Poll CANSTAT until Loopback mode
        +--> Repeating test:
              +--> Send standard frame through TXB0
              +--> Print transmitted ID/DLC/data
              +--> Read RXB0 if RX0IF is set
              +--> Print received ID/DLC/data
              +--> Blink MAX7219 character A

# 16. Code Explanation

Code authority: All code blocks in this section are copied from the finalized attached source/header files. Review notes are intentionally separated so the document does not silently change your code.

## 16.1 Header Constants

//MCP commands
#define MCP2515_CMD_RESET        0xC0
#define MCP2515_CMD_READ         0x03
#define MCP2515_CMD_WRITE        0x02
#define MCP2515_CMD_BIT_MODIFY   0x05
#define MCP2515_CMD_READ_STATUS  0xA0

//CAN modes
#define MCP2515_MODE_NORMAL      0x00
#define MCP2515_MODE_SLEEP       0x20
#define MCP2515_MODE_LOOPBACK    0x40
#define MCP2515_MODE_LISTEN_ONLY 0x60
#define MCP2515_MODE_CONFIG      0x80

//MCP registers
#define MCP2515_CANSTAT   0x0E
#define MCP2515_CANCTRL   0x0F

#define MCP2515_CNF3   0x28
#define MCP2515_CNF2   0x29
#define MCP2515_CNF1   0x2A

#define MCP2515_CANINTE     0x2B
#define MCP2515_CANINTF     0x2C

#define MCP2515_TXB0CTRL    0x30
#define MCP2515_TXB0SIDH    0x31
#define MCP2515_TXB0SIDL    0x32
#define MCP2515_TXB0DLC     0x35
#define MCP2515_TXB0D0      0x36

#define MCP2515_RXB0CTRL    0x60
#define MCP2515_RXB0SIDH    0x61
#define MCP2515_RXB0SIDL    0x62
#define MCP2515_RXB0DLC     0x65
#define MCP2515_RXB0D0      0x66

#define MCP2515_RXB1CTRL	0x70

//Masks
#define MCP2515_MODE_MASK   			0xE0
#define TRANSMIT_REQ_MASK   			0x08
#define MCP2515_CANINTF_RX0IF_MASK   	0x01
#define MCP2515_CANINTF_RX1IF_MASK   	0x02

## 16.2 Application Test Variables

uint16_t id = 0x123;
	uint8_t dlc_len = 0x2;
	uint8_t msg[] = {0x11, 0x22};

	uint16_t rx_id = 0;
	uint8_t rx_dlc_len = 0;
	uint8_t rx_msg[8] = {0,0,0,0,0,0,0,0};

	uint8_t uart_msg_tx[100];
	uint8_t uart_msg_rx[100];

The test frame is a standard 11-bit CAN data frame: ID 0x123, DLC 2, payload 0x11 and 0x22. The receive buffers are initialized to zero so any stale data becomes visible if receive fails.

## 16.3 MCP2515 Initialization

MCP2515_Init(void)
{
	// Reset to reach config mode
	MCP2515_Reset();

	//check and wait till config mode
	while((MCP2515_ReadRegister(MCP2515_CANSTAT) & MCP2515_MODE_MASK) != MCP2515_MODE_CONFIG);

	/*Baudrate config , 500kbps ,8 N_TQ, SyncSeg = 1 TQ,
	PropSeg = 2 TQ ,PS1     = 3 TQ ,PS2     = 2 TQ
	Total   = 8 TQ*/
	MCP2515_WriteRegister(MCP2515_CNF1, 0x00);
	MCP2515_WriteRegister(MCP2515_CNF2, 0x91);
	MCP2515_WriteRegister(MCP2515_CNF3, 0x01);

	//Filter,mask settings, receive any message ID
	MCP2515_BitModify(MCP2515_RXB0CTRL, 0x60 , 0x60);
	MCP2515_BitModify(MCP2515_RXB1CTRL, 0x60 , 0x60);

	//enable RXB0 rollover to RXB1 */
	MCP2515_BitModify(MCP2515_RXB0CTRL, 0x04, 0x04);

	/* Clear interrupt flags */
	MCP2515_WriteRegister(MCP2515_CANINTF, 0x00);
}

## 16.4 SPI Command Primitives

MCP2515_Reset(void)
{
	MCP2515_CS_LOW();
	SPI2_TransferByte(MCP2515_CMD_RESET);
	MCP2515_CS_HIGH();

	// Needed for oscillator stabilization
	HAL_Delay(100);
}

MCP2515_ReadRegister(uint8_t reg)
{
	uint8_t reg_data;

	MCP2515_CS_LOW();
	SPI2_TransferByte(MCP2515_CMD_READ);
	SPI2_TransferByte(reg);
	reg_data = SPI2_TransferByte(0xFF);
	MCP2515_CS_HIGH();

	return reg_data;
}

MCP2515_WriteRegister(uint8_t address, uint8_t data)
{
	MCP2515_CS_LOW();
	SPI2_TransferByte(MCP2515_CMD_WRITE);
	SPI2_TransferByte(address);
	SPI2_TransferByte(data);
	MCP2515_CS_HIGH();
}

MCP2515_BitModify(uint8_t address,uint8_t mask,uint8_t data)
{
	MCP2515_CS_LOW();
	SPI2_TransferByte(MCP2515_CMD_BIT_MODIFY);
	SPI2_TransferByte(address);
	SPI2_TransferByte(mask);
	SPI2_TransferByte(data);
	MCP2515_CS_HIGH();
}

## 16.5 Mode Control

MCP2515_GetMode(void)
{
    uint8_t canstat;

    canstat = MCP2515_ReadRegister(MCP2515_CANSTAT);

    return (canstat & MCP2515_MODE_MASK);
}

MCP2515_SetMode(uint8_t mode)
{
    MCP2515_BitModify(MCP2515_CANCTRL,
                      MCP2515_MODE_MASK,
                      mode);
}

MCP2515_SetModeVerify(uint8_t mode)
{
    uint32_t timeout = 100000;

    MCP2515_SetMode(mode);

    while(timeout--)
    {
        if(MCP2515_GetMode() == mode)
        {
            return 1;   // success
        }
    }

    return 0;           // failed
}

The helper MCP2515_SetModeVerify() already implements bounded polling, but main.c currently uses an unbounded while loop for loopback entry. This is documented as a correction note rather than changing the finalized code.

## 16.6 Standard Frame Transmission

MCP2515_SendMessage(uint16_t id, uint8_t *data, uint8_t len)
{
    uint8_t sidh;
    uint8_t sidl;

    /* Standard CAN ID is 11-bit */
    if(id > 0x7FF)
    {
        return 0;
    }

    /* CAN data length max is 8 bytes */
    if(len > 8)
    {
        return 0;
    }

    if((data == 0) && (len > 0))
    {
        return 0;
    }

    /* Check TXB0 is free: TXREQ must be 0 */
    if(MCP2515_ReadRegister(MCP2515_TXB0CTRL) & 0x08)
    {
        return 0;
    }

    /*
     * Standard ID mapping:
     * TXB0SIDH = SID10..SID3
     * TXB0SIDL[7:5] = SID2..SID0
     */
    sidh = (uint8_t)(id >> 3);
    sidl = (uint8_t)((id & 0x07) << 5);

    /* Load standard ID */
    MCP2515_WriteRegister(MCP2515_TXB0SIDH, sidh);
    MCP2515_WriteRegister(MCP2515_TXB0SIDL, sidl);

    /* Standard data frame, DLC = len */
    MCP2515_WriteRegister(MCP2515_TXB0DLC, len & 0x0F);

    /* Load data bytes */
    for(uint8_t i = 0; i < len; i++)
    {
        MCP2515_WriteRegister(MCP2515_TXB0D0 + i, data[i]);
    }

    /* Request transmission: set TXB0CTRL.TXREQ */
    MCP2515_BitModify(MCP2515_TXB0CTRL, 0x08, 0x08);

    return 1;
}

| Step | Code action | Register effect |
| --- | --- | --- |
| Validate ID | id <= 0x7FF | Allows standard 11-bit identifier only |
| Validate DLC | len <= 8 | CAN classic payload size limit |
| Check TXB0 | TXB0CTRL & 0x08 | TXREQ must be clear before loading TXB0 |
| Pack ID | SIDH=id>>3, SIDL=(id&7)<<5 | Maps SID10..SID0 into standard-ID registers |
| Write DLC | TXB0DLC = len & 0x0F | Defines frame payload length |
| Write data | TXB0D0+i | Loads payload bytes |
| Request TX | BitModify TXB0CTRL mask 0x08 data 0x08 | Sets TXREQ |

## 16.7 RXB0 Readback

MCP2515_ReadMessage(uint16_t *id, uint8_t *data, uint8_t *len)
{
    uint8_t sidh;
    uint8_t sidl;
    uint8_t dlc;
    uint8_t canintf;

    /* Check if RXB0 has received message */
    canintf = MCP2515_ReadRegister(MCP2515_CANINTF);

    if((canintf & MCP2515_CANINTF_RX0IF_MASK) == 0)
    {
        return 0;   /* No message available */
    }

    /* Read standard identifier registers */
    sidh = MCP2515_ReadRegister(MCP2515_RXB0SIDH);
    sidl = MCP2515_ReadRegister(MCP2515_RXB0SIDL);

    /*
     * Reconstruct 11-bit standard ID:
     * SIDH contains SID10..SID3
     * SIDL[7:5] contains SID2..SID0
     */
    *id = ((uint16_t)sidh << 3) | (sidl >> 5);

    /* Read DLC */
    dlc = MCP2515_ReadRegister(MCP2515_RXB0DLC);
    dlc = dlc & 0x0F;

    if(dlc > 8)
    {
        dlc = 8;
    }

    *len = dlc;

    /* Read data bytes */
    for(uint8_t i = 0; i < dlc; i++)
    {
        data[i] = MCP2515_ReadRegister(MCP2515_RXB0D0 + i);
    }

    /*
     * Clear RX0IF after reading message.
     * This tells MCP2515 RXB0 is free again.
     */
    MCP2515_BitModify(MCP2515_CANINTF,
                      MCP2515_CANINTF_RX0IF_MASK,
                      0x00);

    return 1;
}

The receive function checks CANINTF.RX0IF before touching RXB0. It reconstructs the standard ID using the inverse of the transmit mapping, bounds DLC to 8, reads the data bytes, and clears RX0IF using BIT MODIFY so RXB0 can accept another frame.

## 16.8 Application Initialization and Loop

USART2_Init();

  USART2_SendString("UART Initialized....!!\r\n");

  SPI2_GPIO_Init();
  SPI2_Init();
  USART2_SendString("SPI Initialized.....!!\r\n");

  SPI2_Test();

  // Initialize MCP and move to loopback mode
  MCP2515_Init();

  //Enter loopback mode
  MCP2515_SetMode(MCP2515_MODE_LOOPBACK);

  //check and wait till loopback mode
  while((MCP2515_ReadRegister(MCP2515_CANSTAT) & MCP2515_MODE_MASK) != MCP2515_MODE_LOOPBACK);

  uint8_t canstat = MCP2515_ReadRegister(MCP2515_CANSTAT);

  if((canstat & MCP2515_MODE_MASK) == MCP2515_MODE_LOOPBACK)
  {
      USART2_SendString("MCP2515 Loopback mode OK\r\n");
  }
  else
  {
      USART2_SendString("MCP2515 Loopback mode failed\r\n");
  }

  // Simple hardware test
  MAX7219_Write(0x0F, 0x01);   // All LEDs ON
  HAL_Delay(2000);
  MAX7219_Write(0x0F, 0x00);   // Test OFF

  MAX7219_Init();
  MAX7219_Clear();

//Transmitting standard CAN frame
	  MCP2515_SendMessage(id,msg,dlc_len);
	  sprintf(uart_msg_tx,"Message Transmitted : ID = 0x%02X, DLC = 0x%02X, DATA1 = 0x%02X, DATA2 = 0x%02X\r\n",id,dlc_len,msg[0],msg[1]);
	  USART2_SendString(uart_msg_tx);

	  //Receiving frame
	  MCP2515_ReadMessage(&rx_id,rx_msg,&rx_dlc_len);
	  sprintf(uart_msg_rx,"Message Received : ID = 0x%02X, DLC = 0x%02X, DATA1 = 0x%02X, DATA2 = 0x%02X\r\n",rx_id,rx_dlc_len,rx_msg[0],rx_msg[1]);
	  USART2_SendString(uart_msg_rx);

	  MAX7219_CharBlinking('A');
	  //MAX7219_StringBlinking("Hello Via Matrix");

	  //MAX7219_ScrollText_RtoL("HELLO", 100);

## 16.9 SPI Transport

SPI2_GPIO_Init(void)
{
    /* Enable GPIOB clock */
    RCC->AHB1ENR |= RCC_AHB1ENR_GPIOBEN;

    /*
     * PB9  = CS_MAX7219
     * PB12 = CS_MCP2515
     * PB13 = SPI2_SCK AF
     * PB14 = SPI2_MISO AF
     * PB15 = SPI2_MOSI AF
     */

    /* Clear mode bits */
    GPIOB->MODER &= ~((3UL << (9 * 2))  |
    				  (3UL << (12 * 2)) |
                      (3UL << (13 * 2)) |
                      (3UL << (14 * 2)) |
                      (3UL << (15 * 2)));

    /* Set modes */
    GPIOB->MODER |=  ((1UL << (9 * 2))  |   /* PB9 output */
    				  (1UL << (12 * 2)) |   /* PB12 output */
                      (2UL << (13 * 2)) |   /* PB13 AF */
                      (2UL << (14 * 2)) |   /* PB14 AF */
                      (2UL << (15 * 2)));   /* PB15 AF */

    /* Push-pull */
    GPIOB->OTYPER &= ~((1UL << 9)  |
    				   (1UL << 12) |
                       (1UL << 13) |
                       (1UL << 14) |
                       (1UL << 15));

    /* Speed: medium/high for SPI */
    GPIOB->OSPEEDR &= ~((3UL << (9 * 2))  |
    					(3UL << (12 * 2)) |
                        (3UL << (13 * 2)) |
                        (3UL << (14 * 2)) |
                        (3UL << (15 * 2)));

    GPIOB->OSPEEDR |=  ((2UL << (9 * 2)) |
    					(2UL << (12 * 2)) |
                        (2UL << (13 * 2)) |
                        (2UL << (14 * 2)) |
                        (2UL << (15 * 2)));

    /* Pull config: CS pull-up, others no pull */
    GPIOB->PUPDR &= ~((3UL << (9 * 2))  |
    				  (3UL << (12 * 2)) |
                      (3UL << (13 * 2)) |
                      (3UL << (14 * 2)) |
                      (3UL << (15 * 2)));

    GPIOB->PUPDR |=  (1UL << (12 * 2));     /* PB12 pull-up */
    GPIOB->PUPDR |=  (1UL << (9  * 2));     /* PB9 pull-up */

    /* AF5 for PB13, PB14, PB15 */
    GPIOB->AFR[1] &= ~((0xFUL << ((13 - 8) * 4)) |
                       (0xFUL << ((14 - 8) * 4)) |
                       (0xFUL << ((15 - 8) * 4)));

    GPIOB->AFR[1] |=  ((5UL << ((13 - 8) * 4)) |
                       (5UL << ((14 - 8) * 4)) |
                       (5UL << ((15 - 8) * 4)));

    /* Keep CS high initially */
    MCP2515_CS_HIGH();
    MAX7219_CS_HIGH();

}

SPI2_Init(void)
{
    /* Enable SPI2 clock */
    RCC->APB1ENR |= RCC_APB1ENR_SPI2EN;

    /* Disable SPI before configuration */
    SPI2->CR1 &= ~SPI_CR1_SPE;

    /* Clear CR1 configuration bits we care about */
    SPI2->CR1 = 0;

    /*
     * Master mode
     * Software slave management
     * Internal slave select high
     */
    SPI2->CR1 |= SPI_CR1_MSTR;
    SPI2->CR1 |= SPI_CR1_SSM;
    SPI2->CR1 |= SPI_CR1_SSI;

    /*
     * Mode 0: CPOL = 0, CPHA = 0
     */
    SPI2->CR1 &= ~SPI_CR1_CPOL;
    SPI2->CR1 &= ~SPI_CR1_CPHA;

    /*
     * 8-bit frame, MSB first
     */
    SPI2->CR1 &= ~SPI_CR1_DFF;
    SPI2->CR1 &= ~SPI_CR1_LSBFIRST;

    /*
     * Baud rate = PCLK / 32
     * If PCLK1 = 32 MHz, SCK = 1 MHz
     */
    SPI2->CR1 &= ~SPI_CR1_BR;
    SPI2->CR1 |= SPI_CR1_BR_2;

    /*
     * Full duplex 2-line
     */
    SPI2->CR1 &= ~SPI_CR1_BIDIMODE;
    SPI2->CR1 &= ~SPI_CR1_RXONLY;

    /* Enable SPI */
    SPI2->CR1 |= SPI_CR1_SPE;
}

SPI2_TransferByte(uint8_t byte)
{
    uint8_t received;

    //waiting for TX buffer empty
    while(!(SPI2->SR & SPI_SR_TXE));
    *((volatile uint8_t *)&SPI2->DR) = byte;

    //waiting for RX buffer Not empty
    while(!(SPI2->SR & SPI_SR_RXNE));
    received = *((volatile uint8_t *)&SPI2->DR);

    //Wait till SPI completes
    while(SPI2->SR & SPI_SR_BSY);

    return received;
}

# 17. Testing and Validation

| Test | Expected result | Why it proves progress |
| --- | --- | --- |
| SPI waveform test | PB12 CS low, 0xAA 0x55 0x96 0x3C on MOSI | Confirms SPI2 pin mapping and clocking |
| Reset/read mode | CANSTAT mode bits become 0x80 | Confirms MCP2515 responds and enters Configuration mode |
| CNF write | CNF1=00, CNF2=91, CNF3=01 if read back | Confirms bit timing registers were written |
| Loopback entry | CANSTAT & 0xE0 equals 0x40 | Confirms requested mode became actual mode |
| TX frame | UART prints ID=0x123, DLC=0x02, data 11 22 | Confirms transmit function accepted the frame |
| RX frame | UART prints the same ID, DLC, and data | Confirms internal loopback path and RXB0 decode |
| RX0IF clear | Next loop continues receiving | Confirms RX buffer is freed after reading |

# 18. Results

Expected UART result: For a passing loopback test, the transmitted and received UART lines should match: ID 0x123, DLC 0x02, DATA1 0x11, DATA2 0x22. If the receive line stays at zero values, check RX0IF, loopback mode, acceptance mode, and TXREQ behavior.

Expected repeated UART pattern:

Message Transmitted : ID = 0x123, DLC = 0x02, DATA1 = 0x11, DATA2 = 0x22
Message Received : ID = 0x123, DLC = 0x02, DATA1 = 0x11, DATA2 = 0x22

The current printf format uses %02X for the 16-bit ID, so it will still print enough digits for 0x123 in typical printf implementations. For clarity in future code, %03X is a better visual fit for standard 11-bit CAN IDs.

# 19. Problems Faced and Debugging Notes

| Symptom | Likely cause | Debug action |
| --- | --- | --- |
| CANSTAT never becomes 0x80 after reset | CS/SPI wiring, oscillator startup, wrong MISO, or no power | Capture RESET then READ CANSTAT; verify 100 ms delay and SO activity |
| Loopback wait never exits | Mode request not accepted or pending TX blocks transition | Use MCP2515_SetModeVerify() timeout and read CANCTRL/CANSTAT |
| TX function returns 0 | Invalid ID/DLC/data pointer or TXB0 TXREQ still set | Inspect return value and TXB0CTRL bit 3 |
| RX returns 0 | RX0IF not set | Read CANINTF, check loopback mode and RXB0CTRL RXM bits |
| Received ID mismatch | SIDH/SIDL packing or extended-frame bit issue | Probe TXB0SIDH/SIDL and RXB0SIDH/SIDL |
| Data mismatch | DLC/data register order or stale RX buffer | Read RXB0DLC and RXB0D0..D7 directly |
| Only first loop works | RX0IF not cleared | Confirm BIT MODIFY on CANINTF mask 0x01 data 0x00 |
| Works in loopback but not on bus | No transceiver, no termination, no ACK node, or wrong bit timing | Move to Normal mode only after physical-layer validation |

# 20. Embedded Engineering Review

| Finding | Impact | Recommended correction |
| --- | --- | --- |
| Unbounded waits in MCP2515_Init() and main.c | Firmware can hang permanently if MCP2515 is disconnected or mode switch fails | Use timeout-return helpers consistently |
| MCP2515_SendMessage() return value ignored | UART can claim transmit success even when driver rejected the frame | Check return value before printing transmitted message |
| MCP2515_ReadMessage() return value ignored | UART can print stale/zero receive data when RX0IF was not set | Print received frame only after function returns 1 |
| No TX-complete or error status check | TXREQ, TXERR, MLOA, ABTF, TX0IF are not reported | Poll TXB0CTRL/CANINTF/EFLG after TXREQ |
| CANINTF fully cleared during init | Acceptable at startup, but BIT MODIFY is safer for live flags | Use BIT MODIFY for individual flags during runtime |
| spi.h has no include guard | Repeated includes may cause maintenance issues | Add #ifndef/#define/#endif |
| MAX7219 remains in loopback test application | Adds unrelated delays and shared-bus traffic | Split MCP2515 loopback and MAX7219 demos or guard with compile-time options |
| No RXB1 handling despite rollover enabled | A busy loop could miss overflow/rollover behavior | Check RX1IF and read RXB1 in later receive driver |
| No normal-mode physical validation | Loopback success does not prove CANH/CANL operation | Later test with transceiver, 120 ohm termination, and another ACKing CAN node |

# 21. Limitations

This project validates MCP2515 internal loopback, not real CAN bus signaling.

Only standard data frames are implemented; extended IDs and remote frames are not handled.

Only TXB0 and RXB0 are used in the frame API.

Polling and HAL_Delay make the test simple but block concurrent work.

Interrupt pin INT, CANINTE enables, RXB1, error counters, and EFLG are not integrated into the main loop.

The code assumes an 8 MHz MCP2515 oscillator for the selected CNF values.

# 22. Future Improvements

Change MCP2515_Init() to return success/failure and replace all endless waits with bounded polling.

Check return values from MCP2515_SendMessage() and MCP2515_ReadMessage() before printing status.

Add direct CNF/CANCTRL/CANSTAT/CANINTF readback diagnostics after initialization.

Add TX completion and error reporting through TXB0CTRL, CANINTF.TX0IF, and EFLG.

Add RXB1 support and overflow detection using RX1IF, RX0OVR, and RX1OVR.

Add extended ID support by mapping EID registers and EXIDE.

Move to interrupt-driven receive using the MCP2515 INT pin.

Create a Normal-mode CAN bus test with transceiver, termination, and a second node.

# 23. Conclusion

Conclusion: The finalized firmware correctly demonstrates MCP2515 loopback fundamentals: SPI register access, configuration-mode setup, 8 MHz / 500 kbps bit timing, loopback-mode request/verification, standard-frame packing into TXB0, internal receive into RXB0, and RX0IF clearing. The most valuable next step is making the polling flow robust with timeouts and explicit status checks before moving to Normal-mode bus communication.

# 24. Appendix: Source Reference

| File | Used for |
| --- | --- |
| D:\Embedded Communication Protocols\SPI2_MCP2515_LoopBack\Core\Src\main.c | Application flow, test frame, mode wait, UART printouts, loopback loop |
| D:\Embedded Communication Protocols\SPI2_MCP2515_LoopBack\Core\Src\mcp2515.c | MCP2515 command layer, initialization, mode control, TX/RX APIs |
| D:\Embedded Communication Protocols\SPI2_MCP2515_LoopBack\Core\Src\spi.c | SPI2 GPIO/peripheral setup and polling byte transfer |
| D:\Embedded Communication Protocols\SPI2_MCP2515_LoopBack\Core\Inc\mcp2515.h | MCP2515 commands, modes, registers, masks, prototypes |
| D:\Embedded Communication Protocols\SPI2_MCP2515_LoopBack\Core\Inc\spi.h | SPI declarations and chip-select macros |
| https://chatgpt.com/share/6a3093d7-1ca4-83ee-b534-92ea538b7356 | Loopback learning history, 8 MHz oscillator confirmation, bit timing discussion |
| https://ww1.microchip.com/downloads/aemDocuments/documents/APID/ProductDocuments/DataSheets/MCP2515-Family-Data-Sheet-DS20001801K.pdf | Official MCP2515 behavior, registers, SPI instructions, modes, flags, and timing |

# 25. Appendix: Revision Checklist

| Check | Status |
| --- | --- |
| All five finalized source/header files read | Done |
| Shared ChatGPT loopback conversation downloaded and extracted | Done |
| Official MCP2515 data sheet used for verification | Done |
| Loopback mode, CANCTRL/CANSTAT, TXB0, RXB0, TXREQ, RX0IF covered | Done |
| CNF1/CNF2/CNF3 8 MHz / 500 kbps calculation included | Done |
| Exact source snippets included for key driver functions | Done |
| Corrections and risks separated from finalized code | Done |
| No screenshots included because none were attached in this turn | Done |