SPI Polling-Based Project

Theory, Bare-Metal Implementation, Register Review, and Logic Analyzer Validation

Document intent: This document converts the SPI polling learning data from the shared Embedded Protocol Mastery chat into a long-term professional reference. It uses the shared-chat implementation as the current code reference because separate finalized source files were not attached in this thread.

| Field | Value |
| --- | --- |
| Protocol | SPI - Serial Peripheral Interface |
| Project type | Bare-metal SPI2 polling validation |
| MCU family | STM32F4 / STM32F407-style register model |
| Peripheral | SPI2 on APB1 |
| Pins | PB12 CS GPIO, PB13 SCK, PB14 MISO, PB15 MOSI, AF5 for SPI2 pins |
| Validation evidence | Logic analyzer: CS low, SCK bursts, MOSI 0xAA 0x55 0x96 0x3C, MISO 0xFF |
| Code source boundary | Reviewed shared-chat snippets; final .c/.h files not attached here |

# 1. Document Control / Metadata

| Item | Details |
| --- | --- |
| Version | v1.0 |
| Source chat | https://chatgpt.com/share/6a2864ec-4480-83ec-8632-441f7ec42f84 |
| Extraction scope | SPI polling-based theory and implementation only |
| Excluded scope | MCP2515 CAN driver internals, SPI interrupt/DMA, unrelated USART/DMA material |
| Document style | doc-m compact reference guide |
| Verification note | Embedded-engineering audit performed against STM32F4 SPI behavior and the attached analyzer evidence |

# 2. Learning Objectives

Explain why polling SPI waits on TXE, RXNE, and BSY.

Understand why every SPI transmit also produces a received byte.

Configure SPI2 pins PB12/PB13/PB14/PB15 for software CS and AF5 SPI signals.

Configure SPI2 as master, Mode 0, 8-bit, MSB-first, 2-line full-duplex.

Calculate SCK from APB1 clock and SPI baud-rate prescaler.

Write a safe byte-transfer primitive using explicit 8-bit DR access.

Validate the waveform using a logic analyzer before connecting a real SPI peripheral.

# 3. Project Objective

Build a reliable bare-metal SPI polling foundation on STM32F4 using SPI2. The first milestone is not reading a real sensor or CAN controller; it is proving that the MCU generates correct CS, SCK, and MOSI waveforms and that the firmware handles SPI's simultaneous receive path correctly.

# 4. Problem Statement

Core problem: A polling SPI driver can appear to transmit correctly while still being architecturally incomplete if it ignores RXNE, never reads DR, releases CS before BSY clears, or configures GPIO/SPI fields by OR-ing bits without clearing old state.

# 5. System Overview

The project configures STM32 SPI2 as a controller/master. PB12 is used as software-controlled chip select, and PB13/PB14/PB15 are mapped to SPI2 SCK/MISO/MOSI through alternate function AF5. The first test sends known bytes without a connected slave and verifies the MOSI and SCK lines on a logic analyzer.

| Signal | Pin | Mode | Purpose | Validation |
| --- | --- | --- | --- | --- |
| CS / NSS | PB12 | GPIO output | Software chip select, active low | One low window around the transaction |
| SCK | PB13 | AF5 SPI2 | Controller-generated clock | 1 MHz bursts for each byte |
| MISO | PB14 | AF5 SPI2 | Peripheral-to-controller data | 0xFF when no slave drives the line |
| MOSI | PB15 | AF5 SPI2 | Controller-to-peripheral data | 0xAA, 0x55, 0x96, 0x3C decoded |

# 6. Requirements

| Requirement | Chosen value | Reason |
| --- | --- | --- |
| SPI instance | SPI2 | Available on APB1 and mapped to PB13/PB14/PB15 |
| Peripheral clock | APB1 SPI clock = 32 MHz | Shared-chat implementation basis |
| Baud prescaler | /32 using BR[2:0] | Produces 1 MHz SCK for clean debug captures |
| Mode | Mode 0: CPOL=0, CPHA=0 | Simple starting mode and useful for many SPI devices |
| Frame | 8-bit, MSB first | Matches common SPI device command/data byte transfers |
| NSS handling | Software CS on PB12 | Keeps transaction framing explicit and visible |

# 7. Hardware Components

| Component | Use |
| --- | --- |
| STM32F407-development board | Runs bare-metal SPI2 polling firmware |
| Logic analyzer | Verifies CS, SCK, MOSI, and MISO timing |
| Jumper wires / probe leads | Connect analyzer channels to PB12-PB15 and ground |
| Future SPI device | MCP2515 or other peripheral can be connected after waveform validation |

# 8. Software and Firmware Tools

| Tool | Purpose |
| --- | --- |
| STM32CubeIDE / bare-metal project | Build and flash the polling driver |
| CMSIS device headers | Register names such as RCC_APB1ENR_SPI2EN, SPI_CR1_MSTR, SPI_SR_TXE |
| STM32F4 reference manual RM0090 | Register behavior and status flag semantics |
| Logic 2 / Saleae analyzer | Decode SPI Mode 0 waveform and byte values |
| Terminal/debugger | Inspect register values and watch RXNE/TXE/BSY behavior |

# 9. System Block Diagram

STM32F4 CPU / APB1 bus
        |
        +--> RCC enables GPIOB and SPI2 clocks
        |
        +--> GPIOB
        |     PB12  GPIO output  -> CS / Enable
        |     PB13  AF5 SPI2     -> SCK
        |     PB14  AF5 SPI2     -> MISO
        |     PB15  AF5 SPI2     -> MOSI
        |
        +--> SPI2
              CR1: master, Mode 0, /32, 8-bit, MSB first, full duplex
              SR : TXE, RXNE, BSY, OVR, MODF
              DR : software-visible transmit/receive data register

# 10. Firmware Architecture

| Layer | Responsibility | Key APIs |
| --- | --- | --- |
| GPIO layer | Pin mode, speed, pull, alternate-function setup, CS idle high | SPI2_GPIO_Init(), SPI2_CS_LOW(), SPI2_CS_HIGH() |
| SPI config layer | SPI2 CR1 setup and SPE enable | SPI2_Init() |
| Polling transfer layer | Wait TXE, write DR, wait RXNE, read DR, wait BSY | SPI2_TransferByte() |
| Buffer helper layer | Transmit/receive arrays using the byte primitive | SPI2_TransmitBuffer(), SPI2_ReceiveBuffer(), SPI2_TransferBuffer() |
| Validation layer | Send known bytes and inspect analyzer capture | SPI2_Test(), analyzer decode |

# 11. Communication Protocol Used

This document focuses on raw SPI transport, not a device protocol. The tested transaction is a controller-driven SPI Mode 0 byte stream. Since no real slave is attached in the validation screenshot, MISO data is not treated as a valid device response.

| Protocol aspect | Current project behavior |
| --- | --- |
| Clocking | Controller generates SCK at about 1 MHz |
| Data direction | MOSI carries known test bytes; MISO is sampled but not meaningful without a slave |
| Chip select | PB12 goes low before the byte group and high after transfer completion |
| Frame size | 8-bit frames |
| Bit order | MSB first |
| Mode | CPOL=0 and CPHA=0, also called SPI Mode 0 |

# 12. Module-Wise Implementation

| Module | Implementation detail | Engineering check |
| --- | --- | --- |
| RCC | Enable GPIOB on AHB1 and SPI2 on APB1 | Touch registers only after clock enable |
| GPIO MODER | PB12 output, PB13/PB14/PB15 alternate function | Clear 2-bit fields before setting |
| GPIO AFR | AF5 in GPIOB->AFR[1] for PB13-PB15 | Use AFR[1] for pins 8..15 |
| GPIO pulls | CS pull-up optional; SPI pins normally no pull | MISO may need pull when no slave selected |
| SPI CR1 | Master, SSM/SSI, Mode 0, 8-bit, MSB first, /32, full duplex | Clear BR before setting /32 |
| Transfer | Wait TXE, write DR, wait RXNE, read DR, wait BSY | Read received byte to avoid OVR |

# 13. Core Concept Deep Dive: Polling SPI

Polling model: Polling means the CPU repeatedly checks SPI status flags and moves forward only when hardware reports that the next operation is safe.

| Flag | Meaning | Polling use | Mistake to avoid |
| --- | --- | --- | --- |
| TXE | Transmit buffer empty | Wait before writing DR | TXE does not mean the whole SPI frame is complete |
| RXNE | Receive buffer not empty | Wait before reading DR | Do not ignore it even during transmit-only tests |
| BSY | SPI busy | Wait before deasserting CS | Do not raise CS while the last bit is still shifting |
| OVR | Overrun | Debug if RX data was not read | SPI can overrun if received data is left unread |
| MODF | Mode fault | Check NSS/master configuration | Software NSS usually needs SSM=1 and SSI=1 in master mode |

# 14. SPI Polling Flow

Polling byte transfer:

Start
  |
  +-- Wait until TXE = 1
  +-- Write 8-bit value into SPI2->DR
  +-- Hardware generates SCK and shifts MOSI/MISO
  +-- Wait until RXNE = 1
  +-- Read 8-bit value from SPI2->DR
  +-- Wait until BSY = 0
  |
End

The key difference from UART polling is that SPI transmit and receive are coupled. Writing one byte starts a full-duplex exchange; therefore a robust transfer primitive should return the received byte.

# 15. Register Configuration Reference

| Register | Bits / fields | Purpose in this project |
| --- | --- | --- |
| RCC_AHB1ENR | GPIOBEN | Enable GPIOB before configuring PB12-PB15 |
| RCC_APB1ENR | SPI2EN | Enable SPI2 peripheral clock |
| GPIOB_MODER | PB12 output, PB13-PB15 AF | Select GPIO/alternate-function behavior |
| GPIOB_AFR[1] | AF5 for PB13/PB14/PB15 | Connect pins to SPI2 peripheral |
| GPIOB_BSRR | BS12 / BR12 | Set or reset CS atomically |
| SPI2_CR1 | MSTR, SSM, SSI, CPOL, CPHA, DFF, LSBFIRST, BR, BIDIMODE, RXONLY, SPE | Configure SPI master mode |
| SPI2_SR | TXE, RXNE, BSY, OVR, MODF | Polling and debug flags |
| SPI2_DR | 8-bit data access | Write TX byte and read RX byte |

# 16. Code Explanation

Code source boundary: The following code is based on the reviewed shared-chat implementation. Separate finalized source files were not attached in this Codex thread, so a future pass should replace these snippets with exact file-based code if you provide main.c, spi.c, and spi.h.

## 16.1 Corrected GPIO Initialization

void SPI2_GPIO_Init(void)
{
    RCC->AHB1ENR |= RCC_AHB1ENR_GPIOBEN;

    GPIOB->MODER &= ~((3UL << (12 * 2)) |
                      (3UL << (13 * 2)) |
                      (3UL << (14 * 2)) |
                      (3UL << (15 * 2)));

    GPIOB->MODER |=  ((1UL << (12 * 2)) |   /* PB12 output */
                      (2UL << (13 * 2)) |   /* PB13 AF */
                      (2UL << (14 * 2)) |   /* PB14 AF */
                      (2UL << (15 * 2)));   /* PB15 AF */

    GPIOB->OTYPER &= ~((1UL << 12) | (1UL << 13) |
                       (1UL << 14) | (1UL << 15));

    GPIOB->OSPEEDR &= ~((3UL << (12 * 2)) |
                        (3UL << (13 * 2)) |
                        (3UL << (14 * 2)) |
                        (3UL << (15 * 2)));

    GPIOB->OSPEEDR |=  ((2UL << (12 * 2)) |
                        (2UL << (13 * 2)) |
                        (2UL << (14 * 2)) |
                        (2UL << (15 * 2)));

    GPIOB->PUPDR &= ~((3UL << (12 * 2)) |
                      (3UL << (13 * 2)) |
                      (3UL << (14 * 2)) |
                      (3UL << (15 * 2)));

    GPIOB->PUPDR |=  (1UL << (12 * 2));     /* PB12 pull-up */

    GPIOB->AFR[1] &= ~((0xFUL << ((13 - 8) * 4)) |
                       (0xFUL << ((14 - 8) * 4)) |
                       (0xFUL << ((15 - 8) * 4)));

    GPIOB->AFR[1] |=  ((5UL << ((13 - 8) * 4)) |
                       (5UL << ((14 - 8) * 4)) |
                       (5UL << ((15 - 8) * 4)));

    GPIOB->BSRR = GPIO_BSRR_BS12;
}

## 16.2 Corrected SPI2 Initialization

void SPI2_Init(void)
{
    RCC->APB1ENR |= RCC_APB1ENR_SPI2EN;

    SPI2->CR1 &= ~SPI_CR1_SPE;
    SPI2->CR1 = 0;

    SPI2->CR1 |= SPI_CR1_MSTR;
    SPI2->CR1 |= SPI_CR1_SSM;
    SPI2->CR1 |= SPI_CR1_SSI;

    SPI2->CR1 &= ~SPI_CR1_CPOL;
    SPI2->CR1 &= ~SPI_CR1_CPHA;
    SPI2->CR1 &= ~SPI_CR1_DFF;
    SPI2->CR1 &= ~SPI_CR1_LSBFIRST;

    SPI2->CR1 &= ~SPI_CR1_BR;
    SPI2->CR1 |= SPI_CR1_BR_2;       /* PCLK / 32 */

    SPI2->CR1 &= ~SPI_CR1_BIDIMODE;
    SPI2->CR1 &= ~SPI_CR1_RXONLY;

    SPI2->CR1 |= SPI_CR1_SPE;
}

## 16.3 Transfer Primitive

uint8_t SPI2_TransferByte(uint8_t byte)
{
    uint8_t received;

    while(!(SPI2->SR & SPI_SR_TXE));

    *((volatile uint8_t *)&SPI2->DR) = byte;

    while(!(SPI2->SR & SPI_SR_RXNE));

    received = *((volatile uint8_t *)&SPI2->DR);

    while(SPI2->SR & SPI_SR_BSY);

    return received;
}

| Line / choice | Reason |
| --- | --- |
| 8-bit DR access | Matches DFF=0 and avoids accidental 16-bit data access behavior |
| Return received byte | SPI is full-duplex; every transmitted byte samples MISO |
| Read DR after RXNE | Clears receive data path and prevents overrun in repeated transfers |
| Wait BSY | Ensures final bit has left before CS is released |

## 16.4 Buffer and CS API Layer

#define SPI2_CS_LOW()   (GPIOB->BSRR = GPIO_BSRR_BR12)
#define SPI2_CS_HIGH()  (GPIOB->BSRR = GPIO_BSRR_BS12)

void SPI2_TransmitBuffer(const uint8_t *tx_buf, uint16_t len)
{
    for(uint16_t i = 0; i < len; i++)
    {
        (void)SPI2_TransferByte(tx_buf[i]);
    }
}

void SPI2_ReceiveBuffer(uint8_t *rx_buf, uint16_t len)
{
    for(uint16_t i = 0; i < len; i++)
    {
        rx_buf[i] = SPI2_TransferByte(0xFF);
    }
}

void SPI2_TransferBuffer(const uint8_t *tx_buf, uint8_t *rx_buf, uint16_t len)
{
    for(uint16_t i = 0; i < len; i++)
    {
        uint8_t tx = (tx_buf != 0) ? tx_buf[i] : 0xFF;
        uint8_t rx = SPI2_TransferByte(tx);
        if(rx_buf != 0)
        {
            rx_buf[i] = rx;
        }
    }
}

# 17. Main Algorithm and Flowchart

Main test flow:

SPI2_GPIO_Init()
SPI2_Init()

uint8_t tx_data[] = {0xAA, 0x55, 0x96, 0x3C};

SPI2_CS_LOW();
SPI2_TransmitBuffer(tx_data, sizeof(tx_data));
SPI2_CS_HIGH();

Expected analyzer result:
CS low for one complete transaction
SCK bursts at about 1 MHz
MOSI decodes 0xAA, 0x55, 0x96, 0x3C
MISO may decode 0xFF if no slave is connected

# 18. Testing and Validation

| Validation point | Observed / expected result | Engineering interpretation |
| --- | --- | --- |
| CS / Enable | Low for the full 4-byte group | Transaction framing is correct |
| SCK | Four clean 8-clock bursts | SPI2 master clock generation works |
| MOSI | 0xAA, 0x55, 0x96, 0x3C | Byte order and output path are correct |
| MISO | 0xFF repeated | Normal if no slave drives MISO or the line floats/pulls high |
| Byte duration | About 8 us per decoded byte | Consistent with 8 bits at 1 MHz SCK |
| CS release | After final byte | Good practice; ensure BSY is clear before release |

Screenshot interpretation: The attached Logic 2 screenshot shows D0 as SPI Enable/CS, D1 as SPI Clock, D2 as MISO, and D3 as MOSI. The analyzer decodes MOSI as 0xAA, 0x55, 0x96, 0x3C and MISO as 0xFF for each byte.

Analyzer-level view:



Conclusion: first SPI polling waveform validation passed.

# 19. Results

SPI2 GPIO mapping and alternate function selection are conceptually correct for PB13/PB14/PB15 AF5.

APB1 32 MHz divided by /32 produces a 1 MHz SPI clock, matching analyzer timing.

The MOSI path is verified by decoded test bytes.

The SCK path is verified by clean clock bursts.

The MISO 0xFF result is not a real peripheral response because no receiver/slave was connected.

The polling transfer primitive should return the received byte and always read DR after RXNE.

# 20. Problems Faced and Debugging Notes

| Issue / observation | Root cause | Correction / learning |
| --- | --- | --- |
| MISO shows 0xFF | No slave drives MISO; line may float or be pulled high | Treat as idle/floating, not valid data |
| Only one SPI_DR register visible | Software sees one DR, but hardware has internal TX and RX paths | Write DR for TX, read DR for RX |
| OR-ing GPIO MODER bits | Two-bit fields can retain old state | Clear the field first, then set desired mode |
| GPIOB->OSPEEDR assignment | Whole-port overwrite risk | Mask only PB12-PB15 fields |
| GPIOB->AFR instead of AFR[1] | AFR is split for pins 0..7 and 8..15 | Use GPIOB->AFR[1] for PB13-PB15 |
| Ignoring RXNE in transmit-only | SPI samples MISO during every transmit | Read DR to clear RXNE and prevent OVR |
| CS high too early | Last bit may still be shifting while TXE is already set | Check BSY before deasserting CS |

# 21. Limitations

The test did not include a real SPI slave, so received data was not functionally validated.

No finalized source files were attached in this Codex thread; code snippets are based on shared-chat review.

The polling driver is simple and blocks the CPU during each transfer.

No timeout handling is shown; production polling code should avoid infinite waits.

No OVR/MODF recovery helper is implemented yet.

The document does not cover interrupt or DMA SPI operation.

# 22. Future Improvements

| Next improvement | Why it matters |
| --- | --- |
| Attach final main.c/spi.c/spi.h | Allows exact source-code fidelity audit |
| Add timeout wrappers | Prevents permanent CPU lock if TXE/RXNE/BSY gets stuck |
| Add SPI2_ClearOVR() | Improves recovery after missed DR reads |
| Connect MCP2515 as SPI device only | Validate real command/response before CAN logic |
| Read MCP2515 register | Proves MISO response path using actual slave data |
| Add interrupt/DMA later | Reduces CPU blocking after polling foundation is trusted |

# 23. Conclusion

The polling SPI foundation is validated at the waveform level. The analyzer capture proves that SPI2 can assert CS, generate 1 MHz SCK, and transmit known bytes on MOSI. The important embedded lesson is that the driver must still treat every transmit as a full-duplex exchange: wait TXE, write DR, wait RXNE, read DR, and wait BSY before ending the transaction.

# Appendix A: Source Reference

| Source | Used for |
| --- | --- |
| Shared ChatGPT chat | SPI polling theory, register review, implementation review, analyzer validation |
| Attached screenshot | Logic analyzer evidence: CS/SCK/MISO/MOSI decoded bytes |
| STM32F4 reference manual RM0090 | SPI2 register/flag behavior and GPIO alternate function model |
| doc-m skill | Document structure, source boundary, and verification workflow |

# Appendix B: Verification Checklist

| Check | Status | Evidence |
| --- | --- | --- |
| Shared link accessible | Pass | Saved local HTML copy and decoded conversation payload |
| Polling-only extraction | Pass | 20 selected source chunks from 66 SPI candidates |
| Code boundary documented | Pass | Document states no separate final source files attached |
| SPI2 pin map covered | Pass | PB12/PB13/PB14/PB15 table and implementation notes |
| Register corrections covered | Pass | BR clear, GPIO field clear, AFR[1], 8-bit DR access |
| Analyzer evidence covered | Pass | MOSI bytes, MISO 0xFF, 1 MHz byte timing |

# Appendix C: Revision Questions

| Question | Expected answer |
| --- | --- |
| Why does SPI receive while transmitting? | The same SCK shifts MOSI out and samples MISO in simultaneously. |
| Why read DR after transmit? | RXNE is set for every frame; reading DR clears the received data path and avoids OVR. |
| Why can MISO decode 0xFF with no slave? | A floating or pulled-high MISO line samples as all ones. |
| Why wait for BSY before CS high? | TXE can set before the final bit has fully left the pin. |
| Why use GPIOB->AFR[1]? | PB13-PB15 are pins 8..15, which use AFR[1]. |
| Why explicit 8-bit DR access? | DFF=0 configures 8-bit frames; byte access avoids 16-bit access confusion. |