SPI Protocol Foundation

Basics, Frame Format, Internal Working, Modes, and Block Diagram

Document intent: This document converts the SPI learning data from the shared Embedded Protocol Mastery chat into a structured long-term reference. It is verified and expanded from an embedded-engineering perspective.

| Field | Value |
| --- | --- |
| Protocol | SPI - Serial Peripheral Interface |
| Learning scope | Basics, signal lines, frame timing, CPOL/CPHA, modes, internal hardware, block diagram |
| Source | Shared ChatGPT conversation: Embedded Protocol Mastery |
| Document style | doc-m compact reference guide |
| Code status | No finalized SPI source files attached; code snippets are conceptual reference only |

# 1. Document Control / Metadata

| Item | Details |
| --- | --- |
| Version | v1.0 |
| Prepared for | Embedded protocol learning and long-term revision |
| Primary source | Shared chat link, filtered to SPI protocol learning data |
| Verification basis | STM32F4 SPI register model, official reference material, and embedded driver design practice |
| Boundary | This is a concept and architecture document, not a finalized project-code document |

# 2. Learning Objectives

Explain what SPI is and why it is synchronous.

Understand SCK, MOSI, MISO, and NSS/CS from both signal and hardware viewpoints.

Describe why SPI is full-duplex and why reception happens during every transmission.

Read an SPI waveform and identify frame boundaries, bit order, clock idle level, and sample edge.

Select CPOL/CPHA correctly using a device datasheet instead of memorizing modes blindly.

Explain STM32-style SPI internals: TX buffer, RX buffer, shift register, status flags, and control logic.

Build the mental model needed for polling, interrupt, and DMA SPI drivers later.

# 3. Project Objective

Create an industry-grade foundation reference for SPI that captures the learning progress completed so far: protocol basics, frame format, internal operation, modes, and hardware block diagram. The document is designed to be used before writing bare-metal SPI driver code.

# 4. Problem Statement

Core problem: SPI looks simple because it has only a few wires, but many bugs come from misunderstanding timing, full-duplex exchange, RXNE/OVR behavior, chip-select lifetime, and the difference between TXE and actual transfer completion.

# 5. System Overview

SPI is a synchronous serial bus normally used for short-distance communication between one controller and one or more peripherals. It is common in sensors, flash memories, ADCs, DACs, displays, external CAN controllers, and many board-level devices.

| Concept | SPI meaning | Engineering implication |
| --- | --- | --- |
| Synchronous | A clock line controls bit movement | The controller decides transfer speed and exact bit timing |
| Controller/peripheral | One device initiates clocks and selects targets | Peripherals do not transmit unless clocked and selected |
| Full-duplex | One bit shifts out while one bit shifts in | Every transmitted byte produces a received byte, even if it is dummy |
| Frame-oriented | A frame is commonly 8 or 16 bits on STM32F4 | No start/stop bits; CS/NSS and device protocol define transaction boundaries |
| Short distance | Usually board-level or short cable | Signal integrity, ground reference, and timing matter at higher SCK speeds |

# 6. Requirements

| Requirement | Why it matters |
| --- | --- |
| Understand binary bit order | SPI devices commonly expect MSB-first, though LSB-first exists in some peripherals |
| Read a timing diagram | Datasheets specify CPOL, CPHA, setup time, hold time, and CS timing |
| Know memory-mapped registers | Bare-metal SPI uses CR1, CR2, SR, and DR register access |
| Use a logic analyzer | SCK, MOSI, MISO, and CS must be inspected to debug real failures |
| Read device datasheets | SPI mode, maximum SCK, command format, and dummy cycles are device-specific |

# 7. Hardware Components and Signal Lines

| Line | Direction in controller mode | Function | Debug expectation |
| --- | --- | --- | --- |
| SCK | Controller -> peripheral | Serial clock generated by controller | Clock toggles only during active transfers |
| MOSI | Controller -> peripheral | Controller output, peripheral input | Carries command/address/data from controller |
| MISO | Peripheral -> controller | Controller input, peripheral output | Carries response data; may be idle or high-Z when CS inactive |
| NSS/CS | Controller -> peripheral | Selects one peripheral, usually active low | Goes low before clocks, high after transaction completes |

# 8. Software and Firmware Tools

| Tool | Use in SPI learning |
| --- | --- |
| STM32CubeIDE or similar | Project setup, build, flash, and register-level debugging |
| STM32F4 reference manual | Verify CR1, CR2, SR, DR, flags, and block diagram behavior |
| Device datasheet | Find SPI mode, command frame, maximum SCK, CS timing, and dummy cycles |
| Saleae or logic analyzer | Observe CS, SCK, MOSI, MISO, sample edge, and bit order |
| Oscilloscope | Check signal integrity, ringing, rise/fall time, and voltage levels |

# 9. System Block Diagram

This diagram is the mental model behind the common polling sequence: wait for TXE, write DR, wait for RXNE, read DR, then wait for BSY to clear before releasing chip select.

# 10. Firmware Architecture

| Layer | Responsibility | Typical functions |
| --- | --- | --- |
| GPIO layer | Configure SCK/MOSI/MISO alternate functions and CS output | SPI_GPIO_Init(), CS_Low(), CS_High() |
| SPI peripheral layer | Configure master mode, baud prescaler, frame format, CPOL/CPHA, SSM/SSI | SPI_Init(), SPI_Enable() |
| Transfer layer | Move one frame or a buffer through DR while handling RXNE/TXE/BSY | SPI_TransferByte(), SPI_TransferBuffer() |
| Device protocol layer | Build command/address/dummy/data sequences | Flash_ReadID(), ADC_ReadChannel() |
| Application layer | Use parsed sensor/storage data | Read temperature, log data, fetch device ID |

# 11. Communication Protocol Used

SPI itself is only a transport mechanism. It defines clocked bit exchange and chip selection, but it does not define universal addresses, start bits, stop bits, acknowledgments, checksums, or packet boundaries. Those are defined by each peripheral device.

| SPI provides | Device protocol provides |
| --- | --- |
| Clocking and bit exchange | Meaning of command bytes |
| MOSI/MISO data movement | Register address format |
| NSS/CS selection | Read/write bit convention |
| Frame length and bit order | Dummy cycles and response length |
| Status flags in MCU | Device-specific status register bits |

# 12. Module-Wise Implementation Plan

| Module | Configuration / behavior | Verification point |
| --- | --- | --- |
| Clock setup | Enable GPIO and SPI peripheral clocks | Peripheral registers become writable |
| GPIO setup | SCK/MOSI/MISO in alternate function; CS as output if software controlled | Pins show expected AF behavior |
| SPI_CR1 | MSTR, BR, CPOL, CPHA, DFF, LSBFIRST, SSM, SSI, SPE | Mode and frame timing match datasheet |
| SPI_CR2 | Interrupt/DMA enables later; SSOE if hardware NSS is used | Polling driver may leave most CR2 bits off |
| Transfer function | Write DR, read DR for every frame | No RXNE backlog or OVR |
| CS wrapper | Assert CS around the complete command/data transaction | CS remains low until BSY is clear |

# 13. Core Concept Deep Dive: SPI Basics

| Question | Professional answer |
| --- | --- |
| What is SPI? | A synchronous serial communication protocol used for short-distance controller-to-peripheral communication. |
| Why synchronous? | A shared SCK signal defines when bits shift and when they are sampled. |
| Who generates SCK? | The controller/master generates SCK. The selected peripheral follows it. |
| Why controller/peripheral? | One device coordinates timing and selection; peripherals respond when selected and clocked. |
| Why full-duplex? | The same clock shifts one bit out and one bit in at the same time. |
| Why CS/NSS? | It selects the intended peripheral and frames the transaction. |

Important correction from the learning chat: MOSI means Master Out, Slave In. MISO means Master In, Slave Out. The controller receives on MISO, not MOSI.

# 14. Frame Format and Signal Timing

Unlike UART, SPI does not use start and stop bits. A frame is a configured number of bits shifted under SCK while the peripheral is selected. On STM32F4, the DFF bit selects 8-bit or 16-bit frame format.

| Frame element | SPI behavior | Common bug |
| --- | --- | --- |
| CS low | Peripheral becomes selected and listens to clocks | CS released too early before BSY clears |
| SCK edges | Each edge pair advances bit exchange | Wrong CPOL/CPHA samples unstable data |
| MOSI bit | Controller sends command/address/data/dummy | Wrong bit order or wrong command sequence |
| MISO bit | Peripheral returns data at the same time | Ignoring RXNE causes overrun later |
| Frame length | Usually 8 or 16 bits in STM32F4 | Device expects multiple bytes but driver treats one byte as complete transaction |

Example conceptual 8-bit transaction:

CS    ----\________________________/----
SCK   ____/\__/\__/\__/\__/\__/\__/\__/\____
MOSI      b7  b6  b5  b4  b3  b2  b1  b0
MISO      r7  r6  r5  r4  r3  r2  r1  r0

One transmitted bit and one received bit exist for each clocked bit time.

# 15. CPOL, CPHA, and SPI Modes

Memory rule: CPOL decides the idle level of SCK. CPHA decides whether sampling happens on the first or second clock transition after CS becomes active.

| Mode | CPOL | CPHA | SCK idle | Sample edge | Change edge |
| --- | --- | --- | --- | --- | --- |
| Mode 0 | 0 | 0 | Low | Rising / leading | Falling / trailing |
| Mode 1 | 0 | 1 | Low | Falling / trailing | Rising / leading |
| Mode 2 | 1 | 0 | High | Falling / leading | Rising / trailing |
| Mode 3 | 1 | 1 | High | Rising / trailing | Falling / leading |

The reliable engineering rule is not to memorize the mode number first. Read the device datasheet, identify clock idle level and sampling edge, then translate that timing into CPOL and CPHA.

| Term | Meaning | How to see it on a logic analyzer |
| --- | --- | --- |
| CPOL | Clock polarity / idle level | Look at SCK level when CS is inactive |
| CPHA | Clock phase / sample timing | Determine whether data is sampled on the first or second active edge |
| Leading edge | First edge after CS activates | Rising if CPOL=0, falling if CPOL=1 |
| Trailing edge | Second edge in the clock cycle | Falling if CPOL=0, rising if CPOL=1 |

# 16. Practical SPI Modes and Device Examples

| Device type | Why useful for learning | Mode / timing caution |
| --- | --- | --- |
| MCP3008 / MCP3208 ADC | Simple command-response ADC flow | Good first SPI ADC; check datasheet timing and sample command format |
| W25Q64 SPI flash | Commands, addresses, dummy cycles, status register | Commonly supports mode 0 and mode 3 variants depending on datasheet |
| MCP2515 CAN controller | Real register-based external controller | SPI mode and CS timing must match datasheet |
| ADXL345 accelerometer | Sensor register reads/writes | Wrong mode often shows bad device ID or shifted values |
| MAX31855 thermocouple converter | Read-only SPI-style transfer | Clocking and sample edge are the main concerns |
| AD7705 sigma-delta ADC | Advanced ADC with calibration/register complexity | Useful later; not the easiest first SPI device |

# 17. Internal Working: What Happens During One Transfer

CPU checks TXE to confirm the transmit buffer can accept data.

CPU writes a frame into SPIx->DR.

SPI hardware moves the value into its transmit path and starts clocking if enabled and selected.

The shift register drives MOSI while simultaneously sampling MISO.

After the received frame is ready, RXNE is set.

CPU reads SPIx->DR to obtain the received frame and clear RXNE.

CPU waits for BSY to clear before ending the transaction or deasserting CS.

Conceptual polling transfer byte, not finalized project code:

uint8_t SPI_TransferByte(uint8_t tx)
{
    while(!(SPIx->SR & SPI_SR_TXE));
    SPIx->DR = tx;

    while(!(SPIx->SR & SPI_SR_RXNE));
    uint8_t rx = SPIx->DR;

    while(SPIx->SR & SPI_SR_BSY);
    return rx;
}

Driver-design caution: For multi-byte transfers, do not wait for BSY after every byte unless the design intentionally separates frames. Usually CS stays low across the whole transaction, and BSY is checked before releasing CS at the end.

# 18. SPI Block Diagram Explanation

| Block | Meaning | Firmware-visible behavior |
| --- | --- | --- |
| Address/data bus | CPU/APB connection to peripheral registers | Memory-mapped writes like SPIx->DR = value |
| TX buffer | Temporary holding point for data written by CPU/DMA | TXE set means this buffer can accept another frame |
| Shift register | Serializes TX bits and captures RX bits | Actual MOSI/MISO exchange happens here |
| RX buffer | Holds received frame after shifting completes | RXNE set means DR must be read |
| Clock generator | Divides peripheral clock to generate SCK | Controlled by BR bits and active only during transfer in master mode |
| NSS management | Controls or monitors slave select behavior | Software CS uses GPIO; hardware NSS uses SPI control bits |
| Communication control | State machine for SPE, MSTR, CPOL/CPHA, flags, interrupts, DMA | Updates TXE, RXNE, BSY, MODF, OVR |

# 19. Register and Flag Reference

| Register | Important bits | Why you care |
| --- | --- | --- |
| SPI_CR1 | SPE, MSTR, BR[2:0], CPOL, CPHA, DFF, LSBFIRST, SSM, SSI, BIDIMODE, CRCEN | Main SPI mode and transfer configuration |
| SPI_CR2 | TXEIE, RXNEIE, ERRIE, TXDMAEN, RXDMAEN, SSOE | Interrupt, DMA, and hardware NSS behavior |
| SPI_SR | TXE, RXNE, BSY, OVR, MODF, CRCERR | Runtime transfer and error status |
| SPI_DR | Transmit write path and receive read path | Writing starts exchange; reading consumes received data |

| Flag | Meaning | Common misconception | Correct handling |
| --- | --- | --- | --- |
| TXE | Transmit buffer empty | Transfer is complete | Only means DR/TX buffer can accept data |
| RXNE | Receive buffer not empty | Only matters for read commands | Read DR after every frame, even dummy responses |
| BSY | SPI is busy shifting or finishing transfer | Can ignore after TXE | Wait until clear before raising CS |
| OVR | New received data arrived before old data was read | Only happens in receive mode | Read DR then SR on STM32F4 to clear |
| MODF | Mode fault due NSS/master conflict | Random peripheral failure | Configure NSS/SSM/SSI correctly in master mode |

# 20. Main Algorithm and Flowcharts

Single-byte exchange:

Start
  |
  +-- Assert CS low
  |
  +-- Wait TXE = 1
  +-- Write transmit byte to DR
  +-- Wait RXNE = 1
  +-- Read received byte from DR
  +-- Wait BSY = 0
  |
  +-- Deassert CS high
  |
End

Typical register-read transaction:

CS low
  send command/address byte
  send dummy byte(s) to generate clocks
  read returned data byte(s)
wait BSY clear
CS high

# 21. Code Explanation

No finalized SPI code attached: Because this request did not include SPI project source files, the snippets below are intentionally conceptual. When you attach finalized SPI code later, the doc-m rule will switch to exact source-code extraction.

| Code idea | What it teaches | What to verify in real code |
| --- | --- | --- |
| SPI_Init() | Peripheral clock, GPIO AF, CR1 mode setup, SPE enable | CPOL/CPHA and BR match the peripheral datasheet |
| CS_Low()/CS_High() | Software chip-select framing | CS high only after BSY is clear |
| SPI_TransferByte() | Full-duplex byte exchange | Every write has a matching DR read |
| SPI_ReadRegister() | Command/address + dummy clocks + response | Dummy byte count and read length match datasheet |
| SPI_ClearOVR() | Recover from receive overrun | Clear sequence matches MCU reference manual |

# 22. Testing and Validation

| Test | Expected observation | Failure points caught |
| --- | --- | --- |
| Idle capture | SCK idle level matches selected CPOL | Wrong CPOL or wrong mode table |
| Single byte 0xA5 | MOSI bit pattern visible in selected bit order | LSBFIRST/MSB-first mismatch |
| Loopback MOSI->MISO | Received byte equals transmitted byte | GPIO AF, transfer sequence, RXNE handling |
| Device ID read | Known ID returned from peripheral | Wrong mode, CS timing, dummy cycles, wiring |
| Overrun stress | No OVR when reading DR for every frame | Forgotten RX read or too-slow ISR |
| CS timing | CS remains low for complete command/data transaction | Early deassertion before BSY clear |

# 23. Results: What You Should Be Able to Explain

SPI is synchronous because SCK defines bit timing.

The controller generates SCK and selects the peripheral using CS/NSS.

MOSI and MISO transfer simultaneously, so SPI is naturally full-duplex.

TXE is not transfer-complete; it only means the transmit buffer can accept more data.

RXNE matters even during transmit-only style operations because every clock captures input.

BSY must be considered before ending a transaction or raising CS.

CPOL and CPHA should be selected from the peripheral datasheet, then verified on a logic analyzer.

The internal block diagram maps directly to firmware code: DR, SR flags, shift register, buffers, and control logic.

# 24. Problems Faced and Debugging Notes

| Symptom | Likely cause | Debug action |
| --- | --- | --- |
| Always reads 0x00 or 0xFF | MISO floating, wrong CS, device not powered, wrong mode | Probe MISO and CS; verify pull state and datasheet mode |
| Device ID shifted/wrong | CPHA mismatch or bit-order mismatch | Check sample edge and LSBFIRST |
| First byte garbage | Device returns dummy/status during command phase | Understand command-response timing; send dummy clocks |
| OVR flag sets | RXNE data not read before next frame | Read DR after every transfer; clear OVR by DR then SR on STM32F4 |
| CS rises but last bit missing | Released CS before BSY cleared | Wait for BSY=0 before CS high |
| Mode fault | NSS handling incorrect in master mode | For software CS, configure SSM=1 and SSI=1 where appropriate |
| Works slow, fails fast | Signal integrity or device max SCK exceeded | Lower BR prescaler; inspect ringing and setup/hold margins |

# 25. Limitations

SPI has no built-in acknowledgment like I2C; success is usually checked through device-specific response/status bytes.

There is no universal SPI packet format. Each peripheral defines its own command framing.

Multiple peripherals require careful CS management; only one MISO driver should be active at a time.

Higher speeds require board-level signal-integrity attention.

This document does not yet cover interrupt-driven SPI, DMA SPI, RTOS integration, or a complete device driver.

# 26. Future Improvements

| Next topic | Recommended learning goal |
| --- | --- |
| Polling SPI driver | Implement init, CS control, TransferByte, and TransferBuffer |
| MCP3008/MCP3208 lab | Read ADC channels and validate command frame on logic analyzer |
| W25Q64 flash lab | Read JEDEC ID, status register, and memory bytes with dummy cycles |
| Interrupt SPI | Understand TXEIE/RXNEIE and ISR latency constraints |
| DMA SPI | Transfer buffers efficiently and learn RX/TX DMA pairing |
| Production driver design | Timeouts, lock state, error recovery, mode switching, and multi-device bus ownership |

# 27. Conclusion

The important milestone is that SPI should now feel like hardware, not just a protocol name. A write to DR creates clocks, clocks shift MOSI and MISO simultaneously, RXNE must be serviced, TXE is only buffer availability, and CS must frame the complete device-level transaction. This foundation is enough to start a clean polling SPI driver and then grow toward interrupt and DMA implementations.

# Appendix A: Source Traceability

| Learning source | How it was used |
| --- | --- |
| Embedded Protocol Mastery shared chat | Extracted SPI plan, basics, signal/frame explanations, CPOL/CPHA, mode table, internal hardware, and block diagram discussion |
| doc-m skill | Applied professional embedded-document workflow, verification checklist, and long-term learning structure |
| No SPI code attached | Document avoids claiming exact project code; snippets are marked conceptual |

# Appendix B: Verification Sources

| Source | Verification use | Link |
| --- | --- | --- |
| STMicroelectronics RM0090 | STM32F4 SPI registers, flags, block behavior | https://www.st.com/resource/en/reference_manual/dm00031020.pdf |
| Microchip MCP3008 | Example simple SPI ADC device | https://www.microchip.com/en-us/product/MCP3008 |
| Microchip MCP2515 | Example external CAN controller over SPI | https://www.microchip.com/en-us/product/MCP2515 |
| Analog Devices ADXL345 | Example SPI/I2C accelerometer | https://www.analog.com/en/products/adxl345.html |
| Analog Devices MAX31855 | Example SPI-compatible read-only sensor converter | https://www.analog.com/en/products/max31855.html |
| Winbond W25Q64 family | Example SPI flash memory family | https://www.winbond.com/ |

# Appendix C: Revision Checklist

| Check | Question |
| --- | --- |
| [ ] | Can I explain why SPI receives while transmitting? |
| [ ] | Can I identify CPOL and CPHA from a waveform? |
| [ ] | Can I explain TXE, RXNE, BSY, and OVR without memorizing definitions? |
| [ ] | Can I sketch the SPI internal block diagram from memory? |
| [ ] | Can I write a safe polling transfer sequence with CS low/high boundaries? |
| [ ] | Can I debug wrong device ID reads using mode, CS, dummy clocks, and wiring checks? |