EMBEDDED STORAGE REFERENCE

W25Q64FV SPI Flash Driver Implementation

STM32F407 SPI2 transaction design, source walkthrough, engineering audit, and production-driver evolution

Document intent: A long-term implementation reference that maps the shared W25Q64FV driver discussion to the finalized firmware currently attached in this chat. Literal C code remains source-faithful; corrections and future architecture are separated and labeled.

| Field | Implementation snapshot |
| --- | --- |
| Target | W25Q64FV-class 3 V SPI NOR flash |
| Host | STM32F407 using bare-metal SPI2 polling |
| Flash CS | PB9, software-controlled and active low |
| Shared SPI2 slave | MCP2515 on PB12 chip select |
| SPI signals | PB13 SCK, PB14 MISO, PB15 MOSI, AF5 |
| Test address | 0x001000, sector-aligned and page-aligned |
| Test payload | Five bytes: H E L L O |
| Design preset | compact_reference_guide |

# 1. Document Control and Scope

| Control item | Value |
| --- | --- |
| Theory source | https://chatgpt.com/share/6a41dcea-c2a8-83ee-996c-a4cf121eec95 |
| Code source | D:\Embedded Communication Protocols\CAN1_RX_Interrupt |
| Primary technical reference | Winbond W25Q64FV Serial Flash Memory Datasheet |
| Code authority | The seven attached .c/.h files listed in Appendix A |
| Driver scope | Polling SPI transport, reset, identification, status, read, write enable/disable, sector erase, and page program |
| Excluded implementation | No invented interrupt, DMA, filesystem, wear-leveling, or RTOS code |
| Screenshot status | No new screenshot file was attached with this request; analyzer expectations are documented instead |
| Document footer | Embedded Learning Document |

# 2. Learning Objectives

Trace every public W25Q function into its exact SPI command, address, data, and chip-select phases.

Understand why Write Enable, WEL, BUSY, and chip-select termination form one correctness contract.

Distinguish byte-addressable read behavior from page-limited program and sector-based erase behavior.

Follow the finalized main.c smoke test from initialization through erase, program, and read-back.

Recognize the difference between the current learning driver and a production-grade driver architecture.

Debug failures using UART, debugger watch variables, status bits, and logic-analyzer transactions.

# 3. Project Objective

Bring up external serial NOR storage on STM32F407 SPI2 and prove the essential command path. The current project resets the flash, reads its identity and status, erases the sector containing 0x001000, programs HELLO, and reads the same five bytes back.

# 4. Engineering Problem Statement

Core problem: SPI only transports bits. The driver must create valid flash instructions, hold chip select across each complete instruction, respect erase/program state rules, and report failures instead of hanging or silently continuing.

# 5. System Overview

The W25Q device and MCP2515 share SPI2. SCK, MOSI, and MISO are common; each slave owns a separate active-low chip select. The target driver must guarantee that PB12 remains high while PB9 is low so that only the flash can respond on MISO.

## 5.1 System-Level Hardware Diagram

STM32F407
  |
  +-- SPI2 clock domain (APB1)
  |
  +-- PB13 / SPI2_SCK  ------------------------+--> W25Q64FV CLK
  |                                             +--> MCP2515 SCK
  +-- PB15 / SPI2_MOSI -------------------------+--> W25Q64FV DI
  |                                             +--> MCP2515 SI
  +-- PB14 / SPI2_MISO <------------------------+--- W25Q64FV DO
  |                                             +--- MCP2515 SO
  +-- PB9  / GPIO output --------------------------> W25Q64FV /CS
  +-- PB12 / GPIO output --------------------------> MCP2515 /CS

Bus invariant: PB9 and PB12 must never be low at the same time.

## 5.2 Driver Layer Diagram

Application / smoke test
        main.c
          |
          v
W25Q command driver
        w25q64fv.c / w25q64fv.h
          |  reset, ID, status, read, erase, program
          v
SPI transport and chip-select control
        spi.c / spi.h
          |  TXE -> DR write -> RXNE -> DR read -> BSY clear
          v
STM32F407 SPI2 + GPIOB hardware
          |
          v
W25Q64FV command decoder, status logic, page buffer, memory array

## 5.3 Internal Flash Functional Diagram

/CS, CLK, DI, DO
      |
      v
SPI command decoder -----> Status Register 1: BUSY, WEL
      |                              |
      |                              +--> command acceptance / completion
      v
24-bit address decoder
      |
      +--> read path ----------------------> serial DO
      |
      +--> page program buffer (256 B) ----> memory array
      |
      +--> erase controller (4 KB sector) -> memory array

Memory geometry:
8 MB chip -> 128 x 64 KB blocks -> 2048 x 4 KB sectors -> 32768 x 256 B pages

# 6. Requirements and Live Configuration

| Requirement | Final-source value | Verification |
| --- | --- | --- |
| SPI peripheral | SPI2 | RCC_APB1ENR_SPI2EN |
| SPI mode | Mode 0: CPOL=0, CPHA=0 | SPI2->CR1 clears CPOL and CPHA |
| Frame | 8-bit, MSB first | DFF and LSBFIRST cleared |
| Clock divider | PCLK / 32 | BR_2 set; comment gives 1 MHz at 32 MHz PCLK1 |
| Software NSS | SSM=1, SSI=1 | SPI2 master remains internally deselected from fault |
| Flash CS | PB9 GPIO output | W25Q_FLASH_CS_LOW/HIGH macros |
| Shared-slave CS | PB12 GPIO output | MCP2515_CS_HIGH before selected flash paths |
| Address width | 24 bits | A23-A16, A15-A8, A7-A0 transmitted in that order |
| Polling flags | TXE, RXNE, BSY | SPI2_TransferByte |

# 7. Hardware and Tooling

| Item | Role | Engineering note |
| --- | --- | --- |
| W25Q64FV-class module | External non-volatile storage | Confirm exact part marking and 2.7-3.6 V FV supply range |
| STM32F407 | SPI master and application host | 3.3 V logic aligns with FV, not 1.8 V FW |
| Logic analyzer | Decode /CS, CLK, DI, DO | Use SPI Mode 0 and inspect instruction boundaries |
| Debugger | Inspect ID, status, TX and RX buffers | Required because current test does not print flash results |
| USART2 terminal | Future PASS/FAIL output | USART2 is initialized but flash result reporting is absent |
| Winbond datasheet | Opcode, timing, voltage, status semantics | Treat exact part revision as primary authority |

# 8. SPI Instruction Framing

General instruction

/CS  ----\________________________________________/----
CLK       clock clocks clock clocks clock clocks
MOSI      OPCODE | ADDRESS[23:16] | ADDRESS[15:8] | ADDRESS[7:0] | DATA...
MISO      ignored | ignored        | ignored       | ignored      | RESPONSE...

The rising /CS edge terminates the instruction. For erase and page program,
that edge is part of the command contract, not cosmetic GPIO cleanup.

## 8.1 Command Matrix

| Driver operation | Opcode | Address | Data phase | Post-condition |
| --- | --- | --- | --- | --- |
| Enable Reset | 0x66 | None | None | Arms reset command |
| Reset Device | 0x99 | None | None | Device resets; delay follows |
| Read JEDEC ID | 0x9F | None | Read 3 bytes | Identity bytes captured |
| Read Status-1 | 0x05 | None | Read SR1 | BUSY/WEL observed |
| Read Status-2 | 0x35 | None | Read SR2 | Configuration/protection observed |
| Write Enable | 0x06 | None | None | WEL should become 1 |
| Write Disable | 0x04 | None | None | WEL clears |
| Read Data | 0x03 | 24-bit | Read N bytes | Address auto-increments |
| Sector Erase | 0x20 | 24-bit | None | BUSY starts; target 4 KB sector erases |
| Page Program | 0x02 | 24-bit | Write 1-256 bytes | BUSY starts; WEL later clears |

# 9. Flash Rules That Drive the API

The chip is byte-addressable for reads, but erase and program use different granularities.

An erased byte is 0xFF; programming changes selected bits from 1 to 0.

A 0-to-1 transition requires erase of the containing erase unit.

Page Program must remain inside one 256-byte page; a higher layer must split cross-page writes.

Sector Erase affects the entire 4 KB sector containing the supplied address.

Write Enable is required before each erase or program because WEL clears after a modifying operation.

BUSY polling is mandatory after erase/program, but production code must use a bounded timeout.

Critical writes must be read back and compared; transport success alone is not data integrity proof.

Power loss during erase/program can leave data invalid, so atomic records or redundancy are application concerns.

Erase endurance is finite; storage design should avoid repeatedly erasing one sector in a tight loop.

## 9.1 Address and Geometry Reference

| Unit | Size | Mask / step | Example at 0x001234 |
| --- | --- | --- | --- |
| Byte | 1 byte | +1 | Exact byte 0x001234 |
| Page | 0x100 = 256 B | low 8 bits are offset | base 0x001200, offset 0x34 |
| Sector | 0x1000 = 4 KB | low 12 bits are offset | base 0x001000, offset 0x234 |
| 64 KB block | 0x10000 | low 16 bits are offset | base 0x000000, offset 0x1234 |
| Chip | 0x800000 = 8 MB | 0x000000-0x7FFFFF | address is in range |

## 9.2 Status State Model

READY, WEL=0
   |
   | Write Enable (0x06)
   v
READY, WEL=1
   |
   | Page Program or Erase instruction ends with /CS high
   v
BUSY=1, internal operation active
   |
   | Read Status-1 until BUSY=0, with timeout
   v
READY, WEL=0

Failure branches:
- WEL never sets: reject operation and report error.
- BUSY never clears before timeout: stop and report timeout.
- Read-back differs: report verify failure.

# 10. Firmware Architecture and API Contracts

| API | Inputs / output | Transaction | Current contract gap |
| --- | --- | --- | --- |
| W25Q_FlashReset | void | 0x66, /CS cycle, then 0x99, /CS cycle | Fixed 10 ms delay; no status return |
| W25Q_FlashReadJEDEC | id[3] output | 0x9F then 3 dummy transfers | No ID validation or status return |
| W25Q_FlashRead_StatusRegister | status[2] output | 0x05 read, then 0x35 read | No null check |
| W25Q_FlashRead_Data | addr, buffer, size | 0x03 + 24-bit address + N dummy transfers | No address-range check |
| W25Q_FlashReadStatusReg1 | returns SR1 | 0x05 + dummy transfer | Cannot report transport failure |
| W25Q_FlashWaitUntilReady | void | Polls SR1 BUSY | No timeout; can block forever |
| W25Q_FlashWriteEnable | void | 0x06 | Does not verify WEL |
| W25Q_FlashWriteDisable | void | 0x04 | No status return |
| W25Q_FlashSectorErase | addr | 0x06, then 0x20 + address | Critical /CS sequencing defect |
| W25Q_FlashPageProgram | addr, data, size | 0x06, then 0x02 + address + data | Caps size but does not reject page crossing |

# 11. Initialization Sequence

HAL_Init()
SystemClock_Config()
USART2_Init()
SPI2_GPIO_Init()
SPI2_Init()
...
W25Q_FlashReset()
W25Q_FlashReadJEDEC(jedec_id)
W25Q_FlashRead_StatusRegister(w25q_status)

The flash path is initialized after SPI2 GPIO and peripheral setup.
The application also initializes other CAN/display modules, but they are
project context rather than part of the W25Q driver algorithm.

# 12. Exact Source Walkthrough

Source fidelity: Every C snippet in this section is extracted from the attached finalized files. The document explains defects after the snippet so the code record remains exact.

## 12.1 Geometry, Commands, and Public API

#ifndef INC_W25Q64FV_H_
#define INC_W25Q64FV_H_

#include "stm32f407xx.h"
#include <stdint.h>

/* Memory geometry */
#define W25Q_PAGE_SIZE       0x100UL     /* 256 bytes */
#define W25Q_SECTOR_SIZE     0x1000UL    /* 4 KB */
#define W25Q_BLOCK64_SIZE    0x10000UL   /* 64 KB */
#define W25Q_CHIP_SIZE       0x800000UL  /* 8 MB */

#define PAGE_OFFSET(addr)    ((addr) & 0xFFUL)
#define PAGE_BASE(addr)      ((addr) & ~0xFFUL)

#define SECTOR_OFFSET(addr)  ((addr) & 0xFFFUL)
#define SECTOR_BASE(addr)    ((addr) & ~0xFFFUL)

#define BLOCK64_OFFSET(addr) ((addr) & 0xFFFFUL)
#define BLOCK64_BASE(addr)   ((addr) & ~0xFFFFUL)

/* W25Q64FV command set */
#define W25Q_CMD_READ_STATUS_REG1   0x05
#define W25Q_CMD_READ_STATUS_REG2   0x35
#define W25Q_CMD_READ_DATA          0x03
#define W25Q_CMD_READ_JEDEC_ID      0x9F
#define W25Q_CMD_ENABLE_RESET       0x66
#define W25Q_CMD_RESET_DEVICE       0x99
#define W25Q_CMD_WRITE_ENABLE       0x06
#define W25Q_CMD_WRITE_DISABLE      0x04
#define W25Q_CMD_SECTOR_ERASE_4KB   0x20
#define W25Q_CMD_PAGE_PROGRAM   	0x02

#define W25Q_DUMMY_BYTE             0xFF

#define W25Q_SR1_BUSY    0x01
#define W25Q_SR1_WEL     0x02

void W25Q_FlashReset(void);
void W25Q_FlashReadJEDEC(uint8_t *id);
void W25Q_FlashRead_StatusRegister(uint8_t *status);
void W25Q_FlashRead_Data(uint32_t addr, uint8_t *buffer, uint32_t size);
uint8_t W25Q_FlashReadStatusReg1(void);
void W25Q_FlashWaitUntilReady(void);
void W25Q_FlashWriteEnable(void);
void W25Q_FlashWriteDisable(void);
void W25Q_FlashSectorErase(uint32_t addr);
void W25Q_FlashPageProgram(uint32_t addr, uint8_t *data, uint32_t size);

#endif /* INC_W25Q64FV_H_ */

## 12.2 SPI2 GPIO Configuration

void SPI2_GPIO_Init(void)
{
    /* Enable GPIOB clock */
    RCC->AHB1ENR |= RCC_AHB1ENR_GPIOBEN;

    /*
     * PB9	= CS_W25Q_FLASH
     * PB12 = CS_MCP2515_NODE1
     * PB13 = SPI2_SCK AF
     * PB14 = SPI2_MISO AF
     * PB15 = SPI2_MOSI AF
     */

    /* Clear mode bits */
    GPIOB->MODER &= ~((3UL << (	9 * 2)) |
    				  (3UL << (12 * 2)) |
                      (3UL << (13 * 2)) |
                      (3UL << (14 * 2)) |
                      (3UL << (15 * 2)));

    /* Set modes */
    GPIOB->MODER |=  ((1UL << ( 9 * 2)) |   /* PB9 output */
    				  (1UL << (12 * 2)) |   /* PB12 output */
                      (2UL << (13 * 2)) |   /* PB13 AF */
                      (2UL << (14 * 2)) |   /* PB14 AF */
                      (2UL << (15 * 2)));   /* PB15 AF */

    /* Push-pull */
    GPIOB->OTYPER &= ~((1UL <<  9) |
    				   (1UL << 12) |
                       (1UL << 13) |
                       (1UL << 14) |
                       (1UL << 15));

    /* Speed: medium/high for SPI */
    GPIOB->OSPEEDR &= ~((3UL << ( 9 * 2)) |
    					(3UL << (12 * 2)) |
                        (3UL << (13 * 2)) |
                        (3UL << (14 * 2)) |
                        (3UL << (15 * 2)));

    GPIOB->OSPEEDR |=  ((2UL << ( 9 * 2)) |
    					(2UL << (12 * 2)) |
                        (2UL << (13 * 2)) |
                        (2UL << (14 * 2)) |
                        (2UL << (15 * 2)));

    /* Pull config: CS pull-up, others no pull */
    GPIOB->PUPDR &= ~((3UL << ( 9 * 2)) |
    				  (3UL << (12 * 2)) |
                      (3UL << (13 * 2)) |
                      (3UL << (14 * 2)) |
                      (3UL << (15 * 2)));

    GPIOB->PUPDR |=  (1UL << (12 * 2));     /* PB12 pull-up */
    GPIOB->PUPDR |=  (1UL << ( 9 * 2));     /* PB9 pull-up */

    /* AF5 for PB13, PB14, PB15 */
    GPIOB->AFR[1] &= ~((0xFUL << ((13 - 8) * 4)) |
                       (0xFUL << ((14 - 8) * 4)) |
                       (0xFUL << ((15 - 8) * 4)));

    GPIOB->AFR[1] |=  ((5UL << ((13 - 8) * 4)) |
                       (5UL << ((14 - 8) * 4)) |
                       (5UL << ((15 - 8) * 4)));

    /* Keep CS high initially */
    MCP2515_CS_HIGH();
    W25Q_FLASH_CS_HIGH();

}

PB9 and PB12 become push-pull outputs with pull-ups; PB13-PB15 become AF5 SPI pins. Both chip selects are driven high before communication begins.

## 12.3 SPI2 Peripheral Configuration

void SPI2_Init(void)
{
    /* Enable SPI2 clock */
    RCC->APB1ENR |= RCC_APB1ENR_SPI2EN;

    /* Disable SPI before configuration */
    SPI2->CR1 &= ~SPI_CR1_SPE;

    /* Clear CR1 configuration bits we care about */
    SPI2->CR1 = 0;

    /*
     * Master mode
     * Software slave management
     * Internal slave select high
     */
    SPI2->CR1 |= SPI_CR1_MSTR;
    SPI2->CR1 |= SPI_CR1_SSM;
    SPI2->CR1 |= SPI_CR1_SSI;

    /*
     * Mode 0: CPOL = 0, CPHA = 0
     */
    SPI2->CR1 &= ~SPI_CR1_CPOL;
    SPI2->CR1 &= ~SPI_CR1_CPHA;

    /*
     * 8-bit frame, MSB first
     */
    SPI2->CR1 &= ~SPI_CR1_DFF;
    SPI2->CR1 &= ~SPI_CR1_LSBFIRST;

    /*
     * Baud rate = PCLK / 32
     * If PCLK1 = 32 MHz, SCK = 1 MHz
     */
    SPI2->CR1 &= ~SPI_CR1_BR;
    SPI2->CR1 |= SPI_CR1_BR_2;

    /*
     * Full duplex 2-line
     */
    SPI2->CR1 &= ~SPI_CR1_BIDIMODE;
    SPI2->CR1 &= ~SPI_CR1_RXONLY;

    /* Enable SPI */
    SPI2->CR1 |= SPI_CR1_SPE;
}

The implementation clears CR1 before rebuilding the mode. This prevents stale peripheral state from surviving a re-initialization sequence.

## 12.4 Blocking Byte Transfer Primitive

uint8_t SPI2_TransferByte(uint8_t byte)
{
    uint8_t received;

    //waiting for TX buffer empty
    while(!(SPI2->SR & SPI_SR_TXE));
    *((volatile uint8_t *)&SPI2->DR) = byte;

    //waiting for RX buffer Not empty
    while(!(SPI2->SR & SPI_SR_RXNE));
    received = *((volatile uint8_t *)&SPI2->DR);

    //Wait till SPI completes
    while(SPI2->SR & SPI_SR_BSY);

    return received;
}

The 8-bit access to SPI2->DR is intentional. Reading every received byte prevents RXNE from remaining set and avoids overrun during repeated full-duplex transfers.

## 12.5 Reset Sequence

void W25Q_FlashReset(void)
{
    /*
     * Software reset sequence:
     * 1. Enable Reset command: 0x66
     * 2. Reset Device command: 0x99
     *
     * These should be sent as separate SPI instructions.
     */

    W25Q_FLASH_CS_LOW();
    SPI2_TransferByte(W25Q_CMD_ENABLE_RESET);
    W25Q_FLASH_CS_HIGH();

    W25Q_FLASH_CS_LOW();
    SPI2_TransferByte(W25Q_CMD_RESET_DEVICE);
    W25Q_FLASH_CS_HIGH();

    /*
     * Device needs a small time after reset.
     * If you have delay_us(), use around 30 us or more.
     * If not, 1 ms delay is safe for now.
     */
    HAL_Delay(10);
}

Enable Reset and Reset Device are separate instructions, each with its own /CS low-high boundary. The final delay gives the device time to recover before the next command.

## 12.6 JEDEC Identification

void W25Q_FlashReadJEDEC(uint8_t *id)
{
    if (id == 0)
    {
        return;
    }

    W25Q_FLASH_CS_LOW();

    SPI2_TransferByte(W25Q_CMD_READ_JEDEC_ID);

    id[0] = SPI2_TransferByte(W25Q_DUMMY_BYTE);
    id[1] = SPI2_TransferByte(W25Q_DUMMY_BYTE);
    id[2] = SPI2_TransferByte(W25Q_DUMMY_BYTE);

    W25Q_FLASH_CS_HIGH();
}

The master transmits dummy 0xFF bytes to generate the clocks needed for the flash to shift out manufacturer, memory-type, and capacity bytes.

## 12.7 Status Reads and Ready Polling

void W25Q_FlashRead_StatusRegister(uint8_t *status)
{
	//status register 1
	W25Q_FLASH_CS_LOW();
	SPI2_TransferByte(W25Q_CMD_READ_STATUS_REG1);
	status[0] = SPI2_TransferByte(W25Q_DUMMY_BYTE);
	W25Q_FLASH_CS_HIGH();

	//status register 2
	W25Q_FLASH_CS_LOW();
	SPI2_TransferByte(W25Q_CMD_READ_STATUS_REG2);
	status[1] = SPI2_TransferByte(W25Q_DUMMY_BYTE);
	W25Q_FLASH_CS_HIGH();
}

uint8_t W25Q_FlashReadStatusReg1(void)
{
    uint8_t status;

    MCP2515_CS_HIGH();

    W25Q_FLASH_CS_LOW();
    SPI2_TransferByte(W25Q_CMD_READ_STATUS_REG1);
    status = SPI2_TransferByte(W25Q_DUMMY_BYTE);
    W25Q_FLASH_CS_HIGH();

    return status;
}

void W25Q_FlashWaitUntilReady(void)
{
    while (W25Q_FlashReadStatusReg1() & W25Q_SR1_BUSY)
    {
        /* wait while BUSY = 1 */
    }
}

Important: The polling logic is conceptually correct for learning, but an unbounded while loop is not acceptable in production firmware. A missing flash or malformed command can trap the CPU forever.

## 12.8 Write Enable and Write Disable

void W25Q_FlashWriteEnable(void)
{
    W25Q_FLASH_CS_LOW();
    SPI2_TransferByte(W25Q_CMD_WRITE_ENABLE);
    W25Q_FLASH_CS_HIGH();
}

void W25Q_FlashWriteDisable(void)
{
    W25Q_FLASH_CS_LOW();
    SPI2_TransferByte(W25Q_CMD_WRITE_DISABLE);
    W25Q_FLASH_CS_HIGH();
}

Each is a complete instruction ending at /CS high. A hardened Write Enable API should read SR1 afterward and confirm WEL=1 before erase or program.

## 12.9 Sequential Read Data

void W25Q_FlashRead_Data(uint32_t addr, uint8_t *buffer, uint32_t size)
{
    if ((buffer == 0) || (size == 0))
    {
        return;
    }

    /* Make sure other SPI2 slave is deselected */
    MCP2515_CS_HIGH();

    W25Q_FLASH_CS_LOW();

    SPI2_TransferByte(W25Q_CMD_READ_DATA);

    /* Send 24-bit address: A23-A16, A15-A8, A7-A0 */
    SPI2_TransferByte((uint8_t)((addr >> 16) & 0xFF));
    SPI2_TransferByte((uint8_t)((addr >> 8)  & 0xFF));
    SPI2_TransferByte((uint8_t)( addr        & 0xFF));

    for (uint32_t i = 0; i < size; i++)
    {
        buffer[i] = SPI2_TransferByte(W25Q_DUMMY_BYTE);
    }

    W25Q_FLASH_CS_HIGH();
}

Read Data sends the address most-significant byte first, then keeps /CS low while N dummy bytes generate N response bytes. Sequential read may cross page and sector boundaries.

## 12.10 Sector Erase

void W25Q_FlashSectorErase(uint32_t addr)
{
    W25Q_FLASH_CS_LOW();

    W25Q_FlashWriteEnable();

    SPI2_TransferByte(W25Q_CMD_SECTOR_ERASE_4KB);

    /* Send 24-bit address: A23-A16, A15-A8, A7-A0 */
    SPI2_TransferByte((uint8_t)((addr >> 16) & 0xFF));
    SPI2_TransferByte((uint8_t)((addr >> 8)  & 0xFF));
    SPI2_TransferByte((uint8_t)( addr        & 0xFF));

    W25Q_FLASH_CS_LOW();

    W25Q_FlashWaitUntilReady();

}

Critical correction: After the address, W25Q_FLASH_CS_LOW() must be W25Q_FLASH_CS_HIGH(). Remove the leading CS-low too, because WriteEnable owns its own /CS cycle. Otherwise the erase instruction may never terminate and latch.

## 12.11 Page Program

void W25Q_FlashPageProgram(uint32_t addr, uint8_t *data, uint32_t size)
{
    if ((data == 0) || (size == 0))
    {
        return;
    }

    if (size > W25Q_PAGE_SIZE)
    {
        size = W25Q_PAGE_SIZE;
    }

    MCP2515_CS_HIGH();

    W25Q_FlashWriteEnable();

    W25Q_FLASH_CS_LOW();

    SPI2_TransferByte(W25Q_CMD_PAGE_PROGRAM);

    SPI2_TransferByte((uint8_t)((addr >> 16) & 0xFF));
    SPI2_TransferByte((uint8_t)((addr >> 8)  & 0xFF));
    SPI2_TransferByte((uint8_t)( addr        & 0xFF));

    for (uint32_t i = 0; i < size; i++)
    {
        SPI2_TransferByte(data[i]);
    }

    W25Q_FLASH_CS_HIGH();

    W25Q_FlashWaitUntilReady();
}

This function limits the transfer count to 256 bytes, but it does not calculate the remaining room in the current page. A 256-byte write starting at a nonzero page offset can cross the page boundary and must be split by a wrapper.

## 12.12 Application Smoke Test

#define W25Q_TEST_ADDR   0x001000UL

extern const uint8_t font_8x8_digits[10][8];
extern const uint8_t font_8x8_alpha[26][8];
extern const uint8_t font_space[8];


volatile uint8_t mcp2515_node2_int_pending;
volatile uint8_t mcp2515_node1_int_pending;
volatile uint8_t can1_rx0_pending = 0;
uint8_t current_txctrl_addr;

//W25Q
	uint8_t jedec_id[3];
	uint8_t w25q_status[2];
	uint8_t w25q_buffer[8] = {0,0,0,0,0,0,0,0};

	uint8_t rx[5];
	uint8_t tx_data[] = { 'H', 'E', 'L', 'L', 'O' };

W25Q_FlashReset();
  W25Q_FlashReadJEDEC(jedec_id);
  W25Q_FlashRead_StatusRegister(w25q_status);

  //Programming sequence - Erase sector,program page, read data
  W25Q_FlashSectorErase(W25Q_TEST_ADDR);
  W25Q_FlashPageProgram(W25Q_TEST_ADDR,tx_data,5);
  W25Q_FlashRead_Data(W25Q_TEST_ADDR, rx, 5);

# 13. Current End-to-End Algorithm

Start
  |
  +-- Initialize GPIOB and SPI2
  +-- Keep PB9 and PB12 high
  +-- Reset flash: 0x66 then 0x99
  +-- Read JEDEC ID: 0x9F + 3 response bytes
  +-- Read SR1 and SR2
  +-- Sector erase at 0x001000
  |     +-- Write Enable
  |     +-- 0x20 + 00 10 00
  |     +-- /CS high          <-- missing in current source
  |     +-- Wait BUSY=0
  +-- Page program HELLO
  |     +-- Write Enable
  |     +-- 0x02 + 00 10 00 + 48 45 4C 4C 4F
  |     +-- /CS high
  |     +-- Wait BUSY=0
  +-- Read 5 bytes from 0x001000
  +-- Compare RX with TX       <-- not implemented in current source
  v
End / report result

# 14. Page-Boundary-Safe Write Design

The shared chat develops a higher-level no-erase write service. It repeatedly computes the remaining bytes in the current page, programs only that chunk, advances the address and data pointer, and repeats until the request is exhausted. This service assumes the target bits are already erased or only require 1-to-0 transitions.

Example: 40 bytes starting at 0x0000F0

Current page room = 0x100 - 0xF0 = 16 bytes

Transaction 1: Page Program 16 bytes at 0x0000F0
Transaction 2: Page Program 24 bytes at 0x000100

Invariant: no individual Page Program crosses a 256-byte page boundary.

# 15. Sector-Safe Update Design

A generic application update may need both 1-to-0 and 0-to-1 changes while preserving neighboring data. The safe method operates one sector at a time: read the complete sector into RAM, patch the RAM copy, erase the sector, program the full sector back page by page, and verify. The STM32 needs a 4096-byte working buffer or an equivalent streaming design.

Requested update
  |
  +-- Find containing 4 KB sector
  +-- Read full sector to RAM
  +-- Modify requested byte range in RAM
  +-- Erase sector
  +-- Program 16 pages x 256 bytes
  +-- Read back and verify
  +-- Continue with next sector if request crosses boundary

Power-loss note: without redundancy or journaling, failure between erase and
reprogram can destroy both old and new sector contents.

# 16. Production-Driver Evolution from the Shared Chat

| Evolution step | Why it matters | Recommended contract |
| --- | --- | --- |
| Status return type | void APIs hide failure | Return OK, invalid parameter, timeout, ID mismatch, or verify failure |
| Parameter validation | Null pointers and zero sizes can fault or mislead | Reject invalid input before asserting /CS |
| Range validation | 24-bit arithmetic can exceed the 8 MB device | Require addr < chip size and size <= chip size - addr |
| Bounded ready wait | Missing hardware can hang CPU | Use decrementing counter or HAL_GetTick deadline |
| WEL verification | 0x06 may be ignored while BUSY or blocked | Read SR1 and require WEL=1 |
| Page-safe write | Application writes can span pages | Split request into page-local Page Program calls |
| Sector-safe update | 0-to-1 changes need erase while preserving neighbors | Read-modify-erase-program one sector at a time |
| Verify API | Command completion is not data correctness | Read back and compare critical regions |
| Self-test | Bring-up needs deterministic proof | Use reserved sector, erase/program/read/compare, then report immediately |
| MISRA-oriented flow | Automotive teams may enforce single-return style | Use project rules consistently without hiding errors |
| Concurrency guard | Shared SPI2 and future RTOS use can interleave transactions | Own a bus lock and centralized deselect-all helper |

# 17. Test and Validation Plan

| Test | Expected result | Failure interpretation |
| --- | --- | --- |
| Idle bus | PB9=1 and PB12=1 | A slave remains selected or GPIO init is missing |
| Reset | Two separate /CS pulses containing 0x66 and 0x99 | Commands combined into one invalid instruction |
| JEDEC | 0x9F followed by realistic response, not all FF/00 | Power, CS, wiring, SPI mode, or part mismatch |
| Write Enable | SR1.WEL becomes 1 | Command ignored, device BUSY, protection, or CS framing issue |
| Erase | 0x20 + 00 10 00, /CS rises, BUSY sets then clears | Current source CS defect prevents latching |
| Erased data | Target bytes read 0xFF before programming | Erase failed or wrong address |
| Program | 0x02 + 00 10 00 + HELLO, then BUSY clears | WEL/page/CS/protection failure |
| Read-back | 48 45 4C 4C 4F | Program failed, wrong address, or data integrity problem |
| Shared bus | PB12 remains high for all flash transactions | Multiple slaves may drive MISO |

## 17.1 Logic-Analyzer Decode Checklist

Set analyzer SPI mode to CPOL=0, CPHA=0, MSB first, 8 bits.

Map PB9 as flash enable, PB13 as clock, PB15 as MOSI, and PB14 as MISO.

Confirm every instruction begins with /CS falling and ends with /CS rising.

Confirm the 24-bit address order is 00 10 00 for test address 0x001000.

Check that the status-read loop creates short 0x05 transactions rather than holding /CS across unrelated commands.

Confirm MCP2515 PB12 chip select stays high during the complete flash transaction.

# 18. Expected Debug Values

Watch variables after a corrected run:

jedec_id[0..2]  : realistic manufacturer/type/capacity bytes
w25q_status[0] : SR1, BUSY=0 after operation completes
w25q_status[1] : SR2 snapshot
tx_data         : 48 45 4C 4C 4F
rx              : 48 45 4C 4C 4F

Current limitation:
main.c stores these values but does not compare or print the W25Q result.

# 19. Embedded Engineering Review

| Severity | Finding | Impact | Required correction |
| --- | --- | --- | --- |
| Critical | SectorErase asserts /CS low again after the address | Erase may not latch; BUSY never represents the intended operation | Raise /CS after the third address byte |
| High | SectorErase asserts /CS before calling WriteEnable | Nested transaction framing is invalid/confusing | Call WriteEnable first, then assert /CS for erase |
| High | WaitUntilReady has no timeout | Firmware can hang permanently | Return timeout after a bounded wait |
| High | WriteEnable does not verify WEL | Program/erase can fail silently | Read SR1 and require WEL=1 |
| High | PageProgram does not enforce remaining page room | Cross-page write can wrap or corrupt intended layout | Reject crossing or split in a wrapper |
| Medium | Read_StatusRegister has no null check | Null output pointer causes fault | Validate status before /CS low |
| Medium | Read/PageProgram lack full address-range checks | Request can extend beyond 0x7FFFFF | Validate address and length without overflow |
| Medium | Read-back is not compared or reported | Test has no explicit PASS/FAIL | Compare rx with tx_data and log result |
| Medium | Non-target CS handling is inconsistent | Shared bus depends on call-site discipline | Centralize deselect-all behavior |
| Low | spi.h has no include guard | Duplicate inclusion can cause build problems | Add conventional include guard |
| Low | SPI1 comment says Enable SPI2 clock and cites PCLK1 | Documentation can mislead future maintenance | Correct comments to SPI1/APB2/PCLK2 |

# 20. Troubleshooting Matrix

| Symptom | Most likely causes | First checks |
| --- | --- | --- |
| JEDEC = FF FF FF | Flash deselected, MISO floating, no power, wrong wiring | Probe PB9, VCC/GND, PB13-PB15 |
| JEDEC = 00 00 00 | MISO stuck low or signal/pin mapping error | Disconnect other SPI2 slave and inspect DO line |
| ID capacity byte differs | Module marking/seller listing does not match silicon | Trust read ID and verify exact datasheet before sizing |
| WEL stays 0 | Device BUSY, write protected, bad /CS frame | Read SR1, inspect /WP/protection bits and 0x06 pulse |
| BUSY never sets after erase | Erase command never latched | Look for /CS rising edge after address |
| BUSY never clears | Electrical/command fault or no timeout | Stop test, inspect status traffic, add deadline |
| Read-back remains FF | Erase worked but program did not | Check WEL, 0x02 frame, address and /CS edge |
| Neighbor data lost | Raw sector erase used for a small update | Use sector read-modify-erase-program |
| Only later page is wrong | Cross-page Page Program | Split write at each 256-byte boundary |

# 21. Limitations

The attached driver is synchronous, polling-based, and blocking.

SPI2_TransferByte has no peripheral-error or timeout reporting.

The driver has no general multi-page write, sector-safe update, verify, or self-test API.

No filesystem, wear-leveling, bad-record recovery, or power-fail atomicity layer is present.

The current application combines CAN, display, UART, and flash experiments, so W25Q testing is not isolated.

No new screenshot was attached for this request; diagrams and expected analyzer frames are included instead.

# 22. Recommended Next Implementation Order

Correct W25Q_FlashSectorErase chip-select sequencing and capture the fixed frame on a logic analyzer.

Add a driver status type and bounded timeout behavior to all blocking paths.

Add parameter and address-range validation before touching the bus.

Verify WEL after Write Enable and propagate failures.

Add read-back comparison to the current HELLO smoke test.

Implement a page-boundary-safe no-erase write wrapper.

Implement sector-safe read-modify-erase-program with verification.

Add a reserved-sector self-test and explicit UART PASS/FAIL output.

Add bus locking and centralized chip-select ownership before RTOS or concurrent use.

# 23. Conclusion

Conclusion: The project has the essential SPI NOR building blocks. First correct the Sector Erase /CS sequence. Then add status returns, bounded waits, WEL checks, page-safe writes, sector-safe updates, and read-back verification.

# 24. Appendix A: Source Snapshot

| File | Finalized path | SHA-256 prefix |
| --- | --- | --- |
| main.c | D:\Embedded Communication Protocols\CAN1_RX_Interrupt\Core\Src\main.c | 2d4ec14ef52ed997... |
| spi.c | D:\Embedded Communication Protocols\CAN1_RX_Interrupt\Core\Src\spi.c | 9daa385a8a7795cc... |
| usart.c | D:\Embedded Communication Protocols\CAN1_RX_Interrupt\Core\Src\usart.c | ab0473504bf0db8a... |
| w25q64fv.c | D:\Embedded Communication Protocols\CAN1_RX_Interrupt\Core\Src\w25q64fv.c | 446bd6df3716d6e2... |
| spi.h | D:\Embedded Communication Protocols\CAN1_RX_Interrupt\Core\Inc\spi.h | 8853e0d22933aeb9... |
| usart.h | D:\Embedded Communication Protocols\CAN1_RX_Interrupt\Core\Inc\usart.h | 8564d36616457e40... |
| w25q64fv.h | D:\Embedded Communication Protocols\CAN1_RX_Interrupt\Core\Inc\w25q64fv.h | c31dfbb64ef8bec2... |

# 25. Appendix B: References

| Reference | Use |
| --- | --- |
| https://chatgpt.com/share/6a41dcea-c2a8-83ee-996c-a4cf121eec95 | Shared theory and driver-evolution discussion |
| https://www.winbond.com/hq/support/documentation/?__locale=en&line=/product/code-storage-flash/index.html&family=/product/code-storage-flash/qspi-nor/index.html&category=/.categories/resources/datasheet/&pno=W25Q64FV | Official W25Q64FV datasheet catalog entry and device authority |
| C:\Users\HP\Documents\Codex\2026-06-04\i-m-an-embedded-engineer-currently\work\w25q64fv_driver_clean_source.txt | Filtered shared-chat material used for authoring |

# 26. Appendix C: Revision Checklist

| Verification item | Status |
| --- | --- |
| Shared link downloaded and decoded | Complete |
| Scope filtered to W25Q64FV driver implementation | Complete |
| All seven attached files read | Complete |
| Literal code snippets sourced only from attached files | Complete |
| System, driver-layer, internal-function, and state diagrams included | Complete |
| Current implementation separated from future driver design | Complete |
| Sector Erase /CS defect documented | Complete |
| Timeout, WEL, boundary, range, and verification gaps documented | Complete |
| Footer set to Embedded Learning Document | Complete |
| Filename avoids the word advanced | Complete |