W25Q64FV SPI Flash Project

STM32F407 SPI2 driver reference for W25Q64FV rules, commands, initialization, erase/program/read flow, and safe usage

Document intent: Industry-grade learning reference for the W25Q64FV SPI NOR flash module used in the CAN1_RX_Interrupt project. It combines the shared-chat theory with the finalized firmware files and adds embedded-engineering verification notes.

| Field | Value |
| --- | --- |
| Project scope | W25Q64FV flash rules, functions, initialization, and usage using SPI |
| MCU peripheral | STM32F407 SPI2, polling transfer |
| Flash chip select | PB9, controlled by W25Q_FLASH_CS_LOW/HIGH |
| SPI pins | PB13 SCK, PB14 MISO, PB15 MOSI, AF5 |
| Memory geometry | 8 MB chip, 256-byte page, 4 KB sector, 64 KB block |
| Current test address | W25Q_TEST_ADDR = 0x001000UL |
| Current test data | Five bytes: 'H', 'E', 'L', 'L', 'O' |
| Theory source | https://chatgpt.com/share/6a3cd3b1-539c-83e8-b215-ebb12dc6e2ea |

# 1. Document Control / Metadata

| Item | Details |
| --- | --- |
| Design preset | compact_reference_guide |
| Firmware reference | D:\Embedded Communication Protocols\CAN1_RX_Interrupt |
| Shared-chat reference | https://chatgpt.com/share/6a3cd3b1-539c-83e8-b215-ebb12dc6e2ea |
| Code fidelity rule | Only the attached finalized source files are used for code snippets and live flow |
| Footer rule | Embedded Learning Document |
| Screenshot boundary | No screenshot file was attached in this turn; expected validation observations are documented |
| Variant caution | The shared chat discusses W25QXX variants; always verify the actual IC marking and voltage range before wiring |

# 2. Learning Objectives

Understand how an STM32 communicates with a W25Q64FV SPI NOR flash using command, address, and data phases.

Explain flash memory rules: erase-before-program, 1-to-0 programming, 0-to-1 erase, page limits, sector erase, BUSY, and WEL.

Trace the finalized firmware sequence: reset, JEDEC ID read, status read, sector erase, page program, and read-back.

Map SPI2 GPIO configuration and chip-select ownership to the flash hardware.

Identify source-level risks that matter in real firmware, especially chip-select latching, timeouts, WEL verification, and page-boundary-safe writes.

Use system-level diagrams to understand where SPI, flash array organization, and software driver functions meet.

# 3. Project Objective

The project brings up a W25Q64FV-style SPI NOR flash connected to STM32F407 SPI2. The current firmware performs a practical storage smoke test: reset the flash, read identity/status, erase one 4 KB sector at 0x001000, program the bytes HELLO, then read the same address back into a receive buffer.

# 4. Problem Statement

Core engineering problem: SPI flash is not RAM. Reads are byte-addressable, but programming is constrained by page boundaries and erase state. A correct driver must sequence chip select, write enable, erase/program commands, BUSY polling, and read-back verification carefully.

# 5. System Overview

STM32F407
  |
  | SPI2 peripheral, Mode 0, 8-bit, MSB first, software CS
  |
  +-- PB13  SPI2_SCK  ---> W25Q CLK
  +-- PB15  SPI2_MOSI ---> W25Q DI
  +-- PB14  SPI2_MISO <--- W25Q DO
  +-- PB9   GPIO CS   ---> W25Q /CS
  |
  +-- PB12 GPIO CS    ---> MCP2515 on same SPI2 bus

Important shared-bus rule:
Only one SPI2 chip-select may be low at a time.
The flash driver raises MCP2515 CS before flash read/page-program paths.

# 6. Requirements

| Requirement | Project value | Verification point |
| --- | --- | --- |
| SPI instance | SPI2 | SPI2_Init enables RCC_APB1ENR_SPI2EN |
| SPI mode | Mode 0 | CPOL=0, CPHA=0 |
| Data frame | 8-bit, MSB first | DFF cleared, LSBFIRST cleared |
| SPI clock | PCLK / 32; comment says 1 MHz for 32 MHz PCLK1 | SPI_CR1_BR_2 |
| Flash CS | PB9 output | W25Q_FLASH_CS_LOW/HIGH macros |
| Read ID | 0x9F, three returned bytes | W25Q_FlashReadJEDEC |
| Status | SR1 0x05, SR2 0x35 | W25Q_FlashRead_StatusRegister |
| Erase | 4 KB sector erase command 0x20 | W25Q_FlashSectorErase |
| Program | Page program command 0x02, capped to 256 bytes | W25Q_FlashPageProgram |
| Read data | Read command 0x03 and 24-bit address | W25Q_FlashRead_Data |

# 7. Hardware Components

| Component | Role | Important caution |
| --- | --- | --- |
| W25Q64FV SPI NOR flash | External non-volatile storage | Verify exact part suffix and voltage before powering |
| STM32F407 SPI2 | Command/address/data transport | SPI2 is shared with MCP2515, so CS discipline matters |
| PB9 | Flash chip select | Must return high to end/latch most commands |
| PB13/PB14/PB15 | SCK/MISO/MOSI | Configured as AF5 |
| USART2 | Debug output path | Current W25Q test does not print JEDEC/readback by default |
| MCP2515 on SPI2 | Other SPI2 slave | Must be deselected during flash operations |

# 8. Software and Firmware Tools

STM32CubeIDE or equivalent for building/flashing the project.

UART terminal on USART2 for debug messages if readback printing is added.

Logic analyzer for SPI decode of CS, SCK, MOSI, and MISO.

Debugger watch window for jedec_id, w25q_status, tx_data, and rx buffers.

W25Q64FV datasheet for exact voltage, timing, command, and endurance limits.

# 9. System-Level Diagrams

## 9.1 SPI Flash Transaction Pattern

Every flash command follows this basic shape:

/CS low
  |
  +-- instruction byte
  +-- optional 24-bit address: A23-A16, A15-A8, A7-A0
  +-- optional data write bytes or dummy bytes for reads
  |
/CS high
  |
Flash latches/executes the command boundary

## 9.2 Memory Organization

W25Q64FV logical address space

Chip size      : 8 MB = 0x800000 bytes
Valid address  : 0x000000 to 0x7FFFFF

Page           : 256 bytes = 0x100
Sector         : 4 KB      = 0x1000
64 KB block    : 64 KB     = 0x10000

Example:
0x001000 is sector-aligned.
It is also page-aligned because 0x001000 % 0x100 = 0.

## 9.3 Current Firmware Test Flow

SPI2_GPIO_Init()
SPI2_Init()
W25Q_FlashReset()
W25Q_FlashReadJEDEC(jedec_id)
W25Q_FlashRead_StatusRegister(w25q_status)
W25Q_FlashSectorErase(0x001000)
W25Q_FlashPageProgram(0x001000, "HELLO", 5)
W25Q_FlashRead_Data(0x001000, rx, 5)
Expected rx buffer: 48 45 4C 4C 4F

# 10. Firmware Architecture

| Module | Responsibility | Important APIs / data |
| --- | --- | --- |
| w25q64fv.h | Flash geometry, command opcodes, status bits, driver prototypes | W25Q_PAGE_SIZE, W25Q_SECTOR_SIZE, W25Q_CMD_* |
| w25q64fv.c | Flash reset, JEDEC/status/read/erase/program helpers | W25Q_FlashReset, W25Q_FlashPageProgram |
| spi.h | SPI API and chip-select macros | W25Q_FLASH_CS_LOW/HIGH, MCP2515_CS_HIGH |
| spi.c | SPI2 GPIO/init/byte transfer | SPI2_GPIO_Init, SPI2_Init, SPI2_TransferByte |
| main.c | Calls W25Q flash smoke-test sequence | W25Q_TEST_ADDR, tx_data, rx |
| usart.c/h | Debug transport | USART2_SendString |

# 11. Communication Protocol Used

The flash uses standard SPI. The finalized SPI2 configuration uses master mode, software slave management, internal SSI high, CPOL=0, CPHA=0, 8-bit frames, MSB-first transfer, and blocking polling on TXE, RXNE, and BSY. Commands are sent as ordinary SPI bytes while PB9 is manually driven low/high as flash chip select.

# 12. Core Concept Deep Dive

## 12.1 Practical Flash Rules

Read operations can start at any byte address and can read arbitrary lengths.

Page Program writes up to 256 bytes, but it must not be treated as a generic stream write across page boundaries.

Erase operates on larger regions; the minimum common erase unit in this driver is one 4 KB sector.

An erased byte reads as 0xFF.

Programming changes bits from 1 to 0.

Changing any bit from 0 back to 1 requires erase of the containing erase sector.

Write Enable must be issued before program or erase.

BUSY must be polled after program or erase before assuming data is stable.

Erase/program endurance is finite, so repeated fast erase loops should be avoided.

## 12.2 Command Summary

| Operation | Opcode | Current driver support | Purpose |
| --- | --- | --- | --- |
| Enable Reset | 0x66 | Yes | First part of software reset sequence |
| Reset Device | 0x99 | Yes | Second part of software reset sequence |
| Read JEDEC ID | 0x9F | Yes | Verify manufacturer/type/capacity response |
| Read Status Register 1 | 0x05 | Yes | Read BUSY and WEL bits |
| Read Status Register 2 | 0x35 | Yes | Read second status/config byte |
| Write Enable | 0x06 | Yes | Set WEL before erase/program |
| Write Disable | 0x04 | Yes | Clear write enable latch |
| Read Data | 0x03 | Yes | Read from 24-bit address |
| Page Program | 0x02 | Yes | Program up to one page |
| Sector Erase 4 KB | 0x20 | Yes | Erase containing 4 KB sector |

## 12.3 Status Register Rule: BUSY and WEL

Status Register 1 is the operational guardrail. BUSY bit 0 indicates the flash is internally programming or erasing. WEL bit 1 indicates that the Write Enable Latch is set. A robust driver checks WEL after Write Enable and polls BUSY after erase/program with a timeout.

## 12.4 Addressing and Alignment

| Expression | Meaning | Example |
| --- | --- | --- |
| PAGE_OFFSET(addr) | Byte index inside a 256-byte page | 0x0010F0 -> 0xF0 |
| PAGE_BASE(addr) | Start address of containing page | 0x0010F0 -> 0x001000 |
| SECTOR_OFFSET(addr) | Byte index inside a 4 KB sector | 0x001234 -> 0x234 |
| SECTOR_BASE(addr) | Start address of containing sector | 0x001234 -> 0x001000 |
| BLOCK64_BASE(addr) | Start of containing 64 KB block | 0x012345 -> 0x010000 |

## 12.5 Page-Boundary-Safe Writes

The shared chat correctly emphasizes the next driver level: a user write may cross page boundaries, but the low-level Page Program operation should stay within the current 256-byte page. A wrapper should compute space_left = 256 - (addr & 0xFF), program only that chunk, then move to the next page.

Example:
Write 40 bytes at 0x0000F0.

Page 0 ends at 0x0000FF.
Bytes left in page 0 = 16.

Correct split:
  16 bytes at 0x0000F0
  24 bytes at 0x000100

## 12.6 Sector-Safe Updates

A sector-safe update is different from raw page programming. To change existing data without losing neighboring bytes, read the full 4 KB sector into RAM, modify the RAM copy, erase the sector, then program the sector back page by page. This is the safe pattern for configuration records and small updates inside a sector.

## 12.7 Variant and Voltage Caution

Verify the exact part: The shared chat included an important W25QXX variant caution: part suffixes matter. Do not assume W25Q64FV and W25Q64FW have the same voltage range. Confirm the actual IC marking and datasheet before connecting directly to STM32 3.3 V logic.

# 13. Module-Wise Implementation

| Phase | Final-source behavior | Engineering purpose |
| --- | --- | --- |
| SPI2 GPIO | PB9/PB12 outputs, PB13/PB14/PB15 AF5 | Prepare flash CS, other SPI2 CS, and SPI signals |
| SPI2 peripheral | Master, SSM/SSI, Mode 0, 8-bit, MSB first, BR_2 | Generate flash-compatible SPI transfers |
| Reset | 0x66 followed by 0x99 and delay | Return flash to known state |
| Read ID | 0x9F then three dummy transfers | Confirm communication/device response |
| Read status | 0x05 and 0x35 | Observe BUSY/WEL/protection state |
| Erase | Write Enable then 0x20 + 24-bit address | Set erased bytes in containing 4 KB sector to 0xFF |
| Program | Write Enable then 0x02 + address + up to 256 bytes | Program test bytes |
| Read back | 0x03 + address + dummy bytes | Verify programmed data |

# 14. Main Algorithm and Flowchart

Power-on
  |
  +-- HAL_Init and SystemClock_Config
  +-- USART2_Init
  +-- SPI2_GPIO_Init
  +-- SPI2_Init
  |
  +-- W25Q_FlashReset
  +-- W25Q_FlashReadJEDEC(jedec_id)
  +-- W25Q_FlashRead_StatusRegister(w25q_status)
  |
  +-- W25Q_FlashSectorErase(0x001000)
  +-- W25Q_FlashPageProgram(0x001000, "HELLO", 5)
  +-- W25Q_FlashRead_Data(0x001000, rx, 5)
  |
  +-- Debugger should show rx = {'H','E','L','L','O'}

# 15. Code Explanation

Code authority: The snippets below are copied from the attached finalized source files. Review notes are documented separately so the document remains honest about what the current code actually does.

## 15.1 Flash Geometry and Opcodes

#ifndef INC_W25Q64FV_H_
#define INC_W25Q64FV_H_

#include "stm32f407xx.h"
#include <stdint.h>

/* Memory geometry */
#define W25Q_PAGE_SIZE       0x100UL     /* 256 bytes */
#define W25Q_SECTOR_SIZE     0x1000UL    /* 4 KB */
#define W25Q_BLOCK64_SIZE    0x10000UL   /* 64 KB */
#define W25Q_CHIP_SIZE       0x800000UL  /* 8 MB */

#define PAGE_OFFSET(addr)    ((addr) & 0xFFUL)
#define PAGE_BASE(addr)      ((addr) & ~0xFFUL)

#define SECTOR_OFFSET(addr)  ((addr) & 0xFFFUL)
#define SECTOR_BASE(addr)    ((addr) & ~0xFFFUL)

#define BLOCK64_OFFSET(addr) ((addr) & 0xFFFFUL)
#define BLOCK64_BASE(addr)   ((addr) & ~0xFFFFUL)

/* W25Q64FV command set */
#define W25Q_CMD_READ_STATUS_REG1   0x05
#define W25Q_CMD_READ_STATUS_REG2   0x35
#define W25Q_CMD_READ_DATA          0x03
#define W25Q_CMD_READ_JEDEC_ID      0x9F
#define W25Q_CMD_ENABLE_RESET       0x66
#define W25Q_CMD_RESET_DEVICE       0x99
#define W25Q_CMD_WRITE_ENABLE       0x06
#define W25Q_CMD_WRITE_DISABLE      0x04
#define W25Q_CMD_SECTOR_ERASE_4KB   0x20
#define W25Q_CMD_PAGE_PROGRAM   	0x02

#define W25Q_DUMMY_BYTE             0xFF

#define W25Q_SR1_BUSY    0x01
#define W25Q_SR1_WEL     0x02

void W25Q_FlashReset(void);
void W25Q_FlashReadJEDEC(uint8_t *id);
void W25Q_FlashRead_StatusRegister(uint8_t *status);
void W25Q_FlashRead_Data(uint32_t addr, uint8_t *buffer, uint32_t size);
uint8_t W25Q_FlashReadStatusReg1(void);
void W25Q_FlashWaitUntilReady(void);
void W25Q_FlashWriteEnable(void);
void W25Q_FlashWriteDisable(void);
void W25Q_FlashSectorErase(uint32_t addr);
void W25Q_FlashPageProgram(uint32_t addr, uint8_t *data, uint32_t size);

#endif /* INC_W25Q64FV_H_ */

## 15.2 SPI2 GPIO and Chip Selects

void SPI2_GPIO_Init(void)
{
    /* Enable GPIOB clock */
    RCC->AHB1ENR |= RCC_AHB1ENR_GPIOBEN;

    /*
     * PB9	= CS_W25Q_FLASH
     * PB12 = CS_MCP2515_NODE1
     * PB13 = SPI2_SCK AF
     * PB14 = SPI2_MISO AF
     * PB15 = SPI2_MOSI AF
     */

    /* Clear mode bits */
    GPIOB->MODER &= ~((3UL << (	9 * 2)) |
    				  (3UL << (12 * 2)) |
                      (3UL << (13 * 2)) |
                      (3UL << (14 * 2)) |
                      (3UL << (15 * 2)));

    /* Set modes */
    GPIOB->MODER |=  ((1UL << ( 9 * 2)) |   /* PB9 output */
    				  (1UL << (12 * 2)) |   /* PB12 output */
                      (2UL << (13 * 2)) |   /* PB13 AF */
                      (2UL << (14 * 2)) |   /* PB14 AF */
                      (2UL << (15 * 2)));   /* PB15 AF */

    /* Push-pull */
    GPIOB->OTYPER &= ~((1UL <<  9) |
    				   (1UL << 12) |
                       (1UL << 13) |
                       (1UL << 14) |
                       (1UL << 15));

    /* Speed: medium/high for SPI */
    GPIOB->OSPEEDR &= ~((3UL << ( 9 * 2)) |
    					(3UL << (12 * 2)) |
                        (3UL << (13 * 2)) |
                        (3UL << (14 * 2)) |
                        (3UL << (15 * 2)));

    GPIOB->OSPEEDR |=  ((2UL << ( 9 * 2)) |
    					(2UL << (12 * 2)) |
                        (2UL << (13 * 2)) |
                        (2UL << (14 * 2)) |
                        (2UL << (15 * 2)));

    /* Pull config: CS pull-up, others no pull */
    GPIOB->PUPDR &= ~((3UL << ( 9 * 2)) |
    				  (3UL << (12 * 2)) |
                      (3UL << (13 * 2)) |
                      (3UL << (14 * 2)) |
                      (3UL << (15 * 2)));

    GPIOB->PUPDR |=  (1UL << (12 * 2));     /* PB12 pull-up */
    GPIOB->PUPDR |=  (1UL << ( 9 * 2));     /* PB9 pull-up */

    /* AF5 for PB13, PB14, PB15 */
    GPIOB->AFR[1] &= ~((0xFUL << ((13 - 8) * 4)) |
                       (0xFUL << ((14 - 8) * 4)) |
                       (0xFUL << ((15 - 8) * 4)));

    GPIOB->AFR[1] |=  ((5UL << ((13 - 8) * 4)) |
                       (5UL << ((14 - 8) * 4)) |
                       (5UL << ((15 - 8) * 4)));

    /* Keep CS high initially */
    MCP2515_CS_HIGH();
    W25Q_FLASH_CS_HIGH();

}

## 15.3 SPI2 Peripheral Setup

void SPI2_Init(void)
{
    /* Enable SPI2 clock */
    RCC->APB1ENR |= RCC_APB1ENR_SPI2EN;

    /* Disable SPI before configuration */
    SPI2->CR1 &= ~SPI_CR1_SPE;

    /* Clear CR1 configuration bits we care about */
    SPI2->CR1 = 0;

    /*
     * Master mode
     * Software slave management
     * Internal slave select high
     */
    SPI2->CR1 |= SPI_CR1_MSTR;
    SPI2->CR1 |= SPI_CR1_SSM;
    SPI2->CR1 |= SPI_CR1_SSI;

    /*
     * Mode 0: CPOL = 0, CPHA = 0
     */
    SPI2->CR1 &= ~SPI_CR1_CPOL;
    SPI2->CR1 &= ~SPI_CR1_CPHA;

    /*
     * 8-bit frame, MSB first
     */
    SPI2->CR1 &= ~SPI_CR1_DFF;
    SPI2->CR1 &= ~SPI_CR1_LSBFIRST;

    /*
     * Baud rate = PCLK / 32
     * If PCLK1 = 32 MHz, SCK = 1 MHz
     */
    SPI2->CR1 &= ~SPI_CR1_BR;
    SPI2->CR1 |= SPI_CR1_BR_2;

    /*
     * Full duplex 2-line
     */
    SPI2->CR1 &= ~SPI_CR1_BIDIMODE;
    SPI2->CR1 &= ~SPI_CR1_RXONLY;

    /* Enable SPI */
    SPI2->CR1 |= SPI_CR1_SPE;
}

## 15.4 SPI2 Blocking Byte Transfer

uint8_t SPI2_TransferByte(uint8_t byte)
{
    uint8_t received;

    //waiting for TX buffer empty
    while(!(SPI2->SR & SPI_SR_TXE));
    *((volatile uint8_t *)&SPI2->DR) = byte;

    //waiting for RX buffer Not empty
    while(!(SPI2->SR & SPI_SR_RXNE));
    received = *((volatile uint8_t *)&SPI2->DR);

    //Wait till SPI completes
    while(SPI2->SR & SPI_SR_BSY);

    return received;
}

## 15.5 Flash Reset

void W25Q_FlashReset(void)
{
    /*
     * Software reset sequence:
     * 1. Enable Reset command: 0x66
     * 2. Reset Device command: 0x99
     *
     * These should be sent as separate SPI instructions.
     */

    W25Q_FLASH_CS_LOW();
    SPI2_TransferByte(W25Q_CMD_ENABLE_RESET);
    W25Q_FLASH_CS_HIGH();

    W25Q_FLASH_CS_LOW();
    SPI2_TransferByte(W25Q_CMD_RESET_DEVICE);
    W25Q_FLASH_CS_HIGH();

    /*
     * Device needs a small time after reset.
     * If you have delay_us(), use around 30 us or more.
     * If not, 1 ms delay is safe for now.
     */
    HAL_Delay(10);
}

## 15.6 Read JEDEC ID

void W25Q_FlashReadJEDEC(uint8_t *id)
{
    if (id == 0)
    {
        return;
    }

    W25Q_FLASH_CS_LOW();

    SPI2_TransferByte(W25Q_CMD_READ_JEDEC_ID);

    id[0] = SPI2_TransferByte(W25Q_DUMMY_BYTE);
    id[1] = SPI2_TransferByte(W25Q_DUMMY_BYTE);
    id[2] = SPI2_TransferByte(W25Q_DUMMY_BYTE);

    W25Q_FLASH_CS_HIGH();
}

## 15.7 Read Status Registers

void W25Q_FlashRead_StatusRegister(uint8_t *status)
{
	//status register 1
	W25Q_FLASH_CS_LOW();
	SPI2_TransferByte(W25Q_CMD_READ_STATUS_REG1);
	status[0] = SPI2_TransferByte(W25Q_DUMMY_BYTE);
	W25Q_FLASH_CS_HIGH();

	//status register 2
	W25Q_FLASH_CS_LOW();
	SPI2_TransferByte(W25Q_CMD_READ_STATUS_REG2);
	status[1] = SPI2_TransferByte(W25Q_DUMMY_BYTE);
	W25Q_FLASH_CS_HIGH();
}

uint8_t W25Q_FlashReadStatusReg1(void)
{
    uint8_t status;

    MCP2515_CS_HIGH();

    W25Q_FLASH_CS_LOW();
    SPI2_TransferByte(W25Q_CMD_READ_STATUS_REG1);
    status = SPI2_TransferByte(W25Q_DUMMY_BYTE);
    W25Q_FLASH_CS_HIGH();

    return status;
}

void W25Q_FlashWaitUntilReady(void)
{
    while (W25Q_FlashReadStatusReg1() & W25Q_SR1_BUSY)
    {
        /* wait while BUSY = 1 */
    }
}

## 15.8 Write Enable / Write Disable

void W25Q_FlashWriteEnable(void)
{
    W25Q_FLASH_CS_LOW();
    SPI2_TransferByte(W25Q_CMD_WRITE_ENABLE);
    W25Q_FLASH_CS_HIGH();
}

void W25Q_FlashWriteDisable(void)
{
    W25Q_FLASH_CS_LOW();
    SPI2_TransferByte(W25Q_CMD_WRITE_DISABLE);
    W25Q_FLASH_CS_HIGH();
}

## 15.9 Read Data

void W25Q_FlashRead_Data(uint32_t addr, uint8_t *buffer, uint32_t size)
{
    if ((buffer == 0) || (size == 0))
    {
        return;
    }

    /* Make sure other SPI2 slave is deselected */
    MCP2515_CS_HIGH();

    W25Q_FLASH_CS_LOW();

    SPI2_TransferByte(W25Q_CMD_READ_DATA);

    /* Send 24-bit address: A23-A16, A15-A8, A7-A0 */
    SPI2_TransferByte((uint8_t)((addr >> 16) & 0xFF));
    SPI2_TransferByte((uint8_t)((addr >> 8)  & 0xFF));
    SPI2_TransferByte((uint8_t)( addr        & 0xFF));

    for (uint32_t i = 0; i < size; i++)
    {
        buffer[i] = SPI2_TransferByte(W25Q_DUMMY_BYTE);
    }

    W25Q_FLASH_CS_HIGH();
}

## 15.10 Sector Erase

void W25Q_FlashSectorErase(uint32_t addr)
{
    W25Q_FLASH_CS_LOW();

    W25Q_FlashWriteEnable();

    SPI2_TransferByte(W25Q_CMD_SECTOR_ERASE_4KB);

    /* Send 24-bit address: A23-A16, A15-A8, A7-A0 */
    SPI2_TransferByte((uint8_t)((addr >> 16) & 0xFF));
    SPI2_TransferByte((uint8_t)((addr >> 8)  & 0xFF));
    SPI2_TransferByte((uint8_t)( addr        & 0xFF));

    W25Q_FLASH_CS_LOW();

    W25Q_FlashWaitUntilReady();

}

## 15.11 Page Program

void W25Q_FlashPageProgram(uint32_t addr, uint8_t *data, uint32_t size)
{
    if ((data == 0) || (size == 0))
    {
        return;
    }

    if (size > W25Q_PAGE_SIZE)
    {
        size = W25Q_PAGE_SIZE;
    }

    MCP2515_CS_HIGH();

    W25Q_FlashWriteEnable();

    W25Q_FLASH_CS_LOW();

    SPI2_TransferByte(W25Q_CMD_PAGE_PROGRAM);

    SPI2_TransferByte((uint8_t)((addr >> 16) & 0xFF));
    SPI2_TransferByte((uint8_t)((addr >> 8)  & 0xFF));
    SPI2_TransferByte((uint8_t)( addr        & 0xFF));

    for (uint32_t i = 0; i < size; i++)
    {
        SPI2_TransferByte(data[i]);
    }

    W25Q_FLASH_CS_HIGH();

    W25Q_FlashWaitUntilReady();
}

## 15.12 Main Test Sequence

#define W25Q_TEST_ADDR   0x001000UL

extern const uint8_t font_8x8_digits[10][8];
extern const uint8_t font_8x8_alpha[26][8];
extern const uint8_t font_space[8];


volatile uint8_t mcp2515_node2_int_pending;
volatile uint8_t mcp2515_node1_int_pending;
volatile uint8_t can1_rx0_pending = 0;
uint8_t current_txctrl_addr;

//W25Q
	uint8_t jedec_id[3];
	uint8_t w25q_status[2];
	uint8_t w25q_buffer[8] = {0,0,0,0,0,0,0,0};

	uint8_t rx[5];
	uint8_t tx_data[] = { 'H', 'E', 'L', 'L', 'O' };

W25Q_FlashReset();
  W25Q_FlashReadJEDEC(jedec_id);
  W25Q_FlashRead_StatusRegister(w25q_status);

  //Programming sequence - Erase sector,program page, read data
  W25Q_FlashSectorErase(W25Q_TEST_ADDR);
  W25Q_FlashPageProgram(W25Q_TEST_ADDR,tx_data,5);
  W25Q_FlashRead_Data(W25Q_TEST_ADDR, rx, 5);

# 16. Testing and Validation

| Test | Expected observation | Meaning |
| --- | --- | --- |
| SPI CS idle | PB9 high before flash operation | Flash is deselected when idle |
| JEDEC read | MISO returns non-0xFF/non-0x00 realistic ID | Wiring/SPI/CS likely working |
| Status read | SR1 BUSY clears when idle | Status path works |
| Erase | Bytes in erased sector read as 0xFF before programming | Sector erase completed |
| Program | Page program accepts HELLO at 0x001000 | Write Enable + program flow works |
| Read-back | rx buffer equals 0x48 0x45 0x4C 0x4C 0x4F | End-to-end erase/program/read path works |
| Shared SPI2 | MCP2515 CS remains high during flash operations | No two slaves drive MISO together |

# 17. Results / Expected Debug Values

Current source does not print W25Q results over UART.
Verify using debugger/watch window:

jedec_id[0..2]     = manufacturer/type/capacity response, not all 0xFF
w25q_status[0]    = SR1 value, BUSY bit should be 0 after operations
tx_data            = 48 45 4C 4C 4F
rx                 = 48 45 4C 4C 4F after read-back

If rx is all 0xFF after program:
  suspect program did not execute, WEL was missing, /CS sequence was wrong,
  or the page/sector was not correctly addressed.

# 18. Problems Faced and Debugging Notes

| Symptom | Likely cause | Debug action |
| --- | --- | --- |
| JEDEC ID all 0xFF | MISO floating, CS not low, wrong wiring, flash not powered | Check VCC/GND, PB9 CS, SCK/MOSI/MISO, and analyzer decode |
| JEDEC ID all 0x00 | MISO stuck low or wrong mode/wiring | Check DO/MISO line and SPI mode |
| Erase never completes | BUSY stuck or command not latched | Check CS high after erase address and add timeout |
| Read-back unchanged | Write Enable missing or WEL cleared | Read SR1 after 0x06 and before program/erase |
| Only part of buffer writes | Page boundary crossed or size capped to 256 | Use page-boundary-safe wrapper |
| Other SPI2 device misbehaves | Two CS lines active or stale selected slave | Force all non-target CS pins high before each transaction |
| Random data after read | Address byte order or dummy transfer issue | Verify 0x03 + A23/A15/A7 then dummy 0xFF reads |

# 19. Embedded Engineering Review

| Finding | Impact | Recommended action |
| --- | --- | --- |
| W25Q_FlashSectorErase appears to leave PB9 CS low after sending the address | Erase instruction may not latch/execute correctly | Drive W25Q_FLASH_CS_HIGH after the third address byte, then poll BUSY |
| W25Q_FlashSectorErase drives CS low before W25Q_FlashWriteEnable | Nested CS sequence is confusing and can distort analyzer traces | Keep WriteEnable as a separate complete transaction before erase |
| W25Q_FlashWaitUntilReady has no timeout | Firmware can hang forever on wiring or command faults | Add timeout and return E_NOT_OK on failure |
| Write Enable is not verified using WEL | Erase/program may silently fail | Read SR1 and verify W25Q_SR1_WEL after 0x06 |
| PageProgram caps size to 256 but does not check page boundary | Writes starting mid-page can cross the page limit | Add page-boundary-safe Write_NoErase wrapper |
| Read-back is stored but not compared or printed | Pass/fail result is invisible without debugger | Compare rx against tx_data and print result over USART2 |
| spi.h has no include guard | Can cause duplicate declarations in larger builds | Add conventional include guard |
| Variant voltage must be confirmed | Wrong FV/FW suffix assumption can damage hardware | Check IC marking and datasheet before wiring |

# 20. Limitations

The current driver is polling-based and blocking.

Only basic commands are implemented: reset, ID/status read, read data, write enable/disable, sector erase, page program.

No timeout/error return exists for BUSY polling.

No page-boundary-safe multi-page write wrapper is implemented yet.

No sector-safe read-modify-erase-program update API is implemented yet.

No UART pass/fail print exists for the flash read-back test.

No screenshots were attached in this turn, so validation images are not embedded.

# 21. Future Improvements

Fix W25Q_FlashSectorErase chip-select termination and verify with a logic analyzer.

Add timeout return values for W25Q_FlashWaitUntilReady.

Add WEL verification after W25Q_FlashWriteEnable.

Add W25Q_FlashWrite_NoErase for page-boundary-safe multi-page writes.

Add W25Q_FlashUpdate_SectorSafe for preserving existing sector data.

Add JEDEC ID validation and UART printing of manufacturer/type/capacity bytes.

Add read-back compare and clear PASS/FAIL logs.

Add block erase and chip erase only after guardrails are in place.

# 22. Conclusion

Conclusion: The attached code demonstrates the essential W25Q64FV bring-up path over STM32 SPI2: command transactions, JEDEC/status reads, erase, page program, and read-back. The next maturity step is to harden chip-select sequencing, timeout/error returns, WEL checks, and page/sector-safe write APIs.

# 23. Appendix: Source Reference

| File / source | Used for |
| --- | --- |
| D:\Embedded Communication Protocols\CAN1_RX_Interrupt\Core\Src\main.c | Finalized firmware file used for source grounding |
| D:\Embedded Communication Protocols\CAN1_RX_Interrupt\Core\Src\spi.c | Finalized firmware file used for source grounding |
| D:\Embedded Communication Protocols\CAN1_RX_Interrupt\Core\Src\usart.c | Finalized firmware file used for source grounding |
| D:\Embedded Communication Protocols\CAN1_RX_Interrupt\Core\Src\w25q64fv.c | Finalized firmware file used for source grounding |
| D:\Embedded Communication Protocols\CAN1_RX_Interrupt\Core\Inc\spi.h | Finalized firmware file used for source grounding |
| D:\Embedded Communication Protocols\CAN1_RX_Interrupt\Core\Inc\usart.h | Finalized firmware file used for source grounding |
| D:\Embedded Communication Protocols\CAN1_RX_Interrupt\Core\Inc\w25q64fv.h | Finalized firmware file used for source grounding |
| https://chatgpt.com/share/6a3cd3b1-539c-83e8-b215-ebb12dc6e2ea | Shared ChatGPT theory source for W25QXX flash rules and driver concepts |
| work/w25q64fv_candidates.txt | Extracted shared-chat candidate text used during generation |

# 24. Appendix: Revision Checklist

| Check | Status |
| --- | --- |
| Shared ChatGPT link fetched | Done |
| W25Q64FV-specific chat content extracted | Done |
| All user-named source/header files read | Done |
| SPI2 system diagram included | Done |
| Memory geometry diagram included | Done |
| Command flow and status rules explained | Done |
| Page-boundary and sector-safe update concepts included | Done |
| Source review corrections documented | Done |
| No invented screenshots | Done |
| Footer name set to Embedded Learning Document | Done |