EMBEDDED PROJECT REPORT

USART Polling-Based Driver Project

STM32F407VG | USART2 | PA2 TX / PA3 RX | 115200 baud | 8N1

| Field | Value |
| --- | --- |
| Document purpose | Long-term learning, project revision, implementation reference |
| Project type | Bare-metal USART polling driver with application-level TX/RX |
| Learning stage | Day 1 foundation before interrupt, circular-buffer, and DMA versions |
| Primary source | Actual project source files |

Preview of the finalized structure: This document follows a Template: objective, problem statement, system overview, requirements, hardware/software setup, architecture, protocol behavior, module-wise implementation, test strategy, results, debugging, limitations, future improvements, and conclusion.

# 1. Project Objective

The objective of this project is to build and understand a polling-based USART driver on the STM32F407VG. The project intentionally keeps the software path simple so that each hardware event is visible: the CPU waits for TXE before writing a byte, waits for TC after the last transmitted frame, and waits for RXNE before reading received data.

Learning outcome: After completing this project, you should be able to explain the difference between the data register and shift register, justify why TXE and TC are separate flags, bring up USART2 using bare-metal register access, and debug a basic UART terminal link.

### Objective Checkpoint

| Check | Learning or validation item | Evidence |
| --- | --- | --- |
| [ ] | I can describe what polling means in a peripheral driver. | Notes / evidence: |
| [ ] | I can explain why this project avoids interrupts and DMA on Day 1. | Notes / evidence: |
| [ ] | I can map the application test to actual USART hardware events. | Notes / evidence: |

# 2. Problem Statement

A new embedded engineer often sees UART as a simple print interface, but reliable serial communication depends on clocking, pin multiplexing, frame format, baud-rate generation, transmit/receive status flags, and correct clearing behavior. The problem solved by this project is to create a minimal but trustworthy USART2 driver that proves both transmit and receive paths without hiding the hardware behind high-level abstractions.

| Problem area | Project response |
| --- | --- |
| Pin routing | Configure PA2 and PA3 as alternate function pins using AF7. |
| Peripheral clocking | Enable GPIOA on AHB1 and USART2 on APB1 before register access. |
| Serial format | Use 8 data bits, no parity, one stop bit for terminal compatibility. |
| TX validation | Send a greeting and a single-byte test using TXE and TC polling. |
| RX validation | Block on RXNE, read DR, and echo the received byte. |

# 3. System Overview

The system contains a host terminal, a USB-UART bridge or board-level serial path, and the STM32F407VG running a bare-metal USART2 driver. The firmware initializes the clock tree, configures GPIOA, enables USART2, sends messages, receives one byte, and echoes it back.

Host terminal
    |
    | 115200 baud, 8N1
    v
USB-UART / board serial path
    |
    | TX/RX cross connection
    v
STM32F407VG USART2
    |-- PA2: TX output
    |-- PA3: RX input
    |-- Driver: polling on TXE, TC, RXNE

| What this project proves | Why it matters |
| --- | --- |
| USART2 can be brought up with bare-metal register access. | You see exactly which clocks, pins, and USART control bits are required. |
| TX and RX are both polled in software. | You observe TXE, TC, and RXNE without interrupt complexity. |
| The code is intentionally blocking. | Blocking behavior makes the hardware events easier to understand. |
| The code path is simple and reproducible. | It becomes a stable base for interrupt and DMA learning. |

# 4. Requirements

## 4.1 Functional Requirements

| ID | Requirement | Acceptance condition |
| --- | --- | --- |
| FR-01 | Initialize USART2 for asynchronous serial communication. | USART2 UE, TE, and RE are set after clocks, pins, baud rate, and frame format are configured. |
| FR-02 | Transmit a null-terminated string. | Characters appear correctly in the terminal at 115200 baud. |
| FR-03 | Transmit one byte and terminate the line. | The byte appears followed by CRLF. |
| FR-04 | Receive one byte using polling. | Firmware blocks until RXNE and then returns the received byte. |
| FR-05 | Echo the received byte. | The terminal shows the same byte sent from the host. |

## 4.2 Non-Functional Requirements

| Area | Requirement |
| --- | --- |
| Readability | Register operations should remain explicit and easy to trace. |
| Learning clarity | No interrupt or DMA path should be introduced in this Day 1 version. |
| Debuggability | The design should expose TXE, TC, and RXNE behavior directly. |
| Portability note | Clock and BRR assumptions must be reviewed if the clock tree changes. |

# 5. Hardware Components

| Item | Selection in this project | Purpose |
| --- | --- | --- |
| MCU | STM32F407VG | Target processor for the polling driver. |
| USART instance | USART2 | Peripheral used for serial communication. |
| TX pin | PA2 | Serial output line from MCU to terminal path. |
| RX pin | PA3 | Serial input line from terminal path to MCU. |
| GPIO mode | Alternate function | Connects PA2/PA3 to USART hardware. |
| AF selection | AF7 | USART2 pin mapping on GPIOA. |
| Frame format | 8N1 | 8 data bits, no parity, 1 stop bit. |
| Baud rate | 115200 | Common terminal/debug baud rate. |
| Clock domain | APB1 | USART2 peripheral clock source domain. |

Hardware caution: Always confirm the board routing. PA2 and PA3 must connect to the terminal path expected by your board or external USB-UART bridge. A correct driver will still look broken if TX/RX are crossed incorrectly, the ground is missing, or the terminal is using a different baud rate.

# 6. Software and Firmware Tools

| Tool / file | Role in this project |
| --- | --- |
| main.c | Application sequence: HAL init, clock config, GPIO init, USART init, TX/RX test. |
| uart.c | Bare-metal USART2 driver implementation. |
| uart.h | USART2 driver API prototypes. |
| main.h | Project-level declarations and generated includes. |
| stm32f4xx_it.c/.h | Default exception handlers and SysTick handler; no USART ISR in polling version. |
| Terminal program | Displays transmitted characters and sends the receive-test byte. |
| Logic analyzer / oscilloscope | Optional evidence tool for verifying idle level, start bit, data bits, and stop bit. |

## 6.1 Source Files Used for This Corrected Edition

| Source file | Verification note |
| --- | --- |
| Core/Src/main.c | Application sequence verified: HAL_Init, SystemClock_Config, MX_GPIO_Init, USART2_Init, greeting TX, byte TX, byte RX, echo. |
| Core/Src/uart.c | USART2_Init and polling helpers copied into this document from the real source. |
| Core/Src/stm32f4xx_it.c | Verified no USART ISR is implemented in Day 1; SysTick calls HAL_IncTick only. |
| Core/Inc/uart.h | Verified exported API prototypes for init, string TX, byte TX, and byte RX. |
| Core/Inc/main.h | Verified generated main header and Error_Handler prototype. |
| Core/Inc/stm32f4xx_it.h | Verified exception-handler prototypes only; no USART-specific handler. |

# 7. System Block Diagram

A useful mental model is to separate the CPU-visible data register from the internal shift register. TXE becomes true when the data register can accept another byte, while TC becomes true only when the final frame has fully left the pin.

CPU write
   |
   v
USART_DR -> TX shift register -> serializer -> PA2/TX pin

PA3/RX pin -> RX shift register -> USART_DR -> CPU read

| Flag | Meaning | Day 1 usage |
| --- | --- | --- |
| TXE | Transmit data register empty. | Wait before writing the next byte to DR. |
| TC | Transmission complete on the wire. | Wait after the final byte before considering TX complete. |
| RXNE | Receive data register not empty. | Wait before reading DR. |
| ORE | Overrun occurred. | Not actively handled in Day 1; important for later versions. |

# 8. Firmware Architecture

The firmware architecture is intentionally flat. The application owns the demonstration sequence; the USART driver owns register-level configuration and byte/string helpers. No queue, ISR, RTOS task, or DMA descriptor is introduced at this stage.

main.c
  HAL_Init()
  SystemClock_Config()
  MX_GPIO_Init()
  USART2_Init()
  USART2_SendString(...)
  USART2_SendByte('A')
  msg = USART2_ReceiveByte()
  USART2_SendByte(msg)

uart.c
  USART2_Init()
  USART2_SendString(char *s)
  USART2_SendByte(char c)
  USART2_ReceiveByte(void)

Architecture decision: Polling is not chosen because it is the most efficient design. It is chosen because it is the clearest first layer for learning the peripheral. Interrupt and DMA versions should reuse this understanding instead of replacing it.

# 9. Communication Protocol Used

USART stands for Universal Synchronous Asynchronous Receiver Transmitter. In this project it is used in asynchronous UART-style mode. The idle line is logic high, each frame begins with a start bit, the receiver samples the data bits according to the configured baud rate, and the frame ends with a stop bit.

| Parameter | Value | Engineering note |
| --- | --- | --- |
| Mode | Asynchronous | No separate clock line is used. |
| Baud rate | 115200 | Both MCU and terminal must match. |
| Frame | 8N1 | Common terminal format: 8 data bits, no parity, one stop bit. |
| BRR | 364 | Matches the chosen clocking scheme in the supplied project. |
| Oversampling | Default STM32 USART behavior | Review if changing CR1 settings or clock assumptions. |

Timing intuition: At 115200 baud with 8N1 framing, one 10-bit frame takes about 86.8 microseconds on the wire. That number helps explain why receive-side latency can create overrun risk in a polling design.

# 10. Module-Wise Implementation

## 10.0 Header API

void USART2_Init(void);
void USART2_SendString(char *s);
void USART2_SendByte(char s);
char USART2_ReceiveByte(void);

The public API in uart.h exposes exactly four functions. This keeps the Day 1 driver small enough to understand while still separating the application code from the USART register-level implementation.

## 10.1 USART2 Initialization

void USART2_Init(void)
{
    /* Enable clocks */
    RCC->AHB1ENR |= RCC_AHB1ENR_GPIOAEN;
    RCC->APB1ENR |= RCC_APB1ENR_USART2EN;


    /* Configure PA2(TX) and PA3(RX) as Alternate Function */

    GPIOA->MODER &= ~(0x3UL << GPIO_MODER_MODER2_Pos);
    GPIOA->MODER &= ~(0x3UL << GPIO_MODER_MODER3_Pos);

    GPIOA->MODER |=  (0x2UL << GPIO_MODER_MODER2_Pos);
    GPIOA->MODER |=  (0x2UL << GPIO_MODER_MODER3_Pos);


    /* Select AF7 for USART2 */

    GPIOA->AFR[0] &= ~(0xFUL << GPIO_AFRL_AFSEL2_Pos);
    GPIOA->AFR[0] &= ~(0xFUL << GPIO_AFRL_AFSEL3_Pos);

    GPIOA->AFR[0] |=  (0x7UL << GPIO_AFRL_AFSEL2_Pos);
    GPIOA->AFR[0] |=  (0x7UL << GPIO_AFRL_AFSEL3_Pos);


    /* Configure baud rate */

    USART2->BRR = 364;


    /* Configure frame format: 8N1 */

    USART2->CR1 &= ~USART_CR1_M;
    USART2->CR1 &= ~USART_CR1_PCE;

    USART2->CR2 &= ~USART_CR2_STOP;


    /* Enable transmitter and receiver */

    USART2->CR1 |= USART_CR1_TE;
    USART2->CR1 |= USART_CR1_RE;


    /* Enable USART */

    USART2->CR1 |= USART_CR1_UE;
}

The initialization sequence must enable the clocks first, configure the GPIO alternate function mapping, program the baud rate register, select the frame format, enable transmitter and receiver paths, and finally enable the USART peripheral.

## 10.2 Transmit String

void USART2_SendString(char *s)
{
	while(*s != '\0')
	{
		while(!(USART2->SR & USART_SR_TXE));
		USART2->DR = *s++;
	}

	//checking for the last character
	while(!(USART2->SR & USART_SR_TC));
}

The string function waits for TXE before each character so the data register is ready. It waits for TC after the final character so the final stop bit has fully left the pin.

## 10.3 Transmit Byte

void USART2_SendByte(char c)
{
	while(!(USART2->SR & USART_SR_TXE));
	USART2->DR = c;

	while(!(USART2->SR & USART_SR_TC));

	USART2_SendString("\r\n");
}

## 10.4 Receive Byte

char USART2_ReceiveByte(void)
{
	while(!(USART2->SR & USART_SR_RXNE));

	return (char)USART2->DR;
}

Implementation warning: The receive function is blocking forever until RXNE becomes true. That is acceptable for Day 1 learning, but production code usually needs timeout, error handling, buffering, or an interrupt/DMA model.

# 11. Main Algorithm and Flowchart

Start
  |
  v
HAL_Init -> SystemClock_Config -> MX_GPIO_Init -> USART2_Init
  |
  v
Send greeting string
  |
  v
Send byte 'A'
  |
  v
Print receive-test prompt
  |
  v
Wait until RXNE == 1
  |
  v
Read DR into msg
  |
  v
Echo msg using USART2_SendByte
  |
  v
End / idle loop

| Step | Expected observation |
| --- | --- |
| USART2_Init() | No terminal output yet, but register configuration becomes valid. |
| Send greeting | Terminal prints USART initialized message. |
| Send 'A' | Terminal displays A and CRLF. |
| Receive byte | Firmware waits until user sends a character. |
| Echo byte | Terminal displays the received character. |

# 12. Code Explanation

The key design pattern is a flag-controlled loop. Each helper function waits for the hardware to advertise readiness, then performs exactly one register action. This makes the code readable and maps directly to the reference manual.

| Code action | Hardware meaning | Reason |
| --- | --- | --- |
| Enable GPIOA clock | GPIOA register block becomes accessible. | Pins cannot be configured safely before their clock is active. |
| Enable USART2 clock | USART2 register block becomes accessible. | BRR, CR1, CR2, SR, and DR need the peripheral clock. |
| Set PA2/PA3 AF mode | GPIO pins are routed away from simple GPIO behavior. | USART cannot drive/read the pins without alternate function routing. |
| Select AF7 | PA2/PA3 connect specifically to USART2. | Alternate mode alone is not enough; the AF number selects the peripheral function. |
| Set BRR | Baud generator is configured. | Receiver and transmitter bit timing must match the terminal. |
| Set TE/RE/UE | TX path, RX path, and peripheral are enabled. | The USART engine must be enabled after configuration. |
| Poll TXE | DR can accept data. | Prevents writing before the data register is ready. |
| Poll TC | Last frame finished on the wire. | Useful at the end of a blocking send operation. |
| Poll RXNE | DR contains received data. | Prevents reading invalid or stale data. |

# 13. Testing and Validation

## 13.1 Terminal Validation

| Test | Action | Expected result | Pass evidence |
| --- | --- | --- | --- |
| TX greeting | Reset or run the firmware. | Initialization message appears in the terminal. | Terminal capture. |
| TX byte | Observe the one-byte TX test. | The character A appears with a line ending. | Terminal capture. |
| RX byte | Send one character from the terminal. | Firmware receives without crashing or skipping. | Echo behavior. |
| Echo | Compare sent and echoed byte. | Echoed byte equals the sent byte. | Terminal capture. |
| Baud sanity | Change terminal baud intentionally, then restore. | Wrong baud produces garbage; correct baud restores readable output. | Observation note. |

## 13.2 Logic Analyzer Expectations

On a scope or logic analyzer, the idle line should sit high. A frame should start with a low start bit, continue through the data bits according to the configured baud rate, and end with a high stop bit.

### Validation Checklist

| Check | Learning or validation item | Evidence |
| --- | --- | --- |
| [ ] | Terminal baud rate is 115200. | Notes / evidence: |
| [ ] | Terminal frame format is 8 data bits, no parity, one stop bit. | Notes / evidence: |
| [ ] | PA2 is routed to the receive side of the host terminal path. | Notes / evidence: |
| [ ] | PA3 is routed to the transmit side of the host terminal path. | Notes / evidence: |
| [ ] | TX line idles high before transmission. | Notes / evidence: |
| [ ] | RXNE becomes true after a byte is sent from the terminal. | Notes / evidence: |

# 14. Results

| Observed behavior | Interpretation |
| --- | --- |
| Greeting prints successfully. | Clock, GPIO alternate function, USART enable, BRR, and TX path are likely correct. |
| Single-byte A test prints successfully. | TXE and TC polling path is operating. |
| Firmware waits during RX test. | Blocking receive behavior is active and expected. |
| Sent byte is echoed back. | RXNE polling, DR read, and transmit echo path are functional. |

Result summary: The project demonstrates a working Day 1 USART2 polling driver. The most important result is not only that text appears in the terminal, but that each visible behavior can be traced to a concrete register flag.

# 15. Problems Faced and Debugging Notes

| Symptom | Likely cause | Debug action |
| --- | --- | --- |
| No terminal output | GPIOA or USART2 clock not enabled; wrong pin path; wrong baud. | Check RCC->AHB1ENR, RCC->APB1ENR, PA2 AF7, terminal settings. |
| Unreadable characters | Baud rate mismatch or wrong peripheral clock assumption. | Recalculate BRR for actual APB1 clock and verify terminal baud. |
| TX works but RX blocks forever | PA3 wiring, terminal send settings, or RX path not enabled. | Check PA3 routing, RE bit, and RXNE in SR. |
| Echo is missing or delayed | Firmware is waiting in a blocking loop or terminal is not sending input. | Confirm the main sequence and terminal transmit behavior. |
| Overrun risk during faster input | Polling code does not read DR quickly enough. | Introduce ORE handling, interrupts, or DMA in later versions. |

## 15.1 Register View Checklist

| Register | Expected state in Day 1 | Quick check |
| --- | --- | --- |
| RCC->AHB1ENR | GPIOAEN set | GPIOA clock is alive. |
| RCC->APB1ENR | USART2EN set | USART2 clock is alive. |
| GPIOA->MODER | PA2 and PA3 alternate function | Pins are not plain GPIO. |
| GPIOA->AFR[0] | AF7 for PA2 and PA3 | USART2 is mapped to the pins. |
| USART2->BRR | 364 in supplied project | Baud generator matches clock assumption. |
| USART2->CR1 | UE, TE, RE set | USART engine, TX, and RX are enabled. |
| USART2->SR | TXE/TC/RXNE change during use | Polling loops are meaningful. |

# 16. Limitations

| Limitation | Why it matters | Next improvement |
| --- | --- | --- |
| Blocking TX and RX | CPU cannot do other work while waiting for flags. | Move to interrupt-driven events. |
| No receive timeout | Firmware can wait forever if no byte arrives. | Add timeout based on SysTick or a timer. |
| No active ORE handling | Fast receive traffic can overrun DR. | Clear and log error flags; use buffering. |
| No receive buffer | Only one byte is handled at a time. | Introduce circular buffer for RX. |
| Hard-coded clock/BRR assumption | Clock changes can break baud accuracy. | Calculate BRR from runtime clock values or constants. |

# 17. Future Improvements

This project is the base layer for a larger USART learning series. Each later stage should preserve the Day 1 mental model while reducing CPU blocking and improving robustness.

| Stage | Upgrade | What changes |
| --- | --- | --- |
| Day 2 | Interrupt USART | NVIC calls the code when TX/RX events occur. |
| Day 3 | RX circular buffer | Received bytes are stored without immediate application handling. |
| Day 4 | TX DMA direct mode | DMA moves bytes to USART without per-byte CPU writes. |
| Day 5 | TX DMA FIFO mode | TX transfer behavior becomes more efficient and configurable. |
| Day 6 | DMA RX | DMA stores received bytes into memory. |
| Day 7 | DMA RX circular mode | Continuous receive stream becomes possible. |
| Day 8 | DMA RX circular + idle detection | Frame boundaries can be detected in stream-style reception. |
| Final | CLI protocol | USART becomes a command interface, not just a byte echo path. |

### Long-Term Learning Tracker

| Check | Learning or validation item | Evidence |
| --- | --- | --- |
| [ ] | I can implement the polling version without looking at the final code. | Notes / evidence: |
| [ ] | I can explain how the interrupt version maps onto the same TXE/RXNE events. | Notes / evidence: |
| [ ] | I can explain why DMA improves repetitive byte movement. | Notes / evidence: |
| [ ] | I can identify when polling is acceptable and when it becomes risky. | Notes / evidence: |

# 18. Conclusion

The USART polling project is a strong first layer because it exposes the conversation between firmware and hardware. The code is small, but the lessons are foundational: clocks must be enabled before register access, GPIO pins must be routed to the peripheral, the baud generator must match the clock tree, and status flags must be interpreted precisely.

Core takeaway: If you remember only one lesson from this project, remember this: USART flags are not abstract software states. They are hardware events that tell the CPU when the peripheral can accept, complete, or provide data.

# Appendix A. Concise Event Matrix

| Flag | Meaning | How Day 1 uses it | Clearing concept |
| --- | --- | --- | --- |
| TXE | Data register empty | Wait before writing next byte. | Writing DR consumes the empty slot. |
| TC | Transmission complete | Wait after final byte. | Frame has fully left the pin. |
| RXNE | Receive data available | Wait before reading DR. | Reading DR consumes the byte. |
| ORE | Receive overrun | Not actively handled in this version. | Handled later in interrupt/DMA docs. |

# Appendix B. One-Page Revision Summary

USART2 is initialized by enabling clocks, selecting AF7 for PA2/PA3, setting BRR, selecting 8N1, and enabling TE, RE, and UE.

Polling TX waits for TXE before each byte and TC after the last byte.

Polling RX waits for RXNE and then reads DR.

The Day 1 design is intentionally blocking so the hardware behavior remains easy to see.

This exact driver becomes the base for interrupt, buffering, and DMA versions later in the series.