EMBEDDED PROJECT REPORT

USART Day 2 Interrupt-Based RX Project

STM32F407VG | USART2 | RXNE interrupt | NVIC | PA2 TX / PA3 RX | 115200 baud | 8N1

| Field | Value |
| --- | --- |
| Document purpose | Long-term learning, implementation reference, and project revision. |
| Project type | Bare-metal USART2 receive interrupt with polling-based transmit helper. |
| Content source | USART_Day2_Interrupt_Based_Expert_Reference_v1.pdf. |
| Code authority | Only the attached finalized source files are used for code snippets. |
| Reasoning correction | The PDF discusses full TXE/TC interrupt design, but this finalized code implements RXNE interrupt reception and polling TX. |

Implementation truth: In this Day 2 code, receive is interrupt-driven. Transmit is still polling through USART2_SendString and USART2_SendByte. That distinction is important for debugging and for planning the next version.

# 1. Project Objective

The objective of Day 2 is to move the receive path from blocking polling to interrupt-driven event handling. Instead of waiting inside USART2_ReceiveByte, the firmware enables RXNEIE and the USART2 IRQ line, lets the peripheral interrupt the CPU when a byte arrives, stores the byte in shared state, and lets the main loop perform the echo.

Learning outcome: After this project, you should be able to explain how RXNE, RXNEIE, NVIC_EnableIRQ, USART2_IRQHandler, rx_data, and rx_flag work together to create an event-driven receive path.

### Objective Checkpoint

| Check | Learning or validation item | Evidence |
| --- | --- | --- |
| [ ] | I can explain how an interrupt differs from polling. | Notes / capture: |
| [ ] | I can identify which exact line enables RXNE interrupt generation. | Notes / capture: |
| [ ] | I can trace one received byte from RX pin to ISR to main-loop echo. | Notes / capture: |

# 2. Problem Statement

Polling is excellent for learning the raw USART flags, but it wastes CPU time because the application must repeatedly check whether RXNE is set. Day 2 solves the first practical limitation of Day 1: receive-side blocking. The CPU can now remain in the main loop and respond only when USART2 reports a received byte.

| Problem area | Day 2 response |
| --- | --- |
| Blocking receive | Enable RXNEIE and USART2_IRQn so received bytes trigger an ISR. |
| ISR-to-main communication | Use shared variables rx_data and rx_flag. |
| Transmit simplicity | Keep TX as polling so only one new concept is introduced at a time. |
| Debug visibility | Echo received bytes from the main loop after the ISR sets the flag. |

# 3. System Overview

The hardware remains close to Day 1: STM32F407VG, USART2, PA2 as TX, PA3 as RX, and a serial terminal or USB-UART path. The main difference is the software event path. A received byte sets RXNE, RXNEIE allows RXNE to request an interrupt, the NVIC dispatches USART2_IRQHandler, and the ISR reads DR.

Terminal / USB-UART
    |
    v
PA3 RX -> USART2 receive shift register -> USART2->DR -> RXNE = 1
    |
    v
RXNEIE + NVIC USART2_IRQn -> USART2_IRQHandler
    |
    v
rx_data = USART2->DR; rx_flag = 1
    |
    v
main loop sees rx_flag and echoes through polling TX

| Project focus | What changes from Day 1 |
| --- | --- |
| RX path | Polling receive is replaced by RXNE interrupt handling. |
| TX path | Transmit remains polling using TXE and TC waits. |
| CPU behavior | The main loop no longer blocks inside USART2_ReceiveByte for the echo demo. |
| Design style | Small ISR plus shared flag, not a full driver state machine yet. |

# 4. Requirements

## 4.1 Functional Requirements

| ID | Requirement | Acceptance condition |
| --- | --- | --- |
| FR-01 | Initialize USART2 for 115200-style 8N1 serial communication. | USART2 is enabled with TE, RE, BRR = 364, and 8N1 frame configuration. |
| FR-02 | Enable receive interrupt generation. | USART2->CR1 has RXNEIE set. |
| FR-03 | Enable the USART2 IRQ line in the NVIC. | NVIC_EnableIRQ(USART2_IRQn) is called. |
| FR-04 | Store received bytes in ISR-shared state. | USART2_IRQHandler reads DR into rx_data and sets rx_flag. |
| FR-05 | Echo received byte from main loop. | When rx_flag is set, main sends rx_data and clears rx_flag. |

## 4.2 Non-Functional Requirements

| Area | Requirement |
| --- | --- |
| ISR length | ISR should remain short and deterministic. |
| Shared state | Variables shared by ISR and main must be treated as volatile state. |
| Learning scope | Keep TX polling to avoid mixing RX interrupt learning with full TX state-machine design. |
| Traceability | All code shown in the document must come from the finalized attached source files. |

# 5. Hardware Components

| Item | Selection in this project | Purpose |
| --- | --- | --- |
| MCU | STM32F407VG / STM32F407VGT6 family | Target processor for the USART2 driver. |
| USART instance | USART2 | Peripheral used for serial communication. |
| TX pin | PA2 AF7 | Polling transmit output. |
| RX pin | PA3 AF7 | Interrupt-driven receive input. |
| Clock domain | APB1 | USART2 peripheral clock domain. |
| External tool | Terminal / USB-UART adapter | Sends and receives test characters. |
| Frame format | 8N1 | Terminal-compatible asynchronous serial framing. |

# 6. Software and Firmware Tools

| Source file | Verified role |
| --- | --- |
| Core/Src/main.c | Declares rx_flag/rx_data, initializes USART2, sends greeting, echoes received bytes from main loop. |
| Core/Src/usart.c | Configures USART2, enables RXNEIE and NVIC, provides polling TX helper functions. |
| Core/Src/stm32f4xx_it.c | Implements USART2_IRQHandler that reads DR and sets rx_flag. |
| Core/Inc/usart.h | Exports USART2 driver prototypes. |
| Core/Inc/stm32f4xx_it.h | Declares external ISR-shared variables and USART2_IRQHandler prototype. |
| Core/Inc/main.h | Provides generated HAL include and Error_Handler prototype. |

Reasoning correction: The PDF is conceptually broader than the finalized code. This document corrects the implemented scope: RXNE interrupt reception is implemented; TXEIE and TCIE transmit interrupts are not implemented in the attached code.

# 7. System Block Diagram

Application main loop
  |
  | if(rx_flag) USART2_SendByte(rx_data)
  v
USART polling TX helper

USART2 RX hardware
  |
  | RXNE event
  v
NVIC USART2_IRQn
  |
  v
USART2_IRQHandler
  |
  | read DR, set flag
  v
Shared state: rx_data, rx_flag

| Signal / event | Meaning | Day 2 handling |
| --- | --- | --- |
| RXNE | Receive data register contains a byte. | Generates IRQ because RXNEIE is enabled. |
| TXE | Transmit data register can accept a byte. | Used inside polling transmit helpers. |
| TC | Final transmitted frame completed on wire. | Used after polling transmit writes. |
| ORE | New byte arrived before old byte was read. | Explained as a risk; not actively handled in finalized ISR. |

# 8. Firmware Architecture

The architecture has three layers: the application loop, the USART register driver, and the interrupt handler. The application no longer calls a blocking receive function in the main demo path. Instead, the ISR captures the byte and the main loop performs the echo.

main.c
  volatile uint8_t rx_flag;
  volatile char rx_data;
  USART2_Init();
  while(1) { if(rx_flag) { USART2_SendByte(rx_data); rx_flag = 0; } }

usart.c
  USART2_Init() enables RXNEIE and USART2_IRQn
  USART2_SendString()/USART2_SendByte() use polling TX

stm32f4xx_it.c
  USART2_IRQHandler() reads USART2->DR and sets rx_flag

Architecture decision: This is a controlled transition document. RX becomes event-driven first; TX remains simple. That makes the interrupt mechanism easy to isolate and debug.

# 9. Communication Protocol Used

The serial protocol remains asynchronous USART/UART-style communication. The configured frame is 8N1: idle high, one start bit, eight data bits, no parity, and one stop bit. The code programs BRR as 364 and uses PA2/PA3 alternate function 7 for USART2.

| Parameter | Value from finalized code | Engineering note |
| --- | --- | --- |
| Clock source | HSI through PLL in main.c | This Day 2 project uses HSI, unlike the earlier Day 1 HSE-based clock tree. |
| PLL | M=8, N=84, P=DIV2, Q=7 | SYSCLK is configured lower than the Day 1 document version. |
| APB1 divider | HCLK / 2 | USART2 is on APB1. |
| BRR | 364 | Matches the current project assumption; revisit if the clock tree changes. |
| Frame | 8N1 | CR1 M/PCE cleared and CR2 STOP cleared. |
| Receive event | RXNE | Interrupt source for this finalized code. |

Timing intuition: At 115200 baud and 8N1 framing, one byte frame takes about 86.8 microseconds. An RX ISR should read DR quickly enough to avoid receive overrun when traffic increases.

# 10. Module-Wise Implementation

## 10.0 Header API from usart.h

void USART2_Init(void);
void USART2_SendString(char *s);
void USART2_SendByte(char s);
char USART2_ReceiveByte(void);


#endif /* INC_USART_H_ */

## 10.1 USART2 Initialization from usart.c

void USART2_Init(void)
{
    /* Enable clocks */
    RCC->AHB1ENR |= RCC_AHB1ENR_GPIOAEN;
    RCC->APB1ENR |= RCC_APB1ENR_USART2EN;


    /* Configure PA2(TX) and PA3(RX) as Alternate Function */

    GPIOA->MODER &= ~(0x3UL << GPIO_MODER_MODER2_Pos);
    GPIOA->MODER &= ~(0x3UL << GPIO_MODER_MODER3_Pos);

    GPIOA->MODER |=  (0x2UL << GPIO_MODER_MODER2_Pos);
    GPIOA->MODER |=  (0x2UL << GPIO_MODER_MODER3_Pos);


    /* Select AF7 for USART2 */

    GPIOA->AFR[0] &= ~(0xFUL << GPIO_AFRL_AFSEL2_Pos);
    GPIOA->AFR[0] &= ~(0xFUL << GPIO_AFRL_AFSEL3_Pos);

    GPIOA->AFR[0] |=  (0x7UL << GPIO_AFRL_AFSEL2_Pos);
    GPIOA->AFR[0] |=  (0x7UL << GPIO_AFRL_AFSEL3_Pos);


    /* Configure baud rate */

    USART2->BRR = 364;


    /* Configure frame format: 8N1 */

    USART2->CR1 &= ~USART_CR1_M;
    USART2->CR1 &= ~USART_CR1_PCE;

    USART2->CR2 &= ~USART_CR2_STOP;


    /* Enable transmitter and receiver */

    USART2->CR1 |= USART_CR1_TE;
    USART2->CR1 |= USART_CR1_RE;


    /* Enable USART */

    USART2->CR1 |= USART_CR1_UE;

    /* Enable RX interrupt and USART2 interrupt in NVIC*/
    USART2->CR1 |= USART_CR1_RXNEIE;
    NVIC_EnableIRQ(USART2_IRQn);
}

The final two initialization actions are the Day 2 change: set RXNEIE in USART2->CR1 and enable USART2_IRQn in the NVIC. The current code enables UE before RXNEIE/NVIC, then enables interrupt reception.

## 10.2 Polling Transmit Helpers from usart.c

void USART2_SendString(char *s)
{
	while(*s != '\0')
	{
		while(!(USART2->SR & USART_SR_TXE));
		USART2->DR = *s++;
	}

	//checking for the last character
	while(!(USART2->SR & USART_SR_TC));
}

void USART2_SendByte(char c)
{
	while(!(USART2->SR & USART_SR_TXE));
	USART2->DR = c;

	while(!(USART2->SR & USART_SR_TC));
}

Transmit remains polling in the finalized code. SendString waits for TXE before each character and TC after the final byte. SendByte waits for TXE, writes one byte, and waits for TC.

## 10.3 Blocking Receive Helper Still Present

char USART2_ReceiveByte(void)
{
	while(!(USART2->SR & USART_SR_RXNE));

	return (char)USART2->DR;
}

The blocking receive function is still present in the driver file, but the Day 2 main-loop demo does not use it for echo. The interrupt path handles incoming bytes instead.

## 10.4 USART2 Interrupt Handler from stm32f4xx_it.c

void USART2_IRQHandler(void)
{
	if(USART2->SR & USART_SR_RXNE)
	{
		rx_data = USART2->DR;
		rx_flag = 1;
	}
}

/* USER CODE END 1 */

The ISR checks RXNE, reads USART2->DR into rx_data, and sets rx_flag. Reading DR consumes the received byte and clears RXNE on STM32F4 USART behavior.

## 10.5 ISR Shared Declarations

main.c:
volatile uint8_t rx_flag;
volatile char rx_data;

stm32f4xx_it.h:
extern volatile uint8_t rx_flag;
extern char rx_data;

Recommended correction: For consistency, rx_data should ideally be declared volatile in the extern declaration too: extern volatile char rx_data;. The actual code is documented as-is, but this is the correction I would make for ISR-shared state hygiene.

# 11. Main Algorithm and Flowchart

Start
  |
  v
HAL_Init -> SystemClock_Config -> MX_GPIO_Init -> USART2_Init
  |
  v
Send "USART Initialized...!!"
  |
  v
Main loop
  |
  |-- if rx_flag == 0: continue normal loop
  |
  |-- if rx_flag == 1: USART2_SendByte(rx_data); rx_flag = 0

Interrupt path
  Byte arrives -> RXNE -> USART2_IRQHandler -> rx_data = DR; rx_flag = 1

| Step | Expected observation |
| --- | --- |
| Startup | Initialization message appears in terminal. |
| Byte sent from terminal | USART2_IRQHandler runs because RXNEIE and NVIC IRQ are enabled. |
| ISR executes | rx_data stores received byte and rx_flag becomes 1. |
| Main loop executes | rx_flag is detected, byte is echoed, rx_flag resets to 0. |

# 12. Code Explanation

| Code action | Hardware / software meaning | Why it matters |
| --- | --- | --- |
| USART2->CR1 ¦= USART_CR1_RXNEIE | Allows RXNE to request an interrupt. | Without this, RXNE can set but no USART IRQ is generated. |
| NVIC_EnableIRQ(USART2_IRQn) | Enables the Cortex-M interrupt line for USART2. | Without this, the peripheral request cannot reach the CPU handler. |
| USART2_IRQHandler | Named ISR called by vector table for USART2 IRQ. | Handler name must match the startup vector. |
| if(USART2->SR & USART_SR_RXNE) | Confirms received-byte event. | Prevents reading DR for unrelated interrupts. |
| rx_data = USART2->DR | Reads the received byte and clears RXNE. | Consumes hardware receive data. |
| rx_flag = 1 | Signals main loop that a byte is ready. | Keeps ISR short by moving echo work outside the ISR. |
| USART2_SendByte(rx_data) | Echoes the byte using polling TX. | Demonstrates RX interrupt success without TX interrupt complexity. |

# 13. Testing and Validation

| Test | Action | Expected result | Evidence |
| --- | --- | --- | --- |
| Startup TX | Run or reset firmware. | Terminal prints initialization message. | Terminal capture. |
| RX interrupt | Send one character from terminal. | ISR stores byte and sets rx_flag. | Debugger watch or echo result. |
| Echo path | Observe terminal after sending byte. | Same byte is echoed back. | Terminal capture. |
| Flag reset | Send multiple separate characters. | Each character echoes once. | Watch rx_flag return to 0. |
| Baud sanity | Verify terminal settings. | Readable output at configured baud and 8N1. | Terminal settings screenshot. |

### Validation Checklist

| Check | Learning or validation item | Evidence |
| --- | --- | --- |
| [ ] | USART2->CR1 contains UE, TE, RE, and RXNEIE after init. | Notes / capture: |
| [ ] | NVIC has USART2 IRQ enabled. | Notes / capture: |
| [ ] | USART2_IRQHandler symbol exists and matches the startup handler name. | Notes / capture: |
| [ ] | Sending a byte from the terminal changes rx_flag to 1. | Notes / capture: |
| [ ] | Reading DR in the ISR clears RXNE. | Notes / capture: |
| [ ] | Main loop clears rx_flag after echoing rx_data. | Notes / capture: |

# 14. Results

| Observed / expected behavior | Interpretation |
| --- | --- |
| Initialization message prints. | TX polling helpers, GPIO AF7, USART2 enable, BRR, and terminal settings are likely correct. |
| A received character is echoed. | RXNE interrupt path, ISR shared variables, and main-loop handoff are working. |
| The firmware does not block waiting for receive in main. | Day 2 objective is achieved for the RX path. |
| TX does not use TXEIE/TCIE. | This is expected in the finalized code; full non-blocking TX remains a future step. |

Result summary: Day 2 successfully changes reception from active waiting to event-driven receive notification while keeping transmit deliberately simple.

# 15. Problems Faced and Debugging Notes

| Symptom | Likely cause | Debug action |
| --- | --- | --- |
| Initialization text does not print | TX path, baud, GPIO AF7, or clock issue. | Check PA2 AF7, BRR, UE/TE, and terminal baud. |
| No echo when byte is sent | RXNEIE not set, NVIC IRQ not enabled, wrong handler name, or PA3 routing issue. | Check CR1 RXNEIE, NVIC enable, USART2_IRQHandler symbol, and RX pin path. |
| rx_flag never changes | ISR is not running or RXNE never sets. | Set breakpoint in USART2_IRQHandler and inspect USART2->SR. |
| Echo repeats unexpectedly | rx_flag not cleared or main-loop logic modified. | Verify rx_flag = 0 after USART2_SendByte. |
| Bytes missing under fast input | Single-byte flag design is too small for bursts. | Move to RX circular buffer in the next document. |
| Compiler/shared-state confusion | ISR-shared declaration mismatch. | Use volatile consistently for rx_flag and rx_data declarations. |

## 15.1 Register View Checklist

| Register / symbol | Expected state | Quick check |
| --- | --- | --- |
| RCC->AHB1ENR | GPIOAEN set | GPIOA clock is active. |
| RCC->APB1ENR | USART2EN set | USART2 clock is active. |
| GPIOA->MODER | PA2 and PA3 alternate function | Pins are not plain GPIO. |
| GPIOA->AFR[0] | AF7 for PA2 and PA3 | USART2 maps to pins. |
| USART2->BRR | 364 | Baud register matches code. |
| USART2->CR1 | UE, TE, RE, RXNEIE set | USART and RX interrupt are enabled. |
| USART2->SR | RXNE sets when byte arrives | Receive event reaches peripheral. |
| USART2_IRQHandler | Defined in stm32f4xx_it.c | Vector can dispatch to your handler. |

# 16. Limitations

| Limitation | Why it matters | Next improvement |
| --- | --- | --- |
| Single-byte handoff | A second byte can overwrite rx_data before main consumes it. | Use a circular receive buffer. |
| No active ORE handling | Data loss is possible if DR is not read in time. | Add error handling and SR/DR clear sequence. |
| TX remains polling | Main loop still blocks during transmit. | Add TXEIE/TCIE state-machine transmit later. |
| No callback abstraction | Application logic is tied to global flag behavior. | Add callback or buffer API. |
| Volatile mismatch for rx_data extern | Shared ISR variable declaration is not fully consistent. | Declare extern volatile char rx_data in the interrupt header. |

# 17. Future Improvements

The PDF correctly points toward a fuller interrupt-driven driver, but the finalized code is an intermediate step. The next high-value improvement is an RX circular buffer. After that, non-blocking TX using TXEIE and TCIE can be introduced cleanly.

| Stage | Upgrade | What changes |
| --- | --- | --- |
| Day 3 | RX circular buffer | ISR pushes received bytes into a buffer instead of overwriting one byte. |
| Day 4 | Interrupt-driven TX | TXEIE feeds bytes from a transmit buffer; TCIE marks final completion. |
| Day 5 | Error handling | ORE, FE, NE, and PE are handled deterministically. |
| Day 6 | DMA TX/RX | DMA moves bytes between memory and USART with less CPU work. |
| Later | Idle detection + CLI parser | Frame boundaries and commands become easier to process. |

### Long-Term Learning Tracker

| Check | Learning or validation item | Evidence |
| --- | --- | --- |
| [ ] | I can implement RXNE interrupt echo without looking at the final code. | Notes / capture: |
| [ ] | I can explain why the ISR only stores data and sets a flag. | Notes / capture: |
| [ ] | I can explain why a single rx_data variable is not enough for burst traffic. | Notes / capture: |
| [ ] | I can describe how TXEIE/TCIE would improve transmit later. | Notes / capture: |

# 18. Conclusion

Day 2 is the first event-driven version of the USART learning path. The receive side now depends on hardware notification instead of software waiting, while the transmit side remains deliberately simple. This is a useful and realistic learning step because it isolates the interrupt mechanism before adding buffers, non-blocking transmit, DMA, and protocol parsing.

Core takeaway: RXNEIE plus NVIC_EnableIRQ does not receive data by itself. The complete path is RXNE flag, enabled interrupt request, vector dispatch, ISR DR read, shared-state update, and main-loop processing.

# Appendix A. Actual Source Evidence

| File | Evidence included |
| --- | --- |
| usart.c | USART2_Init, polling TX helpers, blocking receive helper. |
| stm32f4xx_it.c | USART2_IRQHandler implementation. |
| main.c | rx_flag/rx_data state and main-loop echo behavior. |
| usart.h | Driver API declarations. |
| stm32f4xx_it.h | Extern shared-state declarations and handler prototype. |

# Appendix B. One-Page Revision Summary

USART2 is initialized by enabling GPIOA and USART2 clocks, selecting AF7 on PA2/PA3, setting BRR to 364, selecting 8N1, and enabling TE, RE, and UE.

Day 2 adds receive interrupt behavior by setting RXNEIE and enabling USART2_IRQn in the NVIC.

The ISR reads USART2->DR into rx_data and sets rx_flag.

The main loop echoes rx_data with USART2_SendByte when rx_flag is set, then clears rx_flag.

This version does not implement interrupt-driven TXE/TC transmit or receive buffering yet.