EMBEDDED PROJECT REPORT

USART Day 3 RX Circular Buffer Project

STM32F407VG | USART2 | RXNE ISR | 64-byte ring buffer | PA2 TX / PA3 RX | 115200 baud | 8N1

| Field | Value |
| --- | --- |
| Document purpose | Long-term learning, implementation reference, and circular-buffer revision guide. |
| Project type | Bare-metal USART2 interrupt RX circular buffer with polling TX echo. |
| Content source | Source files and PDF template |
| Code authority | Only the attached finalized source files are used for code snippets. |
| Core concept | ISR is producer, main loop is consumer, head and tail decouple receive timing from processing timing. |

Implementation truth: This finalized Day 3 code uses global volatile buffer state: rx_buffer[64], rx_head, rx_tail, rx_overflow, and next_head. The PDF's struct-based model is useful conceptually, but the code shown here follows your attached source exactly.

# 1. Project Objective

The objective of Day 3 is to improve the Day 2 receive interrupt design by replacing single-byte storage with a circular buffer. The USART interrupt handler stores each received byte into RAM as quickly as possible, while the main loop reads bytes later using USART2_ReadByte.

Learning outcome: After this project, you should be able to explain why rx_head is the producer index, why rx_tail is the consumer index, how empty/full conditions are detected, why one buffer slot is intentionally left unused, and how overflow is signaled.

### Objective Checkpoint

| Check | Learning or validation item | Evidence |
| --- | --- | --- |
| [ ] | I can describe the circular buffer as a producer-consumer system. | Notes / capture: |
| [ ] | I can explain what rx_head and rx_tail represent. | Notes / capture: |
| [ ] | I can trace one received byte from USART2->DR into rx_buffer and back out through USART2_ReadByte. | Notes / capture: |

# 2. Problem Statement

Day 2 solved blocking receive by using an RXNE interrupt, but it still had a single-byte handoff problem. If a new byte arrived before the main loop consumed the previous byte, the application could lose data. Day 3 solves this by adding a ring buffer so short bursts can be absorbed even when the main loop is temporarily busy.

| Problem area | Day 3 response |
| --- | --- |
| Single-byte overwrite risk | Use rx_buffer with head and tail indexes instead of one rx_data variable. |
| Burst traffic | Store multiple bytes before the main loop processes them. |
| Main-loop timing variation | Application reads from tail when it has CPU time. |
| Buffer full condition | Detect next_head == rx_tail and set rx_overflow. |

# 3. System Overview

USART2 still receives bytes through PA3. RXNE causes USART2_IRQHandler to run. The ISR calculates the next head position, checks for available space, stores the byte at rx_buffer[rx_head], and advances rx_head. The main loop checks whether rx_head differs from rx_tail, reads one byte with USART2_ReadByte, and echoes that byte using polling transmit.

RX pin -> USART2 DR -> RXNE interrupt
    |
    v
USART2_IRQHandler producer
    |-- next_head = (rx_head + 1) % RX_BUFFER_SIZE
    |-- if next_head != rx_tail: store byte, advance rx_head
    |-- else: read DR and set rx_overflow
    |
    v
rx_buffer[64]
    |
    v
main loop consumer -> USART2_ReadByte -> USART2_SendByte

| Project focus | What improves from Day 2 |
| --- | --- |
| RX storage | Single rx_data becomes rx_buffer[RX_BUFFER_SIZE]. |
| ISR role | ISR stores bytes instead of only raising a one-byte flag. |
| Main role | Main loop consumes buffered bytes one at a time. |
| Overflow policy | Drop newest byte when buffer is full and raise rx_overflow. |

# 4. Requirements

## 4.1 Functional Requirements

| ID | Requirement | Acceptance condition |
| --- | --- | --- |
| FR-01 | Initialize USART2 with RXNE interrupt enabled. | USART2 CR1 has UE, TE, RE, and RXNEIE set; USART2_IRQn is enabled. |
| FR-02 | Create a 64-byte circular receive buffer. | RX_BUFFER_SIZE is 64 and rx_buffer is declared as volatile uint8_t array. |
| FR-03 | Store received bytes in ISR context. | USART2_IRQHandler writes received DR value into rx_buffer at rx_head. |
| FR-04 | Prevent unread data overwrite. | ISR refuses write when next_head equals rx_tail and sets rx_overflow. |
| FR-05 | Read buffered bytes from main context. | USART2_ReadByte blocks while empty, reads rx_buffer[rx_tail], and advances rx_tail. |

## 4.2 Non-Functional Requirements

| Area | Requirement |
| --- | --- |
| ISR discipline | No parsing, printing, or long loops inside USART2_IRQHandler. |
| Shared state | rx_head, rx_tail, rx_buffer, and rx_overflow are volatile because ISR and main share them. |
| Overflow visibility | Overflow should be explicit rather than silent. |
| Learning scope | TX remains polling; RX buffering is the primary Day 3 concept. |

# 5. Hardware Components

| Item | Selection in this project | Purpose |
| --- | --- | --- |
| MCU | STM32F407VG / STM32F407VGT6 family | Target processor for USART2 receive buffering. |
| USART instance | USART2 | Peripheral used for serial communication. |
| TX pin | PA2 AF7 | Polling transmit echo output. |
| RX pin | PA3 AF7 | Interrupt-driven receive input. |
| Clock domain | APB1 | USART2 peripheral clock domain. |
| External tool | Terminal / USB-UART adapter | Sends byte bursts and observes echo behavior. |
| Frame format | 8N1 | Terminal-compatible asynchronous serial framing. |

# 6. Software and Firmware Tools

| Source file | Verified role |
| --- | --- |
| Core/Src/main.c | Declares rx_buffer, rx_head, rx_tail, rx_overflow; reads and echoes buffered bytes. |
| Core/Src/usart.c | Configures USART2, enables RXNEIE/NVIC, provides polling TX and USART2_ReadByte. |
| Core/Src/stm32f4xx_it.c | Implements circular-buffer write logic in USART2_IRQHandler. |
| Core/Inc/usart.h | Defines RX_BUFFER_SIZE as 64 and declares USART APIs. |
| Core/Inc/stm32f4xx_it.h | Declares USART2_IRQHandler prototype. |
| Core/Inc/main.h | Provides generated HAL include and Error_Handler prototype. |

Reasoning correction: The PDF recommends reading SR first and handling ORE explicitly. Your finalized ISR checks RXNE and reads DR, but it does not separately handle USART_SR_ORE. I document the code as-is and call this out as the next robustness correction.

# 7. System Block Diagram

Producer side
  USART2_IRQHandler
    reads USART2->DR
    writes rx_buffer[rx_head]
    advances rx_head

Shared ring buffer
  rx_buffer[0..63]
  rx_head = next write position
  rx_tail = next read position

Consumer side
  main loop checks rx_head != rx_tail
  USART2_ReadByte reads rx_buffer[rx_tail]
  advances rx_tail
  USART2_SendByte echoes byte

| Variable | Meaning | Updated by |
| --- | --- | --- |
| rx_buffer[64] | Storage for received bytes. | ISR writes; main reads. |
| rx_head | Next write position. | USART2_IRQHandler. |
| rx_tail | Next read position. | USART2_ReadByte / main context. |
| next_head | Candidate next write index. | USART2_IRQHandler. |
| rx_overflow | Buffer-full event flag. | ISR sets; main checks. |
| rx_flag | Set when ISR stores a byte. | ISR sets; current main loop does not use it for flow control. |

# 8. Firmware Architecture

Day 3 uses a producer-consumer architecture. The USART interrupt is the producer because it receives bytes from hardware and places them into the buffer. The main loop is the consumer because it removes bytes from the buffer and decides what to do with them.

main.c
  volatile uint8_t rx_buffer[RX_BUFFER_SIZE];
  volatile uint16_t rx_head = 0;
  volatile uint16_t rx_tail = 0;
  volatile uint8_t rx_overflow = 0;

stm32f4xx_it.c
  USART2_IRQHandler() pushes received bytes into rx_buffer

usart.c
  USART2_ReadByte() pops bytes from rx_buffer

Architecture decision: The ISR and main loop each own one index most of the time: ISR owns rx_head, and main owns rx_tail. That separation is what makes a simple interrupt ring buffer practical.

# 9. Communication Protocol Used

The serial protocol is still asynchronous USART/UART-style communication with 8N1 framing. The key Day 3 change is not the wire protocol; it is the software receive model behind RXNE.

| Parameter | Value from finalized code | Engineering note |
| --- | --- | --- |
| Clock source | HSI through PLL in main.c | Same clock style as the Day 2 interrupt project. |
| PLL | M=8, N=84, P=DIV2, Q=7 | SYSCLK is configured by HAL clock setup. |
| APB1 divider | HCLK / 2 | USART2 belongs to APB1. |
| BRR | 364 | Current project baud-register value. |
| Frame | 8N1 | M/PCE cleared and STOP cleared. |
| Receive event | RXNE | Interrupt source that feeds the circular buffer. |

Capacity intuition: At 115200 baud with 8N1 framing, roughly 115 bytes can arrive in 10 ms. A 64-byte buffer can absorb a small burst, but it cannot cover a long application stall at that rate.

# 10. Module-Wise Implementation

## 10.0 Header API and Buffer Size from usart.h

#define RX_BUFFER_SIZE 64

void USART2_Init(void);
void USART2_SendString(char *s);
void USART2_SendByte(char s);
char USART2_ReceiveByte(void);
uint8_t USART2_ReadByte(void);


#endif /* INC_USART_H_ */

## 10.1 Global Buffer State from main.c

volatile uint8_t rx_flag;

//For circular buffer
volatile uint8_t rx_buffer[RX_BUFFER_SIZE];
volatile uint16_t rx_head = 0;
volatile uint16_t rx_tail = 0;
volatile uint8_t rx_overflow = 0;

## 10.2 USART2 Initialization from usart.c

void USART2_Init(void)
{
    /* Enable clocks */
    RCC->AHB1ENR |= RCC_AHB1ENR_GPIOAEN;
    RCC->APB1ENR |= RCC_APB1ENR_USART2EN;


    /* Configure PA2(TX) and PA3(RX) as Alternate Function */

    GPIOA->MODER &= ~(0x3UL << GPIO_MODER_MODER2_Pos);
    GPIOA->MODER &= ~(0x3UL << GPIO_MODER_MODER3_Pos);

    GPIOA->MODER |=  (0x2UL << GPIO_MODER_MODER2_Pos);
    GPIOA->MODER |=  (0x2UL << GPIO_MODER_MODER3_Pos);


    /* Select AF7 for USART2 */

    GPIOA->AFR[0] &= ~(0xFUL << GPIO_AFRL_AFSEL2_Pos);
    GPIOA->AFR[0] &= ~(0xFUL << GPIO_AFRL_AFSEL3_Pos);

    GPIOA->AFR[0] |=  (0x7UL << GPIO_AFRL_AFSEL2_Pos);
    GPIOA->AFR[0] |=  (0x7UL << GPIO_AFRL_AFSEL3_Pos);


    /* Configure baud rate */

    USART2->BRR = 364;


    /* Configure frame format: 8N1 */

    USART2->CR1 &= ~USART_CR1_M;
    USART2->CR1 &= ~USART_CR1_PCE;

    USART2->CR2 &= ~USART_CR2_STOP;


    /* Enable transmitter and receiver */

    USART2->CR1 |= USART_CR1_TE;
    USART2->CR1 |= USART_CR1_RE;


    /* Enable USART */

    USART2->CR1 |= USART_CR1_UE;

    /* Enable RX interrupt and USART2 interrupt in NVIC*/
    USART2->CR1 |= USART_CR1_RXNEIE;
    NVIC_EnableIRQ(USART2_IRQn);
}

## 10.3 Circular Buffer Read Function from usart.c

uint8_t USART2_ReadByte(void)
{
	uint8_t data;

	// block while buffer is empty
	while(rx_head == rx_tail);

	//read with tail var
	data = rx_buffer[rx_tail];

	//incrementing buffer index
	rx_tail++;

	if(rx_tail >= RX_BUFFER_SIZE)
	{
		// making it circular
		rx_tail = 0;
	}

	return data;
}

USART2_ReadByte is the consumer-side pop operation. It waits while the buffer is empty, reads the byte at rx_tail, advances rx_tail, wraps rx_tail to zero if it reaches RX_BUFFER_SIZE, and returns the byte.

## 10.4 USART2 Interrupt Handler from stm32f4xx_it.c

void USART2_IRQHandler(void)
{
	// checking for data in register
	if(USART2->SR & USART_SR_RXNE)
	{
		// for wrapping condition, head reaching tail in circular buffer
		next_head = (rx_head + 1) % RX_BUFFER_SIZE;

		if(next_head != rx_tail)
		{
			rx_buffer[rx_head] = USART2->DR;
			rx_flag = 1;

			rx_head = next_head;

			if(rx_head >= RX_BUFFER_SIZE)
			{
				rx_head = 0;
			}
		}
		else
		{
		    volatile uint8_t dummy;

		    dummy = USART2->DR;
		    rx_overflow = 1;
		}
	}
}

/* USER CODE END 1 */

The ISR is the producer-side push operation. It calculates next_head, checks whether that would collide with rx_tail, stores the new byte when space exists, and sets rx_overflow when the buffer is full.

# 11. Circular Buffer Logic Deep Dive

This is the most important Day 3 concept. A circular buffer is a fixed-size array treated as a loop. The write index moves forward as bytes arrive. The read index moves forward as the application consumes bytes. When either index reaches the end, it wraps back to zero.

| Condition | Expression in this project | Meaning |
| --- | --- | --- |
| Empty | rx_head == rx_tail | No unread bytes are available. |
| Data available | rx_head != rx_tail | At least one unread byte is present. |
| Candidate write slot | next_head = (rx_head + 1) % RX_BUFFER_SIZE | Where head would move after storing the new byte. |
| Full | next_head == rx_tail | Writing would overwrite unread data. |
| Usable capacity | RX_BUFFER_SIZE - 1 | One slot is left unused so empty and full are distinguishable. |

## 11.1 Producer Logic: ISR Push

next_head = (rx_head + 1) % RX_BUFFER_SIZE;

if(next_head != rx_tail)
{
    rx_buffer[rx_head] = USART2->DR;
    rx_flag = 1;
    rx_head = next_head;
}
else
{
    dummy = USART2->DR;
    rx_overflow = 1;
}

The ISR must read DR to consume the hardware byte. If there is room, the byte is stored at the current head and head advances to next_head. If there is no room, the byte is read into a dummy variable so RXNE is cleared, and rx_overflow records that data was dropped.

## 11.2 Consumer Logic: Main Pop

while(rx_head == rx_tail);

data = rx_buffer[rx_tail];
rx_tail++;

if(rx_tail >= RX_BUFFER_SIZE)
{
    rx_tail = 0;
}

return data;

The consumer waits until head and tail differ. It then reads from the current tail, advances tail, wraps tail when needed, and returns the byte to the application.

## 11.3 Example Walkthrough

| Step | rx_head | rx_tail | Buffer event |
| --- | --- | --- | --- |
| Initial empty | 0 | 0 | head equals tail, so no data is available. |
| Byte A arrives | 1 | 0 | A stored at index 0; head advances to 1. |
| Byte B arrives | 2 | 0 | B stored at index 1; two bytes are queued. |
| Main reads | 2 | 1 | A is read from index 0; tail advances to 1. |
| Main reads again | 2 | 2 | B is read from index 1; buffer becomes empty. |
| Head near end | 63 | 10 | Next received byte writes index 63, then head wraps to 0. |

Key mental model: Head points to where the next byte will be written. Tail points to where the next byte will be read. The data is the distance between tail and head, moving around the array in a circle.

# 12. Main Algorithm and Flowchart

Start
  |
  v
HAL_Init -> SystemClock_Config -> MX_GPIO_Init -> USART2_Init
  |
  v
Send initialization message
  |
  v
Main loop
  |
  |-- if rx_head == rx_tail: no buffered data
  |
  |-- if rx_head != rx_tail: msg = USART2_ReadByte(); USART2_SendByte(msg)
  |
  |-- if rx_overflow == 1: Error_Handler()

Interrupt path
  Byte arrives -> RXNE -> USART2_IRQHandler -> push byte or set overflow

| Step | Expected observation |
| --- | --- |
| Startup | Initialization message appears in terminal. |
| Single byte | Byte is stored by ISR and echoed by main loop. |
| Burst input | Several bytes can be stored before main loop catches up. |
| Overflow | If buffer fills, rx_overflow becomes 1 and main eventually enters Error_Handler. |

# 13. Code Explanation

| Code action | Meaning | Why it matters |
| --- | --- | --- |
| #define RX_BUFFER_SIZE 64 | Creates a fixed-size receive queue. | Defines memory cost and burst capacity. |
| rx_head | Next ISR write index. | Tracks where the producer will store data. |
| rx_tail | Next application read index. | Tracks where the consumer will read data. |
| next_head = (rx_head + 1) % RX_BUFFER_SIZE | Computes wrapped next write position. | Prevents running past array end. |
| next_head != rx_tail | Space is available. | Prevents unread data overwrite. |
| rx_buffer[rx_head] = USART2->DR | Stores received byte and clears RXNE. | Moves byte from hardware into RAM. |
| rx_overflow = 1 | Records full-buffer condition. | Makes data loss visible. |
| USART2_ReadByte | Pops one byte from tail. | Consumer-side API for application logic. |

# 14. Testing and Validation

| Test | Action | Expected result | Evidence |
| --- | --- | --- | --- |
| Startup TX | Run or reset firmware. | Terminal prints initialization message. | Terminal capture. |
| Single byte echo | Send one character. | Character is echoed back. | Terminal capture. |
| Small burst | Paste a short word. | Characters echo in order. | Terminal capture. |
| Wrap behavior | Send more than 64 total bytes over time while consuming. | Indexes wrap without crashing. | Debugger watch. |
| Overflow behavior | Send a burst faster than processing can consume. | rx_overflow becomes 1. | Debugger watch / Error_Handler breakpoint. |

### Validation Checklist

| Check | Learning or validation item | Evidence |
| --- | --- | --- |
| [ ] | RX_BUFFER_SIZE is 64 in usart.h. | Notes / capture: |
| [ ] | rx_head and rx_tail start at 0. | Notes / capture: |
| [ ] | USART2_IRQHandler stores bytes into rx_buffer. | Notes / capture: |
| [ ] | next_head is compared against rx_tail before writing. | Notes / capture: |
| [ ] | USART2_ReadByte advances rx_tail and wraps to 0. | Notes / capture: |
| [ ] | A pasted terminal burst echoes in the same order. | Notes / capture: |

# 15. Results

| Observed / expected behavior | Interpretation |
| --- | --- |
| Initialization message prints. | GPIO, USART2 setup, BRR, and polling TX are working. |
| Single characters echo correctly. | RXNE interrupt and consumer read path are working. |
| Short bursts echo in order. | Circular buffer decouples receive ISR timing from main-loop processing. |
| Overflow is flagged when full. | The code drops the newest byte instead of overwriting unread data. |

Result summary: Day 3 turns RX from a single-byte notification into a small queued receive system. This is the foundation for command parsing, line editing, and DMA receive designs.

# 16. Problems Faced and Debugging Notes

| Symptom | Likely cause | Debug action |
| --- | --- | --- |
| Bytes disappear | Buffer overflow or slow main-loop consumer. | Watch rx_overflow, rx_head, and rx_tail. |
| Echo order is wrong | Head/tail update bug or accidental overwrite. | Trace rx_buffer indexes during a known burst. |
| RXNE never clears | DR is not read or ISR not entered. | Set breakpoint in USART2_IRQHandler. |
| No IRQ entry | RXNEIE or NVIC USART2 IRQ disabled. | Check USART2->CR1 and NVIC state. |
| HardFault/Error_Handler | rx_overflow reached 1 and main called Error_Handler. | Increase buffer size or consume faster. |
| Repeated ORE | Hardware DR was not drained fast enough. | Add explicit ORE handling and keep ISR short. |

## 16.1 Register and Buffer Checklist

| Item | Expected state | Quick check |
| --- | --- | --- |
| USART2->CR1 | UE, TE, RE, RXNEIE set | USART and RX interrupt are enabled. |
| USART2->SR | RXNE changes when byte arrives | Receive hardware event occurs. |
| rx_head | Moves when ISR stores bytes | Producer index works. |
| rx_tail | Moves when USART2_ReadByte runs | Consumer index works. |
| rx_buffer | Contains terminal bytes in order | RAM storage works. |
| rx_overflow | 0 during normal tests | No buffer-full loss occurred. |

# 17. Limitations

| Limitation | Why it matters | Next improvement |
| --- | --- | --- |
| Usable capacity is 63 bytes | One slot is reserved to distinguish full from empty. | Document capacity clearly or add a count field. |
| USART2_ReadByte blocks when empty | Useful for simple demos, but risky in production. | Use non-blocking read that returns status. |
| rx_flag is set but not used by main | It is currently redundant for buffer flow. | Remove it or use it consistently. |
| RX_BUFFER_SIZE duplicated in stm32f4xx_it.c | Macro can drift from usart.h. | Include usart.h or centralize the definition. |
| No explicit ORE handling | Hardware overrun can still occur. | Check and clear ORE with the SR/DR sequence. |
| Overflow enters Error_Handler | Useful for learning, harsh for real products. | Count overflow and recover gracefully. |

# 18. Future Improvements

The next natural step is turning this byte queue into a line or packet parser. Once bytes are buffered, the application can search for delimiters, build commands, validate packets, and eventually move reception into DMA with idle-line detection.

| Stage | Upgrade | What changes |
| --- | --- | --- |
| Day 4 | Line parser | Main loop reads until newline and processes complete commands. |
| Day 5 | Non-blocking read API | USART2_ReadByte returns success/failure instead of blocking. |
| Day 6 | Error handling | ORE, FE, NE, and PE are recorded and cleared. |
| Day 7 | DMA RX circular | DMA writes incoming bytes into memory without per-byte ISR work. |
| Later | Idle detection + CLI | Frame boundaries and commands become robust. |

### Long-Term Learning Tracker

| Check | Learning or validation item | Evidence |
| --- | --- | --- |
| [ ] | I can explain why one buffer slot remains unused. | Notes / capture: |
| [ ] | I can draw the head/tail movement for a burst of bytes. | Notes / capture: |
| [ ] | I can debug full, empty, and wrap conditions. | Notes / capture: |
| [ ] | I can explain why the ISR should not parse commands. | Notes / capture: |

# 19. Conclusion

Day 3 is the bridge from interrupt notification to practical receive buffering. The USART ISR now behaves like a fast producer, and the application behaves like a consumer that can process data later. This small change makes the receiver much more useful for real terminal input, command strings, and later protocol parsing.

Core takeaway: A circular buffer is not just an array. It is a timing boundary between hardware arrival and application processing. Head, tail, wrap, empty, full, and overflow are the rules that make that boundary reliable.

# Appendix A. Actual Source Evidence

| File | Evidence included |
| --- | --- |
| usart.h | RX_BUFFER_SIZE and API declarations. |
| main.c | Global circular buffer variables and main-loop consumer behavior. |
| usart.c | USART2_Init and USART2_ReadByte implementation. |
| stm32f4xx_it.c | USART2_IRQHandler producer and overflow behavior. |
| stm32f4xx_it.h | USART2_IRQHandler prototype. |

# Appendix B. One-Page Revision Summary

RX_BUFFER_SIZE is 64, so the practical queue capacity is 63 bytes with this full/empty detection method.

rx_head is advanced by the ISR after storing a received byte.

rx_tail is advanced by USART2_ReadByte after the application reads a byte.

The buffer is empty when rx_head equals rx_tail.

The buffer is full when next_head equals rx_tail.

Overflow is handled by reading DR into a dummy variable and setting rx_overflow to 1.