EMBEDDED PROJECT REPORT

USART Day 4 TX DMA Direct Mode Project

STM32F407VG | USART2 TX | DMA1 Stream6 Channel4 | Direct Mode | PA2 TX / PA3 RX

| Field | Value |
| --- | --- |
| Document purpose | Long-term learning, implementation reference, and DMA configuration guide. |
| Project type | Bare-metal USART2 TX DMA direct mode with RX circular buffer retained. |
| Content source | Source files and PDF template |
| Code authority | Only the attached finalized source files are used for code snippets. |
| Core concept | CPU configures DMA once; DMA feeds USART2->DR when USART TX requests are generated. |

Implementation truth: This finalized code configures DMA1 Stream6 Channel4 for USART2 TX direct mode, uses USART_CR3_DMAT to enable TX DMA requests, and uses DMA TC followed by USART TC to distinguish buffer-fed completion from wire completion.

# 1. Project Objective

The objective of Day 4 is to move USART transmit from CPU-fed byte writes to DMA-fed streaming. The CPU prepares the DMA stream with a memory buffer, transfer length, and USART2 data-register address; then DMA writes bytes into USART2->DR as the USART transmit path becomes ready.

Learning outcome: After this project, you should be able to explain DMA1 Stream6 Channel4, PAR, M0AR, NDTR, DIR, MINC, PINC, TCIE, direct mode, USART DMAT, DMA TC, and USART TC in one complete TX path.

### Objective Checkpoint

| Check | Learning or validation item | Evidence |
| --- | --- | --- |
| [ ] | I can identify why USART2 TX maps to DMA1 Stream6 Channel4. | Notes / capture: |
| [ ] | I can explain what PAR, M0AR, and NDTR mean. | Notes / capture: |
| [ ] | I can explain why DMA TC is not the same as USART TC. | Notes / capture: |

# 2. Problem Statement

Polling TX and interrupt TX both require the CPU to participate in each byte movement. This becomes inefficient for long strings, logs, command responses, or binary frames. Day 4 solves that by making DMA the data feeder for USART2->DR.

| Problem area | Day 4 response |
| --- | --- |
| Per-byte CPU work | DMA moves bytes from RAM to USART2->DR. |
| Long string transmit | Application provides buffer address and length once. |
| Completion ambiguity | DMA TC marks buffer consumed; USART TC marks final wire completion. |
| Future scaling | Direct mode prepares for FIFO mode and later RX DMA. |

# 3. System Overview

The transmit data path starts in an application buffer, passes through DMA1 Stream6 Channel4, enters USART2->DR, then moves through the USART shift register to PA2/TX. USART2 generates DMA requests when TXE and DMAT allow the next data register write.

Application buffer in SRAM
    |
    | M0AR + NDTR
    v
DMA1 Stream6 Channel4
    |
    | PAR = &USART2->DR
    v
USART2->DR -> TX shift register -> PA2/TX pin

Completion path:
DMA TCIF6 -> DMA1_Stream6_IRQHandler -> enable USART TCIE
USART TC -> USART2_IRQHandler -> clear DMAT/TCIE and mark tx_complete

| Project focus | What changes from Day 3 |
| --- | --- |
| TX path | Polling TX echo is replaced in main usage by DMA send APIs. |
| RX path | RX circular buffer remains present from Day 3. |
| DMA role | DMA feeds USART2->DR automatically. |
| Completion model | Two-stage completion: DMA complete, then USART wire complete. |

# 4. Requirements

## 4.1 Functional Requirements

| ID | Requirement | Acceptance condition |
| --- | --- | --- |
| FR-01 | Initialize USART2 for TX/RX serial communication. | USART2 has UE, TE, RE, BRR, and 8N1 configuration. |
| FR-02 | Initialize DMA1 Stream6 Channel4 for USART2 TX. | DMA1 clock enabled, stream disabled, flags cleared, CHSEL=4, DIR memory-to-peripheral. |
| FR-03 | Use direct mode. | DMA_SxFCR_DMDIS is cleared so FIFO is disabled. |
| FR-04 | Transmit strings using DMA. | M0AR points to string, NDTR equals string length, stream EN set, USART DMAT set. |
| FR-05 | Handle DMA completion. | DMA1_Stream6_IRQHandler clears CTCIF6, sets dma_tx_complete, and enables USART TCIE. |
| FR-06 | Handle USART wire completion. | USART2_IRQHandler handles TC, sets tx_complete, disables TCIE and DMAT. |

## 4.2 Non-Functional Requirements

| Area | Requirement |
| --- | --- |
| Buffer lifetime | Source buffer must remain valid until transfer completion. |
| Reconfiguration safety | DMA stream must be disabled and EN cleared before reconfiguration. |
| Flag hygiene | Old DMA flags must be cleared before enabling a new transfer. |
| Learning scope | Direct mode only; FIFO configuration is deferred to the next stage. |

# 5. Hardware Components

| Item | Selection in this project | Purpose |
| --- | --- | --- |
| MCU | STM32F407VG / STM32F407VGT6 family | Target MCU with USART2 and DMA1. |
| USART instance | USART2 | Serial peripheral. |
| TX pin | PA2 AF7 | USART2 transmit line. |
| RX pin | PA3 AF7 | USART2 receive line retained from earlier days. |
| DMA controller | DMA1 | DMA controller serving USART2 TX. |
| DMA stream | Stream 6 | USART2_TX stream used by finalized code. |
| DMA channel | Channel 4 | USART2_TX channel selection. |
| Mode | Direct mode | FIFO disabled for simple transfer behavior. |

# 6. Software and Firmware Tools

| Source file | Verified role |
| --- | --- |
| Core/Src/main.c | Initializes USART2 and DMA; sends startup DMA string; echoes received bytes using DMA send byte. |
| Core/Src/usart.c | Configures USART2, DMA1 Stream6, and DMA send APIs. |
| Core/Src/stm32f4xx_it.c | Handles RX circular buffer, USART TC, and DMA1 Stream6 TC interrupt. |
| Core/Inc/usart.h | Declares DMA APIs and RX buffer size. |
| Core/Inc/stm32f4xx_it.h | Declares USART2 and DMA1 Stream6 interrupt handlers. |

Reasoning correction: The PDF shows an ideal busy-state driver. Your finalized code does not yet guard against starting a new DMA TX while one is active. That is acceptable for learning, but a production driver should add a busy state or transmit queue.

# 7. System Block Diagram

CPU setup phase
  USART2_DMA_SendString / USART2_DMA_SendByte
    disable DMA stream
    clear HIFCR flags
    set M0AR and NDTR
    enable DMA stream
    set USART2->CR3 DMAT

Hardware transfer phase
  USART TXE request -> DMA writes USART2->DR -> USART shifts byte out

Completion phase
  DMA TCIF6 -> DMA IRQ -> enable USART TCIE
  USART TC -> USART IRQ -> disable TCIE and DMAT

| Register / flag | Meaning | Used for |
| --- | --- | --- |
| DMA1_Stream6->PAR | Peripheral address. | Points to USART2->DR. |
| DMA1_Stream6->M0AR | Memory source address. | Points to string buffer or byte source. |
| DMA1_Stream6->NDTR | Number of bytes. | Transfer length. |
| DMA_SxCR_CHSEL | DMA channel select. | Channel 4 for USART2_TX. |
| DMA_SxCR_DIR_0 | Memory-to-peripheral. | TX data direction. |
| DMA_SxCR_MINC | Memory increment. | Walk through a string buffer. |
| DMA_SxCR_TCIE | Transfer complete interrupt enable. | Generate DMA completion interrupt. |
| USART_CR3_DMAT | USART TX DMA request enable. | Allows USART to request DMA writes. |

# 8. Firmware Architecture

The firmware keeps the previous RX circular buffer and adds a TX DMA path. The application reads received bytes from the RX buffer and sends the echo byte through USART2_DMA_SendByte. String transmit is also available through USART2_DMA_SendString.

main.c
  USART2_Init();
  USART2_DMA_Init();
  USART2_DMA_SendString("DMA Initialized for Transmit\r\n");
  while(1) { if(rx_head != rx_tail) { msg = USART2_ReadByte(); USART2_DMA_SendByte(msg); } }

usart.c
  USART2_DMA_Init()
  USART2_DMA_SendString(char *str)
  USART2_DMA_SendByte(uint8_t c)

stm32f4xx_it.c
  DMA1_Stream6_IRQHandler()
  USART2_IRQHandler() handles final TC

Architecture decision: The code uses DMA for transmit data movement but still relies on USART TC to know when the final byte has physically finished on the TX wire.

# 9. Communication Protocol Used

The serial protocol remains USART/UART-style 8N1 communication. DMA does not change baud rate, framing, idle level, or TX/RX pins. It changes only how bytes are delivered from memory to USART2->DR.

| Parameter | Value from finalized code | Engineering note |
| --- | --- | --- |
| Clock source | HSI through PLL in main.c | Same clock style as Day 2 and Day 3 projects. |
| APB1 divider | HCLK / 2 | USART2 belongs to APB1. |
| BRR | 364 | Current project baud-register value. |
| Frame | 8N1 | M/PCE cleared and STOP cleared. |
| DMA mapping | DMA1 Stream6 Channel4 | USART2_TX mapping on STM32F407 family. |
| Mode | Direct mode | FIFO disabled by clearing DMDIS. |

# 10. Module-Wise Implementation

## 10.0 Header API from usart.h

#define RX_BUFFER_SIZE 64

void USART2_Init(void);
void USART2_SendString(char *s);
void USART2_SendByte(char s);
char USART2_ReceiveByte(void);
uint8_t USART2_ReadByte(void);
void USART2_DMA_Init(void);
void USART2_DMA_SendString(char *str);
void USART2_DMA_SendByte(uint8_t c);


#endif /* INC_USART_H_ */

## 10.1 DMA-Related Global State from main.c

volatile uint8_t dma_tx_complete;
volatile uint8_t tx_complete;

uint8_t msg;

## 10.2 USART2 DMA Initialization from usart.c

void USART2_DMA_Init(void)
{
    RCC->AHB1ENR |= RCC_AHB1ENR_DMA1EN;

    /* Disable stream */
    DMA1_Stream6->CR &= ~DMA_SxCR_EN;

    while(DMA1_Stream6->CR & DMA_SxCR_EN);

    /* Clear old flags */
    DMA1->HIFCR =
          DMA_HIFCR_CTCIF6  |
          DMA_HIFCR_CHTIF6  |
          DMA_HIFCR_CTEIF6  |
          DMA_HIFCR_CDMEIF6 |
          DMA_HIFCR_CFEIF6;

    /* Clear CR */
    DMA1_Stream6->CR = 0;

    /* Channel 4 */
    DMA1_Stream6->CR |=
            (4UL << DMA_SxCR_CHSEL_Pos);

    /* Memory-to-peripheral */
    DMA1_Stream6->CR |= DMA_SxCR_DIR_0;

    /* Memory increment */
    DMA1_Stream6->CR |= DMA_SxCR_MINC;

    /* Peripheral increment disabled */
    DMA1_Stream6->CR &= ~DMA_SxCR_PINC;

    /* Transfer complete interrupt */
    DMA1_Stream6->CR |= DMA_SxCR_TCIE;

    /* Direct mode */
    DMA1_Stream6->FCR &= ~DMA_SxFCR_DMDIS;

    /* Peripheral address */
    DMA1_Stream6->PAR =
            (uint32_t)&USART2->DR;

    /* Enable DMA IRQ */
    NVIC_EnableIRQ(DMA1_Stream6_IRQn);

}

## 10.3 DMA String Transfer from usart.c

void USART2_DMA_SendString(char *str)
{
	uint16_t strln = 0;

	//length of the string
	while(str[strln]){
		strln++;
	}

    /* Disable stream */
    DMA1_Stream6->CR &= ~DMA_SxCR_EN;

    while(DMA1_Stream6->CR & DMA_SxCR_EN);

    /* Clear old flags */
    DMA1->HIFCR =
          DMA_HIFCR_CTCIF6  |
          DMA_HIFCR_CHTIF6  |
          DMA_HIFCR_CTEIF6  |
          DMA_HIFCR_CDMEIF6 |
          DMA_HIFCR_CFEIF6;

    /* Transfer complete interrupt */
    DMA1_Stream6->CR |= DMA_SxCR_TCIE;

    /* Memory address */
    DMA1_Stream6->M0AR = (uint32_t)str;

    /* Number of bytes */
    DMA1_Stream6->NDTR = strln;

    /* Enable DMA stream */
    DMA1_Stream6->CR |= DMA_SxCR_EN;

    /* Enable USART DMA TX */
    USART2->CR3 |= USART_CR3_DMAT;
}

## 10.4 DMA Byte Transfer from usart.c

void USART2_DMA_SendByte(uint8_t c)
{
    /* Disable stream */
    DMA1_Stream6->CR &= ~DMA_SxCR_EN;
    while(DMA1_Stream6->CR & DMA_SxCR_EN);

    /* Clear old flags */
    DMA1->HIFCR =
          DMA_HIFCR_CTCIF6  |
          DMA_HIFCR_CHTIF6  |
          DMA_HIFCR_CTEIF6  |
          DMA_HIFCR_CDMEIF6 |
          DMA_HIFCR_CFEIF6;

    /* Transfer complete interrupt */
    DMA1_Stream6->CR |= DMA_SxCR_TCIE;

    /* Memory address */
    DMA1_Stream6->M0AR = (uint32_t *)&c;

    /* Number of bytes */
    DMA1_Stream6->NDTR = 1;

    /* Enable DMA stream */
    DMA1_Stream6->CR |= DMA_SxCR_EN;

    /* Enable USART DMA TX */
    USART2->CR3 |= USART_CR3_DMAT;

}

Recommended correction: USART2_DMA_SendByte currently passes the address of local variable c to DMA. Because DMA may continue after the function returns, use a static/global byte buffer or wait synchronously before returning. This is the most important code-safety correction for Day 4.

## 10.5 DMA and USART Completion Handlers from stm32f4xx_it.c

void DMA1_Stream6_IRQHandler(void)
{
    //clear DMA TC flag
    DMA1->HIFCR |= DMA_HIFCR_CTCIF6;

    dma_tx_complete = 1;

    /*Waiting for USART completion*/
    USART2->CR1 |= USART_CR1_TCIE;

}

/* USER CODE END 1 */

if(USART2->SR & USART_SR_TC)
	{
		tx_complete = 1;

	    //clear transmit completion status
	    USART2->SR &= ~USART_SR_TC;

		//clear transmit completion interrupt
	    USART2->CR1 &= ~USART_CR1_TCIE;

	    //clear DMA request
	    USART2->CR3 &= ~USART_CR3_DMAT;

# 11. DMA Configuration Deep Dive

DMA configuration is the heart of Day 4. The stream must be disabled before changing it, all stale status flags must be cleared, the channel and direction must match USART2_TX, the address registers must point to the correct source and destination, and USART DMAT must be enabled so USART can request DMA service.

| Configuration step | Finalized code | Why it matters |
| --- | --- | --- |
| Enable DMA clock | RCC->AHB1ENR ¦= RCC_AHB1ENR_DMA1EN | DMA registers are not usable until the controller clock is enabled. |
| Disable stream | DMA1_Stream6->CR &= ~DMA_SxCR_EN | STM32 DMA stream registers should be changed only when EN is clear. |
| Wait EN clear | while(DMA1_Stream6->CR & DMA_SxCR_EN) | Prevents reconfiguring an active stream. |
| Clear old flags | DMA1->HIFCR = CTCIF6 ¦ CHTIF6 ¦ CTEIF6 ¦ CDMEIF6 ¦ CFEIF6 | Avoids immediate stale interrupts. |
| Select channel | (4UL << DMA_SxCR_CHSEL_Pos) | Maps stream to USART2_TX request. |
| Set direction | DMA_SxCR_DIR_0 | Memory-to-peripheral transfer. |
| Enable MINC | DMA_SxCR_MINC | Moves through each byte in a string buffer. |
| Disable PINC | ~DMA_SxCR_PINC | USART2->DR address must stay fixed. |
| Enable TC interrupt | DMA_SxCR_TCIE | Generates DMA completion interrupt. |
| Direct mode | DMA1_Stream6->FCR &= ~DMA_SxFCR_DMDIS | FIFO disabled. |
| Peripheral address | PAR = (uint32_t)&USART2->DR | DMA destination is USART data register. |
| USART request | USART2->CR3 ¦= USART_CR3_DMAT | USART TX requests can trigger DMA writes. |

## 11.1 Direct Mode Meaning

Direct mode means the DMA FIFO is disabled. The stream responds directly to peripheral requests and transfers data from memory toward USART2->DR without using the DMA FIFO buffering stage. It is the simplest mode for learning and works well for straightforward byte streams.

## 11.2 Address and Count Registers

| Register | Value in this project | Interpretation |
| --- | --- | --- |
| PAR | &USART2->DR | Fixed peripheral destination register. |
| M0AR | str or &c | Memory source address. For byte send, this should outlive the DMA transfer. |
| NDTR | string length or 1 | Number of data items DMA must move. |
| CR | CHSEL=4, DIR_0, MINC, TCIE | Core stream behavior. |
| FCR | DMDIS cleared | Direct mode enabled. |

## 11.3 DMA TC vs USART TC

| Event | Meaning | Handler action in finalized code |
| --- | --- | --- |
| DMA TCIF6 | DMA has finished writing the requested bytes into the USART data path. | DMA1_Stream6_IRQHandler clears CTCIF6, sets dma_tx_complete, and enables USART TCIE. |
| USART TC | Final frame has fully completed on the wire. | USART2_IRQHandler sets tx_complete, clears TCIE, and clears DMAT. |

Key DMA lesson: DMA transfer complete means the buffer has been consumed by DMA. It does not guarantee that the last stop bit has left PA2. Your code correctly adds a USART TC follow-up stage for wire completion.

# 12. Main Algorithm and Flowchart

Startup
  HAL_Init -> clock config -> GPIO init
  USART2_Init -> USART2_DMA_Init
  polling TX greeting
  DMA TX initialization string

Main loop
  if rx_head != rx_tail:
      msg = USART2_ReadByte()
      tx_complete = 0
      dma_tx_complete = 0
      USART2_DMA_SendByte(msg)

DMA completion
  DMA TCIF6 -> DMA1_Stream6_IRQHandler -> dma_tx_complete = 1 -> enable USART TCIE
  USART TC -> USART2_IRQHandler -> tx_complete = 1 -> disable TCIE and DMAT

| Step | Expected observation |
| --- | --- |
| USART init | Terminal receives polling initialization message. |
| DMA init string | Terminal receives DMA initialized message. |
| RX byte arrives | Byte enters RX circular buffer. |
| Main reads byte | Byte is sent back through DMA TX. |
| DMA TC | dma_tx_complete becomes 1. |
| USART TC | tx_complete becomes 1 after final wire completion. |

# 13. Code Explanation

| Code action | Meaning | Why it matters |
| --- | --- | --- |
| USART2_DMA_Init | One-time DMA stream setup. | Establishes stream, channel, direction, direct mode, PAR, and IRQ. |
| USART2_DMA_SendString | Calculates length, configures M0AR/NDTR, enables stream and DMAT. | Starts a multi-byte DMA transmit. |
| USART2_DMA_SendByte | Configures a one-byte DMA transfer. | Provides uniform DMA TX API for echo. |
| DMA1_Stream6_IRQHandler | Handles DMA transfer complete. | Marks DMA completion and enables USART TCIE. |
| USART2_IRQHandler TC branch | Handles wire-level transmit completion. | Disables TCIE and DMAT after final frame. |
| DMA1->HIFCR flag clears | Clears stale or completed stream flags. | Prevents immediate false interrupts. |

# 14. Testing and Validation

| Test | Action | Expected result | Evidence |
| --- | --- | --- | --- |
| Startup polling TX | Run firmware. | USART initialized text appears. | Terminal capture. |
| Startup DMA TX | Observe second startup line. | DMA initialized message appears. | Terminal capture. |
| DMA byte echo | Send one terminal byte. | Byte echoes through DMA send byte. | Terminal capture. |
| DMA flags | Watch dma_tx_complete and tx_complete. | DMA flag sets before or around USART completion flag. | Debugger watch. |
| Register view | Inspect CR3, stream CR, NDTR, PAR, M0AR. | Values match configured transfer. | Debugger screenshot. |

### DMA Validation Checklist

| Check | Learning or validation item | Evidence |
| --- | --- | --- |
| [ ] | DMA1 clock is enabled before accessing DMA1_Stream6. | Notes / capture: |
| [ ] | DMA1_Stream6 EN is cleared before reconfiguration. | Notes / capture: |
| [ ] | HIFCR flags for Stream6 are cleared before enabling the transfer. | Notes / capture: |
| [ ] | CHSEL is Channel 4. | Notes / capture: |
| [ ] | DIR is memory-to-peripheral. | Notes / capture: |
| [ ] | PAR points to USART2->DR. | Notes / capture: |
| [ ] | M0AR points to a valid transmit buffer. | Notes / capture: |
| [ ] | NDTR equals the intended byte count. | Notes / capture: |
| [ ] | USART2->CR3 DMAT is set after stream setup. | Notes / capture: |

# 15. Results

| Observed / expected behavior | Interpretation |
| --- | --- |
| DMA initialization string prints. | String transmit through DMA is functional. |
| Received byte echoes through DMA. | RX circular buffer plus DMA TX path is connected. |
| dma_tx_complete sets in DMA IRQ. | DMA transfer complete interrupt path is functional. |
| tx_complete sets in USART IRQ. | USART wire-completion follow-up is functional. |

# 16. Problems Faced and Debugging Notes

| Symptom | Likely cause | Debug action |
| --- | --- | --- |
| No DMA output | DMAT not set, DMA stream not enabled, or wrong channel/stream. | Check USART2->CR3, DMA1_Stream6->CR, CHSEL=4. |
| Only first byte appears | NDTR or M0AR incorrect. | Inspect M0AR and NDTR before enabling stream. |
| Immediate DMA interrupt | Old TC flag not cleared. | Clear HIFCR before enabling Stream6. |
| Output corrupted | Source buffer changed or went out of scope. | Keep buffer valid until completion. |
| Wire completion late | Confusing DMA TC with USART TC. | Use USART TC stage when protocol needs final-bit completion. |
| Next transfer unreliable | No busy-state guard. | Add tx busy state before allowing new sends. |

## 16.1 Register Checklist

| Item | Expected state | Quick check |
| --- | --- | --- |
| RCC->AHB1ENR | GPIOAEN and DMA1EN set | GPIO and DMA clocks active. |
| RCC->APB1ENR | USART2EN set | USART2 clock active. |
| USART2->CR1 | UE, TE, RE set; TCIE only during final completion wait | USART enabled and completion interrupt staged. |
| USART2->CR3 | DMAT set during transfer | USART DMA TX request path enabled. |
| DMA1_Stream6->CR | CHSEL=4, DIR=M2P, MINC, TCIE, EN during transfer | Core DMA stream config. |
| DMA1_Stream6->PAR | &USART2->DR | Fixed peripheral target. |
| DMA1_Stream6->NDTR | Length or 1 | Correct transfer count. |

# 17. Limitations

| Limitation | Why it matters | Next improvement |
| --- | --- | --- |
| USART2_DMA_SendByte uses local variable address | DMA may read after function returns. | Use static/global byte storage or blocking wait. |
| No TX busy state | New transfer can be started before previous one completes. | Add busy flag or queue. |
| DMA IRQ does not check TCIF6 before acting | Other stream events could be misinterpreted. | Read HISR and branch on TCIF6/TEIF6. |
| No DMA error handling | TEIF/DMEIF/FEIF are cleared at setup but not handled at runtime. | Add error interrupt handling. |
| RX buffer macro duplicated in interrupt file | Definition drift risk. | Use shared header consistently. |
| Direct mode only | No FIFO tuning yet. | Introduce FIFO mode in Day 5. |

# 18. Future Improvements

| Stage | Upgrade | What changes |
| --- | --- | --- |
| Day 5 | TX DMA FIFO mode | Enable and tune DMA FIFO behavior. |
| Day 6 | DMA error handling | Handle TEIF, DMEIF, FEIF, and transfer aborts. |
| Day 7 | Busy-state TX API | Prevent overlapping DMA transfers. |
| Day 8 | DMA RX | Move receive data movement into DMA. |
| Later | CLI / packet protocol | Use DMA-backed TX responses and buffered RX parsing. |

### Long-Term Learning Tracker

| Check | Learning or validation item | Evidence |
| --- | --- | --- |
| [ ] | I can configure DMA1 Stream6 Channel4 from memory. | Notes / capture: |
| [ ] | I can explain direct mode versus FIFO mode. | Notes / capture: |
| [ ] | I can explain every field used in CR, PAR, M0AR, NDTR, and FCR. | Notes / capture: |
| [ ] | I can explain why source buffer lifetime matters. | Notes / capture: |
| [ ] | I can explain DMA TC versus USART TC. | Notes / capture: |

# 19. Conclusion

Day 4 makes the USART transmit path scalable. Instead of spending CPU time on every byte, the application schedules a DMA transfer and lets hardware feed USART2->DR. The most important learning is not only how to start DMA, but how to configure it safely, protect the source buffer, clear flags, and interpret completion correctly.

Core takeaway: DMA direct mode is a hardware data-movement contract: M0AR says where bytes come from, PAR says where they go, NDTR says how many, CR says how, DMAT lets USART request the movement, and TC handling tells you what kind of completion has happened.

# Appendix A. Actual Source Evidence

| File | Evidence included |
| --- | --- |
| usart.c | USART2_DMA_Init, USART2_DMA_SendString, USART2_DMA_SendByte. |
| stm32f4xx_it.c | DMA1_Stream6_IRQHandler and USART TC completion branch. |
| main.c | DMA init and DMA send usage in main loop. |
| usart.h | DMA API declarations. |
| stm32f4xx_it.h | DMA1_Stream6_IRQHandler prototype. |

# Appendix B. One-Page Revision Summary

USART2 TX uses DMA1 Stream6 Channel4 in direct mode.

PAR points to USART2->DR, M0AR points to the source buffer, and NDTR stores the transfer length.

DIR is memory-to-peripheral, MINC is enabled, PINC is disabled, and TCIE is enabled.

USART2->CR3 DMAT enables USART TX DMA requests.

DMA TC means the DMA stream is done feeding data; USART TC means the final frame is done on the wire.