EMBEDDED PROJECT REPORT

USART Day 5 TX DMA FIFO Mode Project

STM32F407VG | USART2 TX | DMA1 Stream6 Channel4 | FIFO Mode | Full threshold | PA2 TX / PA3 RX

| Field | Value |
| --- | --- |
| Document purpose | Long-term learning, implementation reference, and DMA FIFO configuration guide. |
| Project type | Bare-metal USART2 TX DMA FIFO mode with RX circular buffer retained. |
| Content source | Source files and PDF template document |
| Code authority | Only the attached finalized source files are used for code snippets. |
| Core concept | DMA FIFO adds an internal DMA staging buffer before USART2->DR writes. |

Implementation truth: This finalized code enables FIFO mode with DMA_SxFCR_DMDIS, sets FIFO threshold through DMA_SxFCR_FTH, uses byte-sized memory/peripheral transfers, and adds FIFO error tracking through FEIF6.

# 1. Project Objective

The objective of Day 5 is to extend the Day 4 TX DMA design from direct mode to FIFO mode. The USART still transmits one frame at a time, but the DMA stream can stage memory data internally before serving USART2 transmit requests.

Learning outcome: After this project, you should be able to explain DMDIS, FTH, MSIZE, PSIZE, MBURST, PBURST, FEIF6, DMA TC versus USART TC, and why FIFO mode is mainly useful for larger or repeated transmit buffers.

### Objective Checkpoint

| Check | Learning or validation item | Evidence |
| --- | --- | --- |
| [ ] | I can explain what changes from Day 4 direct mode. | Notes / capture: |
| [ ] | I can locate the FIFO enable and threshold configuration in the finalized code. | Notes / capture: |
| [ ] | I can explain how FIFO errors are detected and cleared. | Notes / capture: |

# 2. Problem Statement

Direct mode is simple and useful, but it does not use the DMA stream's internal FIFO. For longer strings, repeated frames, and busier systems, FIFO mode can stage data internally and make the DMA side less sensitive to short memory-access timing variations.

| Problem area | Day 5 response |
| --- | --- |
| Direct-mode simplicity limit | Enable DMA FIFO mode through FCR. |
| Large payload testing | Transmit a long RSA-key string with USART2_DMA_SendBuffer. |
| FIFO diagnostics | Track FEIF6 through fifo_error_count. |
| Completion ambiguity | Keep the Day 4 two-stage DMA TC then USART TC model. |

# 3. System Overview

The data path begins in SRAM, enters DMA1 Stream6 FIFO, drains from the DMA stream to USART2->DR when USART requests TX data, then leaves through the USART shift register and PA2.

Application buffer in SRAM
    |
    v
DMA1 Stream6 FIFO stage
    |
    v
DMA1 Stream6 writes USART2->DR
    |
    v
USART2 shift register -> PA2/TX

Completion and diagnostics:
DMA TCIF6 -> DMA IRQ -> dma_tx_complete + USART TCIE
DMA FEIF6 -> fifo_error_count++ + CFEIF6 clear
USART TC -> tx_complete + DMAT clear

| Project focus | What changes from Day 4 |
| --- | --- |
| DMA mode | FIFO enabled instead of direct mode. |
| FCR role | FCR now actively controls FIFO enable and threshold. |
| Transfer test | Long buffer transfer is tested using USART2_DMA_SendBuffer. |
| Diagnostics | FIFO error flag FEIF6 is checked in the DMA IRQ. |

# 4. Requirements

## 4.1 Functional Requirements

| ID | Requirement | Acceptance condition |
| --- | --- | --- |
| FR-01 | Initialize USART2 and DMA1 Stream6 Channel4. | USART2, DMA1, GPIOA clocks and USART/DMA registers are configured. |
| FR-02 | Enable DMA FIFO mode. | DMA1_Stream6->FCR has DMDIS set. |
| FR-03 | Configure FIFO threshold. | DMA1_Stream6->FCR sets FTH. |
| FR-04 | Use byte-sized transfers. | MSIZE and PSIZE are cleared for 8-bit transfer width. |
| FR-05 | Support string, byte, and buffer DMA TX. | USART2_DMA_SendString, SendByte, and SendBuffer are available. |
| FR-06 | Detect FIFO error. | DMA IRQ checks FEIF6, increments fifo_error_count, and clears CFEIF6. |

## 4.2 Non-Functional Requirements

| Area | Requirement |
| --- | --- |
| Buffer lifetime | Source buffers must remain valid until DMA and USART completion. |
| Reconfiguration safety | Stream must be disabled and EN cleared before updating registers. |
| Flag hygiene | Old DMA flags must be cleared before each transfer. |
| Security hygiene | Do not embed real private keys in documentation or logs; use test-only keys or redacted fixtures. |

# 5. Hardware Components

| Item | Selection in this project | Purpose |
| --- | --- | --- |
| MCU | STM32F407VG / STM32F407VGT6 family | Target MCU with USART2 and DMA1. |
| USART instance | USART2 | Serial peripheral. |
| TX pin | PA2 AF7 | USART2 transmit line. |
| RX pin | PA3 AF7 | USART2 receive line retained. |
| DMA controller | DMA1 | DMA controller serving USART2 TX. |
| DMA stream | Stream 6 | USART2_TX stream used by finalized code. |
| DMA channel | Channel 4 | USART2_TX channel selection. |
| DMA mode | FIFO mode | Internal DMA FIFO enabled. |

# 6. Software and Firmware Tools

| Source file | Verified role |
| --- | --- |
| Core/Src/main.c | Initializes USART and DMA FIFO TX, transmits startup strings and a long buffer, then echoes received bytes. |
| Core/Src/usart.c | Configures USART2, DMA1 Stream6 FIFO mode, and DMA TX APIs. |
| Core/Src/stm32f4xx_it.c | Handles RX circular buffer, USART TC, DMA TC, and FIFO error flag. |
| Core/Inc/usart.h | Declares DMA APIs including USART2_DMA_SendBuffer. |
| Core/Inc/stm32f4xx_it.h | Declares USART2 and DMA1 Stream6 interrupt handlers. |

Reasoning correction: main.c calls USART2_DMA_TX_Init(), but usart.c/usart.h define USART2_DMA_Init(). Unless a macro exists outside the attached files, rename the call or function so they match.

# 7. System Block Diagram

CPU setup phase
  USART2_DMA_SendString / SendByte / SendBuffer
    disable DMA stream
    clear HIFCR flags
    set M0AR and NDTR
    enable DMA stream
    set USART2->CR3 DMAT

FIFO transfer phase
  DMA fetches from memory into FIFO
  USART TX request drains FIFO toward USART2->DR

Completion phase
  DMA TCIF6 -> DMA IRQ -> USART TCIE
  USART TC -> USART IRQ -> clear TCIE and DMAT
  FEIF6 -> fifo_error_count++

| Register / bit | Meaning | Used for |
| --- | --- | --- |
| DMA_SxFCR_DMDIS | Direct mode disable. | Set to enable FIFO mode. |
| DMA_SxFCR_FTH | FIFO threshold field. | Configured as full FIFO in finalized code. |
| DMA_SxCR_MSIZE | Memory data size. | Cleared for byte-sized transfers. |
| DMA_SxCR_PSIZE | Peripheral data size. | Cleared for byte-sized USART DR writes. |
| DMA_SxCR_MBURST | Memory burst setting. | Cleared for string/byte; set in SendBuffer. |
| DMA_SxCR_PBURST | Peripheral burst setting. | Cleared for single peripheral beats. |
| DMA_HISR_FEIF6 | FIFO error flag. | Counted and cleared in DMA IRQ. |
| USART_CR3_DMAT | USART TX DMA request enable. | Allows USART2 TX to request DMA service. |

# 8. Firmware Architecture

The firmware retains the RX circular buffer from Day 3 and the DMA TX completion model from Day 4, then changes the DMA stream configuration to use FIFO mode. The main application also sends a long static buffer to demonstrate FIFO-mode transfer behavior.

main.c usage excerpt:
USART2_Init();
USART2_DMA_TX_Init();
USART2_SendString("USART Initialized...!!\r\n");
USART2_DMA_SendString("DMA Initialized for Transmit\r\n");
HAL_Delay(500);
USART2_DMA_SendBuffer(rsa_private_key, strlen(rsa_private_key));

Security note: The attached main.c contains a PEM-formatted private key string as test payload. The generated document does not reproduce the key body. For long-payload tests, use a dummy PEM-like fixture or non-secret test data.

# 9. Communication Protocol Used

USART wire behavior remains 8N1 asynchronous serial communication. FIFO mode changes only the DMA stream's internal memory staging behavior; it does not change the USART frame, baud rate, pins, or TX completion meaning.

| Parameter | Value from finalized code | Engineering note |
| --- | --- | --- |
| Clock source | HSI through PLL in main.c | Same clock style as previous DMA projects. |
| BRR | 364 | Current project baud-register value. |
| Frame | 8N1 | M/PCE cleared and STOP cleared. |
| DMA mapping | DMA1 Stream6 Channel4 | USART2_TX mapping on STM32F407 family. |
| FIFO mode | DMDIS set | Direct mode disabled, FIFO enabled. |
| FIFO threshold | FTH set | Finalized code labels this as full FIFO threshold. |

# 10. Module-Wise Implementation

## 10.0 Header API from usart.h

#define RX_BUFFER_SIZE 64

void USART2_Init(void);
void USART2_SendString(char *s);
void USART2_SendByte(char s);
char USART2_ReceiveByte(void);
uint8_t USART2_ReadByte(void);
void USART2_DMA_Init(void);
void USART2_DMA_SendString(char *str);
void USART2_DMA_SendByte(uint8_t c);
void USART2_DMA_SendBuffer(const char *buffer, uint16_t length);


#endif /* INC_USART_H_ */

## 10.1 DMA FIFO Initialization from usart.c

void USART2_DMA_Init(void)
{
    RCC->AHB1ENR |= RCC_AHB1ENR_DMA1EN;

    /* Disable stream */
    DMA1_Stream6->CR &= ~DMA_SxCR_EN;
    while(DMA1_Stream6->CR & DMA_SxCR_EN);

    /* Clear old flags */
    DMA1->HIFCR =
          DMA_HIFCR_CTCIF6  |
          DMA_HIFCR_CHTIF6  |
          DMA_HIFCR_CTEIF6  |
          DMA_HIFCR_CDMEIF6 |
          DMA_HIFCR_CFEIF6;

    /* Clear CR */
    DMA1_Stream6->CR = 0;

    /* Channel 4 */
    DMA1_Stream6->CR |=
            (4UL << DMA_SxCR_CHSEL_Pos);

    /* Memory-to-peripheral */
    DMA1_Stream6->CR |= DMA_SxCR_DIR_0;

    /* Memory increment */
    DMA1_Stream6->CR |= DMA_SxCR_MINC;

    /* Peripheral increment disabled */
    DMA1_Stream6->CR &= ~DMA_SxCR_PINC;

    /* Transfer complete interrupt */
    DMA1_Stream6->CR |= DMA_SxCR_TCIE;

    /* FIFO Mode Enabled */
    DMA1_Stream6->FCR |= DMA_SxFCR_DMDIS;

    /* FIFO Threshold - Full FIFO */
    DMA1_Stream6->FCR |= DMA_SxFCR_FTH;

    /* MSIZE - 1 Byte transfer */
    DMA1_Stream6->CR &= ~DMA_SxCR_MSIZE;

    /* PSIZE - 1 Byte transfer */
    DMA1_Stream6->CR &= ~DMA_SxCR_PSIZE;

    /* MBURST - Single beat transfer from memory */
    DMA1_Stream6->CR &= ~DMA_SxCR_MBURST;

    /* PBURST - Single beat transfer from Peripheral */
    DMA1_Stream6->CR &= ~DMA_SxCR_PBURST;

    /* Peripheral address */
    DMA1_Stream6->PAR = (uint32_t)&USART2->DR;

    /* Enable DMA IRQ */
    NVIC_EnableIRQ(DMA1_Stream6_IRQn);

}

## 10.2 DMA String Transfer from usart.c

void USART2_DMA_SendString(char *str)
{
	uint16_t strln = 0;

	//length of the string
	while(str[strln]){
		strln++;
	}

    /* Disable stream */
    DMA1_Stream6->CR &= ~DMA_SxCR_EN;
    while(DMA1_Stream6->CR & DMA_SxCR_EN);

    /* Clear old flags */
    DMA1->HIFCR =
          DMA_HIFCR_CTCIF6  |
          DMA_HIFCR_CHTIF6  |
          DMA_HIFCR_CTEIF6  |
          DMA_HIFCR_CDMEIF6 |
          DMA_HIFCR_CFEIF6;

    /* Transfer complete interrupt */
    DMA1_Stream6->CR |= DMA_SxCR_TCIE;

    /* configuring to single transfer */
    DMA1_Stream6->CR &= ~DMA_SxCR_MBURST;

    /* Memory address */
    DMA1_Stream6->M0AR = (uint32_t)str;

    /* Number of bytes */
    DMA1_Stream6->NDTR = strln;

    /* Enable DMA stream */
    DMA1_Stream6->CR |= DMA_SxCR_EN;

    /* Enable USART DMA TX */
    USART2->CR3 |= USART_CR3_DMAT;
}

## 10.3 DMA Byte Transfer from usart.c

void USART2_DMA_SendByte(uint8_t c)
{
    /* Disable stream */
    DMA1_Stream6->CR &= ~DMA_SxCR_EN;
    while(DMA1_Stream6->CR & DMA_SxCR_EN);

    /* Clear old flags */
    DMA1->HIFCR =
          DMA_HIFCR_CTCIF6  |
          DMA_HIFCR_CHTIF6  |
          DMA_HIFCR_CTEIF6  |
          DMA_HIFCR_CDMEIF6 |
          DMA_HIFCR_CFEIF6;

    /* Transfer complete interrupt */
    DMA1_Stream6->CR |= DMA_SxCR_TCIE;

    /* configuring to single transfer */
    DMA1_Stream6->CR &= ~DMA_SxCR_MBURST;

    /* Memory address */
    DMA1_Stream6->M0AR = (uint32_t)&c;

    /* Number of bytes */
    DMA1_Stream6->NDTR = 1;

    /* Enable DMA stream */
    DMA1_Stream6->CR |= DMA_SxCR_EN;

    /* Enable USART DMA TX */
    USART2->CR3 |= USART_CR3_DMAT;

}

Recommended correction: USART2_DMA_SendByte still passes the address of local variable c to DMA. Use a static/global byte buffer or wait for completion before returning.

## 10.4 DMA Buffer Transfer from usart.c

void USART2_DMA_SendBuffer(const char *buffer, uint16_t length)
{
    /* Disable stream */
    DMA1_Stream6->CR &= ~DMA_SxCR_EN;
    while(DMA1_Stream6->CR & DMA_SxCR_EN);

    /* Clear old flags */
    DMA1->HIFCR =
          DMA_HIFCR_CTCIF6  |
          DMA_HIFCR_CHTIF6  |
          DMA_HIFCR_CTEIF6  |
          DMA_HIFCR_CDMEIF6 |
          DMA_HIFCR_CFEIF6;

    /* Transfer complete interrupt */
    DMA1_Stream6->CR |= DMA_SxCR_TCIE;

    /* MBURST - 16 beat transfer from memory */
    DMA1_Stream6->CR |= DMA_SxCR_MBURST;

    /* Memory address */
    DMA1_Stream6->M0AR = (uint32_t)buffer;

    /* Number of bytes */
    DMA1_Stream6->NDTR = length;

    /* Enable DMA stream */
    DMA1_Stream6->CR |= DMA_SxCR_EN;

    /* Enable USART DMA TX */
    USART2->CR3 |= USART_CR3_DMAT;

}

## 10.5 DMA FIFO Interrupt Handling from stm32f4xx_it.c

void DMA1_Stream6_IRQHandler(void)
{
    //clear DMA TC flag
    DMA1->HIFCR |= DMA_HIFCR_CTCIF6;

    dma_tx_complete = 1;

    /*Waiting for USART completion*/
    USART2->CR1 |= USART_CR1_TCIE;


    if(DMA1->HISR & DMA_HISR_FEIF6)
    {
        fifo_error_count++;

        DMA1->HIFCR = DMA_HIFCR_CFEIF6;
    }

}

/* USER CODE END 1 */

# 11. DMA FIFO Configuration Deep Dive

FIFO configuration is the main Day 5 learning area. The stream is still DMA1 Stream6 Channel4, memory-to-peripheral, but FCR now enables FIFO mode and selects a threshold. The CR register also explicitly sets transfer sizes and burst behavior.

| Configuration step | Finalized code | Why it matters |
| --- | --- | --- |
| Enable FIFO | DMA1_Stream6->FCR ¦= DMA_SxFCR_DMDIS | DMDIS disables direct mode, which enables FIFO mode. |
| Set threshold | DMA1_Stream6->FCR ¦= DMA_SxFCR_FTH | Chooses when FIFO threshold behavior triggers. |
| Memory size | DMA1_Stream6->CR &= ~DMA_SxCR_MSIZE | Memory transfers are byte-sized. |
| Peripheral size | DMA1_Stream6->CR &= ~DMA_SxCR_PSIZE | USART DR writes are byte-sized. |
| Memory burst | DMA1_Stream6->CR &= ~DMA_SxCR_MBURST | Single memory beat for string/byte APIs. |
| Peripheral burst | DMA1_Stream6->CR &= ~DMA_SxCR_PBURST | Single peripheral beat, suitable for USART DR. |
| Buffer burst variant | DMA1_Stream6->CR ¦= DMA_SxCR_MBURST | SendBuffer attempts memory burst behavior for long payloads. |
| FIFO error check | DMA1->HISR & DMA_HISR_FEIF6 | Detects FIFO configuration/operation error. |

## 11.1 Direct Mode vs FIFO Mode

| Area | Direct mode Day 4 | FIFO mode Day 5 |
| --- | --- | --- |
| FCR DMDIS | Cleared | Set |
| Internal DMA FIFO | Disabled | Enabled |
| Best learning use | Simple byte movement | Longer buffers and FIFO behavior |
| Extra error flag | FIFO error less central | FEIF6 becomes important |
| USART behavior | Unchanged | Unchanged |

## 11.2 FIFO Threshold

The FIFO threshold controls how much FIFO fill level is used before the DMA stream services the transfer path. The PDF describes quarter, half, three-quarter, and full thresholds as tuning options. The finalized code sets the FTH field and comments it as full FIFO.

## 11.3 DMA TC vs USART TC

| Event | Meaning | Handler action in finalized code |
| --- | --- | --- |
| DMA TCIF6 | DMA finished feeding the USART data path. | DMA IRQ clears CTCIF6, sets dma_tx_complete, and enables USART TCIE. |
| USART TC | Final serial frame completed on the wire. | USART IRQ sets tx_complete, clears TCIE, and clears DMAT. |
| DMA FEIF6 | FIFO error occurred. | DMA IRQ increments fifo_error_count and clears CFEIF6. |

Key FIFO lesson: FIFO mode changes DMA-side buffering, not USART-side framing. USART still accepts byte writes into DR and still needs TC if wire-level completion matters.

# 12. Main Algorithm and Flowchart

Startup
  HAL_Init -> clock config -> GPIO init
  USART2_Init -> USART2_DMA_TX_Init call in main.c
  polling TX greeting
  DMA TX startup string
  delay
  USART2_DMA_SendBuffer(long static payload, strlen(payload))

Main loop
  if rx_head != rx_tail:
      msg = USART2_ReadByte()
      tx_complete = 0
      dma_tx_complete = 0
      USART2_DMA_SendByte(msg)

Completion
  DMA TC -> DMA IRQ -> USART TCIE
  USART TC -> USART IRQ -> DMAT clear
  FIFO error -> fifo_error_count++

| Step | Expected observation |
| --- | --- |
| Startup polling TX | USART initialized text appears. |
| Startup DMA string | DMA initialized text appears. |
| Long buffer DMA TX | Large payload transmits through FIFO-mode DMA. |
| RX echo | Received byte echoes through DMA byte transfer. |
| FIFO error watch | fifo_error_count remains zero in a healthy transfer. |

# 13. Code Explanation

| Code action | Meaning | Why it matters |
| --- | --- | --- |
| USART2_DMA_Init | Configures DMA stream with FIFO mode. | Establishes the Day 5 DMA behavior. |
| DMA_SxFCR_DMDIS | Enables FIFO mode. | Main difference from direct mode. |
| DMA_SxFCR_FTH | Sets FIFO threshold. | Tuning point for FIFO operation. |
| MSIZE/PSIZE cleared | Byte-size memory and peripheral transfers. | Matches USART byte data path. |
| USART2_DMA_SendBuffer | Starts DMA transfer for explicit buffer and length. | Used for long payload testing. |
| DMA_HISR_FEIF6 | FIFO error status. | Allows diagnosing FIFO-mode problems. |

# 14. Testing and Validation

| Test | Action | Expected result | Evidence |
| --- | --- | --- | --- |
| Startup text | Run firmware. | USART and DMA initialization lines appear. | Terminal capture. |
| Long buffer transfer | Observe post-delay payload transmission. | Large buffer prints without truncation. | Terminal capture. |
| FIFO register view | Inspect DMA1_Stream6->FCR. | DMDIS and FTH are set. | Debugger watch. |
| DMA completion | Watch dma_tx_complete. | Flag sets after DMA TC. | Debugger watch. |
| USART completion | Watch tx_complete. | Flag sets after USART TC. | Debugger watch. |
| FIFO error | Watch fifo_error_count. | Should remain zero during normal run. | Debugger watch. |

### DMA FIFO Validation Checklist

| Check | Learning or validation item | Evidence |
| --- | --- | --- |
| [ ] | DMA1 clock is enabled. | Notes / capture: |
| [ ] | DMA1_Stream6 EN is cleared before reconfiguration. | Notes / capture: |
| [ ] | HIFCR Stream6 flags are cleared before enabling. | Notes / capture: |
| [ ] | CHSEL is Channel 4. | Notes / capture: |
| [ ] | DIR is memory-to-peripheral. | Notes / capture: |
| [ ] | FCR DMDIS is set for FIFO mode. | Notes / capture: |
| [ ] | FCR FTH is set as intended. | Notes / capture: |
| [ ] | MSIZE and PSIZE are byte-sized. | Notes / capture: |
| [ ] | PAR points to USART2->DR. | Notes / capture: |
| [ ] | NDTR equals the intended byte count. | Notes / capture: |

# 15. Results

| Observed / expected behavior | Interpretation |
| --- | --- |
| DMA initialization string prints. | DMA TX path is functional. |
| Long buffer sends after delay. | FIFO-mode buffer transfer is functional. |
| Received byte echoes through DMA. | RX circular buffer and DMA TX path remain integrated. |
| fifo_error_count remains zero. | FIFO configuration is likely stable for the test payload. |

# 16. Problems Faced and Debugging Notes

| Symptom | Likely cause | Debug action |
| --- | --- | --- |
| Build error on DMA init | main.c calls USART2_DMA_TX_Init but header defines USART2_DMA_Init. | Rename call or function consistently. |
| No DMA output | DMAT not set, stream not enabled, wrong channel/stream. | Check CR3 DMAT and Stream6 CR. |
| FIFO error count increases | FCR, burst, or stream setup issue. | Inspect FCR, MBURST/PBURST, MSIZE/PSIZE. |
| Output truncated | NDTR length wrong or buffer lifetime issue. | Check strlen length and buffer storage duration. |
| Corrupted byte echo | DMA SendByte uses local variable address. | Use static/global byte source. |
| Immediate interrupt | Old flags not cleared. | Clear HIFCR before enabling stream. |

## 16.1 Register Checklist

| Item | Expected state | Quick check |
| --- | --- | --- |
| DMA1_Stream6->CR | CHSEL=4, DIR=M2P, MINC, TCIE | Core DMA stream behavior. |
| DMA1_Stream6->FCR | DMDIS set, FTH set | FIFO mode and threshold enabled. |
| DMA1_Stream6->PAR | &USART2->DR | Fixed peripheral destination. |
| DMA1_Stream6->M0AR | Source buffer address | Valid memory source. |
| DMA1_Stream6->NDTR | Length or 1 | Correct transfer count. |
| USART2->CR3 | DMAT set during transfer | USART DMA request path enabled. |
| DMA1->HISR | TCIF6 or FEIF6 as events occur | Interrupt status source. |

# 17. Limitations

| Limitation | Why it matters | Next improvement |
| --- | --- | --- |
| Init function name mismatch | Attached main and driver/header do not agree. | Use USART2_DMA_Init or rename prototype/definition. |
| Real private key used as payload | Secrets should not be used as test data. | Replace with dummy payload. |
| USART2_DMA_SendByte local pointer | DMA can outlive the stack variable. | Use static/global byte storage. |
| No TX busy state | Overlapping DMA transfers can corrupt output. | Add busy flag or queue. |
| DMA IRQ clears TC before checking status | Other DMA events can be misclassified. | Branch based on HISR flags first. |
| Only FIFO error is counted | TEIF/DMEIF/HTIF are not handled at runtime. | Add full DMA error handling. |

# 18. Future Improvements

| Stage | Upgrade | What changes |
| --- | --- | --- |
| Day 6 | DMA RX | Move receive data movement into DMA. |
| Day 7 | DMA RX circular | Use circular DMA receive memory. |
| Day 8 | Idle detection | Detect frame boundaries without per-byte parsing. |
| Driver polish | Busy-state and safe byte storage | Make TX API production-safe. |
| Diagnostics | Full DMA error handling | Handle TEIF, DMEIF, HTIF, FEIF, and TCIF distinctly. |

### Long-Term Learning Tracker

| Check | Learning or validation item | Evidence |
| --- | --- | --- |
| [ ] | I can explain why DMDIS enables FIFO mode. | Notes / capture: |
| [ ] | I can explain the FTH threshold field. | Notes / capture: |
| [ ] | I can compare direct mode and FIFO mode. | Notes / capture: |
| [ ] | I can debug FEIF6 FIFO errors. | Notes / capture: |
| [ ] | I can explain DMA TC versus USART TC. | Notes / capture: |

# 19. Conclusion

Day 5 adds a DMA-side FIFO stage to the USART transmit path. The CPU still sets up the transfer and handles completion, while DMA manages memory-to-USART movement. The important engineering lesson is that FIFO mode adds configuration power and diagnostics, but it also makes correct FCR, burst, size, flag, and buffer-lifetime handling more important.

Core takeaway: FIFO mode is not a USART feature. It is a DMA stream behavior. USART still shifts one serial frame at a time, while the DMA FIFO stages memory data before writing USART2->DR.

# Appendix A. Actual Source Evidence

| File | Evidence included |
| --- | --- |
| usart.c | FIFO DMA init, SendString, SendByte, and SendBuffer. |
| stm32f4xx_it.c | DMA TC, USART TC, and FIFO error handling. |
| main.c | DMA FIFO startup usage and long buffer transfer call, with secret payload redacted from document. |
| usart.h | DMA FIFO API declarations. |

# Appendix B. One-Page Revision Summary

Day 5 enables DMA FIFO mode by setting DMA_SxFCR_DMDIS.

FIFO threshold is configured through DMA_SxFCR_FTH.

MSIZE and PSIZE are cleared for byte-sized transfers.

DMA1 Stream6 Channel4 remains the USART2 TX DMA path.

DMA FEIF6 is the FIFO error flag and is cleared with CFEIF6.

DMA TC still means DMA completion; USART TC still means final wire completion.