EMBEDDED PROJECT REPORT

USART Day 6 RX DMA Project

STM32F407VG | USART2 RX | DMA1 Stream5 Channel4 | Fixed 16-byte receive buffer | PA3 RX

| Field | Value |
| --- | --- |
| Document purpose | Long-term learning, implementation reference, and DMA RX configuration guide. |
| Project type | Bare-metal USART2 RX DMA receive with TX DMA echo response. |
| Content source | Source files and PDF template document |
| Code authority | Only the attached finalized source files are used for code snippets. |
| Core concept | DMA1 Stream5 transfers incoming USART2->DR bytes into a fixed RAM buffer. |

Implementation truth: The PDF describes RX DMA direct mode, but the finalized attached code sets DMA_SxFCR_DMDIS and FTH for DMA1 Stream5, which enables FIFO mode. The document explains the intended direct-mode concept and the actual source-backed configuration separately.

# 1. Project Objective

The objective of Day 6 is to move receive data movement from CPU-side byte handling into DMA. The application arms DMA1 Stream5 with a memory buffer and length, USART2 receives bytes into DR, DMA copies them into RAM, and the DMA interrupt marks the fixed-length receive complete.

Learning outcome: After this project, you should be able to explain DMA1 Stream5 Channel4, peripheral-to-memory direction, PAR, M0AR, NDTR, DMAR, TCIF5, CTCIF5, and why this version must be rearmed after each fixed-size receive.

### Objective Checkpoint

| Check | Learning or validation item | Evidence |
| --- | --- | --- |
| [ ] | I can explain why USART2 RX uses DMA1 Stream5 Channel4. | Notes / capture: |
| [ ] | I can explain why DIR is cleared for peripheral-to-memory. | Notes / capture: |
| [ ] | I can trace 16 received bytes into dmarx_buffer and back out through DMA TX. | Notes / capture: |

# 2. Problem Statement

Interrupt receive still involves CPU work for every byte. DMA RX removes that repetitive byte movement for known-length payloads. The application arms a buffer, waits for completion, processes the complete block, and rearms receive.

| Problem area | Day 6 response |
| --- | --- |
| Per-byte CPU receive work | DMA copies USART2->DR into RAM. |
| Known-length receive | dmarx_buffer is armed for 16 bytes. |
| Completion handling | DMA1_Stream5_IRQHandler sets dma_rx_complete. |
| Rearming model | Main loop restarts DMA receive after processing each block. |

# 3. System Overview

The receive data path starts at PA3/RX, passes through USART2 receive logic into USART2->DR, then DMA1 Stream5 moves each received byte into dmarx_buffer. When NDTR reaches zero, TCIF5 is set and the DMA ISR signals completion.

PA3/RX -> USART2 receive logic -> USART2->DR
    |
    | USART2->CR3 DMAR enables RX DMA requests
    v
DMA1 Stream5 Channel4
    |-- PAR  = &USART2->DR
    |-- M0AR = dmarx_buffer
    |-- NDTR = 16
    v
dmarx_buffer[16]
    |
    v
DMA1_Stream5_IRQHandler -> dma_rx_complete = 1
    |
    v
main echoes block with USART2_DMA_SendBuffer and rearms receive

| Project focus | What changes from Day 5 |
| --- | --- |
| RX path | DMA handles fixed-length receive data movement. |
| TX path | TX DMA remains available for echoing received buffers. |
| DMA stream | Stream5 is added for USART2_RX. |
| Completion model | DMA TCIF5 means the expected receive buffer is filled. |

# 4. Requirements

## 4.1 Functional Requirements

| ID | Requirement | Acceptance condition |
| --- | --- | --- |
| FR-01 | Initialize USART2 for serial RX/TX. | USART2 has UE, TE, RE, BRR, and 8N1 configuration. |
| FR-02 | Initialize DMA1 Stream5 Channel4 for USART2 RX. | DMA1 clock enabled, stream disabled, flags cleared, CHSEL=4. |
| FR-03 | Set peripheral-to-memory direction. | DMA1_Stream5 CR DIR bits are cleared. |
| FR-04 | Configure receive buffer and length. | M0AR points to dmarx_buffer and NDTR equals 16. |
| FR-05 | Enable USART RX DMA request. | USART2->CR3 has DMAR set during receive. |
| FR-06 | Handle RX DMA completion. | DMA1_Stream5_IRQHandler clears CTCIF5 and sets dma_rx_complete. |

## 4.2 Non-Functional Requirements

| Area | Requirement |
| --- | --- |
| Buffer ownership | Application must not process dmarx_buffer until dma_rx_complete is set. |
| Reconfiguration safety | Stream5 must be disabled and EN cleared before rearming. |
| Flag hygiene | Old Stream5 flags must be cleared before starting receive. |
| Scope clarity | This is fixed-length RX DMA, not circular RX DMA. |

# 5. Hardware Components

| Item | Selection in this project | Purpose |
| --- | --- | --- |
| MCU | STM32F407VG / STM32F407VGT6 family | Target MCU with USART2 and DMA1. |
| USART instance | USART2 | Serial peripheral. |
| RX pin | PA3 AF7 | USART2 receive line. |
| TX pin | PA2 AF7 | USART2 transmit line. |
| DMA controller | DMA1 | DMA controller serving USART2 RX/TX. |
| RX DMA stream | Stream 5 | USART2_RX stream used by finalized code. |
| RX DMA channel | Channel 4 | USART2_RX channel selection. |
| RX buffer | dmarx_buffer[16] | Fixed-size receive block. |

# 6. Software and Firmware Tools

| Source file | Verified role |
| --- | --- |
| Core/Src/main.c | Declares dmarx_buffer and dma_rx_complete; arms RX DMA and echoes completed blocks. |
| Core/Src/usart.c | Configures USART2, DMA TX, DMA RX, and receive buffer API. |
| Core/Src/stm32f4xx_it.c | Handles DMA1 Stream5 receive completion and existing TX/RX interrupts. |
| Core/Inc/usart.h | Declares USART2_DMA_RX_Init and USART2_DMA_ReceiveBuffer. |
| Core/Inc/stm32f4xx_it.h | Declares DMA1_Stream5_IRQHandler. |

Reasoning correction: The finalized RX DMA init configures FIFO mode even though the PDF title says direct mode. For strict direct mode, clear DMDIS instead of setting it. Keep the current code if your Day 6 intent is RX DMA with FIFO enabled.

# 7. System Block Diagram

RX setup
  USART2_DMA_RX_Init()
    DMA1 Stream5 Channel4
    DIR = peripheral-to-memory
    PAR = &USART2->DR
    NVIC_EnableIRQ(DMA1_Stream5_IRQn)

Receive start
  USART2_DMA_ReceiveBuffer(dmarx_buffer, 16)
    M0AR = dmarx_buffer
    NDTR = 16
    EN = 1
    USART2->CR3 DMAR = 1

Completion
  TCIF5 -> DMA1_Stream5_IRQHandler
  CTCIF5 clear -> dma_rx_complete = 1

| Register / bit | Meaning | Used for |
| --- | --- | --- |
| DMA1_Stream5->CR | Stream control register. | Channel, direction, increment, size, TC interrupt. |
| DMA1_Stream5->PAR | Peripheral address. | Fixed at USART2->DR. |
| DMA1_Stream5->M0AR | Memory destination address. | dmarx_buffer or caller buffer. |
| DMA1_Stream5->NDTR | Expected receive count. | 16 in current main usage. |
| USART_CR3_DMAR | USART RX DMA request enable. | Allows RX data to trigger DMA writes into RAM. |
| DMA_HISR_TCIF5 | Transfer complete flag. | Signals full buffer received. |
| DMA_HIFCR_CTCIF5 | Transfer complete clear bit. | Clears Stream5 TC flag. |

# 8. Firmware Architecture

The firmware now has separate DMA streams for transmit and receive. Stream6 handles USART2 TX, while Stream5 handles USART2 RX. The main loop treats dma_rx_complete as a block-ready signal.

main.c usage excerpt:
USART2_Init();
USART2_DMA_TX_Init();
USART2_DMA_RX_Init();
USART2_DMA_ReceiveBuffer(dmarx_buffer, 16);

while (1)
{
    if(dma_rx_complete)
    {
        dma_rx_complete = 0;
        USART2_DMA_SendBuffer(dmarx_buffer, 16);
        USART2_DMA_ReceiveBuffer(dmarx_buffer, 16);
    }
}

Architecture decision: This is a ping-style fixed block receive: fill 16 bytes, echo 16 bytes, rearm 16 bytes. It is not continuous streaming yet; Day 7 circular DMA is the natural next step.

# 9. Communication Protocol Used

The USART frame remains 8N1. RX DMA does not change the wire protocol; it changes how received bytes are moved from USART2->DR into memory.

| Parameter | Value from finalized code | Engineering note |
| --- | --- | --- |
| BRR | 364 | Current project baud-register value. |
| Frame | 8N1 | M/PCE cleared and STOP cleared. |
| RX DMA mapping | DMA1 Stream5 Channel4 | USART2_RX mapping on STM32F407 family. |
| RX direction | Peripheral to memory | DIR bits cleared. |
| Receive length | 16 bytes | Fixed block receive in main.c. |
| Request bit | USART_CR3_DMAR | Enables USART RX DMA request path. |

# 10. Module-Wise Implementation

## 10.0 Header API from usart.h

#define RX_BUFFER_SIZE 64

void USART2_Init(void);
void USART2_SendString(char *s);
void USART2_SendByte(char s);
char USART2_ReceiveByte(void);
uint8_t USART2_ReadByte(void);
void USART2_DMA_TX_Init(void);
void USART2_DMA_SendString(char *str);
void USART2_DMA_SendByte(uint8_t c);
void USART2_DMA_SendBuffer(const char *buffer, uint16_t length);
void USART2_DMA_RX_Init(void);
void USART2_DMA_ReceiveBuffer(char *buffer, uint16_t length);


#endif /* INC_USART_H_ */

## 10.1 DMA RX State from main.c

char dmarx_buffer[16];
volatile uint8_t dma_rx_complete;

## 10.2 DMA RX Initialization from usart.c

void USART2_DMA_RX_Init(void)
{
	/* Enable clock for DMA1 peripheral */
    RCC->AHB1ENR |= RCC_AHB1ENR_DMA1EN;

    /* Disable stream */
    DMA1_Stream5->CR &= ~DMA_SxCR_EN;
    while(DMA1_Stream5->CR & DMA_SxCR_EN);

    /* Clear old flags */
    DMA1->HIFCR =
          DMA_HIFCR_CTCIF5  |
          DMA_HIFCR_CHTIF5  |
          DMA_HIFCR_CTEIF5  |
          DMA_HIFCR_CDMEIF5 |
          DMA_HIFCR_CFEIF5;

    /* Clear CR */
    DMA1_Stream5->CR = 0;

    /* Channel 4 */
    DMA1_Stream5->CR |=
            (4UL << DMA_SxCR_CHSEL_Pos);

    /* peripheral to memory */
    DMA1_Stream5->CR &= ~DMA_SxCR_DIR;

    /* Memory increment */
    DMA1_Stream5->CR |= DMA_SxCR_MINC;

    /* Peripheral increment disabled */
    DMA1_Stream5->CR &= ~DMA_SxCR_PINC;

    /* Transfer complete interrupt */
    DMA1_Stream5->CR |= DMA_SxCR_TCIE;

    /* FIFO Mode Enabled */
    DMA1_Stream5->FCR |= DMA_SxFCR_DMDIS;

    /* FIFO Threshold - Full FIFO */
    DMA1_Stream5->FCR |= DMA_SxFCR_FTH;

    /* MSIZE - 1 Byte transfer */
    DMA1_Stream5->CR &= ~DMA_SxCR_MSIZE;

    /* PSIZE - 1 Byte transfer */
    DMA1_Stream5->CR &= ~DMA_SxCR_PSIZE;

    /* MBURST - Single beat transfer from memory */
    DMA1_Stream5->CR &= ~DMA_SxCR_MBURST;

    /* PBURST - Single beat transfer from Peripheral */
    DMA1_Stream5->CR &= ~DMA_SxCR_PBURST;

    /* Peripheral address */
    DMA1_Stream5->PAR = (uint32_t)&USART2->DR;

    /* Enable DMA IRQ */
    NVIC_EnableIRQ(DMA1_Stream5_IRQn);

}

## 10.3 DMA Receive Start from usart.c

void USART2_DMA_ReceiveBuffer(char *buffer, uint16_t length)
{
    /* Disable stream */
    DMA1_Stream5->CR &= ~DMA_SxCR_EN;
    while(DMA1_Stream5->CR & DMA_SxCR_EN);

    /* Clear old flags */
    DMA1->HIFCR =
          DMA_HIFCR_CTCIF5  |
          DMA_HIFCR_CHTIF5  |
          DMA_HIFCR_CTEIF5  |
          DMA_HIFCR_CDMEIF5 |
          DMA_HIFCR_CFEIF5;

    /* Transfer complete interrupt */
    DMA1_Stream5->CR |= DMA_SxCR_TCIE;

    /* MBURST - 1 beat transfer from memory */
    DMA1_Stream5->CR &= ~DMA_SxCR_MBURST;

    /* Memory address */
    DMA1_Stream5->M0AR = (uint32_t)buffer;

    /* Number of bytes */
    DMA1_Stream5->NDTR = length;

    /* Enable DMA stream */
    DMA1_Stream5->CR |= DMA_SxCR_EN;

    /* Enable USART DMA RX */
    USART2->CR3 |= USART_CR3_DMAR;

}

## 10.4 DMA RX Completion Handler from stm32f4xx_it.c

void DMA1_Stream5_IRQHandler(void)
{
    if(DMA1->HISR & DMA_HISR_TCIF5)
    {
    	//clear TC interrupt
        DMA1->HIFCR = DMA_HIFCR_CTCIF5;

        dma_rx_complete = 1;
    }
}

/* USER CODE END 1 */

# 11. DMA RX Configuration Deep Dive

DMA RX configuration is the core Day 6 skill. The stream must be disabled, old flags cleared, Channel4 selected, direction set to peripheral-to-memory, memory increment enabled, peripheral increment disabled, transfer complete interrupt enabled, and USART DMAR enabled after M0AR/NDTR are prepared.

| Configuration step | Finalized code | Why it matters |
| --- | --- | --- |
| Enable DMA1 | RCC->AHB1ENR ¦= RCC_AHB1ENR_DMA1EN | DMA registers require the DMA controller clock. |
| Disable Stream5 | DMA1_Stream5->CR &= ~DMA_SxCR_EN | Stream config should change only while disabled. |
| Wait EN clear | while(DMA1_Stream5->CR & DMA_SxCR_EN) | Prevents reconfiguring active DMA. |
| Clear old flags | DMA1->HIFCR = CTCIF5 ¦ CHTIF5 ¦ CTEIF5 ¦ CDMEIF5 ¦ CFEIF5 | Avoids stale completion/error flags. |
| Select Channel4 | (4UL << DMA_SxCR_CHSEL_Pos) | Maps Stream5 to USART2_RX. |
| Direction P2M | DMA1_Stream5->CR &= ~DMA_SxCR_DIR | Peripheral-to-memory receive direction. |
| Memory increment | DMA_SxCR_MINC | Stores each byte into the next RAM location. |
| Peripheral fixed | ~DMA_SxCR_PINC | USART2->DR address must not increment. |
| Transfer complete IRQ | DMA_SxCR_TCIE | Generates interrupt when NDTR reaches zero. |
| Peripheral address | PAR = (uint32_t)&USART2->DR | DMA source is USART data register. |
| Memory address | M0AR = (uint32_t)buffer | DMA destination is caller-provided buffer. |
| Receive count | NDTR = length | Number of bytes expected. |
| USART RX request | USART2->CR3 ¦= USART_CR3_DMAR | USART RX events can request DMA service. |

## 11.1 Direct Mode Versus Finalized FIFO Setting

| Area | PDF direct-mode concept | Finalized attached code |
| --- | --- | --- |
| FIFO control | DMDIS cleared | DMDIS set |
| Threshold | Not used | FTH set |
| Learning meaning | Simplest RX DMA path | RX DMA with FIFO enabled |
| Document treatment | Explained as reference concept | Used as the source-backed implementation |

## 11.2 Fixed-Length Receive Ownership

During DMA receive, dmarx_buffer belongs to DMA. The application should wait until dma_rx_complete becomes 1 before echoing or parsing the buffer. After processing, the code rearms the same buffer for the next 16-byte receive.

Key DMA RX lesson: DMA TCIF5 means the expected number of bytes reached RAM. It does not mean the bytes have been parsed, validated, null-terminated, or framed by a protocol.

# 12. Main Algorithm and Flowchart

Startup
  USART2_Init
  USART2_DMA_TX_Init
  USART2_DMA_RX_Init
  TX greeting and long test payload
  USART2_DMA_ReceiveBuffer(dmarx_buffer, 16)

Main loop
  if dma_rx_complete:
      dma_rx_complete = 0
      USART2_DMA_SendBuffer(dmarx_buffer, 16)
      USART2_DMA_ReceiveBuffer(dmarx_buffer, 16)

RX DMA IRQ
  if TCIF5:
      clear CTCIF5
      dma_rx_complete = 1

| Step | Expected observation |
| --- | --- |
| Arm RX DMA | Stream5 EN and USART DMAR are set. |
| Receive 16 bytes | NDTR decrements to zero. |
| DMA IRQ fires | dma_rx_complete becomes 1. |
| Main echoes block | The same 16 bytes are transmitted back. |
| Rearm receive | Stream5 is configured again for the next 16 bytes. |

# 13. Code Explanation

| Code action | Meaning | Why it matters |
| --- | --- | --- |
| USART2_DMA_RX_Init | One-time RX DMA Stream5 setup. | Establishes channel, direction, sizes, increment, PAR, and IRQ. |
| USART2_DMA_ReceiveBuffer | Starts a fixed-length RX DMA transfer. | Sets M0AR, NDTR, EN, and DMAR. |
| DMA1_Stream5_IRQHandler | Handles receive DMA complete. | Clears TCIF5 and signals main loop. |
| dmarx_buffer[16] | Receive destination buffer. | Stores one fixed-size block. |
| dma_rx_complete | ISR-to-main completion flag. | Main loop knows when buffer is ready. |
| USART2_DMA_SendBuffer | Echoes completed receive block. | Validates RX DMA by sending block back. |

# 14. Testing and Validation

| Test | Action | Expected result | Evidence |
| --- | --- | --- | --- |
| Startup | Run firmware. | Initialization/DMA strings transmit. | Terminal capture. |
| 16-byte receive | Send exactly 16 bytes. | dma_rx_complete sets and block echoes. | Terminal capture. |
| NDTR watch | Observe DMA1_Stream5->NDTR. | Counts down from 16 to 0. | Debugger watch. |
| DMAR bit | Inspect USART2->CR3. | DMAR is set while receive is armed. | Debugger watch. |
| Rearm behavior | Send another 16-byte block. | Second block also echoes. | Terminal capture. |

### DMA RX Validation Checklist

| Check | Learning or validation item | Evidence |
| --- | --- | --- |
| [ ] | DMA1 clock is enabled. | Notes / capture: |
| [ ] | DMA1_Stream5 EN is cleared before reconfiguration. | Notes / capture: |
| [ ] | Stream5 flags are cleared before arming receive. | Notes / capture: |
| [ ] | CHSEL is Channel 4. | Notes / capture: |
| [ ] | DIR is peripheral-to-memory. | Notes / capture: |
| [ ] | MINC is enabled and PINC is disabled. | Notes / capture: |
| [ ] | PAR points to USART2->DR. | Notes / capture: |
| [ ] | M0AR points to dmarx_buffer. | Notes / capture: |
| [ ] | NDTR equals 16. | Notes / capture: |
| [ ] | USART2->CR3 DMAR is set. | Notes / capture: |

# 15. Results

| Observed / expected behavior | Interpretation |
| --- | --- |
| 16 bytes echo after block completion. | RX DMA transfer and TX DMA response path are integrated. |
| dma_rx_complete sets at NDTR zero. | DMA Stream5 completion interrupt is working. |
| Second block can be received after rearm. | Fixed-block receive cycle is repeatable. |
| Byte-by-byte RX is no longer required for block test. | DMA handles receive data movement. |

# 16. Problems Faced and Debugging Notes

| Symptom | Likely cause | Debug action |
| --- | --- | --- |
| No bytes in dmarx_buffer | DMAR not set, Stream5 not enabled, wrong channel/stream. | Check CR3 DMAR, Stream5 EN, CHSEL=4. |
| DMA IRQ never fires | NDTR never reaches zero or IRQ not enabled. | Send exact length and check NVIC_EnableIRQ(DMA1_Stream5_IRQn). |
| Only partial data received | Sender did not send full 16 bytes or length mismatch. | Check NDTR and terminal send count. |
| Immediate DMA IRQ | Old Stream5 flags not cleared. | Clear HIFCR before enabling stream. |
| Data overwritten before processing | Buffer rearmed too early. | Only rearm after processing or copying data. |
| Mode mismatch confusion | PDF says direct mode, source enables FIFO. | Decide whether to clear or set DMDIS for your intended lesson. |

## 16.1 Register Checklist

| Item | Expected state | Quick check |
| --- | --- | --- |
| DMA1_Stream5->CR | CHSEL=4, DIR=P2M, MINC, TCIE, EN during receive | Core RX DMA config. |
| DMA1_Stream5->PAR | &USART2->DR | Fixed peripheral source. |
| DMA1_Stream5->M0AR | dmarx_buffer | RAM destination. |
| DMA1_Stream5->NDTR | 16 then decrements | Expected byte count. |
| USART2->CR3 | DMAR set during receive | USART RX DMA request path enabled. |
| DMA1->HISR | TCIF5 after 16 bytes | Completion source. |
| dma_rx_complete | 1 after DMA IRQ | Main-loop signal. |

# 17. Limitations

| Limitation | Why it matters | Next improvement |
| --- | --- | --- |
| Fixed 16-byte receive only | Variable-length input waits forever until 16 bytes arrive. | Add timeout or idle-line detection. |
| Manual rearm | Bytes arriving between completion and rearm can be missed. | Use circular DMA for continuous RX. |
| PDF/direct-mode mismatch | Source enables FIFO while document title says direct mode. | Choose one mode intentionally. |
| No RX DMA error handling | TEIF/DMEIF/FEIF are not handled for Stream5. | Add runtime error branches. |
| DMAR not cleared on completion | USART RX DMA request remains enabled after TC. | Consider clearing DMAR before processing if not immediately rearming. |
| Private key test payload remains in main.c | Secret-looking data should not be used as test payload. | Replace with dummy text fixture. |

# 18. Future Improvements

| Stage | Upgrade | What changes |
| --- | --- | --- |
| Day 7 | DMA RX circular mode | Receive runs continuously without manual rearm gaps. |
| Day 8 | Idle-line detection | Variable-length frames can be detected. |
| Driver polish | RX busy state and error handling | Prevents overlapping starts and records DMA failures. |
| Protocol layer | Frame parser | Process complete received blocks safely. |

### Long-Term Learning Tracker

| Check | Learning or validation item | Evidence |
| --- | --- | --- |
| [ ] | I can configure DMA1 Stream5 Channel4 for USART2 RX. | Notes / capture: |
| [ ] | I can explain PAR, M0AR, NDTR, DMAR, and TCIF5. | Notes / capture: |
| [ ] | I can explain why fixed-length RX must be rearmed. | Notes / capture: |
| [ ] | I can explain the difference between direct mode and the finalized FIFO setting. | Notes / capture: |

# 19. Conclusion

Day 6 moves the receive side into DMA. The key value is that USART2 can fill a RAM buffer without CPU byte handling, and the application can react once the expected block has arrived. The finalized code demonstrates fixed-length block reception and immediate rearming, which is a strong bridge toward circular DMA and idle detection.

Core takeaway: RX DMA is a buffer-ownership contract. DMA owns the buffer until TCIF5; the application owns it after completion; then DMA must be armed again for the next fixed-length block.

# Appendix A. Actual Source Evidence

| File | Evidence included |
| --- | --- |
| usart.c | USART2_DMA_RX_Init and USART2_DMA_ReceiveBuffer. |
| stm32f4xx_it.c | DMA1_Stream5_IRQHandler completion path. |
| main.c | dmarx_buffer, dma_rx_complete, RX rearm and echo usage; secret payload redacted from document. |
| usart.h | RX DMA API declarations. |

# Appendix B. One-Page Revision Summary

USART2 RX uses DMA1 Stream5 Channel4.

PAR points to USART2->DR, M0AR points to dmarx_buffer, and NDTR is set to 16.

DIR is cleared for peripheral-to-memory transfer.

USART2->CR3 DMAR enables USART RX DMA requests.

TCIF5 means the fixed receive buffer is full and dma_rx_complete is set.

The finalized code enables FIFO settings even though the PDF describes direct mode.