EMBEDDED PROJECT REPORT

USART Day 7 DMA RX Circular Mode Project

STM32F407VG | USART2 RX | DMA1 Stream5 Channel4 | Hardware Circular Buffer | NDTR Head Tracking

| Field | Value |
| --- | --- |
| Document purpose | Long-term learning, implementation reference, and hardware circular DMA RX guide. |
| Project type | Bare-metal USART2 DMA RX circular mode with NDTR-polled software tail. |
| Content source | Source code and PDF template |
| Code authority | Only the attached finalized source files are used for code snippets. |
| Core concept | DMA writes continuously into a hardware circular buffer; software derives head from NDTR and owns rx_tail. |

Implementation truth: The finalized code uses DMA1 Stream5 circular mode (`CIRC`) and uses NDTR polling in the main loop to derive the DMA write head from `DMA1_Stream5->NDTR`. It does not use RX DMA HT/TC interrupts yet; `DMA1_Stream5_IRQHandler` is empty.

# 1. Project Objective

The objective of Day 7 is to convert fixed-length RX DMA into continuous hardware circular reception. DMA stays armed, wraps automatically at the end of the buffer, and the application consumes bytes by tracking its own software tail.

Learning outcome: After this project, you should be able to explain `DMA_SxCR_CIRC`, `NDTR` reload behavior, hardware write-head derivation, software tail tracking, wraparound, and how this differs from the Day 3 software circular buffer.

### Objective Checkpoint

| Check | Learning or validation item | Evidence |
| --- | --- | --- |
| [ ] | I can explain how DMA circular mode removes manual RX rearming. | Notes / capture: |
| [ ] | I can calculate the DMA write head from RX_BUFFER_SIZE and NDTR. | Notes / capture: |
| [ ] | I can explain why rx_tail still belongs to application code. | Notes / capture: |

# 2. Problem Statement

Day 6 fixed-length RX DMA still had a rearm gap after every completed block. Continuous streams can lose bytes during that gap. Day 7 solves this by enabling DMA circular mode so the hardware write pointer wraps automatically and reception stays active.

| Problem area | Day 7 response |
| --- | --- |
| Manual RX rearm gap | Use DMA circular mode so Stream5 keeps receiving. |
| Continuous stream | DMA writes into dma_rx_buffer repeatedly. |
| Software read tracking | Application tracks rx_tail separately. |
| Write head visibility | Software derives dma_head from NDTR. |

# 3. System Overview

DMA1 Stream5 receives USART2 bytes into `dma_rx_buffer[RX_BUFFER_SIZE]`. When the stream reaches the end, hardware wraps back to index 0. The main loop calculates the current DMA write position and consumes unread bytes one by one.

USART2->DR
  |
  v
DMA1 Stream5 Channel4
  |-- CIRC = 1
  |-- M0AR = dma_rx_buffer
  |-- NDTR = RX_BUFFER_SIZE
  v
dma_rx_buffer[0..63]
  ^       |
  |       v
wrap    dma_head = (RX_BUFFER_SIZE - DMA1_Stream5->NDTR) % RX_BUFFER_SIZE

Application:
  if dma_head != rx_tail:
      msg = dma_rx_buffer[rx_tail]
      USART2_DMA_SendByte(msg)
      rx_tail++ with wrap

| Project focus | What changes from Day 6 |
| --- | --- |
| Receive mode | Fixed block RX becomes continuous circular RX. |
| Completion concept | No final completion event is required for normal operation. |
| Head source | DMA hardware head is inferred from NDTR. |
| Tail source | Application maintains rx_tail. |

# 4. Requirements

## 4.1 Functional Requirements

| ID | Requirement | Acceptance condition |
| --- | --- | --- |
| FR-01 | Configure USART2 and DMA1 Stream5 Channel4 for RX. | USART2 and DMA1 clocks/configuration are valid. |
| FR-02 | Enable hardware circular DMA mode. | DMA1_Stream5->CR has DMA_SxCR_CIRC set. |
| FR-03 | Start circular reception once. | USART2_DMA_CircularReceive(dma_rx_buffer, RX_BUFFER_SIZE) is called. |
| FR-04 | Derive DMA write head from NDTR. | main.c calculates dma_head = (RX_BUFFER_SIZE - DMA1_Stream5->NDTR) % RX_BUFFER_SIZE. |
| FR-05 | Consume bytes with software tail. | main reads dma_rx_buffer[rx_tail] and wraps rx_tail. |
| FR-06 | Avoid overlapping TX DMA byte sends. | main checks tx_complete before starting USART2_DMA_SendByte. |

## 4.2 Non-Functional Requirements

| Area | Requirement |
| --- | --- |
| Continuous reception | DMA RX should remain active without per-frame rearm. |
| Ownership | DMA owns write position; application owns read position. |
| Buffer safety | Application must not overtake the DMA write position. |
| Scope clarity | Current code uses NDTR polling, not HT/TC RX DMA interrupts. |

# 5. Hardware Components

| Item | Selection in this project | Purpose |
| --- | --- | --- |
| MCU | STM32F407VG / STM32F407VGT6 family | Target MCU with USART2 and DMA1. |
| USART instance | USART2 | Serial peripheral. |
| RX pin | PA3 AF7 | USART2 receive line. |
| TX pin | PA2 AF7 | Echo transmit line. |
| DMA controller | DMA1 | DMA controller serving USART2 RX/TX. |
| RX DMA stream | Stream 5 | USART2_RX stream. |
| RX DMA channel | Channel 4 | USART2_RX channel selection. |
| RX circular buffer | dma_rx_buffer[64] | Hardware-filled circular receive buffer. |

# 6. Software and Firmware Tools

| Source file | Verified role |
| --- | --- |
| Core/Src/main.c | Declares dma_rx_buffer, dma_head, rx_tail; calculates head from NDTR and echoes unread bytes. |
| Core/Src/usart.c | Configures DMA1 Stream5 circular mode and starts circular receive. |
| Core/Src/stm32f4xx_it.c | Contains USART TX completion and TX DMA IRQ; RX DMA IRQ is empty in this finalized design. |
| Core/Inc/usart.h | Declares USART2_DMA_CircularReceive API. |

Reasoning correction: The PDF discusses HT/TC checkpoint interrupts, but your finalized code does not enable HTIE/TCIE for RX circular mode and leaves Stream5 IRQ empty. That is fine for an NDTR-polling design, but the document separates those approaches clearly.

# 7. System Block Diagram

RX circular setup
  USART2_DMA_RX_Init()
    Stream5 configured with CIRC
    PAR = &USART2->DR

Start once
  USART2_DMA_CircularReceive(dma_rx_buffer, RX_BUFFER_SIZE)
    M0AR = dma_rx_buffer
    NDTR = 64
    EN = 1
    DMAR = 1

Runtime
  DMA writes continuously
  main derives dma_head from NDTR
  main consumes from rx_tail
  TX DMA echoes one byte at a time when tx_complete is true

| Register / variable | Meaning | Used for |
| --- | --- | --- |
| DMA_SxCR_CIRC | Circular mode enable. | Hardware wraps to buffer start automatically. |
| DMA1_Stream5->NDTR | Remaining items before wrap. | Derives current write head. |
| DMA1_Stream5->M0AR | Start of circular buffer. | Points to dma_rx_buffer. |
| DMA1_Stream5->PAR | Peripheral source. | Points to USART2->DR. |
| USART_CR3_DMAR | USART RX DMA request enable. | Allows USART2 RX to request DMA. |
| dma_head | Derived hardware write position. | Main-loop comparison against rx_tail. |
| rx_tail | Software read position. | Application consumes bytes. |

# 8. Firmware Architecture

The Day 7 architecture is a hardware producer/software consumer model. DMA is the producer and writes continuously into RAM. The application is the consumer and reads from `rx_tail` only when `dma_head != rx_tail`.

Main loop core:
dma_head = (RX_BUFFER_SIZE - DMA1_Stream5->NDTR) % RX_BUFFER_SIZE;

if(dma_head != rx_tail)
{
    if(tx_complete)
    {
        tx_complete = 0;
        dma_tx_complete = 0;
        msg = dma_rx_buffer[rx_tail];
        USART2_DMA_SendByte(msg);
        rx_tail++;
        if(rx_tail >= RX_BUFFER_SIZE) rx_tail = 0;
    }
}

Architecture decision: This version does not wait for fixed-size receive completion. It continuously checks whether DMA has written ahead of the software tail.

# 9. Communication Protocol Used

The USART wire format remains 8N1. Circular DMA changes the receive storage model, not the serial protocol.

| Parameter | Value from finalized code | Engineering note |
| --- | --- | --- |
| RX DMA mapping | DMA1 Stream5 Channel4 | USART2_RX mapping. |
| RX direction | Peripheral to memory | DIR bits cleared. |
| Circular mode | DMA_SxCR_CIRC set | Hardware wraps buffer automatically. |
| Buffer size | RX_BUFFER_SIZE = 64 | NDTR reload size and modulo base. |
| Head formula | (64 - NDTR) % 64 | Current DMA write position. |
| Request bit | USART_CR3_DMAR | Enables USART RX DMA requests. |

# 10. Module-Wise Implementation

## 10.0 Header API from usart.h

#define RX_BUFFER_SIZE 64

void USART2_Init(void);
void USART2_SendString(char *s);
void USART2_SendByte(char s);
char USART2_ReceiveByte(void);
uint8_t USART2_ReadByte(void);
void USART2_DMA_TX_Init(void);
void USART2_DMA_SendString(char *str);
void USART2_DMA_SendByte(uint8_t c);
void USART2_DMA_SendBuffer(const char *buffer, uint16_t length);
void USART2_DMA_RX_Init(void);
void USART2_DMA_ReceiveBuffer(char *buffer, uint16_t length);
void USART2_DMA_CircularReceive(uint8_t *buffer, uint16_t length);

#endif /* INC_USART_H_ */

## 10.1 Main Circular DMA State from main.c

volatile uint8_t rx_buffer[RX_BUFFER_SIZE];
volatile uint16_t dma_head = 0;
volatile uint16_t rx_tail = 0;
uint8_t dma_rx_buffer[RX_BUFFER_SIZE];

## 10.2 DMA RX Circular Init from usart.c

void USART2_DMA_RX_Init(void)
{
	/* Enable clock for DMA1 peripheral */
    RCC->AHB1ENR |= RCC_AHB1ENR_DMA1EN;

    /* Disable stream */
    DMA1_Stream5->CR &= ~DMA_SxCR_EN;
    while(DMA1_Stream5->CR & DMA_SxCR_EN);

    /* Clear old flags */
    DMA1->HIFCR =
          DMA_HIFCR_CTCIF5  |
          DMA_HIFCR_CHTIF5  |
          DMA_HIFCR_CTEIF5  |
          DMA_HIFCR_CDMEIF5 |
          DMA_HIFCR_CFEIF5;

    /* Clear CR */
    DMA1_Stream5->CR = 0;

    /* Channel 4 */
    DMA1_Stream5->CR |=
            (4UL << DMA_SxCR_CHSEL_Pos);

    /* peripheral to memory */
    DMA1_Stream5->CR &= ~DMA_SxCR_DIR;

    /* Memory increment */
    DMA1_Stream5->CR |= DMA_SxCR_MINC;

    /* Peripheral increment disabled */
    DMA1_Stream5->CR &= ~DMA_SxCR_PINC;

    /* Transfer complete interrupt */

    /* FIFO Mode Enabled */


    /* FIFO Threshold - Full FIFO */


    DMA1_Stream5->FCR = 0;

    /* MSIZE - 1 Byte transfer */
    DMA1_Stream5->CR &= ~DMA_SxCR_MSIZE;

    /* PSIZE - 1 Byte transfer */
    DMA1_Stream5->CR &= ~DMA_SxCR_PSIZE;

    /* MBURST - Single beat transfer from memory */
    DMA1_Stream5->CR &= ~DMA_SxCR_MBURST;

    /* PBURST - Single beat transfer from Peripheral */
    DMA1_Stream5->CR &= ~DMA_SxCR_PBURST;

    /* Enable circular mode */
    DMA1_Stream5->CR |= DMA_SxCR_CIRC;

    /* Peripheral address */
    DMA1_Stream5->PAR = (uint32_t)&USART2->DR;

    /* Enable DMA IRQ */

}

## 10.3 Circular Receive Start from usart.c

void USART2_DMA_CircularReceive(uint8_t *buffer, uint16_t length)
{
    /* Disable stream */
    DMA1_Stream5->CR &= ~DMA_SxCR_EN;
    while(DMA1_Stream5->CR & DMA_SxCR_EN);

    /* Clear old flags */
    DMA1->HIFCR =
          DMA_HIFCR_CTCIF5  |
          DMA_HIFCR_CHTIF5  |
          DMA_HIFCR_CTEIF5  |
          DMA_HIFCR_CDMEIF5 |
          DMA_HIFCR_CFEIF5;

    /* MBURST - 1 beat transfer from memory */
    DMA1_Stream5->CR &= ~DMA_SxCR_MBURST;

    /* Memory address */
    DMA1_Stream5->M0AR = (uint32_t)buffer;

    /* Number of bytes */
    DMA1_Stream5->NDTR = length;

    /* Enable DMA stream */
    DMA1_Stream5->CR |= DMA_SxCR_EN;

    /* Enable USART DMA RX */
    USART2->CR3 |= USART_CR3_DMAR;

}

## 10.4 Main Loop NDTR Tracking from main.c

/* calculating the head */
	  dma_head = (RX_BUFFER_SIZE - DMA1_Stream5->NDTR) % RX_BUFFER_SIZE;

	  /* if data is received */
	  if(dma_head != rx_tail)
	  {
		  if(tx_complete)
		  {
			  tx_complete = 0;
			  dma_tx_complete = 0;

			  msg = dma_rx_buffer[rx_tail];
			  USART2_DMA_SendByte(msg);

			  rx_tail++;

			  if(rx_tail >= RX_BUFFER_SIZE)
			  {
				  rx_tail = 0;
			  }

		  }
	  }

## 10.5 RX DMA IRQ from stm32f4xx_it.c

void DMA1_Stream5_IRQHandler(void)
{

}

/* USER CODE END 1 */

# 11. DMA RX Hardware Circular Buffer Deep Dive

This is the important Day 7 concept. In software circular buffers, code increments the head. In DMA circular mode, hardware increments the write position internally. Software cannot directly read a simple `head` variable from DMA, so it derives the current write index from NDTR.

This finalized implementation uses an NDTR polling strategy: the main loop repeatedly reads DMA1_Stream5->NDTR, derives the DMA write head, and compares it with rx_tail to find unread bytes.

| Concept | Expression / setting | Meaning |
| --- | --- | --- |
| Hardware circular enable | DMA1_Stream5->CR ¦= DMA_SxCR_CIRC | DMA reloads and wraps automatically. |
| DMA write head | (RX_BUFFER_SIZE - NDTR) % RX_BUFFER_SIZE | Current hardware write position. |
| Software read tail | rx_tail | Next byte the application will consume. |
| Data available | dma_head != rx_tail | DMA has written at least one unread byte. |
| Wrap handling | if(rx_tail >= RX_BUFFER_SIZE) rx_tail = 0 | Software tail loops through buffer. |
| Continuous receive | No rearm in main loop | Stream remains active. |

## 11.1 NDTR-Based Head Calculation

NDTR counts down from the configured buffer length toward zero. In circular mode, when NDTR reaches zero, DMA reloads it and continues from the start of the buffer. Therefore, `RX_BUFFER_SIZE - NDTR` tells software how far DMA has advanced within the current lap.

| NDTR | Derived dma_head | Interpretation |
| --- | --- | --- |
| 64 | 0 | DMA is at the beginning or just reloaded. |
| 63 | 1 | One byte has been written. |
| 32 | 32 | Half of the buffer has been filled in this lap. |
| 1 | 63 | DMA is near the end. |
| 0/reload | 0 after reload | DMA wraps back to index 0. |

## 11.2 Hardware Circular vs Software Circular

| Area | Day 3 software circular buffer | Day 7 hardware circular DMA |
| --- | --- | --- |
| Producer | USART ISR writes bytes | DMA hardware writes bytes. |
| Head update | Software increments rx_head | Hardware position inferred from NDTR. |
| Tail update | Application increments rx_tail | Application increments rx_tail. |
| CPU per byte | ISR runs for each byte | No RX ISR per byte. |
| Risk | ISR overflow if tail too slow | DMA overwrite if tail too slow. |

Key circular DMA lesson: Circular DMA removes the rearm gap, but it does not remove the need for software flow control. If software tail falls too far behind, DMA can wrap and overwrite unread data.

# 12. Main Algorithm and Flowchart

Startup
  USART2_Init
  USART2_DMA_TX_Init
  USART2_DMA_RX_Init
  USART2_DMA_CircularReceive(dma_rx_buffer, RX_BUFFER_SIZE)

Main loop
  dma_head = (RX_BUFFER_SIZE - DMA1_Stream5->NDTR) % RX_BUFFER_SIZE
  if dma_head != rx_tail and tx_complete:
      msg = dma_rx_buffer[rx_tail]
      USART2_DMA_SendByte(msg)
      rx_tail = rx_tail + 1 with wrap

| Step | Expected observation |
| --- | --- |
| Start circular RX | DMAR and Stream5 EN are set. |
| Byte arrives | DMA writes into dma_rx_buffer and NDTR decrements. |
| Main calculates head | dma_head moves ahead of rx_tail. |
| Main consumes byte | Byte echoes using DMA TX and rx_tail advances. |
| DMA wraps | NDTR reloads and writes continue from index 0. |

# 13. Code Explanation

| Code action | Meaning | Why it matters |
| --- | --- | --- |
| DMA_SxCR_CIRC | Enables hardware circular mode. | Removes manual receive rearming. |
| USART2_DMA_CircularReceive | Starts continuous RX DMA. | Sets M0AR, NDTR, EN, and DMAR. |
| dma_head formula | Derives current DMA write index. | Lets software know what has arrived. |
| rx_tail | Application read pointer. | Prevents rereading old bytes. |
| tx_complete gate | Avoids starting another one-byte TX DMA while previous TX is incomplete. | Protects echo transmit path. |
| Empty RX DMA IRQ | No HT/TC processing currently. | Confirms NDTR polling design. |

# 14. Testing and Validation

| Test | Action | Expected result | Evidence |
| --- | --- | --- | --- |
| Start circular RX | Run firmware. | Stream5 EN and DMAR set. | Debugger watch. |
| Single byte | Send one terminal byte. | NDTR decrements and byte echoes. | Terminal/debugger. |
| Burst shorter than buffer | Paste a short string. | Bytes echo in order. | Terminal capture. |
| Wrap test | Send more than 64 bytes over time. | NDTR reloads and rx_tail wraps. | Debugger watch. |
| Backpressure test | Send fast burst while TX echo is busy. | Check for overwrite risk. | Terminal/debugger. |

### DMA RX Circular Validation Checklist

| Check | Learning or validation item | Evidence |
| --- | --- | --- |
| [ ] | DMA1 Stream5 Channel4 is configured. | Notes / capture: |
| [ ] | DMA_SxCR_CIRC is set. | Notes / capture: |
| [ ] | M0AR points to dma_rx_buffer. | Notes / capture: |
| [ ] | NDTR starts at RX_BUFFER_SIZE. | Notes / capture: |
| [ ] | USART2->CR3 DMAR is set. | Notes / capture: |
| [ ] | dma_head changes as NDTR decrements. | Notes / capture: |
| [ ] | rx_tail advances only after consuming a byte. | Notes / capture: |
| [ ] | rx_tail wraps to 0 at RX_BUFFER_SIZE. | Notes / capture: |

# 15. Results

| Observed / expected behavior | Interpretation |
| --- | --- |
| Bytes echo without rearming RX DMA. | Hardware circular receive is active. |
| NDTR decrements and reloads. | DMA circular counter behavior is visible. |
| rx_tail tracks consumed bytes. | Software consumer side is functioning. |
| DMA1_Stream5_IRQHandler remains unused. | Current design relies on polling NDTR, not HT/TC interrupts. |

# 16. Problems Faced and Debugging Notes

| Symptom | Likely cause | Debug action |
| --- | --- | --- |
| dma_head never changes | DMAR not set, Stream5 not enabled, wrong channel/stream. | Check CR3, Stream5 EN, CHSEL=4. |
| Bytes overwrite before echo | Application tail too slow. | Add overflow distance check or faster processing. |
| No HT/TC interrupt | HTIE/TCIE not enabled and IRQ is empty. | Expected in current NDTR polling design. |
| Echo stalls | tx_complete remains 0. | Check DMA TX and USART TC path. |
| Unexpected RX gaps | ORE or DMA misconfiguration. | Inspect USART SR and DMA Stream5 registers. |
| Secret-like payload present | Test data in main.c includes PEM private key string. | Replace with dummy long payload. |

## 16.1 Register Checklist

| Item | Expected state | Quick check |
| --- | --- | --- |
| DMA1_Stream5->CR | CHSEL=4, DIR=P2M, MINC, CIRC, EN | Core circular RX DMA config. |
| DMA1_Stream5->M0AR | dma_rx_buffer | Circular RAM destination. |
| DMA1_Stream5->NDTR | Counts down and reloads | Head derivation source. |
| DMA1_Stream5->PAR | &USART2->DR | USART receive source. |
| USART2->CR3 | DMAR set | USART RX DMA request path. |
| dma_head | Derived from NDTR | Hardware write position. |
| rx_tail | Application read index | Software consumer position. |

# 17. Limitations

| Limitation | Why it matters | Next improvement |
| --- | --- | --- |
| No overflow distance check | DMA can overwrite unread data if software lags. | Track unread count and detect overrun. |
| No HT/TC RX DMA events | Software must poll NDTR regularly. | Enable HTIE/TCIE if checkpoint processing is desired. |
| No idle-line detection | Variable-length frame boundary is unknown. | Add USART IDLE detection in Day 8. |
| RX DMA IRQ empty | No error/checkpoint handling there. | Implement HT/TC/error branches if using IRQ approach. |
| Real private key-like test payload | Secrets should not be embedded as test data. | Use dummy payload. |

# 18. Future Improvements

| Stage | Upgrade | What changes |
| --- | --- | --- |
| Day 8 | Circular DMA + idle detection | Detect variable-length frame boundaries. |
| Driver polish | Unread distance tracking | Detect when DMA is about to overwrite unread data. |
| Optional IRQ mode | HTIE/TCIE processing | Use half/full buffer checkpoints. |
| Protocol layer | CLI/parser | Consume complete commands from circular DMA buffer. |

### Long-Term Learning Tracker

| Check | Learning or validation item | Evidence |
| --- | --- | --- |
| [ ] | I can explain CIRC and NDTR reload. | Notes / capture: |
| [ ] | I can calculate dma_head from NDTR. | Notes / capture: |
| [ ] | I can compare Day 3 software ring buffer with Day 7 hardware circular DMA. | Notes / capture: |
| [ ] | I can explain why idle detection is the next step. | Notes / capture: |

# 19. Conclusion

Day 7 makes receive continuous. DMA remains armed and wraps through the buffer automatically, while the application reads from a software tail. The practical lesson is that hardware circular mode solves the rearm gap, but the application still needs disciplined head/tail tracking to avoid overwritten unread data.

Core takeaway: In DMA RX circular mode, DMA owns the write side and NDTR tells you where that write side is. Your code owns the read side through rx_tail.

# Appendix A. Actual Source Evidence

| File | Evidence included |
| --- | --- |
| usart.c | USART2_DMA_RX_Init and USART2_DMA_CircularReceive. |
| main.c | dma_rx_buffer, dma_head formula, rx_tail consumption loop. |
| stm32f4xx_it.c | Empty DMA1_Stream5_IRQHandler and TX completion flow. |
| usart.h | USART2_DMA_CircularReceive declaration. |

# Appendix B. One-Page Revision Summary

USART2 RX circular DMA uses DMA1 Stream5 Channel4.

DMA_SxCR_CIRC enables hardware wraparound.

dma_rx_buffer[64] is the hardware-filled receive buffer.

The DMA write head is derived as (RX_BUFFER_SIZE - DMA1_Stream5->NDTR) % RX_BUFFER_SIZE.

rx_tail is the software read pointer.

The current finalized design polls NDTR; it does not use RX DMA HT/TC interrupts.