EMBEDDED PROJECT REPORT

USART Day 8 Circular DMA RX with Idle Detection

STM32F407VG | USART2 RX | DMA1 Stream5 Channel4 | Circular DMA | IDLEIE | SR/DR clear sequence

| Field | Value |
| --- | --- |
| Document purpose | Long-term learning, implementation reference, and idle-line detection guide. |
| Project type | USART2 circular DMA RX with IDLE boundary detection. |
| Content source | Source code and PDF template |
| Code authority | Only the attached finalized source files are used for code snippets. |
| Core concept | DMA continuously captures bytes; USART IDLE interrupt tells software when a burst has paused. |

Implementation truth: The finalized code enables IDLEIE inside USART2_DMA_CircularReceive, clears IDLE by reading SR then DR in USART2_IRQHandler, sets idle_detected, then the main loop snapshots DMA head from NDTR and processes bytes from rx_tail to dma_head.

# 1. Project Objective

The objective of Day 8 is to combine continuous circular DMA reception with USART idle-line detection. Circular DMA solves uninterrupted acquisition, while IDLE detection gives a hardware boundary hint when the sender pauses.

Learning outcome: After this project, you should be able to explain IDLEIE, USART_SR_IDLE, the SR-then-DR clear sequence, idle_detected, NDTR-based write-index calculation, and why parsing should happen outside the ISR.

### Objective Checkpoint

| Check | Learning or validation item | Evidence |
| --- | --- | --- |
| [ ] | I can explain when the USART IDLE flag sets. | Notes / capture: |
| [ ] | I can explain exactly how STM32F4 clears IDLE. | Notes / capture: |
| [ ] | I can trace data from DMA circular buffer to main-loop processing after idle_detected. | Notes / capture: |

# 2. Problem Statement

Circular DMA receives continuously but does not know where a message ends. Fixed lengths work for known frames, but terminal commands and many protocols are variable length. IDLE detection adds a practical boundary signal: when the RX line stays idle for one frame time, the application can process the bytes that arrived since the last read position.

| Problem area | Day 8 response |
| --- | --- |
| Continuous receive | DMA1 Stream5 remains in circular mode. |
| Unknown message length | USART IDLE indicates the sender paused. |
| Boundary handling | Main loop processes bytes only when idle_detected is set. |
| Pointer tracking | dma_head is derived from NDTR and compared with rx_tail. |

# 3. System Overview

USART2 receives bytes into DR, DMA1 Stream5 writes them into dma_rx_buffer, and the USART IDLE interrupt fires after a silent gap. The ISR clears IDLE and sets a flag. The main loop then calculates the current DMA write head and processes the newly received chunk.

RX line -> USART2->DR -> DMA1 Stream5 circular buffer
    |
    v
USART line idle for one frame time
    |
    v
USART2_IRQHandler
    read SR; read DR; idle_detected = 1
    |
    v
main loop
    dma_head = (RX_BUFFER_SIZE - DMA1_Stream5->NDTR) % RX_BUFFER_SIZE
    while(dma_head != rx_tail): process dma_rx_buffer[rx_tail]

| Project focus | What changes from Day 7 |
| --- | --- |
| Boundary signal | IDLE interrupt marks a pause in the receive stream. |
| Processing trigger | Main loop waits for idle_detected before draining new bytes. |
| Buffer size | RX_BUFFER_SIZE is now 2050 in usart.h. |
| RX DMA IRQ | Still unused; USART IDLE IRQ is the key event. |

# 4. Requirements

## 4.1 Functional Requirements

| ID | Requirement | Acceptance condition |
| --- | --- | --- |
| FR-01 | Configure USART2 and DMA1 Stream5 Channel4 for circular RX. | CIRC, MINC, PAR, M0AR, NDTR, EN, and DMAR are configured. |
| FR-02 | Enable idle-line interrupt. | USART2->CR1 has USART_CR1_IDLEIE set. |
| FR-03 | Clear idle flag correctly. | USART2_IRQHandler reads SR then DR when USART_SR_IDLE is set. |
| FR-04 | Signal main context. | ISR sets idle_detected = 1. |
| FR-05 | Snapshot DMA head on idle. | main calculates dma_head from RX_BUFFER_SIZE and DMA1_Stream5->NDTR. |
| FR-06 | Process bytes outside ISR. | main drains bytes from rx_tail to dma_head. |

## 4.2 Non-Functional Requirements

| Area | Requirement |
| --- | --- |
| ISR discipline | USART ISR only clears flags and sets idle_detected. |
| Frame safety | Application processes a snapshot after IDLE rather than parsing inside ISR. |
| Buffer ownership | DMA owns write position; application owns rx_tail. |
| Debuggability | NDTR, IDLE flag, idle_detected, dma_head, and rx_tail must be easy to inspect. |

# 5. Hardware Components

| Item | Selection in this project | Purpose |
| --- | --- | --- |
| MCU | STM32F407VG / STM32F407VGT6 family | Target MCU with USART2 and DMA1. |
| USART instance | USART2 | Serial peripheral. |
| RX pin | PA3 AF7 | Receive input. |
| TX pin | PA2 AF7 | Transmit/debug output. |
| RX DMA stream | DMA1 Stream5 Channel4 | USART2_RX DMA path. |
| RX DMA mode | Circular mode | Continuous receive into RAM. |
| Boundary event | USART IDLE | Detects silent gap after a burst. |
| Buffer | dma_rx_buffer[2050] | Large circular receive buffer. |

# 6. Software and Firmware Tools

| Source file | Verified role |
| --- | --- |
| Core/Src/main.c | Declares idle_detected, dma_rx_buffer, dma_head, rx_tail; processes bytes after IDLE. |
| Core/Src/usart.c | Configures circular DMA RX and enables IDLEIE during circular receive start. |
| Core/Src/stm32f4xx_it.c | Handles USART IDLE clear sequence and sets idle_detected. |
| Core/Inc/usart.h | Defines RX_BUFFER_SIZE = 2050 and declares circular RX API. |
| Core/Inc/stm32f4xx_it.h | Declares USART2 and DMA interrupt handlers. |

Reasoning correction: usart.h defines RX_BUFFER_SIZE as 2050, while stm32f4xx_it.c still contains a local #define RX_BUFFER_SIZE 64. Avoid duplicated size definitions; keep one authoritative value in the shared header.

# 7. System Block Diagram

Setup
  USART2_DMA_CircularReceive(dma_rx_buffer, RX_BUFFER_SIZE)
    Stream5 M0AR = dma_rx_buffer
    Stream5 NDTR = RX_BUFFER_SIZE
    Stream5 EN = 1
    USART2 CR3 DMAR = 1
    USART2 CR1 IDLEIE = 1

Runtime
  DMA writes bytes continuously
  USART IDLE interrupt fires after line silence
  ISR clears IDLE and sets idle_detected
  main drains rx_tail..dma_head

| Register / variable | Meaning | Used for |
| --- | --- | --- |
| USART_CR1_IDLEIE | IDLE interrupt enable. | Allows USART idle event to interrupt CPU. |
| USART_SR_IDLE | Idle line detected. | Indicates RX line silent for one frame time. |
| USART_DR | Data register. | Read after SR to clear IDLE. |
| USART_CR3_DMAR | RX DMA request enable. | Allows USART RX to feed DMA. |
| DMA1_Stream5->NDTR | Remaining DMA count. | Derives dma_head. |
| idle_detected | ISR-to-main flag. | Triggers chunk processing. |
| rx_tail | Software read pointer. | Tracks processed bytes. |

# 8. Firmware Architecture

The Day 8 architecture is event-assisted circular DMA. DMA handles all byte movement. USART IDLE gives a boundary event. The main loop performs the actual data processing.

ISR responsibility:
if(USART2->SR & USART_SR_IDLE)
{
    dummy = USART2->SR;
    dummy = USART2->DR;
    idle_detected = 1;
}

Main responsibility:
if(idle_detected)
{
    idle_detected = 0;
    dma_head = (RX_BUFFER_SIZE - DMA1_Stream5->NDTR) % RX_BUFFER_SIZE;
    while(dma_head != rx_tail) { process byte; advance rx_tail; }
}

Architecture decision: The ISR does not parse or echo the frame. It only acknowledges the hardware event and tells the main loop that a quiet gap occurred.

# 9. Communication Protocol Used

The wire format remains UART-style 8N1. Idle detection is not a data byte and not a terminator character. It is a hardware observation that the line stayed idle long enough after the last frame.

| Parameter | Value from finalized code | Engineering note |
| --- | --- | --- |
| RX DMA mapping | DMA1 Stream5 Channel4 | USART2_RX mapping. |
| Circular mode | DMA_SxCR_CIRC set | Hardware wraps buffer. |
| Buffer size | 2050 | Large circular receive window. |
| Boundary source | USART_SR_IDLE | Line silence after reception. |
| Clear sequence | Read SR then DR | Required STM32F4 sequence. |
| Processing trigger | idle_detected | Main loop chunk-processing flag. |

# 10. Module-Wise Implementation

## 10.0 Header API from usart.h

#define RX_BUFFER_SIZE 2050

void USART2_Init(void);
void USART2_SendString(char *s);
void USART2_SendByte(char s);
char USART2_ReceiveByte(void);
uint8_t USART2_ReadByte(void);
void USART2_DMA_TX_Init(void);
void USART2_DMA_SendString(char *str);
void USART2_DMA_SendByte(uint8_t c);
void USART2_DMA_SendBuffer(const char *buffer, uint16_t length);
void USART2_DMA_RX_Init(void);
void USART2_DMA_ReceiveBuffer(char *buffer, uint16_t length);
void USART2_DMA_CircularReceive(uint8_t *buffer, uint16_t length);

#endif /* INC_USART_H_ */

## 10.1 Main Idle/Circular State from main.c

volatile uint8_t idle_detected =0;
uint8_t dma_rx_buffer[RX_BUFFER_SIZE];
volatile uint16_t dma_head = 0;
volatile uint16_t rx_tail = 0;

## 10.2 Circular Receive Start from usart.c

void USART2_DMA_CircularReceive(uint8_t *buffer, uint16_t length)
{
    /* Disable stream */
    DMA1_Stream5->CR &= ~DMA_SxCR_EN;
    while(DMA1_Stream5->CR & DMA_SxCR_EN);

    /* Clear old flags */
    DMA1->HIFCR =
          DMA_HIFCR_CTCIF5  |
          DMA_HIFCR_CHTIF5  |
          DMA_HIFCR_CTEIF5  |
          DMA_HIFCR_CDMEIF5 |
          DMA_HIFCR_CFEIF5;

    /* MBURST - 1 beat transfer from memory */
    DMA1_Stream5->CR &= ~DMA_SxCR_MBURST;

    /* Memory address */
    DMA1_Stream5->M0AR = (uint32_t)buffer;

    /* Number of bytes */
    DMA1_Stream5->NDTR = length;

    /* Enable DMA stream */
    DMA1_Stream5->CR |= DMA_SxCR_EN;

    /* Enable USART DMA RX */
    USART2->CR3 |= USART_CR3_DMAR;

    /* Enable idle state detection */
    USART2->CR1 |= USART_CR1_IDLEIE;

}

## 10.3 USART IDLE Interrupt Handler from stm32f4xx_it.c

void USART2_IRQHandler(void)
{
	if(USART2->SR & USART_SR_TC)
	{
		tx_complete = 1;

		//clear transmit completion interrupt
	    USART2->CR1 &= ~USART_CR1_TCIE;

	    // clear the DMA request
	    USART2->CR3 &= ~USART_CR3_DMAT;

	}

    if(USART2->SR & USART_SR_IDLE)
    {
        volatile uint32_t dummy;

        //reading status and data register for clearing the idle interrupt
        dummy = USART2->SR;
        dummy = USART2->DR;

        idle_detected = 1;
    }
}

## 10.4 Main Loop Idle Processing from main.c

if(idle_detected)
	  {
		  idle_detected = 0;

		  /* updating/calculating the head */
		  dma_head = (RX_BUFFER_SIZE - DMA1_Stream5->NDTR) % RX_BUFFER_SIZE;

		  /* if data is received */
		  while(dma_head != rx_tail)
		  {
			  msg = dma_rx_buffer[rx_tail];
			  USART2_SendByte(msg);

			  rx_tail++;

			  if(rx_tail >= RX_BUFFER_SIZE)
			  {
				  rx_tail = 0;
			  }
		  }


	  }

# 11. Idle Line Detection Deep Dive

IDLE detection is the center of Day 8. On STM32F4 USART, IDLE becomes set when the RX line stays idle for one frame time after activity. In 8N1 at 115200 baud, one frame is about 86.8 microseconds, so IDLE gives a hardware hint that the sender has paused.

| Concept | Meaning | Why it matters |
| --- | --- | --- |
| IDLE flag | RX line silent for one frame time. | A burst or packet may have ended. |
| IDLEIE | Enables interrupt on IDLE. | Lets software react to pause without polling SR. |
| Clear sequence | Read SR, then read DR. | Prevents repeated idle interrupts. |
| Boundary hint | Not a parser by itself. | Main still decides what the bytes mean. |
| Circular DMA pairing | DMA keeps receiving while IDLE marks chunks. | Supports unknown-length input. |

## 11.1 Why SR Then DR Clears IDLE

The STM32F4 USART clear sequence for IDLE is a status-register read followed by a data-register read. The finalized code follows this pattern with a volatile dummy variable. This acknowledges the idle event while keeping the ISR short.

volatile uint32_t dummy;

dummy = USART2->SR;
dummy = USART2->DR;

idle_detected = 1;

## 11.2 IDLE with Circular DMA

IDLE does not stop DMA. The DMA stream continues to own the write position. On idle, software snapshots the current write position using NDTR, then processes the span from rx_tail to dma_head. If the write position wrapped, processing must wrap too.

| Case | Condition | Processing span |
| --- | --- | --- |
| No wrap | dma_head > rx_tail | Process rx_tail through dma_head - 1. |
| Wrap | dma_head < rx_tail | Process rx_tail through buffer end, then 0 through dma_head - 1. |
| No new data | dma_head == rx_tail | Nothing new to process. |
| Current code | while(dma_head != rx_tail) | Processes one byte at a time with tail wrap. |

## 11.3 IDLE vs Delimiter Bytes

A delimiter such as '\n' is data inside the stream. IDLE is timing outside the stream. For terminal input, both can be useful: IDLE tells you the burst paused, and the parser can still look for newline or command tokens inside the received bytes.

Key idle lesson: IDLE is a boundary hint, not the message itself. The clean pattern is: ISR sets a flag, main snapshots NDTR, main processes bytes, parser validates the content.

# 12. Main Algorithm and Flowchart

Startup
  USART2_Init
  USART2_DMA_TX_Init
  USART2_DMA_RX_Init
  USART2_DMA_CircularReceive(dma_rx_buffer, RX_BUFFER_SIZE)

USART IDLE ISR
  if IDLE:
      read SR
      read DR
      idle_detected = 1

Main loop
  if idle_detected:
      idle_detected = 0
      dma_head = (RX_BUFFER_SIZE - DMA1_Stream5->NDTR) % RX_BUFFER_SIZE
      while dma_head != rx_tail:
          msg = dma_rx_buffer[rx_tail]
          USART2_SendByte(msg)
          rx_tail = rx_tail + 1 with wrap

| Step | Expected observation |
| --- | --- |
| Start circular RX | DMAR and Stream5 EN are set. |
| Send a burst | DMA writes bytes into dma_rx_buffer. |
| Stop sending | USART IDLE flag sets. |
| ISR executes | IDLE is cleared and idle_detected becomes 1. |
| Main processes | Bytes from rx_tail to dma_head are echoed. |

# 13. Code Explanation

| Code action | Meaning | Why it matters |
| --- | --- | --- |
| USART2->CR1 ¦= USART_CR1_IDLEIE | Enables idle interrupt. | Creates variable-length boundary event. |
| USART_SR_IDLE check | Detects idle condition. | Confirms reason for USART interrupt. |
| dummy = USART2->SR; dummy = USART2->DR | Clears IDLE. | Prevents repeated ISR entry. |
| idle_detected = 1 | Signals main loop. | Keeps ISR short. |
| dma_head formula | Snapshots DMA write position. | Defines available data span. |
| while(dma_head != rx_tail) | Drains unread bytes. | Processes current chunk. |

# 14. Testing and Validation

| Test | Action | Expected result | Evidence |
| --- | --- | --- | --- |
| IDLE interrupt | Send characters, then pause. | idle_detected becomes 1. | Debugger watch. |
| IDLE clear | Let ISR run repeatedly. | No repeated stuck idle interrupt. | Breakpoint/register watch. |
| Chunk echo | Send a word and pause. | Word echoes after pause. | Terminal capture. |
| NDTR snapshot | Watch DMA1_Stream5->NDTR. | dma_head matches bytes received. | Debugger watch. |
| Wrap test | Send enough data over time to wrap buffer. | rx_tail and dma_head wrap safely. | Debugger watch. |

### Idle Detection Validation Checklist

| Check | Learning or validation item | Evidence |
| --- | --- | --- |
| [ ] | USART2->CR1 has IDLEIE set after circular receive starts. | Notes / capture: |
| [ ] | USART2->CR3 has DMAR set. | Notes / capture: |
| [ ] | DMA1_Stream5 is enabled in circular mode. | Notes / capture: |
| [ ] | USART2_IRQHandler checks USART_SR_IDLE. | Notes / capture: |
| [ ] | ISR reads SR then DR to clear IDLE. | Notes / capture: |
| [ ] | idle_detected changes to 1 after a receive pause. | Notes / capture: |
| [ ] | main computes dma_head from NDTR. | Notes / capture: |
| [ ] | main advances rx_tail after processing bytes. | Notes / capture: |

# 15. Results

| Observed / expected behavior | Interpretation |
| --- | --- |
| Data is processed after sender pause. | IDLE boundary detection is working. |
| DMA continues receiving while main waits. | Circular DMA remains active. |
| No per-byte RX ISR is needed. | DMA owns receive byte movement. |
| Main drains rx_tail to dma_head. | Application-level chunk processing is working. |

# 16. Problems Faced and Debugging Notes

| Symptom | Likely cause | Debug action |
| --- | --- | --- |
| No idle interrupt | IDLEIE not set or USART IRQ not enabled. | Check CR1 IDLEIE and NVIC USART2 IRQ. |
| Repeated idle interrupts | IDLE not cleared correctly. | Verify SR read followed by DR read. |
| Chunk missing bytes | Wrong NDTR math or tail wrap bug. | Inspect dma_head, NDTR, rx_tail. |
| Parser sees old data | rx_tail not advanced after processing. | Verify tail update and wrap. |
| Data overwritten | Buffer too small or processing too slow. | Increase buffer or process faster. |
| Size confusion | RX_BUFFER_SIZE duplicated as 2050 and 64. | Use one shared define. |

## 16.1 Register Checklist

| Item | Expected state | Quick check |
| --- | --- | --- |
| USART2->CR1 | UE, RE, IDLEIE set | IDLE interrupt enabled. |
| USART2->SR | IDLE sets after pause | Boundary event occurs. |
| USART2->CR3 | DMAR set | RX DMA request enabled. |
| DMA1_Stream5->CR | CIRC, MINC, EN | Circular RX DMA active. |
| DMA1_Stream5->NDTR | Counts down and reloads | Head derivation source. |
| idle_detected | 1 after IDLE ISR | Main-loop trigger. |
| rx_tail | Moves after processing | Consumer index works. |

# 17. Limitations

| Limitation | Why it matters | Next improvement |
| --- | --- | --- |
| IDLE only means silence | It does not validate packet content. | Add parser and framing rules. |
| No overflow distance check | DMA can overwrite unread data. | Track unread count and detect overrun. |
| Blocking USART2_SendByte in processing loop | Processing can be slow during echo. | Use DMA TX or buffer output. |
| No USART ORE handling in idle branch | Overrun could be missed. | Handle ORE/error flags. |
| Duplicate RX_BUFFER_SIZE define | Confusing or dangerous if reused. | Keep size in usart.h only. |
| Private-key-like payload | Secrets should not be test data. | Use dummy payload. |

# 18. Future Improvements

| Stage | Upgrade | What changes |
| --- | --- | --- |
| Day 9 | CLI protocol | Parse command lines from idle-delimited chunks. |
| Driver polish | Overflow distance tracking | Detect unread data overwrite risk. |
| Error handling | ORE/FE/NE/PE support | Make receiver robust. |
| TX path | Non-blocking chunk echo | Avoid blocking while processing received bytes. |

### Long-Term Learning Tracker

| Check | Learning or validation item | Evidence |
| --- | --- | --- |
| [ ] | I can explain IDLE detection without confusing it with a delimiter byte. | Notes / capture: |
| [ ] | I can explain the SR then DR clear sequence. | Notes / capture: |
| [ ] | I can snapshot the DMA write index using NDTR. | Notes / capture: |
| [ ] | I can describe how Day 8 becomes the foundation for CLI parsing. | Notes / capture: |

# 19. Conclusion

Day 8 turns continuous DMA reception into a protocol-friendly receive path. Circular DMA captures bytes continuously, and IDLE detection gives the application a practical event for processing unknown-length bursts. The most important discipline is keeping the ISR short and doing all parsing in main context using rx_tail and the NDTR-derived dma_head.

Core takeaway: Circular DMA answers 'where did the bytes go?' IDLE detection answers 'when should I look at them?' Together they form the receive foundation for CLI and packet protocols.

# Appendix A. Actual Source Evidence

| File | Evidence included |
| --- | --- |
| usart.c | USART2_DMA_CircularReceive with DMAR and IDLEIE enable. |
| stm32f4xx_it.c | USART IDLE check, SR/DR clear sequence, idle_detected flag. |
| main.c | idle_detected processing, dma_head calculation, rx_tail draining. |
| usart.h | RX_BUFFER_SIZE = 2050 and circular receive API declaration. |

# Appendix B. One-Page Revision Summary

Day 8 combines DMA RX circular mode with USART idle-line detection.

USART2_DMA_CircularReceive enables DMAR and IDLEIE.

IDLE is cleared by reading USART2->SR and then USART2->DR.

The ISR sets idle_detected and leaves processing to the main loop.

The main loop calculates dma_head from NDTR and drains bytes from rx_tail to dma_head.

IDLE is a boundary hint, not a parser or delimiter byte.